# 3GPP TSG-RAN WG4 Meeting #116	R4-2509497

# Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.3.4

Source:	Apple

Title:	On RRM requirement for R19 LB CA

Document for:	Discussion

# Introduction

The WF on RRM requirements for NR_LBCA_Sw [1] have been agreed in RAN4#115, and the open issues have been summarized in the topic summary for [115][207] NR_LBCA_Sw [2].

In this contribution, we continue the discussion for RRM requirement of LB CA via switching.

# Assumptions for R19 LB CA via switching

Issue 1-1-3: RRM requirement applicability for R19 LB CA via switching

- Proposal 1 (vivo): For defining RRM requirements for SDL SCell operation, it is assumed at least one SSB/TRS is available either during one switching pattern period or one SSB/TRS period depending on the periodicity of switching pattern and SSB/TRS. - Proposal 2 (Ericsson): RAN4 to further investigate the RRM requirements impact with at least one of the following methods. - Method 1: Agree on the principle of extending the delay based on missed reception occasion of necessary DL signals/channels and apply this principle to all the aforementioned procedures. - Method 2: Specify an applicability condition under which the requirements are valid for low-band CA via switching. - Proposal 3 (ZTE): No need to consider the extension solution at least for L3 measurement, it can not work under current switching pattern periodicity P = 40 approved by RAN1. If one SMTC occasion is missed, then all SMTC occasions would be missed since typical SMTC configuration contains one or multiple complete switching pattern periodicity. - Proposal 4 (Nokia): For an activated SDL SCell with LBCA-sw, for a serving cell which can be either FDD PCell or SDL SCell: - a. If all the SSB occasions of the serving are contained within the LBCA-sw pattern, no delay extension is allowed - b. If a fraction the SSB occasions of the serving are contained within the LBCA-sw pattern, the delay is extended in accordance to the fraction of available SSB occasions - c. If no SSB occasion of the serving cell is contained within the LBCA-sw pattern, the requirements do no apply - Proposal 5 (MTK): - Agree on the principle of extending the delay based on missed reception occasion of necessary DL signals/channels and apply this principle to all the other procedures/measurements.

In our view, this issue is a common issue for SDL SCell and PCell L3 and L1 requirement. However, it’s necessary to differentiate the cases of “fully non-overlapped”, “fully overlapped” and “partially overlapped” between SMTC/SSB/CSI-RS occasion and corresponding duration in switching pattern. We think the following general rules can be applied to 3 cases for both FDD PCell and activated SDL SCell:

- For gap-less based L3 measurement and L1 measurement of a serving cell which can be either FDD PCell or activated SDL SCell:

  - a.	If all {the SSB occasions in SMTC window or CSI-RS for L3 measurement} or {SSB occasions or CSI-RS for L1 measurement} are available based on the LB-CA switching pattern, no delay extension is allowed

  - b.	If a fraction of {the SSB occasions in SMTC window or CSI-RS for L3 measurement} or {SSB occasions or CSI-RS for L1 measurement} are not available based on the LB-CA switching pattern, the delay is scaled in accordance to the fraction of available occasions.

  - c.	If no {SSB occasions in SMTC window or CSI-RS for L3 measurement} or {SSB occasions or CSI-RS for L1 measurement} are available based on the LB-CA switching pattern, the corresponding requirements do not apply.

Proposal 1: for gap-less based L3 measurement and L1 measurement of a serving cell which can be either FDD PCell or activated SDL SCell:

- a.	If all {the SSB occasions in SMTC window or CSI-RS for L3 measurement} or {SSB occasions or CSI-RS for L1 measurement} are available based on the LB-CA switching pattern, no delay extension is allowed

- b.	If a fraction of {the SSB occasions in SMTC window or CSI-RS for L3 measurement} or {SSB occasions or CSI-RS for L1 measurement} are not available based on the LB-CA switching pattern, the delay is scaled in accordance to the fraction of available occasions.

- c.	If no {SSB occasions in SMTC window or CSI-RS for L3 measurement} or {SSB occasions or CSI-RS for L1 measurement} are available based on the LB-CA switching pattern, the corresponding requirements do not apply.

Issue 1-1-4: others for general part

- Proposal 1 (OPPO): The requirements for LB CA via switching is irrelevant to the UE capability of Rel-18 NES.

We agree with proposal 1 but this proposal will not be captured as any kind of requirement in the spec, and therefore we propose to close this issue.

Proposal 2: close this issue 1-1-4 since it will not be captured in the spec.

# Switching delay or interruption requirement

Issue 1-2-1: Necessity of interruption requirement in RRM between FDD CC and SDL CC

Concluded in last meeting WF[1].

Issue 1-2-2: Necessity of interruption requirement in RRM when more carriers are considered

- Option 1 (QC): RAN4 does not define interruption requirements to other bands/carriers in Rel-19. - Option 2 (HW): When more carriers are considered, switching between FDD and SDL would interrupt other combined active SCell, the interruption length is ⌈T_switching/(Slot-length-victim)⌉+1. - Option 3 (Nokia): If a third band is configured without switching pattern, no interruption is allowed on the third band due to LBCA-sw operation on PCell and SDL SCell. - Option 4 (MTK): RAN4 RRM to close the issue of RRM requirements when more carriers are considered, unless it is specified in the WID.

RF session had agreements in RAN4#114:

Issue 1: Scope clarification

-	Specify the switching period(s) based on the scenario with the switching between two low bands in Rel-19 WI

-	It is not precluded that such two low bands are further combined with the additional band

-	The combination of those two low bands with other band can be further discussed in the future release.

Thus, according to RF session agreement, we think •	RAN4 RRM to close the issue of RRM requirements when more carriers are considered, unless it is specified in the WID.

Proposal 3: RAN4 RRM to close the issue of RRM requirements when more carriers are considered, unless it is specified in the WID.

•	Note: If the case of more than 2CCs is introduced in R19, RRM can revisit this issue.

# SDL SCC related RRM requirement

##### 4.1 Activated SDL SCell L3 measurement

Issue 1-3-1: Necessity of activated SDL SCell L3 measurement and corresponding interruption requirement

Concluded in last meeting WF[1].

Issue 1-3-2:  Requirement design of activated SDL SCell L3 measurement

Online Agreement on Tuesday: - For activated SDL SCell measurement, UE performs the SDL SCell measurement on the overlapped occasions of SMTC window and SDL SCell reception duration indicated by switching pattern. FFS: - If no SSB occasion of the serving cell is contained within the LB-CA switching pattern, the requirements do not apply (from Nokia proposal) - FFS on details of requirement design.

If SSB-based SCell operation is assumed for SDL SCell measurement, UE needs to measure SSB for L3 on SDL SCell based on SMTC configuration, and the current requirement is defined as:

The current measurement interval is specified based on SMTC periodicity or max(SMTC periodicity, DRX cycle); however, if some of the SMTC occasions are outside the SDL SCC reception duration of switching pattern, UE can only perform the SDL SCell measurement on the overlapped occasions between SMTC window and SDL SCell reception duration indicated by switching pattern. Thus, the legacy measurement interruption shall be revised accordingly. Additionally, the switching pattern and SMCT configuration are both controlled by network, so we assume network will configure the switching pattern reasonably to make sure UE can at least have one available measurement opportunity in each SDL SCell reception duration.

We need to differentiate the activated SCell measurement is gap-less or MG-based. If the activated SCell measurement is MG based, then UE can just follow MGRP and SMTC to perform the measurement without any modification based on switching pattern.

Proposal 4: if the activated SCC measurement is MG-based in LB-CA, the legacy intra-frequency identification/measurement requirement with MG can be applied.

If the activated SCC measurement is MG-less in LB-CA, then we need to revise existing Kp. Since RAN1 didn’t preclude more than 1 switches within 40ms periodicity of switching pattern, the following example of the switching can occur:

Figure 1. Example to justify why Kp method is better

In the above case, we cannot use max(SMTC periodicity, switching pattern) to decide the measurement delay extension, and the reason is: there might be multiple available SMTCs inside each switching pattern periodicity and they might be continuous or non-continuous. Thus, we prefer to use Kp scaling factor for measurement delay extension, like what we used for MG enhancement case.

Proposal 5: if the activated SCC measurement is MG-less in LB-CA, RAN4 to define a new Kp_LB factor for activated SCC identification/measurement requirement for LB-CA.

The legacy Kp definition is as following:

To make Kp working for LB-CA case, we think the duration window shall be revised as well as the Ntotal and Navailable. If activated SCell measurement is gap-less, the Kp_LB can be defined as :

- Kp_LB = Ntotal_LB / Navailable_LB, where Navailable_LB and Ntotal_LB are calculated as follows:

  - For a window W of duration max(SMTC period,  xRP_max, P), where xRP_max is the maximum xRP across all configured per-UE GAPs, periodic MUSIM gaps, and/or per-FR GAPs within the same FR as the SSB frequency layer, and P is the FDD-SDL switching pattern periodicity, and starting from the beginning of any SMTC occasion:

    - Ntotal_LB is the total number of SMTC occasions within the window, including those overlapped with MG and those overlapped with FDD PCC duration based on the switching pattern within the window, and

    - Navailable_LB is the number of SMTC occasions that are not overlapped with any non-dropped GAP or non-dropped MUSIM gap occasions and not overlapped with FDD PCC duration based on the switching pattern within the window W.

    - Kp = 1 when Navailable = 0.

Proposal 6: new Kp_LB factor for SSB-less activated SCC identification/measurement requirement in LB-CA can be defined as following:

- Kp_LB = Ntotal_LB / Navailable_LB, where Navailable_LB and Ntotal_LB are calculated as follows:

  - For a window W of duration max(SMTC period,  xRP_max, P), where xRP_max is the maximum xRP across all configured per-UE GAPs, periodic MUSIM gaps, and/or per-FR GAPs within the same FR as the SSB frequency layer, and P is the FDD-SDL switching pattern periodicity, and starting from the beginning of any SMTC occasion:

    - Ntotal_LB is the total number of SMTC occasions within the window, including those overlapped with MG and those overlapped with FDD PCC duration based on the switching pattern within the window, and

    - Navailable_LB is the number of SMTC occasions that are not overlapped with any non-dropped GAP or non-dropped MUSIM gap occasions and not overlapped with FDD PCC duration based on the switching pattern within the window W.

    - Kp = 1 when Navailable = 0.

#### 4.2 Deactivated SDL SCell L3 measurement

Issue 1-3-3: Necessity of deactivated SDL SCell measurement and corresponding interruption requirement

Concluded in last meeting WF[1].

Issue 1-3-4: Requirement design of deactivated SDL SCell measurement and corresponding interruption

Agreement on Thursday: - The legacy deactivated SCell measurement delay requirements are applied for UE to perform deactivated SDL SCell measurement. Agreement on Thursday: - For deactivated SDL SCell measurement, interruptions on active serving cell(s) are allowed with up to X% probability of missed ACK/NACK. - FFS: X, and X can be same or different for different SDL SCC measurement cycle (i.e., configured measCycleSCell).

The current deactivated SCell measurement consider the interruption for RF tuning/retuning was specified below,

If measCycleSCell is smaller than 640ms, currently UE is required to stay on deactivated SCC after SCell addition(no interruption is allowed). However, in R19 new LB CA scenario, since UE cannot keep the connection with PCell when it stays on the SDL SCC for measurement, it would be necessary to allow UE switch back to PCC for traffic/control reception/transmission after deactivated SCell measurement regardless of the measCycleSCell. The deactivated SCell measurement in LB-CA is quite similar as intra-band deactivated SCell, i.e., the interruption to PCC will be applied even during the SMTC window for measurement on SCC. The legacy intra-band deactivated SCell interruption requirement is defined as:

Even though the measCycleSCell is configured by network, network is still unaware where UE will perform deactivated SCell measurement, i.e., in which SMTC window. Thus, the CC switching before and after SMTC window may impact the data channel or control channel in the corresponding slot, and we think the whole slot shall be counted as interruption as well.

Proposal 7: Legacy interruption length requirement for intra-band deactivated SCC measurement can be reused for deactivated SCC measurement in R19 LB-CA.

Regarding the interruption rate, as shown in the following figure 2, the single interruption length may cause doubled ACK/NACK loss, including both {interrupted ACK/NACK transmission during SCC measurement window} and {future ACK/NACK corresponding to the interrupted PDSCH during SCC measurement window}. Based on the proposal 7, we can calculate the ACK/NACK rate.

Figure 2. illustration for interruption during deactivated SCell measurement in LB-CA

Based on the above analysis, the interruption rate for deactivated SCC measurement in R19 LB-CA can be different for different SDL SCC measurement cycle (i.e., configured measCycleSCell):

  - probability of missed ACK/NACK: X= YmeasCycleSCell ;

    - Y=2* (2+TSMTC_duation) in slot level, because both scheduled PDSCH/PDCCH and ACK/NACK transmission of schedule PDSCH is interrupted

Proposal 8: Probability of missed ACK/NACK for deactivated SCC measurement in R19 LB-CA is X= YmeasCycleSCell in slot level, where Y=2* (2+TSMTC_duation).

#### 4.3 SDL SCell activation/deactivation

Issue 1-3-5: Scope of SDL SCell activation/deactivation

Concluded in last meeting WF[1].

Issue 1-3-6: side condition for SDL SCell activation/deactivation requirement

Concluded in last meeting WF[1].

Issue 1-3-7: SDL SCell activation/deactivation delay requirement

- Agreement on Thursday: - The R18 SSB based FR1 inter-band SCell activation delay requirement can be reused to SDL SCell activation in LB CA via switching if UE supports corresponding feature. - R16 Direct SCell activation with HO is not considered for R19 LB CA. - R17 PUCCH SCell activation is not considered for R19 LB CA. - R18 SSB-less SCell activation for NES is not considered for R19 LB CA. - R17 Fast SCell activation is not considered for R19 LB CA. - Reuse the existing SCell deactivation delay requirement for SDL SCell deactivation. - Known/unknown conditions are reused from existing SCell activation requirements - FFS: - RAN4 to discuss the UE behavior if the CQI or L1-RSRP reporting is colliding with SSB or SMTC of being-activated SDL SCell during the SDL SCell activation. (Apple) - The starting point and ending point of CSI measurement for the sake of CSI reporting should be determined, for both SSB based and SSB-less SCell activation.(ZTE) - Amend the previous agreement with the following - The UE shall continuously monitor PCell disregarding the configured switching pattern. (Nokia)

It’s still FFS for CQI and L1-RSRP reporting during the SDL SCell activation. In legacy SCell activation requirement, UE is required to report OOR and lowest L1-RSRP right after the MAC CE decoding.

However, such CQI or L1-RSRP reporting is on PCell, and when UE needs to report CQI/L1-RSRP on PCell, UE cannot stay on SCell for DL synchronization if the SSB or SMTC is colliding with the CQI/L1-RSRP reporting occasion. Like in figure 3,

Figure 3. the CQI/L1-RSRP on PCell colliding with the SSB/SMTC for SDL SCell activation

It’s unclear how UE can handle the CQI/L1-RSRP reporting during the SCell activation procedure, and our view is to prioritize the SSB/SMTC measurement over the OOR or lowest L1-RSRP reporting to complete the SDL SCell activation as soon as possible. However, the SMTC and CQI reporting or L1-RSRP reporting resource is configured by network, network can simply avoid such colliding case. Thus, we think the colliding case shall be treated as an error case since there is no benefit to do so. In addition, UE only needs to switch to SCC when SMTC window or RS occasion comes, and therefore UE can stay on PCC for these UL reporting outside the SMTC window or RS occasions on SCC, i.e., no CC switching and interruption is needed for CQI/L1-RSRP reporting.

Proposal 9: If CQI or L1-RSRP reporting is colliding with SSB/CSI-RS or SMTC of being-activated SDL SCell during the SDL SCell activation, no SCell activation requirement shall be applied; otherwise, legacy CQI and L1-RSRP reporting requirement of SCell activation can be reused for SCell activation in LB-CA.

Issue 1-3-8: interruption or scheduling restriction during SDL SCell activation/deactivation

- Recommended WF - Check if following moderator proposal for compromise(considering P1, P2, P3, P4(option 2), P5) is agreeable: - Proposal 6(Moderator): - RAN4 to define the scheduling restriction caused by the measurements during SDL SCell activation procedure. - The scheduling restriction due to CC switching in SDL SCell activation shall follow RS periodicity or SMTC periodicity used for SCell activation. - the scheduling restriction length for each CC switching in SDL SCell activation can be defined as {(symbol(s) for switching duration of FDD-to-SDL) + SSB/SMTC duration + (symbol(s) for switching time of FDD-to-SDL)}.

The existing interruption window design in TS38.133 for SCell activation RRM operation is as following,

However, during the SDL SCell activation procedure, UE may need to switching between PCell and SCell periodically. Like in the following figure, carrier switch between SDL SCell and PCell during SCell activation procedure can follow the SSB periodicity or SMTC periodicity used for SCell activation. The interruption length can be defined as (switching duration for FDD-to-SDL) + SSB/SMTC duration + (switching time for FDD-to-SDL).

Figure 4. the CC switching and interruption for SCell activation

RAN1 agreed the followings:

RAN1 #121 Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, regarding the location of the switching gap for switching from FDD carrier to SDL carrier, only support the switching gap is located at the “switch from” carrier, i.e., FDD carrier. - Switching gap ends at the end of the last slot on the “switch from” carrier RAN1 #120bis Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, regarding the location of the switching gap, - For switching from SDL carrier to FDD carrier, the switching gap is always assumed at the “switch from” carrier, i.e., SDL carrier. Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, the duration of the switching gap is RRC configured by the NW, - Switching gap duration can be different for FDD to SDL switch, and SDL to FDD switch. - Note: NW ensures that the switching gap is enough to cover at least the switching period as in RAN4 LS (R1-2501702) and UL TA (if needed) - For the UL TA, - Switching gap for SDL to FDD switch ends at the end of slot on SDL - Note: RAN1 assumes that switching gap for SDL to FDD switch includes UL TA - FFS: whether it has specification impact Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, it is not expected that the same slot (1ms) contains both symbol(s) configured as FDD carrier and symbol(s) configured as SDL carrier. Agreement For Rel-19 low NR band carrier aggregation via switching, the semi-static switching pattern and corresponding switching gap are based on the downlink timing (DL reception time at UE side) of Pcell. Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, the time unit (resolution) of the semi-static switching pattern configuration is slot (1ms for 15kHz SCS).

Even though RAN4 agreed that the switching pattern is only applied for activated SCell, we think the assumption of RAN1 agreement “For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, it is not expected that the same slot (1ms) contains both symbol(s) configured as FDD carrier and symbol(s) configured as SDL carrier” can still be reused for SCell activation. Like in following figure 5, the CC switching during SCell activation will follow the SMTC periodicity and RS occasion periodicity. However, this is a kind of scheduling restriction since network is aware the CC switching can happen before and after SMTC window and RS occasion.

Figure 5. switching during the SCell activation

In the switching pattern design for activated SCell operation in LB-CA, the CC switching is agreed to locate at the end of the slot for both FDD-to-SDL and SDL-to-FDD switching. The main reason is to avoid the impact to PDCCH of the PCell/SCell. In SCell activation, we think simple UE implementation can be designed as same as activated SCell case, i.e., the switching can be performed at the end of a slot. If UE performs switching at beginning of slot, it will still puncture the PDCCH and then the whole slot can still not be used anyway. To make spec and UE implementation simpler, we think we can reuse the legacy intra-band FR1 SCell activation interruption requirement for this inter-band SDL SCell activation, since PCell cannot be schedule during the SSB/CSI-RS or SMTC measurement occasions.

In addition, the SCell activation is a short procedure, which is different from periodic SCell measurement, and therefore we don’t see big impact if we allow 1 slot before and 1 slot after the SMTC or RS occasion for scheduling restriction.

Proposal 10: RAN4 to define the scheduling restriction caused by the measurements during SDL SCell activation procedure.

Proposal 11: The scheduling restriction due to CC switching in SDL SCell activation shall follow RS periodicity or SMTC periodicity used for SCell activation.

Proposal 12: the scheduling restriction length for each CC switching in SDL SCell activation can be defined as {1 slot + SSB/SMTC duration/CSI-RS in slot level + 1 slot}.

- PDCCH, PDSCH, PRS or CSI-RS in a set of symbols of a DL slot and PUSCH, PUCCH, PRACH, or SRS in a set of symbols of an UL slot on the FDD carrier that overlaps, even partially, with the set of symbols of {1 slot + SSB/SMTC duration/CSI-RS in slot level + 1 slot}, will have scheduling restriction.

Issue 1-3-9: Activation of Switching Pattern

- Proposal 1 (QC): - At the end of SCell activation (i.e., after UE reporting valid CQI for SCell activation in section 8.3.2 TS38.133), spec allows UE a minimum time separation of X ms between transmitting CQI report and applying the semi-statically configured switching pattern. - Value of X is 3ms - Proposal 2 (LGE): - RAN4 needs to clarify when to apply the semi-static switching pattern after compeleting SCell activation, e.g., apply immediately the semi-static switching pattern or apply next cycle semi-static switching pattern.

The ending point of SCell activation is when UE sending valid CQI to network, and UE shall be able to receive PDCCH/PDSCH on the SCell after this timing point. However, for LB-CA, UE also needs to load or enable the switching pattern on its BB/RF control after sending valid CQI back. We agree with proposal 1 that some time margin is needed between transmitting CQI report and applying the semi-statically configured switching pattern.

Proposal 13: At the end of SCell activation (i.e., after UE reporting valid CQI for SCell activation in section 8.3.2 TS38.133), spec allows UE a minimum time separation of 3 ms between transmitting CQI report and applying the semi-statically configured switching pattern.

#### 4.4 SDL SCell L1 requirement

Issue 1-3-10: BFD/CBD requirements for SDL SCell

Agreement: - If RAN4 to define requirement for BFD/CBD for SDL SCell in LB CA via switching, the BFD/CBD on SDL carrier is performed on the overlapped occasions between SSB/CSI-RS and SDL duration in switch pattern.

The same principle in issue 1-1-3 can be applied for BFD/CBD requirement, and new P_LB factor can be introduced to replace the existing P factor in BFD and CBD requirement as following:

The W window of new P_LB can be designed quite similar as for activated SCell measurement requirement Kp_LB, i.e., max(TL1, xRP_max, switching pattern periodicity), and the Ntotal_LB and Navailable_LB can be defined to count for total BFD-RS or CBD-RS in W window and available BFD-RS and CBD-RS in W window respectively.

Proposal 14: follow principle in issue 1-1-3 to define the BFD/CBD requirement for SDL SCell.

Proposal 15: New P_LB factor is used to replace the P factor in the existing BFD/CBD requirement, where,

- W window is max(TL1, xRP_max, switching pattern periodicity)

- P_LB= Ntotal_LB/ Navailable_LB

- Ntotal_LB is total number of BFD-RS or CBD-RS in W window.

- Navailable_LB is number of available BFD-RS or CBD-RS in W window.

Issue 1-3-11: TCI state switch delay requirement for SDL SCell

Agreement: - The L1-RSRP measurement and T/F tracking for TCI switching on SDL carrier is performed on the overlapped occasions between SSB/CSI-RS and SDL duration in switch pattern.

The legacy requirement is as following, and considering the agreement from last meeting, the TL1-RSRP and T/F tracking for TCI switching on SDL carrier can also follow the principle of issue 1-1-3, and the definition of TL1-RSRP and Tfirst-SSB shall be revised to reflect the overlapped occasion, e.g., Tfirst-SSB is time to first available SSB reception based on switching pattern after MAC CE command is decoded by the UE for other QCL types.

Proposal 16: In TCI state switch delay requirement for SDL SCell, Tfirst-SSB_LB is time to first available SSB reception based on switching pattern after MAC CE command is decoded by the UE for other QCL types

Issue 1-3-12: PL-RS switch delay requirement for SDL SCell

Concluded in last meeting WF[1].

Issue 1-3-13: L1-RSRP/SINR measurement requirements for SDL SCell

Agreement in AH: - For L1-RSRP/SINR measurement requirement for SDL SCell, UE can only perform such measurement on the overlapped occasions of L1 RS and SDL SCell reception duration indicated by switching pattern.

Same as BFD and CBD requirement for LB CA, the same principle in issue 1-1-3 can be applied for L1-RSRP/SINR measurement requirements, and new P_LB factor can be introduced to replace the existing P factor in L1-RSRP/SINR measurement requirements.

Proposal 17: follow principle in issue 1-1-3 to define the L1-RSRP/SINR measurement requirements for SDL SCell.

Proposal 18: New P_LB factor is used to replace the P factor in the existing L1-RSRP/SINR measurement requirements, where,

- W window is max(TL1, xRP_max, switching pattern periodicity)

- P_LB= Ntotal_LB/ Navailable_LB

- Ntotal_LB is total number of L1-RSRP/SINR RS in W window.

- Navailable_LB is number of available L1-RSRP/SINR RS in W window.

#### 4.5 Others for SDL SCell

Issue 1-3-5: others for SDL SCell

- Proposal 1 (Apple): - CSSF: - the overlapping condition (i.e., partially overlapping or fully overlapping) to decide the CSSF inside MG shall be revised for LB CA via switching, by considering configured SMTCs, MGs and LB-CA switching pattern.

This is not essential clarification for RRM spec. Due to the limited time before WI completion, we recommend treating it in maintenance stage.

Proposal 19: this issue 1-3-5 can be FFS (contribution-driven) in the maintenance stage.

# Potential impacts to the PCell related requirement

Issue 1-4-1: Impact on timing requirement for PCell

Concluded in last meeting WF[1].

Issue 1-4-2: others RRM requirement of FDD PSCell in LB CA via switching

The PCell RRM requirement revision can be same as what we do for activated SDL SCell requirement revision for LB-CA.

But PCell has RLM requirement which is not supported on SDL SCell. For RLM requirement of PCell in LB-CA, we can follow the principle in issue 1-1-3 because the switch pattern can also indicate which RLM-RS is available for OOS/IS evaluation on PCell. And same as SCell BFD/CBD, new P_LB factor can be introduced to replace the existing P factor in RLM measurement requirements.

Proposal 20: The PCell RRM requirement revision can follow the same approach used for activated SDL SCell requirement in LB-CA.

Proposal 21: For RLM in LB-CA, follow principle in issue 1-1-3 to define RLM requirements for PCell in LB CA.

Proposal 22: New P_LB factor is used to replace the P factor in the existing RLM measurement requirements, where,

- W window is max(TL1, xRP_max, switching pattern periodicity)

- P_LB= Ntotal_LB/ Navailable_LB

- Ntotal_LB is total number of RLM-RS in W window.

- Navailable_LB is number of available RLM-RS in W window.

# Conclusion

In this contribution, we discuss the scope of corresponding RRM requirement for this R19 LB CA scenario.

Assumptions for R19 LB CA via switching

Proposal 1: for gap-less based L3 measurement and L1 measurement of a serving cell which can be either FDD PCell or activated SDL SCell:

- a.	If all {the SSB occasions in SMTC window or CSI-RS for L3 measurement} or {SSB occasions or CSI-RS for L1 measurement} are available based on the LB-CA switching pattern, no delay extension is allowed

- b.	If a fraction of {the SSB occasions in SMTC window or CSI-RS for L3 measurement} or {SSB occasions or CSI-RS for L1 measurement} are not available based on the LB-CA switching pattern, the delay is scaled in accordance to the fraction of available occasions.

- c.	If no {SSB occasions in SMTC window or CSI-RS for L3 measurement} or {SSB occasions or CSI-RS for L1 measurement} are available based on the LB-CA switching pattern, the corresponding requirements do not apply.

Proposal 2: close this issue 1-1-4 since it will not be captured in the spec.

Switching delay or interruption requirement

Proposal 3: RAN4 RRM to close the issue of RRM requirements when more carriers are considered, unless it is specified in the WID.

•	Note: If the case of more than 2CCs is introduced in R19, RRM can revisit this issue.

Activated SDL SCell L3 measurement

Proposal 4: if the activated SCC measurement is MG-based in LB-CA, the legacy intra-frequency identification/measurement requirement with MG can be applied.

Proposal 5: if the activated SCC measurement is MG-less in LB-CA, RAN4 to define a new Kp_LB factor for activated SCC identification/measurement requirement for LB-CA.

Proposal 6: new Kp_LB factor for SSB-less activated SCC identification/measurement requirement in LB-CA can be defined as following:

- Kp_LB = Ntotal_LB / Navailable_LB, where Navailable_LB and Ntotal_LB are calculated as follows:

  - For a window W of duration max(SMTC period,  xRP_max, P), where xRP_max is the maximum xRP across all configured per-UE GAPs, periodic MUSIM gaps, and/or per-FR GAPs within the same FR as the SSB frequency layer, and P is the FDD-SDL switching pattern periodicity, and starting from the beginning of any SMTC occasion:

    - Ntotal_LB is the total number of SMTC occasions within the window, including those overlapped with MG and those overlapped with FDD PCC duration based on the switching pattern within the window, and

    - Navailable_LB is the number of SMTC occasions that are not overlapped with any non-dropped GAP or non-dropped MUSIM gap occasions and not overlapped with FDD PCC duration based on the switching pattern within the window W.

    - Kp = 1 when Navailable = 0.

Deactivated SDL SCell L3 measurement

Proposal 7: Legacy interruption length requirement for intra-band deactivated SCC measurement can be reused for deactivated SCC measurement in R19 LB-CA.

Proposal 8: Probability of missed ACK/NACK for deactivated SCC measurement in R19 LB-CA is X= YmeasCycleSCell in slot level, where Y=2* (2+TSMTC_duation).

SDL SCell activation/deactivation

Proposal 9: If CQI or L1-RSRP reporting is colliding with SSB/CSI-RS or SMTC of being-activated SDL SCell during the SDL SCell activation, no SCell activation requirement shall be applied; otherwise, legacy CQI and L1-RSRP reporting requirement of SCell activation can be reused for SCell activation in LB-CA.

Proposal 10: RAN4 to define the scheduling restriction caused by the measurements during SDL SCell activation procedure.

Proposal 11: The scheduling restriction due to CC switching in SDL SCell activation shall follow RS periodicity or SMTC periodicity used for SCell activation.

Proposal 12: the scheduling restriction length for each CC switching in SDL SCell activation can be defined as {1 slot + SSB/SMTC duration/CSI-RS in slot level + 1 slot}.

- PDCCH, PDSCH, PRS or CSI-RS in a set of symbols of a DL slot and PUSCH, PUCCH, PRACH, or SRS in a set of symbols of an UL slot on the FDD carrier that overlaps, even partially, with the set of symbols of {1 slot + SSB/SMTC duration/CSI-RS in slot level + 1 slot}, will have scheduling restriction.

Proposal 13: At the end of SCell activation (i.e., after UE reporting valid CQI for SCell activation in section 8.3.2 TS38.133), spec allows UE a minimum time separation of 3 ms between transmitting CQI report and applying the semi-statically configured switching pattern.

SDL SCell L1 requirement

Proposal 14: follow principle in issue 1-1-3 to define the BFD/CBD requirement for SDL SCell.

Proposal 15: New P_LB factor is used to replace the P factor in the existing BFD/CBD requirement, where,

- W window is max(TL1, xRP_max, switching pattern periodicity)

- P_LB= Ntotal_LB/ Navailable_LB

- Ntotal_LB is total number of BFD-RS or CBD-RS in W window.

- Navailable_LB is number of available BFD-RS or CBD-RS in W window.

Proposal 16: In TCI state switch delay requirement for SDL SCell, Tfirst-SSB_LB is time to first available SSB reception based on switching pattern after MAC CE command is decoded by the UE for other QCL types

Proposal 17: follow principle in issue 1-1-3 to define the L1-RSRP/SINR measurement requirements for SDL SCell.

Proposal 18: New P_LB factor is used to replace the P factor in the existing L1-RSRP/SINR measurement requirements, where,

- W window is max(TL1, xRP_max, switching pattern periodicity)

- P_LB= Ntotal_LB/ Navailable_LB

- Ntotal_LB is total number of L1-RSRP/SINR RS in W window.

- Navailable_LB is number of available L1-RSRP/SINR RS in W window.

Others for SDL SCell

Proposal 19: this issue 1-3-5 can be FFS (contribution-driven) in the maintenance stage.

Potential impacts to the PCell related requirement

Proposal 20: The PCell RRM requirement revision can follow the same approach used for activated SDL SCell requirement in LB-CA.

Proposal 21: For RLM in LB-CA, follow principle in issue 1-1-3 to define RLM requirements for PCell in LB CA.

Proposal 22: New P_LB factor is used to replace the P factor in the existing RLM measurement requirements, where,

- W window is max(TL1, xRP_max, switching pattern periodicity)

- P_LB= Ntotal_LB/ Navailable_LB

- Ntotal_LB is total number of RLM-RS in W window.

- Navailable_LB is number of available RLM-RS in W window.

# References

[1] R4-2508259, WF on RRM requirements for NR_LBCA_Sw, Apple, RAN4 #115

[2] R4-2505556, Topic summary for [115][207] NR_LBCA_Sw, Apple, RAN4 #115

[3] R4-2503016, WF on low-low band switching, Apple, RAN4 #114