# 3GPP TSG-RAN WG4 Meeting #116	R4-2509503

# Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.27.5.2

Source:	Apple

Title:	On R19 other NTN RRM requirements

Document for:	Discussion

# Introduction

The WF on RRM requirements for Rel-19 NR NTN phase3 [1] has been agreed in RAN4#115, and the open issues have been summarized in the topic summary for [115][228] NR_NTN_Ph3_Part2 [2].

In this contribution, we continue the discussion on the other RRM requirements for R19 NTN.

# Topic #1: SSB periodicity larger than 20 ms

Issue 0: LS from RAN2 on multiple SMTCs with different periodicities

Agreement: (Ad-hoc) From RAN4 perspective, it is feasible to support configuring more than 4 SMTCs per frequency (i.e. 6) for idle/inactive UEs. - In the reply LS (to be prepared by Xiaomi), the following information highlighted in yellow for both intra- and inter-frequency will be added for reference: - FFS on the exact RRM requirement change. - FFS whether to limit the number of SMTCs to no more than 4 is desirable in an earth-moving cell deployment Agreement: online From RAN4 perspective, it is feasible to support SMTCs with up to 2 periodicities and different offsets but requires some clarifications/modifications to the existing RRM requirements. - RAN4 assumes that the configured SMTC periodicity is equal to or greater than the largest SSB periodicity among the cells associated with the relevant SMTC on the frequency layer. Agreement: online The progress of RRM requirement definition for multiple SMTCs with up to two periodicities does not block the completion of RAN4 RRM core part of this WI.

In last RAN2#130 and #129bis meeting, followings were agreed:

## Agreements in #130: ## 1. the maximum configured SMTCs per frequency for idle/inactive UEs is 6 (can come back if we find and issue) ## 2. To support SMTC enhancements for DL CE UE, introduce a new SMTC list instead of extending legacy smtc4list (can consider signalling optimizations for the new list to refer to the content of smtc4list to avoid signalling duplications) ## 3. We introduce a location-based SMTC selection procedure where each SMTC can be associated with a reference location of the intended neighbor cells that need to be measured by the UE. FFS if also an SSB-index based SMTC selection is supported ## 4. No enhancement for cell selection/reselection for DL CE is pursued (can re-discuss this if we will finally introduce a new barring scheme) ## Working Assumption: ## 1. We introduce a mechanism to assist the NW to configure the SMTCs in connected mode, according to one of the following 2 options: ## Alt 1: UE provide the closest N reference locations/neighbor cells to network. ## Alt 2: UE reports the selected SMTCs from configured SMTC set to network. ## We continue in the next meeting on whether to confirm the WA (and go for either Alt1 or Alt2) or whether we don’t introduce any enhancement (i.e. not confirm the WA) ## Agreements in #129bis: ## 3.RAN2 considers to support configuring two different SMTC periodicities (with different offsets) for SMTCs in one frequency layer for idle, inactive and connected mode. We ask RAN4 whether it is feasible to support this in Rel-19 timeframe (also include previous agreement that at any time the UE will not use more SMTCs in parallel than in previous releases). ## 4. We support configuring more than 4 SMTCs per frequency (e.g. 6) for idle/inactive UEs. It will be up to UE implementation to select which of the SMTCs to consider (send this RAN2 decision to RAN4 for checking) ## 5. Network can provide assistance information (for Rel-19 UEs, not necessarily supporting DL CE) on the association between SMTC and location to help UE to perform SMTC selection for idle/inactive mode. FFS on the details of location information, e.g. serving cell SSB index, reference location, etc. In any case it is up to UE implementation on how to utilize the assistance information for SMTC selection in idle/inactive mode. ## In LS to RAN4, RAN2 agreed that, ## For downlink coverage enhancement, RAN2 assumes that it will be possible to have different SSB periodicities among neighbour cells in the same frequency layer, and for the case of intra-frequency neighbouring cells, the SSBs of surrounding neighbour cells may be transmitted at different times. Thus, RAN2 made the following agreements, given that at any time the UE will not use more SMTCs/Gaps in parallel than in previous releases: - RAN2 considers to support configuring two different SMTC periodicities (with different offsets) in one frequency layer for idle, inactive and connected mode neighbor cell measurements. - RAN2 agrees to support configuring more than 4 SMTCs per frequency (i.e. 6) for idle/inactive UEs.

Firstly, we need to differentiate the cases between connected mode and IDLE/inactive mode. For RRC connected mode, network is aware of which serving cell or footprint UE is located in, and therefore the RRC configuration of SMTC shall consider the rough UE location (cell level). And how network can decide such configuration in connected mode is still under discussing in RAN2, i.e., two alternatives as shown in the above RAN2 WA in RAN2#130 meeting.

In IDLE/Inactive mode, network is unaware of any serving cell information for a particular UE. RAN2 agreed that network can provide assistance information (for Rel-19 UEs, not necessarily supporting DL CE) on the association between SMTC and location to help UE to perform SMTC selection for idle/inactive mode. That can really help UE to understand which SMTC(s) can be used for cell searching at a specific location, but again this cannot address the issue when SMTC periodicity is smaller than SSB periodicity of a target satellite as well.

In our view, configuring “two different SMTC periodicities (with different offsets) in one frequency layer for idle, inactive and connected mode neighbor cell measurements” won’t impact UE’s implementation too much, but as agreed in last meeting, RAN4 will define applicability rule that the configured SMTC periodicity is equal to or greater than the largest SSB periodicity among the cells associated with the relevant SMTC on the frequency layer. There also might be some clarification needs to be done in RAN4 NTN requirement, e.g., in 9.3C.4 Inter-frequency measurement with measurement gaps,

The SMTC period in the above requirement for K is not very clear if SMTC configuration can have 2 SMTC periodicities. In our view, here the SMTC period shall be the SMTC which is used for target satellite measurement, like in pervious SMTC1 and SMTC2 discussion for TN, we can consider to clarify that: if different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified/measured.

Proposal 1: to have two different SMTC periodicities (with different offsets) in one frequency layer for idle, inactive and connected mode neighbor cell measurements, some clarifications are needed for existing NTN RRM requirement, e.g., like TN requirement for SMTC1 and SMTC2, it can clarify in the corresponding requirement that, “if different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified/measured”.

# Topic #2: Satellite Beam Hopping (e.g. Cell DTX/DRX)

In last RAN1 FL Summary #4: NR-NTN downlink coverage enhancements (R1-2501728), following issue was discussed:

However, no conclusion has been achieved in last RAN1 meeting. In the current TN FR1 scheduling restriction for L1 or L3 measurement, it has two main kinds:

- TDD based scheduling restriction because UE cannot do UL transmission when UE is operation on DL for neighbor cell measurement.

- Other scheduling restriction applies because UE cannot do e.g., mixed numerology Rx or Tx when UE is performing SSB or CSI-RS measurement.

R17 NTN has new scheduling restriction if UE does’t support parallelMeasurementWithoutRestriction, when UE is performing SS-RSRP/RSRQ/SINR measurement on a neighbor cell served by a different satellite in LEO.

The above scheduling restriction happens during the SSB measurement occasions of the target cells, e.g., UE is doing SSB measurement but cannot receive the data/control or transmit data/control/RACH to the serving satellite. However, in this special case for DL coverage enhancement with Tx beam hopping, even on other time period outside the configured SSB occasions, NW may be still not possible to schedule or receive any channel/signal to or from UE, as shown in following figure 1. To the example UE in cell 2, outside the cell 2 SSB occasions, before T2 and after T2+40ms, NW hopped Tx beams to other cells (cell 1/3/4), and therefore serving satellite cannot transmit/receive anything to or from UE in cell 2. Therefore, we need to reconsider the scheduling restriction applicability in this scenario of DL coverage enhancement.

Figure 1. cell 2 UE(s) may not transmit or receive signaling from satellite when beam hopped away

If the RO periodicity configuration from serving cell is not greater than the SSB periodicity (less than 160ms in the example figure 2), the scheduling restriction applies to UE RACH and RACH related channels transmission outside the time period of serving Tx beam availability, time period of serving Tx beam availability means, e.g., the time period between the beginning of slot containing available SSB (i.e., T2 in example figure 2) and (original SSB periodicity + T2). Thus, the scheduling restriction for RACH could be: If DL coverage enhancement or satellite beam hopping is enabled, the UE is not expected to transmit PRACH on symbols outside the serving satellite Tx/Rx beam availability durations.

Figure 2. example for RO in SSB periodicity

Proposal 2: RAN4 to reconsider the scheduling restriction applicability for DL coverage enhancement, e.g., scheduling restriction for RACH transmission if serving satellite hopped away the beam to not cover the footprint of target UE.

# Conclusion

In this contribution, we discuss the RRM requirement scope for WI of Non-Terrestrial Networks (NTN) for NR Phase 3.

Proposal 1: to have two different SMTC periodicities (with different offsets) in one frequency layer for idle, inactive and connected mode neighbor cell measurements, some clarifications are needed for existing NTN RRM requirement, e.g., like TN requirement for SMTC1 and SMTC2, it can clarify in the corresponding requirement that, “if different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified/measured”.

Proposal 2: RAN4 to reconsider the scheduling restriction applicability for DL coverage enhancement, e.g., scheduling restriction for RACH transmission if serving satellite hopped away the beam to not cover the footprint of target UE.

# References

[1] R4-2508288, WF on RRM requirements for NR_NTN_Ph3_Part2, Qualcomm, RAN4#115

[2] R4-2505577, Topic summary for [115][228] NR_NTN_Ph3_Part2, Qualcomm, RAN4#115