# 3GPP TSG-RAN WG4 Meeting #116	R4-2509507

# Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.14.5

Source:	Apple

Title:	On testing for CSSF enhancement in CA/DC

Document for:	Discussion

# Introduction

The WF of NR Radio Resource Management (RRM) Phase 5 [1] has been agreed in RAN4#115, and the open issues have been summarized in the topic summary for [115][212] NR_RRM_Ph5_Part1 [2].

In this contribution, we continue the discussion for testing of FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC.

# Discussion

Issue 2-2-2: TC list for CSSF optimization solution 1

- Agreement TC index; Scenario; CA/DC Operation mode; Configurations and scopes; Company. 1; Aspect 1: SSB based Intra-frequency measurement without MG; 1 TC per operation mode: FR1+FR2 CA and EN-DC, UE is only required to pass one of them; - Non-DRX, - Without SSB time index detection - CA mode: FR1 PCC+ 2 FR2 intra-band SCCs, network configures MOs on both FR2 SCCs and indicates to measure one FR2 SCC - EN-DC: LTE PCC + FR2 PSCC + 1 FR2 intra-band SCCs, network configures MOs on both FR2 SCCs and UE by-default to measure one FR2 PSCC; Nokia. 2; Aspect 2: SSB based Inter-frequency measurement without MG; 1 TC per operation mode: FR1+FR2 CA and EN-DC, UE is only required to pass one of them; - Non-DRX, - Without SSB time index detection - CA mode: FR1 PCC+ 2 FR2 intra-band SCCs, network configures MOs on both FR2 SCCs and indicates to measure one FR2 SCC - EN-DC: LTE PCC + FR2 PSCC + 1 FR2 intra-band SCCs, network configures MOs on both FR2 SCCs and UE by-default to measure one FR2 PSCC; Ericsson. 3; Aspect 3: Inter-RAT SSB measurement without MG; 1 TC per operation mode: FR1+FR2 CA and EN-DC, UE is only required to pass one of them; - Non-DRX, - Without SSB time index detection - CA mode: FR1 PCC+ 2 FR2 intra-band SCCs, network configures MOs on both FR2 SCCs and indicates to measure one FR2 SCC - EN-DC: LTE PCC + FR2 PSCC + 1 FR2 intra-band SCCs, network configures MOs on both FR2 SCCs and UE by-default to measure one FR2 PSCC; ZTE. Note: network indication is up to the other RAN2/4 core part discussion. In case network indication for a specific SCC measurement is replaced by other new solution(e.g., SCC selected by UE implementation), use the new solution for CA mode test.

If the network indication for a specific SCC measurement is replaced by other new solution(e.g., SCC selected by UE implementation) as discussed in core part, it’s pointless to test a requirement based on UE implementation, and therefore, we propose to remove the TCs related with network indication.

Proposal: for CSSF enhancement solution 1, if the network indication for a specific SCC measurement is replaced by “SCC selection by UE implementation”, remove the TCs related with network indication in last meeting.

# Conclusion

In this contribution, we continue the discussion for FR2-1 SSB based L3 measurement delay reduction for connected mode.

Proposal: for CSSF enhancement solution 1, if the network indication for a specific SCC measurement is replaced by “SCC selection by UE implementation”, remove the TCs related with network indication in last meeting.

# References

[1] R4-2508264, WF on RRM requirements for NR_RRM_Ph5_Part1, Apple, RAN4#115

[2] R4-2505561, Topic summary for [115][212] NR_RRM_Ph5_Part1, Moderator (Apple), RAN4#115