3GPP RAN WG4 Meeting #116																	R4-2509527

Bangalore, India, 25th – 29th August 2025

Agenda item:	5.2

Source:	Apple

Title:	(IoT_NTN_FDD_LS_band-Core) Power back-off for a UE operating in 1610-1626.5 MHz frequency range

WI/SI:	IoT_NTN_FDD_LS_band-Core

Release:	Rel-18

Document for:	Decision

# 1	Introduction

After the RAN#86 meeting, the Rel-17 WI was approved that aimed at enabling the 5G/NR radio access technology for non-terrestrial satellite deployments [1]. One of the WI objectives was to define and introduce the corresponding bands into the 3GPP specifications. The outcome of these discussions was two NTN bands – L-band and S-band – which were added as bands n255 and n256, respectively, to the Rel-17 core specifications. After the RAN#98 meeting new Rel-18 spectrum WIs were agreed with the objective of adding support for new bands where the UL is on the L-band and the DL is on the S-band [2] [3], which finally was specified as the NTN bands 254/n254.

Upon completion of the NTN IOT band 254 specification work some concerns were raised on potentially high power back-off values to comply with the existing ETSI regulations, and thus some AMPR values were left as TBD in TS 36.102. In fact, uncompleted work for band 254 implicitly impacts another spectrum WI [4], which will add the IOT TDD band operating in the same frequency range as the FDD IOT band 254. And as concluded at the RAN4#115 meeting, the corresponding A-MPR values for this band should be completed at least for 12-tone allocations [5]. Thus, in this discussion paper we provide our further technical input on required power back-off values aiming to complete the NTN IOT band 254 specifications.

# 2	Power back-off for band 254

## 2.1	Power back-off with FCC and ETSI emission requirements

Table 2.1-1 below presents a summary of required power back-off for different NB-IOT configurations, such as 12-, 6-, and 3- and 1-tone allocations under the existing FCC and ETSI regulations. The full RB 12-tone allocation can be viewed as the main reference point whereupon the edge allocation can be regarded as a worst-case scenario in terms of required power back-off.

So as can be seen from the table, no power back-off is needed if an NB-IOT UE operates under the FCC rules irrespective of the fact how many tones are scheduled. As for the ETSI regulatory domain, some power back-off is needed for other cases, whereupon the second ETSI sub-range requires higher back-off because of more stringent in-band emission mask. It is also worth noting that for the 1-tone allocation we need to account for the NS_11N and NS_12N flags that can activate the -15dBW/4kHz (i.e. 21.3dBm/15kHz) PSD threshold. If the corresponding flags are set, then we would need to apply at least 2dB power back-off irrespective of the 1-tone position.

Table 2.1-1: Summary of required power back-off values for PC3

| Number of tones | FCC 1610-1626.5 MHz | ETSI |  | Comments |
| --- | --- | --- | --- | --- |
|  |  | 1610-1618.25 MHz | 1618.25-1626.5 MHz |  |
| 12 | 0 | 0 | 0 | Full RB 12-tone allocation |
| 6 | 0 | 0 | 1 | 0-5 and 6-11 tones |
| 3 | 0 | 0.5 | 1.5 | 0-2 and 9-11 tones |
| 1 | 0 | 0.5 | 1.5 | 0th and 11th tone for NS_04N and NS_05N |
|  | 0 | 2 | 2 | 0-11 tones for NS_11N and NS_12N because of a more stringent PSD limit of -15dBW/4kHz (21.3dBm/15kHz) |

And for the sake of completeness, figures below also present the corresponding spectrum plots for FCC and ETSI with the corresponding in-band emission masks.

Figure 2.1-1: 12-tone spectrum plots (FCC, ETSI sub-range 1 and 2)

Figure 2.1-2: 6-tone spectrum plots (FCC, ETSI sub-range 1 and 2)

Figure 2.1-3: 3-tone spectrum plots (FCC, ETSI sub-range 1 and 2)

Figure 2.1-4: 1-tone spectrum plots (FCC, ETSI sub-range 1 and 2)

## 2.2	Further discussion

As captured in WF from the RAN4#115 meeting [5], one of the first open issues is which power back-off values we should adopt for the baseline 12-tone allocations. According to our simulations no power back-off would be needed. In fact, TS 36.102 already allows 2dB MPR allowing the corresponding margin. Nevertheless, there are proponents suggesting adopting even higher values, e.g. 2..4dB for NS_04N and 3.5..5.5dB for NS_05N. Accounting for the fact that there can be different devices with different filter implementations, we can consider larger than 2dB power back-off for the 12-tone allocations.

As for other allocations with fewer than 12 tones, there were simulation results indicating even higher power back-off values at least for the edge allocation. Thus, one of the open issues is whether to specify power back-off for those cases or to conclude that fewer than 12-tone allocations are not defined at all. Our view is that we still can consider defining required power back-off values because the main issue was with the edge allocations, whereas center tones will not require any power back-off. Thus, this approach will allow the network to use at least center allocations to boost resulting PSD irrespective of device filtering capabilities.

Proposal 1:	For the 12-tone allocation, accounting for the fact that there can be different devices with different filter assumptions, larger than 2dB power back-off can be considered.

Proposal 2:	Define power back-off for fewer than 12-tone allocations as it will allow the network to use at least the center allocations.

# 3	Conclusions

In this discussion paper we presented our power back-off simulation results for the NTN NB-IOT operating in the band 254 frequency range. According to our results, some power back-off is needed for a case when a UE operates under ETSI regulations and is scheduled with fewer than 12 tones, whereupon ETSI sub-range 2 requires even higher back-off because of a more stringent in-band emission mask. Based on these results we suggest defining, when needed, the required power back-off values for all cases including 12-tone and fewer than 12-tone allocations.

Proposal 1:	For the 12-tone allocation, accounting for the fact that there can be different devices with different filter assumptions, larger than 2dB power back-off can be considered.

Proposal 2:	Define power back-off for fewer than 12-tone allocations as it will allow the network to use at least the center allocations.

# 4	References

- RP-193234, "Solutions for NR to support non-terrestrial networks (NTN)", Thales

- RP-223431, "New WID on Introduction of the satellite L-/S-band", Globalstar, Apple

- RP-230792, "New WID: FDD LTE band (L+S band) for IoT NTN operation", MediaTek

RP-242415, "New WID for introduction of IoT-NTN TDD mode", Iridium Satellite LLC

R4-2508104, "WF on AMPR for band 254", Sony