3GPP RAN WG4 Meeting #116																	R4-2509538

Bangalore, India, 25th – 29th August 2025

Agenda item:	7.6.3.2.1

Source:	Apple, Globalstar

Title:	Power back-off results for PC2 on NTN L-/S-band

WI/SI:	NR_IoT_NTN_req_test_enh

Release:	Rel-19

Document for:	Decision

# 1	Introduction

After the RAN#86 meeting, the Rel-17 WI was approved that aimed at enabling the 5G/NR radio access technology for non-terrestrial satellite deployments [1]. One of the WI objectives was to define and introduce the corresponding bands into the 3GPP specifications. The outcome of these discussions was two NTN bands – L-band and S-band – which were added as bands n255 and n256, respectively, to the Rel-17 core specifications. However, already that time it was noted that there exist other satellite deployments. To limit the overall workload in Rel-17, it was decided by RAN WG4 to focus on L- and S-bands with the common understanding that other potential NTN bands should follow the "normal" process of submitting a new spectrum WI.

So, during the RAN#98 meeting a new Rel-18 NTN spectrum WI was agreed with the objective of adding support for a new NTN band (band n254) where the UL is on the L-band and the DL is on the S-band [2]. After the RAN#104 meeting a new WI was agreed [3], one of the aims of which is to enable higher power class PC2 for all satellite bands including band n254. Thus, in this discussion paper we present our power back-off simulation results for the PC2 power class operating in band n254 frequency range.

# 2	Power back-off simulations for NTN L-/S-band

## 2.1	Simulation assumptions and scenarios

Satellite L-/S-band (n254) was standardised during Rel-18, whereupon only the PC3 power class was assumed and only for which A-MPR was simulated. The corresponding results are gathered and summarised in TR 38.791. Here we present further power back-off results for the PC2 power class with the common assumptions as presented below:

-	Tx power: PC2 (+26dBm)

-	LO placement: always in the centre of the carrier

-	Regulations: ETSI and FCC

For the sake of completeness, Figure 2.1-1 below shows all configuration cases for the 5, 10 and 15MHz channels with the corresponding centre frequencies used for the power back-off simulations.

Figure 2.1-1: Considered channels for power back-off simulations.

## 2.2	Power back-off results

### 2.2.1	1610-1618MHz ETSI sub-range (NS_04N)

Figure 2.2-1 and Figure 2.2-2 present simulation results for NS_04N, which corresponds to the first frequency sub-range 1610-1618MHz under the ETSI requirements. The simulation results are similar to the one obtained for the PC3 power class with the only major difference that power back-off values are larger for large RB allocations. For small RB allocations they are typically within MPR limits. If the 5MHz channel is shifted further from the left edge by 1.4MHz, as can be seen from Figure2.2-2, then small RB allocations will need only small power back-off.

Figure 2.2-1: A-MPR results for NS_04N (Fc=1612.5MHz).

Figure 2.2-2: A-MPR results for NS_04N (Fc=1613.9MHz).

### 2.2.2	1618-1626.5MHz ETSI sub-range (NS_05N)

Next, Figure 2.2-3 presents simulations results for NS_05N, which is the second frequency sub-range 1618-1626.5MHz operating under the ETSI requirements. Like for PC3, additional power back-off is needed for large allocations and for some small allocations residing at the right edge of the channel. The reason for the latter case is strict ETSI out-of-band emission requirements for the upper edge of this band. If the 5MHz transmission channel is shifted from the upper edge of the band, then required power back-off reduces significantly, and for instance small RB allocations are mostly within the existing MPR limits.

Figure 2.2-3: A-MPR results for NS_05N (Fc=1624MHz).

Figure 2.2-4: A-MPR results for NS_05N (Fc=1622.4MHz).

Next, we present our simulation results for the 10MHz and 15MHz channels. According to the ETSI regulations, if the channel is not fully contained within the first ETSI sub-range, then the requirements from the second range should be followed. That is the reason why we present here 10 and 15MHz channel results, which reside at the lower and the upper edge of the band, under NS_05N. As can be seen from the figures, the overall trend is similar to what we observed earlier with PC3. While small allocations have more or less small back-off, large allocations especially for the band lower edge may require extremely high power back-off values reaching extremely high values up to 15dB.

Figure 2.2-5: A-MPR results for 10MHz NS_05N (Fc=1615MHz).

Figure 2.2-6: A-MPR results for 10MHz NS_05N (Fc=1621.5MHz).

Figure 2.2-7: A-MPR results for 15MHz NS_05N (Fc=1617.5MHz).

Figure 2.2-8: A-MPR results for 15MHz NS_05N (Fc=1619MHz).

# 3	Conclusions

In this discussion paper we have presented our power back-off simulation results for the NTN band n254 with the PC2 power class. The obtained results are generally in line with A-MPR requirements already defined for the PC3 power class with the only major difference that large RB allocations need even larger power back-off. This is explained by the fact that large RB allocations at the edge create larger out-of-band emissions, which amplify further with a higher Tx power. As for the small RB allocations, the preliminary results are very close to the ones obtained earlier for PC3.

Proposal:	Account for the presented power back-off simulations while defining band n254 PC2 RF requirements.

# 4	References

- RP-193234, "Solutions for NR to support non-terrestrial networks (NTN)", Thales

- RP-223431, "New WID on Introduction of the satellite L-/S-band", Globalstar, Apple

- RP-240857, "New WID: Enhanced requirements and test methodology for NR and IoT NTN"