3GPP RAN WG4 Meeting #116		R4-2509542

Bengaluru, India, August 25th – 29th, 2025

Agenda item:	4.2.1

Source:	Apple

Title:	(NR_newRAT-Core) On NS_05 reevaluation

WI/SI:	NR_newRAT-Core

Release:	Rel-15

Document for:	Approval

# 1	Introduction

The WF [1], [2] and [3] capture the state latest agreements on NS_05 and NS_05U for PC3. This contribution provides further tweaks for PC3/PC2 A-MPR region implementation.

# 2  Discussion

##### 2.1 NS_05 simulation results for PC3

As done in our last contribution we would like to propose to use the format as shown in Table 2 which is more often utilised nowadays. Modifications have been made to carrier frequencies by explicitly defining the lower and upper range. The bounds itself were not altered but explicitly written out for 10MHz and 20MHz channel. Those modifications are coloured in orange.

Table 1: A-MPR for Rel-19 NS_05 (Power Class 3)

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS MHz | LCRB*12*SCS MHz |  |
| 10 MHz | 1925 ≤ fc < 1930.7 | ≥ 0 | ≥ 1.35* RBstart*12*SCS + 4.32 | A1 |
|  |  | ≤ 1.8 | ≤ 2.16 | A2 |
| 15 MHz | 1927.5 ≤ fc < 1932.5 | ≥ 0 | ≥ RBstart*12*SCS *1.20 + 4.68 | A3 |
|  |  | ≤ 3.6 >0 | ≤ 3.96 | A10 |
|  |  | 0 | ≤ 3.96 | A4 |
|  | 1932.5 ≤ fc < 1938.2 | ≤ 0.54 | ≥ 8.28 | A5 |
|  |  | ≤ 2.16 | ≤ 1.62 | A6 |
|  |  | ≤ 2.16 | > 1.62 ≤ 2.16 | A11 |
| 20MHz | 1930 ≤ fc < 1945.7 | ≥ 0 | ≥ RBstart*12*SCS *1.5 + 4.14 | A7 |
|  |  | ≤ 1.8 | ≤ 5.76 ≤ RBstart*12*SCS *1.5 + 4.14 | A8 |
|  |  | > 1.8 ≤ 4.68 | ≤ 5.76 | A9 |

Table 2: A-MPR for Rel-19 NS_05

| Modulation/Waveform |  | A1 | A2 | A3 | A4 | A5 | A6 | A7 | A8 | A9 | A10 | A11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | ≤ 3 | ≤ 2 | ≤ 3 | ≤ 2.5 | ≤ 2 | ≤ 2 | ≤ 3.5 | ≤ 3 | ≤ 2.0 | ≤ 1.5 | ≤ 1.0 |
|  | QPSK | ≤ 4 | ≤ 3 | ≤ 4 | ≤ 3 | ≤ 3 | ≤ 2.5 | ≤ 4 | ≤ 3.5 | ≤ 2.5 | ≤ 2.0 | ≤ 1.0 |
|  | 16 QAM | ≤ 4 | ≤ 3 | ≤ 4.5 | ≤ 3.5 | ≤ 3 | ≤ 3 | ≤ 5 | ≤ 4 | ≤ 2.5 | ≤ 2.5 | ≤ 1.0 |
|  | 64 QAM | ≤ 4.5 | ≤ 3.5 | ≤ 4.5 | ≤ 4 | ≤ 3 | ≤ 3 | ≤ 5 | ≤ 4 | ≤ 3.0 | ≤ 3.0 | ≤ 1.0 |
|  | 256 QAM | ≤ 5 | ≤ 4.5 | ≤ 5 | ≤ 4.5 | ≤ 4.5 | ≤ 4.5 | ≤ 5 | ≤ 4.5 | ≤ 4.5 | ≤ 4.5 | ≤ 4.5 |
| CP-OFDM | QPSK | ≤ 5 | ≤ 4.5 | ≤ 6 | ≤ 5 | ≤ 4.5 | ≤ 4.5 | ≤ 6 | ≤ 5.5 | ≤ 4.0 | ≤ 3.5 | ≤ 3.5 |
|  | 16 QAM | ≤ 5 | ≤ 4.5 | ≤ 6 | ≤ 5 | ≤ 4.5 | ≤ 4.5 | ≤ 6 | ≤ 5.5 | ≤ 4.0 | ≤ 3.5 | ≤ 3.5 |
|  | 64 QAM | ≤ 5 | ≤ 4.5 | ≤ 6 | ≤ 5 | ≤ 4.5 | ≤ 4.5 | ≤ 6 | ≤ 5.5 | ≤ 4.5 | ≤ 4.5 | ≤ 3.5 |
|  | 256 QAM | ≤ 6.5 | ≤ 6.5 | ≤ 6.5 | ≤ 6.5 | ≤ 6.5 | ≤ 6.5 | ≤ 7 | ≤ 6.5 | ≤ 6.5 | ≤ 6.5 | ≤ 6.5 |

Proposal 1: NS_05 PC3: Use tables 1 and 2 when writing the CR and include the proposed explicit statement of carrier frequency bounds.

##### 2.2 NS_05 simulation results for PC2

Table 3: A-MPR for Rel-19 NS_05 (PC2)

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS MHz | LCRB*12*SCS MHz |  |
| 5MHz | 1922.5 ≤ fc < 1925 | ≤ 0.18 | ≥ 3.24 | A1 |
| 10 MHz | 1925 ≤ fc < 1930 | ≥ 0 | ≥ 1.35*RBstart*12*SCS + 4.32 | A2 |
|  |  | ≥ 0 | < 1.35*RBstart*12*SCS + 4.32 ≥ 1.2*RBstart*12*SCS + 3.6 | A3 |
|  |  | ≤ 1.8 | ≤ 2.16 | A3 |
|  | 1930 ≤ fc < 1930.7 | ≥ 0 | ≥ 1.35*RBstart*12*SCS + 4.32 | A2 |
|  |  | ≤ 2.16 | ≤ 1.8 | A3 |
| 15 MHz | 1927.5 ≤ fc<1932.5 | ≥ 0 | ≥ 1.2*RBstart*12*SCS + 3.96 | A4 |
|  |  | ≤ 3.6 | ≤ 1.2*RBstart*12*SCS + 3.96 ≤ 4.5 | A5 |
|  |  | ≥ 0 | ≤ 1.2*RBstart + 3.96 ≥ RBstart + 3.6 > 4.5 | A1 |
|  | 1932.5 ≤ fc < 1938.2 | ≤ 1.8 | ≥ 8.28 | A6 |
|  |  | ≤ 2.16 | ≤ 2.16 | A7 |
| 20MHz | 1930 ≤ fc < 1945.7 | ≥ 0 | ≥ RBstart*12*SCS + 4.14 | A8 |
|  |  | ≤ 4.68 | ≤ 6.48 < RBstart*12*SCS + 4.14 | A5 |

The table is based on last way forward and has been converted into the latest format. Modifications have been made to carrier frequencies by explicitly defining the lower and upper range. Those modifications are coloured in orange.

A-MPR values have been combined into a single table. To cut down the number of columns two sets have been merged (highlighted in orange colour). For the first one (which has been assigned to A1) the only difference is that 256QAM CP-OFDM increased from 6.5dB to 7.5dB. For the second one (which has been assigned to A5) the only difference is that 64QAM DFT-s-OFDM is reduced from 5.5dB to 5.0dB. In case those modifications are not agreeable the table can be easily extended with two new columns.

In the following the simulation results for DFT-s-OFDM and CP-OFDM with QPSK modulation are provided. The results cover 5MHz, 10MHz, 15MHz and 20MHz channels at various positions.

Table 4: A-MPR for Rel-19 NS_05 (PC2)

| Modulation/Waveform |  | A1 | A2 | A3 | A4 | A5 | A6 | A7 | A8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner |
| DFT-s-OFDM | PI/2 BPSK | ≤ 3.5 | ≤ 4.5 | ≤ 3.0 | ≤ 4.5 | ≤ 4.0 | ≤ 3.5 | ≤ 3.5 | ≤ 4.5 |
|  | QPSK | ≤ 4.0 | ≤ 5.5 | ≤ 4.0 | ≤ 5.5 | ≤ 4.5 | ≤ 4.5 | ≤ 4.0 | ≤ 5.5 |
|  | 16 QAM | ≤ 4.5 | ≤ 6.0 | ≤ 4.5 | ≤ 6.0 | ≤ 5.0 | ≤ 4.5 | ≤ 4.5 | ≤ 6.5 |
|  | 64 QAM | ≤ 4.5 | ≤ 6.0 | ≤ 4.5 | ≤ 6.0 | ≤ 5.0 | ≤ 5.0 | ≤ 4.5 | ≤ 6.5 |
|  | 256 QAM | ≤ 5.5 | ≤ 6.0 | ≤ 5.0 | ≤ 6.0 | ≤ 5.5 | ≤ 5.5 | ≤ 5.0 | ≤ 6.5 |
| CP-OFDM | QPSK | ≤ 5.5 | ≤ 6.5 | ≤ 5.5 | ≤ 7.5 | ≤ 6.5 | ≤ 5.5 | ≤ 6.0 | ≤ 7.5 |
|  | 16 QAM | ≤ 5.5 | ≤ 6.5 | ≤ 5.5 | ≤ 7.5 | ≤ 6.5 | ≤ 5.5 | ≤ 6.0 | ≤ 7.5 |
|  | 64 QAM | ≤ 5.5 | ≤ 6.5 | ≤ 5.5 | ≤ 7.5 | ≤ 6.5 | ≤ 5.5 | ≤ 6.0 | ≤ 7.5 |
|  | 256 QAM | ≤ 7.5 | ≤ 7.0 | ≤ 7.0 | ≤ 7.5 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.5 |

Proposal 2: NS_05 PC2: Use tables 3 and 4 when writing the CR and include the proposed explicit statement of carrier frequency bounds.

##### 2.3 On modifiedMPR-Behaviour

The agreement from last meeting states that for Rel-15 to Rel-18 specifications, the modifiedMPR bit may be set to 1 to indicate support of the new requirements of NS_05(U). There is also a tentative agreement for Rel-19 to further discuss whether the modifiedMPR bit shall be set to 1.

From our point of view only Rel-19 can have mandatory requirement to set the modifiedMPR bit to 1, as this would be part of the new release specifications. For previous releases (Rel-15 to Rel-18), this can only be optional ("may be set to 1") since making retroactive changes mandatory for frozen releases will disrupt development cycle and release windows of products. It is fine to mandatoryly set modifiedMPR-Behaviour bit to 1 for Rel-19 as it is an open release for which new requirements can be introduced.

Observation: From our point of view only Rel-19 can have mandatory requirement to set the modifiedMPR bit to 1, as this would be part of the new release specifications. For previous releases (Rel-15 to Rel-18), this can only be optional ("may be set to 1") since making retroactive changes mandatory for frozen releases will disrupt development cycle and release windows of products.

Proposal 3: It is fine to mandatoryly set modifiedMPR-Behaviour bit to 1 for Rel-19 as it is an open release for which new requirements can be introduced.

# 4	Conclusions

This contribution provides simulation results and proposals for NS_05. The following proposals are made:

Proposal 1: NS_05 PC3: Use tables 1 and 2 when writing the CR and include the proposed explicit statement of carrier frequency bounds.

Proposal 2: NS_05 PC2: Use tables 3 and 4 when writing the CR and include the proposed explicit statement of carrier frequency bounds.

Observation: From our point of view only Rel-19 can have mandatory requirement to set the modifiedMPR bit to 1, as this would be part of the new release specifications. For previous releases (Rel-15 to Rel-18), this can only be optional ("may be set to 1") since making retroactive changes mandatory for frozen releases will disrupt development cycle and release windows of products.

Proposal 3: It is fine to mandatoryly set modifiedMPR-Behaviour bit to 1 for Rel-19 as it is an open release for which new requirements can be introduced.

# 4	References

- R4-2502980, “WF on UE requirements for PHS protection”, KDDI, OPPO, 3GPP TSG-RAN WG4 Meeting #114

- R4-2505222, “WF on PHS protection update”, KDDI, 3GPP TSG-RAN WG4 Meeting #114bis

- R4-2508023, “WF on PHS protection update”, KDDI, OPPO, Ericsson, TSG-RAN WG4 Meeting #115

- R4-2504569, “A-MPR regions for NS_05/NS_05U and optionality in earlier releases by modified MPR behaviour”, TSG-RAN Working Group 4 (Radio) meeting #114bis