3GPP RAN WG4 Meeting #116		R4-2509545

Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.24.3.2

Source:	Apple

Title:	On LP-WUR Rx requirements

WI/SI:	NR_LPWUS-Core

Release:	Rel-19

Document for:	Discussion

# 1	Introduction

LP-WUS/WUR shall be introduced to NR in Rel-19. The objectives are covered in the WID [1]. During RAN4#114 a fundamental agreement was made with respect to different sets of requirements. It was decided to consider two sets which can feature different performance in noise figure, interference and differ in modulation aspects. Only if those sets turn out to be similar it shall be considered to merge those.

Issue 1-1-1: How to define two LP-WUR types Agreement: - Define two set of RF requirements as the baseline, considering the following aspects including - Noise figures - Interference performance - Modulation - Consider to merge two sets of requirements into one if the requirements are close to each other

# 2  Discussion

Regarding signalling we propose to consider the details provided in Table 1. The crucial aspect is that the UE can select its support of LP-WUR on a per band basis. Designing a dedicated wake-up receiver for all FR1 bands will take more area and the power consumption rises due to LO being required to operate at high bands. Selectively creating a wake-up receiver for selected bands (e.g. low-mid bands) allows to reduce cost and power consumption.

Table 1: Initial take of signalling for LP-WUR

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 55.NR_LPWUS | 55-1 | Wake-up receiver | - Indicating support to receive wake-up signals on a certain band - Indicating capability: Type 1 or Type 2 - Indicate LP-WUS waveform support |  | Yes |  | The feature cannot be supported. | Per Band | No | No |  | Type 1 and Type 2 indication is mapped to RF performance but independent to LP-WUS waveform | Optional with capability signaling |

Proposal 1: When discussing signalling characteristics consider proposal from Table 1.

ACS was discussed last meeting but had no place in the way forward. Therefore, the state of discussion is taken from the topic summary [3]. The summary provides a good overview of different ACS proposals.

To our understanding it is vital to allow relaxation for ACS, IBB and OBB for Type-1 as such devices will focus on low power and low cost. It has been discussed (e.g. in [4]) that it is imperative to consider relaxed requirements for at least ACS, IBB and OBB. Relaxations allow to reduce power consumption which is one of the main goals when considering low power receiver design. However, LP might be mostly used close the BS where interference might be higher if another UE were located at cell edge and configured at neighboring channels.

Observation: Relaxation for applications in the areas of ACS, IBB and OBB is of importance for devices that are designed with a dedicated LP focus on minimizing power consumption and cost. These relaxations enable a reduction in power consumption, which is a primary objective when designing low-power receivers. However, it’s worth noting that LP may be primarily utilized close to the BS, where interference levels tend to be higher if another UE were located at the cell edge and configured on neighboring channels.

Proposal 2: Our analysis on physical hardware suggests that an ACS of 22dB is a reasonable requirement for implementation for Type-1. For Type-2 the value can be similar to main receiver or slightly relaxed.

# 4	Conclusions

This contribution provides a discussion based on last meetings way forward on LP-WUR. The following observations and proposals are made:

IBB and OBB relaxation for the second set can be useful in the field.

Proposal 1: When discussing signalling characteristics consider proposal from Table 1.

Observation: Relaxation for applications in the areas of ACS, IBB and OBB is of importance for devices that are designed with a dedicated LP focus on minimizing power consumption and cost. These relaxations enable a reduction in power consumption, which is a primary objective when designing low-power receivers. However, it’s worth noting that LP may be primarily utilized close to the BS, where interference levels tend to be higher if another UE were located at the cell edge and configured on neighboring channels.

Proposal 2: Our analysis on physical hardware suggests that an ACS of 22dB is a reasonable requirement for implementation for Type-1. For Type-2 the value can be similar to main receiver or slightly relaxed.

# 4	References

- RP-240801, “Low-power Wake-up Signal and Receiver for NR (LP-WUS/WUR)”, vivo, NTT DOCOMO, Ericsson, MediaTek, Samsung, Sony, 3GPP TSG RAN Meeting #103

- R4-2507936, “WF for [115][136] NR_LPWUS_UERF”, vivo, 3GPP TSG-RAN WG4 Meeting #115

- R4-2507575, “Topic summary for [115][136] NR_LPWUS_UERF”, vivo, 3GPP TSG-RAN WG4 Meeting #115

- R4-2500159, “Further consideration on UE RF other Rx requirements for Rel-19 LP-WUS”, Huawei, HiSilicon, TSG-RAN WG4 Meeting #114