3GPP RAN WG4 Meeting #116		R4-2509565

Bengaluru, India, August 25th – 29th, 2025

Agenda item:	6.11.2

Source:	Apple

Title:	Views on CA_n5A-n8A with non-simultaneous Rx-Tx

WI/SI:	LTE_NR_R19_Simult_RxTx-Core

Release:	Rel-19

Document for:	Approval

# 1	Introduction

CA_n5-n8 was first introduced in Rel-18. Due to that the n5 DL band range is overlapping with n8 UL band range, there is no feasible full-band filter to isolate n5 DL and n8 UL. As a result, when n8 UL and n5 DL are operating simultaneously, severe sensitivity degradation in n5 DL carrier is expected. One of the proposals to mitigate the self-interference issue while still allowing UL CA is to schedule non-simultaneous Rx/Tx between n5 DL and n8 UL which however was not considered in Rel-18 due to the potential impact to RAN1 and RAN4 RRM. Nonetheless, this potential solution has been revisited in Rel-19 by the proponent company. Considering that the non-simultaneous Rx/Tx between n5 DL and n8 UL has certain similarity with LB-LB CA via switching [1], several companies have suggested to postpone this configuration to Rel-20 as one of the LB-LB CA via switching scenarios [2]. Though the proponent company did acknowledge that they can consider the LB-LB CA via switching solution in Rel-20 for this band combination, they still tried to push for an unoptimized solution based on RRC reconfiguration between 2UL/2DL and n5 single UL with DL CA in Rel-19, as captured in the approved WF [3] in last RAN4 meeting. However, it was unclear whether this type of the operation would have any impact to RAN1 and/or RAN4 RRM and the WF is encouraging companies to look into this. If there is impact on RRM or other WGs beyond the introduction of UE capability, the solution is not considered in Rel-19.

In this contribution, share our views on CA_n5A-n8A with non-simultaneous Rx/Tx and suggest that the proposed RRC reconfiguration-based solution would require an equivalent 2UL/1DL configuration which is currently not valid in 5G and RAN1 and RAN4 RRM impacts likely would not be avoided. As a result, it is proposed to not consider the RRC reconfiguration-based solution in Rel-19.

# Discussion

CA_n5A-n8A was first introduced in Rel-18. Due to that the n5 DL band range is overlapping with n8 UL band range, there is no feasible full-band filter to isolate n5 DL and n8 UL. As a result, when n8 UL and n5 DL are operating simultaneously, severe sensitivity degradation in n5 DL carrier is expected. Though during the Rel-18 discussions, there were three operation configurations as below considered to mitigate the n8 UL to n5 DL self-interference issue:

- Option 1: DL CA_n5A-n8A with single UL in n5 only

- Option 2: UL/DL CA_n5A-n8A with non-simultaneous Rx/Tx between n5 DL and n8 UL

- Option 3: UL/DL CA_n5A-n8A with additional dedicated filter for the restricted frequency ranges for specific operator

Due to the potential RAN1 and RAN2 impact for Option 2 and the UE implementation/performance challenge in Option 3 [4], only Option 1 operation was specified in Rel-18.

Nevertheless, owing to the demand for better UL throughput performance, the CA_n5A-n8A proponent company has proposed to revisit Option 2 in Rel-19 via a basket WID, with the assumption that there would be no RAN1 and RAN2 impact, or at most only certain capability signaling needs to be introduced to support the feature. However, in our view, the assumption of no RAN1 impact was an overlook in RAN4.

Observation 1: The assumption of no RAN1 impact to support 2UL/2DL CA_n5A-n8A with non-simultaneous Rx/Tx between n5 DL and n8 UL was an overlook in RAN4.

Considering that the non-simultaneous Rx/Tx between n5 DL and n8 UL has certain similarity with LB-LB CA via switching [1], several companies have suggested to postpone this configuration to Rel-20 as one of the LB-LB CA via switching scenarios [2]. Though the proponent company did acknowledge that they can consider the LB-LB CA via switching solution in Rel-20 for this band combination, they still tried to push for an unoptimized solution based on RRC reconfiguration between 2UL/2DL and n5 single UL with DL CA in Rel-19, as captured in the approved WF [3] in last RAN4 meeting. However, it was unclear whether this type of the operation would have any impact to RAN1 and/or RAN4 RRM and the WF is encouraging companies to look into this. If there is impact on RRM or other WGs beyond the introduction of UE capability, the solution is not considered in Rel-19.

Figure 2-1 recaptures the operation diagram for the proposed RRC reconfiguration-based solution [3] where UE would operate between two CA configurations, one with 2UL/2DL but no n5 DL resource allocated at all times during this configuration, the other configuration is DL CA with single UL in n5 only.

Figure 2-1 RRC reconfiguration-based operation for CA_n5-n8 with non-simultaneous Rx/Tx

In our view, the 2UL/2DL configuration without n5 DL resource allocated at all times would be equivalent to a 2UL/1DL CA which is currently not a valid CA configuration in 5G. To enable such configuration for CA_n5-n8 with non-simultaneous Rx/Tx operation, RAN1 and RAN4 RRM impacts likely would not be avoided.

Observation 2: The 2UL/2DL configuration without n5 DL resource allocated at all times would be equivalent to a 2UL/1DL CA which is currently not a valid CA configuration in 5G.

Observation 3: To enable RRC reconfiguration-based solution for CA_n5-n8 with non-simultaneous Rx/Tx operation, RAN1 and RAN4 RRM impacts likely would not be avoided.

Based on the above assessment, we propose to not consider the RRC reconfiguration-based solution for CA_n5-n8 with non-simultaneous Rx/Tx operation in Rel-19.

Proposal: RAN4 do not consider the RRC reconfiguration-based solution for CA_n5-n8 with non-simultaneous Rx/Tx operation in Rel-19.

# 3	Conclusion

In this contribution, share our views on CA_n5A-n8A with non-simultaneous Rx/Tx and suggest that the proposed RRC reconfiguration-based solution would require an equivalent 2UL/1DL configuration which is currently not valid in 5G and RAN1 and RAN4 RRM impacts likely would not be avoided. As a result, it is proposed to not consider the RRC reconfiguration-based solution in Rel-19.

Observation 1: The assumption of no RAN1 impact to support 2UL/2DL CA_n5A-n8A with non-simultaneous Rx/Tx between n5 DL and n8 UL was an overlook in RAN4.

Observation 2: The 2UL/2DL configuration without n5 DL resource allocated at all times would be equivalent to a 2UL/1DL CA which is currently not a valid CA configuration in 5G.

Observation 3: To enable RRC reconfiguration-based solution for CA_n5-n8 with non-simultaneous Rx/Tx operation, RAN1 and RAN4 RRM impacts likely would not be avoided.

Proposal: RAN4 do not consider the RRC reconfiguration-based solution for CA_n5-n8 with non-simultaneous Rx/Tx operation in Rel-19.

# 4	References

RP-243317, “New WID on low band carrier aggregation via switching”, Apple et al, 3GPP TSG RAN Meeting #106, Madrid, Spain, December 9th -12th, 2024

R4-2505451 “Views on CA_n5A-n8A with non-simultaneous Rx-Tx”, Apple, 3GPP RAN WG4 Meeting #115, Malta, May 19th – 23rd, 2025

R4-2507910 “WF on simultaneous Rx/Tx requirements”, Huawei, 3GPP RAN WG4 Meeting #115, Malta, May 19th – 23rd, 2025

RP-231970 “Rescoping CA_n5-n8 in UL CA_n5-n8 support”, Apple, 3GPP TSG-RAN Meeting #101, Bengaluru, India, September 11th – 15th, 2023