3GPP TSG-RAN WG4 Meeting # 116		R4-2509568

Bengaluru , IN, August 25 – 29, 2025

Agenda item:	7.1.2.3.3

Source:	Apple

Title:	On testing the values of Delta_TRxSRS for 6Rx

WI/SI:	NR_ENDC_RF_Ph4-Core

Release:	Rel-19

Document for:	Discussion

# 1	Introduction and Scope

SRS IL imbalance solution is one of the remaining 6Rx aspects. During the previous meeting (Meeting #115, Malta, MT, 19th – 23rd May 2025), it was highly debated and below is a summary of the outcome of the discussions, taken from the  meeting WF on this topic [1].

# <Topic #4> SRS IL imbalance ## <Sub-topic 4-1> Solutions for SRS IL imbalance issue Agreement:(online agreement) - Do not introduce any solution for SRS-IL imbalance issue for UE in Rel-19 - It is FFS on if and how to introduce [R17] RMC and test for SRS. The related discussion should be contribution driven and in the agenda item of R17 maintenance.

As can be seen from the WF summary captured above, although it was agreed to no longer pursue any solution to address the SRS IL imbalance issue, it was however decided to further discuss if and how to introduce RMC and test solutions for the SRS requirements in future meetings with the agenda for Rel-17 maintenance.

In this contribution, we will provide our views on introducing RMC-based tests for the values of ΔTRxSRS and provide a demodulation-based testing approach that could be used.

# Discussion

  - Testing the values of ΔTRxSRS

Specifying the requirements to support SRS antenna switching was one of the main objectives of the 6Rx WID since its introduction [2]. To achieve that objective, RAN4 started discussions on the requirements for the values of ΔTRxSRS since RAN4 meeting #110Bis and came up with an agreement on those values during RAN4 meeting #114. Those requirements are summarized in the table below for all the frequency bands and antenna configurations [3].

| Operating Bands | ΔTRxSRS t3r6 (dB) | ΔTRxSRS t1r6-t3r6 (dB) | ΔTRxSRS t2r6-t3r6 (dB) | ΔTRxSRS t1r6-t2r6-t3r6 (dB) |
| --- | --- | --- | --- | --- |
| Band n41, n77, n78 | 3.0 | 4.5 | 4.0 | 5.5 |
| Band n79 | 4.5 | 6.0 | 5.5 | 7.0 |
| Band n104 | 5.0 | 7.0 | 6.0 | 8.0 |

Table 2.1-1: Agreed values of ΔTRxSRS for 6Rx [3]

As can be observed from the table above, the values of ΔTRxSRS vary considerably depending on NR band and SRS antenna switching configuration. The worst-case value can be as high as 8dB for 1R6R-2T6R-T36R antenna configuration. It should be pointed out that RAN4 defines ΔTRxSRS as the maximum allowed relaxation of the maximum configured transmitted power PCMAX,f,c due to the additional insertion loss differences between the main TX path and a given diversity branch. To derive the values shown in the table above, we assumed worst-case insertion losses from the different  antenna switches, PCB traces, and RF cable losses used to route the signals to and from the antennas.

Since the values of ΔTRxSRS are derived from worst-case assumptions for the insertion losses from the RF front-end components used to route the signal to different antenna ports, testing those values may not provide meaningful information because typical measured values could deviate significantly from the specified values.

Observation #1:	Since the values of ΔTRxSRS are derived from worst-case assumptions for insertion losses from the RF front-end components used to route the signal to different SRS antenna ports, testing those values may not provide meaningful information because typical measured values could deviate significantly from the specified values.

In the previous meeting (3GPP TSG-RAN Meeting #115, Malta, MT, May 19-23, 2025), some contributions were summitted on introducing Reference Measurement Channels (RMCs) to measure the values of ΔTRxSRS for each SRS resource set [4] [5]. Adding these measurement channels in the specifications will require RAN5 to perform ΔTRxSRS measurements. These measurements will have to be performed for each SRS resource, over several frequencies and for all the antenna configurations shown in Table 2.1-1.These multiple additional RAN5 UE conformance tests just for the SRS cannot be fully justified, given the fact that RAN4 did not agree on pursuing any solution for addressing the SRS IL imbalance issue. UE power self-compensation and ΔTRxSRS reporting were considered as possible solutions but we could not agree on a solution, due to a divergence of views. Without a solution, the need to measure the values of ΔTRxSRS becomes less obvious.

Observation #2 : Since RAN4 did not agree on a solution to address the SRS IL imbalance issue, we do not see a strong need to introduce new RMCs just to test the values ΔTRxSRS because this will add multiple RAN5 compliance tests for the UE.

Proposal 1:  Given the reasons highlighted above, we do not support introducing new UL RMCs in the specifications to test the values of ΔTRxSRS.

  - Evaluating ΔTRxSRS based on a demodulation test

In the last RAN4 meeting ((3GPP TSG-RAN Meeting #115, Malta, MT, May 19-23, 2025), we proposed an alternative solution to verify the values of ΔTRxSRS without relying on new UL RMCs (and hance, no RAN5 testing) [6]. This solution is a demodulation test  An alternative solution to verify the values of ΔTRxSRS without relying on new UL RMCs (and hance, no RAN5 testing)

Proposal 2:  The following demodulation test procedure is proposed to determine if the UE did indeed perform power self-compensation on each SRS port and how effective the compensation was:

- The UE is configured for SRS antenna switching

- The UE is configured for CSI reporting – CRI-RI-CQI

- The TE measures SRS and reconstructs the full channel matrix

- The TE determines the rank and CQI of the reconstructed channel

- If the UE has effectively pre-compensated the IL, the reciprocity based CQI and RI will be within certain margin of the UE reported CQI and RI

# 3	Conclusions

Observation #1:	Since the values of ΔTRxSRS are derived from worst-case assumptions for insertion losses from the RF front-end components used to route the signal to different antenna ports, testing those values may not provide meaningful information because typical measured values could deviate significantly from the specified values.

Observation #2 : Since RAN4 did not agree on a solution to address the SRS IL imbalance issue, we do not see a strong need to introduce new RMCs just to test the values ΔTRxSRS because this will add multiple RAN5 compliance tests for the UE.

Proposal 1:  Given the reasons highlighted above, we do not support introducing new UL RMCs in the specifications to test the values of ΔTRxSRS.

Proposal 2:  The following test procedure is proposed to determine if the UE did indeed perform power self-compensation on each SRS port and how effective the compensation was:

- The UE is configured for SRS antenna switching

- The UE is configured for CSI reporting – CRI-RI-CQI

- The TE measures SRS and reconstructs the full channel matrix

- The TE determines the rank and CQI of the reconstructed channel

- If the UE has effectively pre-compensated the IL, the reciprocity based CQI and RI will be within certain margin of the UE reported CQI and RI

# 4	References

R4-2508086 “WF on 6Rx RF requirements and SRS IL imbalance”, by Huawei, HiSilicon (topic moderator) ,  3GPP TSG-RAN Meeting #115, Malta, MT, May 19-23, 2025

RP-240828, New WID: UE RF enhancements for NR FR1/RF2 and EN-DC, Phase 4, Huawei, INC.TSG-RAN 103

R4-2503014, “WF on 6Rx requirements”, by AT&T, 3GPP TSG-RAN Meeting #114,  Athens, GR, February 17 – 21, 2025

R4-2507073, “SRS IL imbalance indication”, by Qualcomm France, 3GPP TSG-RAN Meeting #115, Malta, MT, May 19-23, 2025

R4-2506186, “(NR_RF_TxD-Core) Reference measurement channels for SRS transmissions”, by Ericsson, 3GPP TSG-RAN Meeting #115, Malta, MT, May 19-23, 2025

R4-2505438, “On SRS Antenna Switching IL Imbalance Solution for 6Rx”, by Apple 3GPP TSG-RAN Meeting #115, Malta, MT, May 19-23, 2025