3GPP TSG-RAN WG4 Meeting #116                                       	                      R4-2509573

# Bangalore, India, August 25 – August 29, 2025

Agenda item:	7.1.4

Source:	Apple Inc.

Title:	On UE Demodulation Performance Requirements for 6Rx UEs

Release:	Rel-19

Document for:	Discussion

# Introduction

A new WID on RF enhancements for NR FR1/FR2 and EN-DC has been created for Release 19. The WID covers three main areas: High power UE (HPUE) for CA in terrestrial network (TN), power boosting and/or MPR reduction, and 6Rx for handheld and FWA UEs. The main objectives for 6Rx, as highlighted in the WID [1], are captured below:

Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario

  - Example bands: n41, n77/n78, n79, n104

  - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers

  - Specify the Rx requirements including reference sensitivity requirements for support 6Rx

    - Note: the specified requirements can be applicable to both handheld UE and FWA devices

  - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, t4r6 depending on UE capability

  - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution

Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding

After finalizing the discussion on the maximum number of MIMO layers for 6Rx handheld UEs, in this paper we propose the scope of the 6Rx Demodulation Performance and CSI Reporting Requirements.

# General Aspects

At the previous RAN4#114bis and RAN4#115 meetings, several agreements were made on demodulation and CSI reporting requirements, as well as specific simulation assumptions.

Observation #1: No new PDCCH or PBCH demodulation requirements are to be introduced. New PDSCH requirements will be introduced only for single carrier and no new requirements will be introduced for CA related tests.

Companies have been discussing how to specify applicability rules for the core requirements phase. Some propose that RAN4 can introduce UE declaration on the 6Rx UE type, indicating whether a UE is a handheld or FWA device without capability signalling. Currently, RAN4 specifies separate tests for each UE type, so the only point of contention is whether RAN2 will introduce this capability.

Observation #2: Discussion on capability signaling is on RAN2 discussion, but this parallel discussion does not preclude RAN4 to continue making progress.

Proposal #1: Continue defining separate requirements for 6 MIMO layers for FWA and 4 MIMO layers for handheld UEs. Wait until RAN2’s capability signaling discussion is concluded before finalizing the wording of any applicability rules.

For the rest of this work item, defining the new PDSCH requirements is most important. Based on previous experience during Rel-18 8Rx discussions, we think only a single PDSCH test per case is needed. Additionally, a FWA device should only need to pass a single test case with 6 MIMO layers. If this test passes, the FWA UE should be allowed to skip the 4 MIMO layers test case since all new relevant functionality, e.g. layer mapping and 6Rx functionality, would be already tested.

  - Observation #3: For a 6Rx FWA device, a performance requirement that focuses on 6 MIMO layers exercises the same new funcitonality that a test focusing on 4 MIMO layers.

  - Proposal #2: Define PDSCH requirements for 6Rx UE and capture in test applicability rules that 1) for 6 MIMO layers performance requirements will only apply for FWA devices, while the 4 MIMO layers test case is reserved for handheld UEs.

  - Proposal #3: Discuss the case where if a device passed the 6Rx 4 MIMO layers test case, the UE should be allowed to skip one or multiple 6Rx and/or 4Rx 4 MIMO layers test cases.

# PDSCH Requirements

With respect to specific simulation parameters, the following table consolidates all simulation assumptions agreed after the past RAN4#115. We highlight in yellow those items that still need to be decided.

| Test case type |  | Only single carrier test |
| --- | --- | --- |
| TDD pattern |  | 7DS2U, S: 6D+4G+4U |
| Channel bandwidth / SCS |  | 40 MHz / 30 kHz |
| Propagation channel |  | 4 Layers: Refer to Issue 2-2-1 and 2-2- - TDLA30-10 ULA Low - TDLA30-10 ULA Medium 6 Layers: TDLA30-10 ULA Low |
| Antenna configuration |  | 4 Layers: 4T6R 6 Layers: 8T6R |
| MCS |  | 4 Layers - ULA Low: Option 1: MCS20 (Table 2); Option 2: MCS17(Table 1) - ULA Medium: MCS13 (Table 1) 6 Layers: MCS17(Table 1) |
| PDSCH configuration | Mapping type | Type A |
|  | k0 | 0 |
|  | Starting symbol (S) | 2 |
|  | Length (L) | 12 |
|  | PRB bundling type | Static |
|  | PRB bundling size | 2 |
| TRS configuration |  | Symbol#{5, 9} |
| PT-RS |  | Not configured |
| DM-RS configuration |  | 4 Layer: Rel-15 Type 1 DM-RS with 1+1 6 Layers: Refer to Issue 2-3-3 |
| Overhead |  | 0 |
| Coodebook for PDCCH for PDSCH tests with 4Tx and 8Tx |  | Keep same number of Tx for PDSCH and PDCCH during PDSCH test. Set “codebookMode” to 1 |
| (N1, N2), (O1, O2) |  | 8Tx case: (N1,N2) = (4,1), (O1, O2) = (4,1) 4Tx case: (N1,N2) = (2,1), (O1, O2) = (4,1) |
| Special slot (S slot) scheduling |  | Not schedule PDSCH in special slot for 6 layers cases |
| Number of HARQ Processes |  | 8 |
| HARQ ACK/NACK bundling |  | Not configured |
| Maximum HARQ transmissions |  | 4 |
| Redundancy version coding sequence |  | {0,2,3,1} |
| PDSCH & PDSCH DMRS Precoding configuration |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with PRB bundling granularity |
| Receiver type assumption |  | MMSE-IRC with joint 6Rx MIMO detector |
| Tx EVM assumption |  | - 6% for QPSK/16QAM/64QAM - 3% for 256QAM |
| Test metric |  | SNR@70% max TP |

From the table above, there’s still disagreement on propagation scenarios and antenna correlation models. For the 6 MIMO layers requirements, since this is a FWA test, we suggested reflecting this aspect in the test definition. Therefore, a proper propagation channel should be TDLA30-10, which provides reasonable frequency selectivity and minimises doppler spread. Since there’s no mobility, the network doesn’t need an additional DMRS symbol due to the low doppler spread (which only accounts for surrounding scattering but not UE mobility). Configuring 2+2 for 6 layers isn’t ideal, but RRC controls reconfiguring DMRS positions, which is slow, and higher rank transmissions are likely mostly in low mobility scenarios. We compromised and accepted a 2+2 DMRS configuration, which has the same throughput performance as a 2+0 configuration. Since FWA devices have a large form factor compared to handheld UEs, ULA Low antenna correlation model is reasonable.

  - Observation #4: The 6 MIMO layers PDSCH requirement should reflect in its test parameters that this is a test targeted for FWA devices that experience virtually no mobility. As of today, a single parameter/capability that indicates DCI-based dynamic AddPos scheduling across DMRS capabilities does not exist.

Observation #5: It has been agreed that for 6 MIMO layers, TDLA30-10 is the propagation channel.

Observation #6: It has been agreed that for 6 MIMO layers, ULA Low is the antenna correlation model.

Proposal #4: For 6 MIMO layers, consider Rel-15 DMRS with a 2+2 configuration. Do not consider Rel-18 DMRS.

For the 4 MIMO layers test, since it’s targeted for a handheld UE with limited space and relatively small form factor, parameters should be different. Because of this, a proper propagation channel should be TDLA30-10, since it provides reasonable frequency selectivity while allowing a somewhat higher doppler spread. Since most of the gains of 6Rx vs 4Rx for handheld devices are related to diversity gain, we believe that a relatively low mobility scenario should be targetted, since we still care more for the high throughput conditions. On the other hand, since handheld UEs indeed present form factor constraints, we believe a good approach would be to consider a variation of the existing MediumB antenna correlation model. However, as discussed during the feasibility study before, BS transmitted correlation in MediumB seems quite pessimistic. Hence, we believe a good compromise for testing purposes is to consider Alpha = 0.0 while Beta = 0.07179. Some companies may have concerns that previously used Alpha = 0.3 provides a too high correlation at the Tx side, which might be excessive if compared to existing products in the field. On the other hand, we consider Alpha = Beta = 0.0 too optimistic for a reasonable UE implementation

Observation #7: For 4 MIMO layers, it has been agreed to consider TDLA30-10 as the propagation channel.

Observation #8: Antenna correlation in handheld UEs are expected to be greater than zero, for which assuming Alpha = Beta = 0.0 might be unrealistic and overly restrictive.

Proposal #5: For 4 MIMO layers, define one single test case corresponding to a Handheld UE scenario, considering ULA with Alpha = 0.0 and Beta = 0.07179 for the antenna correlation model.

Finally, to complete the setup for PDSCH requirements we need to consider initial simulation results. Find below the relative throughput curves for the configurations proposed for 4 MIMO layers and 6 MIMO layers.

Figure 1: Relative throughput for Rank-4 and Rank-6 transmission.

In both cases we observe that 256QAM data points operate at a too high SNR to be tested, but we would like to see curves to achieve full throughput well before the 30dB mark. For this, we believe 64QAM MCSs are sufficient for the purposes of these tests.

Observation #9: Our simulation results suggest that 256QAM modulation is not feasible for testing purposes with the proposed test configurations for 4 and 6 MIMO layers.

Observation #10: For 6 MIMO layers PDSCH requirements, it has been agreed to use Table 1 MCS 17.

Proposal #6: For 4 MIMO layers PDSCH requirements, use Table 1 MCS 13.

# SDR Requirements

Furthermore, we would like to address the discussion on SDR requirements, and two aspects have been raised. First, about the static channel and relevant precoding for determine the requirement for 6 MIMO layers with QPSK, 16QAM, 64QAM and 256QAM. We believe the easiest way to move forward is to fix the precoder to determine the maximum MCS for each case. SDR were already defined for 8Rx so we believe they should be defined for 6Rx as well. During the past meetings it was proposed to determine a specific static channel matrix. Current agreements can be summarized as follows:

- For 1T6R (Random precoding)

H=111111

- For 2T6R (Random precoding):

H=111j-jj111-jj-j

- For 4T6R (Random precoding):

- H=111111111-1-1-122j-2j22j3010j305j-33010j-22j2j-22j-3010j305j-3010j

During the past meeting it was agreed that for 8T6R a specific static channel matrix would be defined, and precoding would be fixed (with i1,1=i1,2=i2=0). The last remaining aspect was wether to include a scaling factor in front to the proposed H. This scaling factor was there purely for power normalization. However, the notation in TS 38.101-4 assumes that power normalization should be applied in any case, so the scaling factor is unnecesary. In addition, we can also check that the product between the proposed static matrix and the proposed fixed precoder does not create any rank deficciency.

Observation #11: For 8T6R, the product between the specific fixed precoder (i1,1=i1,2=i2=0) and the proposed H matrix does not result in a rank-deficiency.

Proposal #7: Define SDR requirements for 6Rx UEs and confirm the following matrix to define the static channel for 8T6R and fixed precoder (with i1,1=i1,2=i2=0).

H=wHw-1wH=11111-j1111-1j1-j1-11-1-1j1-11-111-1-11-j11-1-1-1j-1j1-1-111-j1-1-11

The following tables summarise the next step in SDR test definition. First, decide the MCS values as per the agreed static channel matrix and 6 MIMO layers for QPSK/16QAM/64QAM/256QAM as in the table below.

| Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- |
| 6 | 8 | 1 | TBD |
| 6 | 8 | 0.8 | TBD |
| 6 | 8 | 0.75 | TBD |
| 6 | 8 | 0.4 | TBD |
| 6 | 6 | 1 | TBD |
| 6 | 6 | 0.8 | TBD |
| 6 | 6 | 0.75 | TBD |
| 6 | 6 | 0.4 | TBD |
| 6 | 4 | 1 | TBD |
| 6 | 4 | 0.8 | TBD |
| 6 | 4 | 0.75 | TBD |
| 6 | 4 | 0.4 | TBD |
| 6 | 2 | 1 | TBD |
| 6 | 2 | 0.8 | TBD |
| 6 | 2 | 0.75 | TBD |
| 6 | 2 | 0.4 | TBD |
| Note 1: MCS Index for maximum modulation format 2,4 and 6 is based on MCS index Table 1 defined in clause 5.1.3.1 of TS 38.214 [12] Note 2: MCS Index for maximum modulation format 8 is based on MCS index Table 2 defined in clause 5.1.3.1 of TS 38.214 [12] |  |  |  |

Based on our simulation results, we see that MCS 21 for the 256QAM table and MCS23 for the 64QAM table are appropriate.

Proposal #8: For SDR requirements with 6 layers, consider MCS 21 for the 256QAM table and MCS23 for the 64QAM table.

Secondly, to decide the MCS values for 6Rx and 1024QAM and 2 MIMO layers below table summarizes the status.

| Supported RX antenna ports | Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- | --- |
| 6RX | 2 | 10 | 1 | TBD |
|  | 2 | 10 | 0.8 | TBD |
|  | 2 | 10 | 0.75 | TBD |
|  | 2 | 10 | 0.4 | TBD |
| Note 1: MCS Index for maximum modulation format 10 is based on MCS index Table 4 defined in clause 5.1.3.1 of TS 38.214 [12] Note 2: For the band(s) on which UE supporting “Maximum modulation format” of 10, with 2 RX and 2 MIMO layers, the MCS index is derived from the rows with “Maximum modulation format” of 8 in Table 5.5A-5 |  |  |  |  |

Based on our simulation results, we propose that we should not go higher than MCS23 in the 1024QAM table.

Proposal #9: For SDR requirements with 6Rx and 2 layers, we propose not going higher than MCS23 in the 1024QAM table.

# CSI Reporting Requirements

Finally, we propose to base the CSI requirements discussion on the previous experience with the Rel-18 8Rx work item. For CQI reporting requirements, we are not sure about the specific motivation, and at best, we believe static channel should be the minimum consideration. At this point, we prefer not to define new CQI reporting requirements for 6Rx unless a clear motivation is confirmed since the CQI reporting engine in a modem implementation is not really dependent explicitly on the number of Rx paths.

Proposal #10: Do not define CQI reporting requirements specifically for 6Rx UEs.

In addition, we don’t see much value in RI reporting requirements since we did not define such requirements in Rel-18 for 8Rx. In addition, we don’t plan to define requirements for very high ranks.

Proposal #11: Do not define RI requirements for 6Rx UEs.

# Conclusions

Our observations and proposals are summarized below:

Observation #1: No new PDCCH or PBCH demodulation requirements are to be introduced. New PDSCH requirements will be introduced only for single carrier and no new requirements will be introduced for CA related tests.

Observation #2: Discussion on capability signaling is on RAN2 discussion, but this parallel discussion does not preclude RAN4 to continue making progress.

Proposal #1: Continue defining separate requirements for 6 MIMO layers for FWA and 4 MIMO layers for handheld UEs. Wait until RAN2’s capability signaling discussion is concluded before finalizing the wording of any applicability rules.

  - Observation #3: For a 6Rx FWA device, a performance requirement that focuses on 6 MIMO layers exercises the same new funcitonality that a test focusing on 4 MIMO layers.

  - Proposal #2: Define PDSCH requirements for 6Rx UE and capture in test applicability rules that 1) for 6 MIMO layers performance requirements will only apply for FWA devices, while the 4 MIMO layers test case is reserved for handheld UEs.

  - Proposal #3: Discuss the case where if a device passed the 6Rx 4 MIMO layers test case, the UE should be allowed to skip one or multiple 6Rx and/or 4Rx 4 MIMO layers test cases.

  - Observation #4: The 6 MIMO layers PDSCH requirement should reflect in its test parameters that this is a test targeted for FWA devices that experience virtually no mobility. As of today, a single parameter/capability that indicates DCI-based dynamic AddPos scheduling across DMRS capabilities does not exist.

Observation #5: It has been agreed that for 6 MIMO layers, TDLA30-10 is the propagation channel.

Observation #6: It has been agreed that for 6 MIMO layers, ULA Low is the antenna correlation model.

Proposal #4: For 6 MIMO layers, consider Rel-15 DMRS with a 2+2 configuration. Do not consider Rel-18 DMRS.

Observation #7: For 4 MIMO layers, it has been agreed to consider TDLA30-10 as the propagation channel.

Observation #8: Antenna correlation in handheld UEs are expected to be greater than zero, for which assuming Alpha = Beta = 0.0 might be unrealistic and overly restrictive.

Proposal #5: For 4 MIMO layers, define one single test case corresponding to a Handheld UE scenario, considering ULA with Alpha = 0.0 and Beta = 0.07179 for the antenna correlation model.

Observation #9: Our simulation results suggest that 256QAM modulation is not feasible for testing purposes with the proposed test configurations for 4 and 6 MIMO layers.

Observation #10: For 6 MIMO layers PDSCH requirements, it has been agreed to use Table 1 MCS 17.

Proposal #6: For 4 MIMO layers PDSCH requirements, use Table 1 MCS 13.

Observation #11: For 8T6R, the product between the specific fixed precoder (i1,1=i1,2=i2=0) and the proposed H matrix does not result in a rank-deficiency.

Proposal #7: Define SDR requirements for 6Rx UEs and confirm the following matrix to define the static channel for 8T6R and fixed precoder (with i1,1=i1,2=i2=0).

Proposal #8: For SDR requirements with 6 layers, consider MCS 21 for the 256QAM table and MCS23 for the 64QAM table.

Proposal #9: For SDR requirements with 6Rx and 2 layers, we propose not going higher than MCS23 in the 1024QAM table.

Proposal #10: Do not define CQI reporting requirements specifically for 6Rx UEs.

Proposal #11: Do not define RI requirements for 6Rx UEs.

# References

- RP-240720 “New WID: UE RF enhancements for FR1/FR2 and EN-DC, Phase 4”, by RAN4 chair (Huawei), 3GPP TSG-RAN Meeting #103, Maastricht, Netherlands, March 18-21, 2024