# 3GPP TSG-RAN WG4 Meeting #116                                  R4-2509574

# Bangalore, India, August 25 – August 29, 2025

Agenda item:	7.24.5.1

Source:	Apple

Title:	Discussion of Simulation Assumptions for LP-WUR RRM Requirements

Document for:	Discussion

# Introduction

5G systems are designed and developed targeting for both mobile telephony and vertical use cases. Besides latency, reliability, and availability, UE energy efficiency is also critical to 5G. Currently, 5G devices may have to be recharged per week or day, depending on individual’s usage time. In general, 5G devices consume tens of milliwatts in RRC idle/inactive state and hundreds of milliwatts in RRC connected state. Designs to prolong battery life is a necessity for improving energy efficiency as well as for better user experience.

The Rel-18 study item for "Study on low-power wake up signal and receiver for NR" includes investigations to the followings.

- The low-power wake-up signal and receiver, including power saving benefit, coverage, system overhead impact, network energy impact and other related aspects.

- The receiver architecture for low-power wake-up receiver and provide analysis for power consumption, noise figure and etc.

- L1 design and procedure changes needed to support the low-power wake-up signal and evaluations for the link performances.

- Higher layer protocol changes needed to support the low-power wake-up signals

- Related RAN4 impacts

In RRC IDLE/INACTIVE modes, it’s observed that significant UE power saving gain (up to more than 90%) is obtained by using LP-WUS/WUR to trigger UE MR paging monitoring compared with existing I-DRX operation (with and without PEI), if sufficient relaxation to MR RRM measurement is applied. In RRC CONNECTED mode, it’s observed that moderate UE power saving gain (up to more than 10%) is obtained with marginal impact to capacity by using LP-WUS/WUR to trigger UE MR PDCCH monitoring compared with existing UE power saving techniques, across different types of XR traffic and system load scenarios.

In this contribution we continue the analysis on simulation results and assumptions to define L1 filtering delay and accuracy requirements.

# Discussion

RAN4 RRM has been actively engaging in discussions regarding simulation assumptions and parameters, with the aim of establishing the necessary RRM requirements for LP-WUS/WUR once the RAN1 design is completed. After several months, consensus on the LP-SS sequence design, duration, and modulation has mostly been reached in RAN1, and further downselection has taken place. The updated set of candidate sequences is shown below.

For M=1, L=6, the set of LP-SS sequence is:

[1 0 1 0 1 0]

[0 1 0 1 0 1]

[1 0 0 1 0 1]

[1 0 1 0 0 1]

For M=1, L=8, the set of LP-SS sequence is:

[1 0 1 0 0 1 0 1]

[1 0 1 0 1 0 0 1]

[1 0 0 1 0 1 0 1]

[0 1 0 1 0 1 0 1]

For M=2, L=12, the set of LP-SS sequence is:

[1 0 0 1 1 0 0 1 1 0 0 1]

[0 1 1 0 1 0 0 1 1 0 0 1]

[0 1 1 0 0 1 1 0 1 0 0 1]

[0 1 1 0 0 1 0 1 1 0 0 1]

For M=2, L=16, the set of LP-SS sequence is:

[1 0 0 1 0 1 0 1 1 0 0 1 1 0 0 1]

[1 0 0 1 1 0 0 1 0 1 1 0 0 1 0 1]

[1 0 0 1 1 0 1 0 0 1 0 1 1 0 0 1]

[1 0 1 0 0 1 1 0 0 1 1 0 0 1 0 1]

For M=4, L=16, the set of LP-SS sequence is down-selected between:

[0 1 1 0 1 0 0 1 1 0 1 0 1 0 1 0]

[0 1 1 0 1 0 1 0 1 0 0 1 1 0 1 0]

[1 0 1 0 0 1 1 0 1 0 1 0 1 0 0 1]

[1 0 1 0 1 0 0 1 1 0 1 0 0 1 1 0]

For M=4, L=32, the set of LP-SS sequence is down-selected between:

[0 1 0 1 1 0 1 0 1 0 1 0 1 0 0 1 1 0 1 0 0 1 1 0 0 1 1 0 0 1 0 1]

[0 1 1 0 0 1 0 1 0 1 1 0 0 1 0 1 1 0 0 1 1 0 1 0 1 0 1 0 0 1 0 1]

[0 1 0 1 0 1 0 1 1 0 1 0 1 0 0 1 1 0 1 0 1 0 0 1 1 0 1 0 0 1 1 0]

[0 1 0 1 0 1 1 0 0 1 0 1 1 0 1 0 0 1 1 0 0 1 1 0 1 0 1 0 0 1 0 1]

Observation 1: LP-SS candidate binary sequences are down-selected to 1) M=1, L= {6, 8}, 2) M=2, L= {12, 16}, and 3) M=4, L= {16, 32}. The number of occupied OFDM symbols by LP-SS has been narrowed down to 6 symbols and 8 symbols.

In addition, RAN1 has also discussed whether both values of L for the downselected sequences are viable, which has to be taken into account for RAN4 requirements.

Observation 2: For each value of M in existing LP-SS candidate sequences UE supports both L values and no UE capability. There is no consensus in RAN1 that an SNR of -3dB can be fulfilled with the smaller value of L for each M. RAN1 states that definition of the proper requirements for different values of L is up to RAN4.

We continue believing that a critical factor in maintaining progress is achieving alignment on all LP-SS simulation parameters and signal processing assumptions. This, to ensure that results from different companies can be directly compared.

Proposal 1: RAN4 to continue simulation results campaign and alignment until at least RAN4#116bis. Companies to report all 5th, 50th and 95th percentiles of RSRP/RSRQ distributions, and discuss over RAN1 coverage concerns.

Proposal 2: Determine accuracy and delay requirements no earlier than RAN4#116bis to allow simulation results to converge and being fully understood.

# Simulation Assumptions

Simulations conducted by RAN4 are continuously aligning with the evaluations performed in RAN1. In addition, RAN4 RF session continues working on determining the minimum SNR for LP-WUS detection, which in turns will determine the RRM side condition.

During the past RAN4#114bis, some companies have raise the issue of LP-SS into LP-SS interference. This is, when the desired OOK signal is interfered by another OOK signal from another cell, which is configured with the same time and frequency resources. This, in contrast to the scenario that LP-SS receives interference from an OFDM source like PDSCH or another NR channel.

One can observe that the pattern of the interfering LP-SS can play a relevant role, since the definition of RSRP is the total energy seen during the on-duration minus the energy seen during the off-duration, which is clearly different when an OOK waveform is the interference source. As shown above, RAN1 has desiged the LP-SS to have up to four different patterns for each value of M and L.

Observation 3: Companies have raised concerns about the deployment scenario where LP-SS signals are scheduled on the same frequency resources across co-channel cells, suggesting precluding such operation as a way forward.

Observation 4: RSRP/RSRQ accuracy results might be dependent on the binary patterns of desired LP-SS vs the binary patterns of the interfering LP-SS, i.e. the “opposite sequence” problem.

However, it results quite impractical to determine requirements based on which specific LP-SS combination of serving vs interference cell is considered. A better approach might be simply to take the worst case scenario – although determining such scenario might be time consuming.

Proposal 3: RAN4 to discuss if RSRP/RSRQ accuracy requirements should be based on the worst case scenario of colliding LP-SS binary patterns between serving cell and interfering cell.

Finally, another topic to discuss might be even related to the original simulation assumptions being considered. So far this discussion on accuracy requirements is based on the assumption that the serving cell is 6dB above the interfering cell, which apparently has been reused from legacy conditions for NR intra-frequency measurements.

Observation 5: Current Es/Iot > -6dB condition seems to be reused from legacy NR intra-frequency measurements conditions seems overly pessimistic since LP-SS operation is not expected to occur at legacy NR cell edge.

Proposal 4: As an alternative to mandate LP-SS across cells to operate in different time/frequency resources, RAN4 to discuss if the condition of interfering cell vs serving cell power is -6dB is overly pessimistic and whether to relax it to 1) a lower value (<-6dB), or 2) simply remove the interfering cell from the test setup.

Below we show some of our simulation results. Full results are compiled and presented in R4-2509575. In the two figures below, we show the RSRP CDF for a value of M=1 and for different values of L. The case of L=4 has already been discarded by RAN1, but we show it here for completeness. Side condition is kept to Es/Iot = -3dB and we compare between 2 samples and 5 samples for L1 filtering. We can observe that the accuracy gain from L=4 is mostly achieved by L=6 since diminishing returns are observed with L=8. On the other hand, the number of samples does not increase accuracy by a significant amount.

Figure 1: RSRP Distribution for AWGN Channel

In the next figures, we can observe the RSRP CDF when the channel is now TDLC with a low mobility doppler spread. Naturally, we observe that the accuracy reduces due to the channel variation over time.

Figure 2: RSRP Distribution for TDLC Channel

Finally, we compile our results in the tables below. We capture the 5th, 50th and 95th percentiles, which can then be used to compute the absolute accuracy. We observe that the higher the L value the better the accuracy. We also observe that all these values seem to lie in range of the target defined of [±5.5 or ±6] dB for core requirements for LP-RSRP accuracy when the [2 or 2.5] dB RF implementation margins are added to the results below. Note that these cases do not consider the interfering cell setup. For those cases, refer to our companion contribution.

Table 1: RSRP Percentiles for AWGN Channel

Table 2: RSRP Percentiles for TDLC Channel

In our companion distribution, and in some other companies results, we can observe the existence of a bias in the RSRP distribution. For this, we believe it’s important to clarify the root cause of this, since it’s only observed in the case when interference is another LP-SS, i.e. an OOK waveform. When OFDM interference is considered, this bias is gone. Our best explanation is that the on-duration of the colliding OOK waveform is perfectly correlated. Hence, when they collide the addition is fully constructive.

Observation 6: Some companies have observed a bias in the RSRP estimation for the case when LP-SS is interfered by another LP-SS. This bias is not present when the interfering signals is OFDM.

Observation 7: RSRP estimation bias might be explained by the almost perfect correlation between the on-durations of the colliding LP-SS signals, at least in AWGN channel. The bias might be almost perfectly cancelled if serving cell and interfering cell are configured with orthogonal OFDM overlaid sequences, which is what should be deployed in practice.

Observation 8: Network could statically assign a single root index q across cells and use j=1..4 different cyclic shifts associated with root index q for each cell. Alternatively, could assign multiple root indexes q_i across cells with I=1..#Cell where #Cell is determined bit specification or by static variable, in order to reduce the bias observed.

In our simulations we have observed that if we configure orthogonal OFDM overlaid sequences for serving and interference cell respectively, the bias observed initially is now almost entirely cancelled. In this regard, we believe it’s important to take a closer look again at the previous agreement stating that the same requirement shall be applied for OOK-based LP-WUR serving cell measurement based on LP-SS regardless of whether overlaid OFDM sequence is configured or not. If RSRP/RSRQ accuracy is significantly influenced by the configuration or not of overlaid sequences, this is something that should be discussed.

Proposal 5: RAN4 to take a closer look at the decision that same requirement shall be applied for OOK-based LP-WUR serving cell measurement based on LP-SS regardless of whether overlaid OFDM sequence is configured or not, or alternatively, propose a way to deal with the observed estimation bias.

Even though the issues outlined above are important to be clarified, we also propose a concrete way forward so some progress can be made during this meeting. In our results, we observe that beyond 3 samples for L1 filtering the accuracy of the estimation does not diminishes much. At the same time, we see that the obtained values are quite close to the target accuracy discussed during this work item.

Observation 9: There is an obvious tradeoff between RSRP/RSRQ estimation accuracy and the number of samples considered for L1 filtering, but beyond 3 samples we observe diminishing returns.

Observation 10: RSRP accuracy is defined as Actual RSRP vs Ideal RSRP, while RSRQ accuracy is defined by Actual RSRQ vs Ideal RSRQ. Ideal RSRQ is defined as Ideal RSRP vs Actual RSSI.

Proposal 6: RAN4 to consider an RF margin of 2.5 dB for both RSRP and RSRQ accuracy requirements.

Proposal 7: RAN4 to consider relaxing the accuracy requirement to 4dB if considering 2 samples for L1 filtering. Alternatively, RAN4 to relax the delay requirement to 3 samples to achieve a 3.5dB accuracy.

# Conclusion

In this contribution, we continue the discussion on the simulation assumption.

Observation 1: LP-SS candidate binary sequences are down-selected to 1) M=1, L= {6, 8}, 2) M=2, L= {12, 16}, and 3) M=4, L= {16, 32}. The number of occupied OFDM symbols by LP-SS has been narrowed down to 6 symbols and 8 symbols.

Observation 2: For each value of M in existing LP-SS candidate sequences UE supports both L values and no UE capability. There is no consensus in RAN1 that an SNR of -3dB can be fulfilled with the smaller value of L for each M. RAN1 states that definition of the proper requirements for different values of L is up to RAN4.

Proposal 1: RAN4 to continue simulation results campaign and alignment until at least RAN4#116bis. Companies to report all 5th, 50th and 95th percentiles of RSRP/RSRQ distributions, and discuss over RAN1 coverage concerns.

Proposal 2: Determine accuracy and delay requirements no earlier than RAN4#116bis to allow simulation results to converge and being fully understood.

Observation 3: Companies have raised concerns about the deployment scenario where LP-SS signals are scheduled on the same frequency resources across co-channel cells, suggesting precluding such operation as a way forward.

Observation 4: RSRP/RSRQ accuracy results might be dependent on the binary patterns of desired LP-SS vs the binary patterns of the interfering LP-SS, i.e. the “opposite sequence” problem.

Proposal 3: RAN4 to discuss if RSRP/RSRQ accuracy requirements should be based on the worst case scenario of colliding LP-SS binary patterns between serving cell and interfering cell.

Observation 5: Current Es/Iot > -6dB condition seems to be reused from legacy NR intra-frequency measurements conditions seems overly pessimistic since LP-SS operation is not expected to occur at legacy NR cell edge.

Proposal 4: As an alternative to mandate LP-SS across cells to operate in different time/frequency resources, RAN4 to discuss if the condition of interfering cell vs serving cell power is -6dB is overly pessimistic and whether to relax it to 1) a lower value (<-6dB), or 2) simply remove the interfering cell from the test setup.

Observation 6: Some companies have observed a bias in the RSRP estimation for the case when LP-SS is interfered by another LP-SS. This bias is not present when the interfering signals is OFDM.

Observation 7: RSRP estimation bias might be explained by the almost perfect correlation between the on-durations of the colliding LP-SS signals, at least in AWGN channel. The bias might be almost perfectly cancelled if serving cell and interfering cell are configured with orthogonal OFDM overlaid sequences, which is what should be deployed in practice.

Observation 8: Network could statically assign a single root index q across cells and use j=1..4 different cyclic shifts associated with root index q for each cell. Alternatively, could assign multiple root indexes q_i across cells with I=1..#Cell where #Cell is determined bit specification or by static variable, in order to reduce the bias observed.

Proposal 5: RAN4 to take a closer look at the decision that same requirement shall be applied for OOK-based LP-WUR serving cell measurement based on LP-SS regardless of whether overlaid OFDM sequence is configured or not, or alternatively, propose a way to deal with the observed estimation bias.

Observation 9: There is an obvious tradeoff between RSRP/RSRQ estimation accuracy and the number of samples considered for L1 filtering, but beyond 3 samples we observe diminishing returns.

Observation 10: RSRP accuracy is defined as Actual RSRP vs Ideal RSRP, while RSRQ accuracy is defined by Actual RSRQ vs Ideal RSRQ. Ideal RSRQ is defined as Ideal RSRP vs Actual RSSI.

Proposal 6: RAN4 to consider an RF margin of 2.5 dB for both RSRP and RSRQ accuracy requirements.

Proposal 7: RAN4 to consider relaxing the accuracy requirement to 4dB if considering 2 samples for L1 filtering. Alternatively, RAN4 to relax the delay requirement to 3 samples to achieve a 3.5dB accuracy.

# References

[1] RP-241824, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), vivo / NTT DOCOMO, RAN #105, Melbourne, Australia, September 9-12, 2024