# 3GPP TSG-RAN WG4 Meeting #116                                   R4-2509576

# Bangalore, India, August 25 – August 29, 2025

Agenda item:	7.24.6

Source:	Apple

Title:	On RRM Performance for LP-WUS/WUR

Document for:	Discussion

# Introduction

5G systems are designed and developed targeting for both mobile telephony and vertical use cases. Besides latency, reliability, and availability, UE energy efficiency is also critical to 5G. Currently, 5G devices may have to be recharged per week or day, depending on individual’s usage time. In general, 5G devices consume tens of milliwatts in RRC idle/inactive state and hundreds of milliwatts in RRC connected state. Designs to prolong battery life is a necessity for improving energy efficiency as well as for better user experience.

The Rel-18 study item for "Study on low-power wake up signal and receiver for NR" includes investigations to the followings.

- The low-power wake-up signal and receiver, including power saving benefit, coverage, system overhead impact, network energy impact and other related aspects.

- The receiver architecture for low-power wake-up receiver and provide analysis for power consumption, noise figure and etc.

- L1 design and procedure changes needed to support the low-power wake-up signal and evaluations for the link performances.

- Higher layer protocol changes needed to support the low-power wake-up signals

- Related RAN4 impacts

In RRC IDLE/INACTIVE modes, it’s observed that significant UE power saving gain (up to more than 90%) is obtained by using LP-WUS/WUR to trigger UE MR paging monitoring compared with existing I-DRX operation (with and without PEI), if sufficient relaxation to MR RRM measurement is applied. In RRC CONNECTED mode, it’s observed that moderate UE power saving gain (up to more than 10%) is obtained with marginal impact to capacity by using LP-WUS/WUR to trigger UE MR PDCCH monitoring compared with existing UE power saving techniques, across different types of XR traffic and system load scenarios.

In this contribution we continue the analysis on simulation results and assumptions to define L1 filtering delay and accuracy requirements.

# Discussion

During the last RAN4#115 meeting, several companies submitted contriubutions on the performance part of this work item.In their initial contributions, most companies agree that the primary focus of the LP-WUS/WUR test cases should be on measuring the timing, delay, and performance of the UE when operating in various LP-WUR related modes. More specifically, RRM relaxation, RRM offloading, and LP-WUS monitoring. Test cases should verify whether cell reselection and measurement related aspect occur within defined delays. One company emphasizes that the evaluation mainly targets the behavior when a UE is already in these modes, rather than testing the UE’s entry and exit conditions in detail. However, entry/exit condition testing is considered important and deserves further thought.

Observation 1: Testing should verify timing and delay performance of the UE while operating in LP-WUR related modes. It is to be discussed the verification coverage in terms of which exit and entry conditions to test.

In addition, some companies highlight that due to the UE’s choice whether to monitor LP-WUS based on the entry/exit thresholds and conditions (which may be dependend on implementation), the LP-WUS/WUR feature is inherently difficult to verify under normal test conditions. Some companies argue that the absence of a guaranteed test mode makes it impossible to ensure the UE enters or exits LP-WUS mode consistently during testing. We disagree.

Observation 2: Verifying LP-WUR behavior is not unreliable or impractical without a test mode.

However, a dedicated test mode is rather invasive for UE implementation, akin implementing a separate secondary feature only to verify the primary feature.

Observation 3: On the other hand, a test mode is quite invasive in terms of UE hardware/firmware implementation

There are companies advocating that separate test sets should be developed for LP-WUR UEs supporting different reference signals: one set for PSS/SSS-based LP-WUR and another for LP-SS-based LP-WUR. This differentiation arises due to differing measurement and detection techniques involved. Some companies also note the need to discuss applicability when UEs support both reference signals. Ericsson and others suggest streamlining tests to cover both types through a single test case where possible but recommend clarifying applicability rules.

Observation 4: Test cases need to be separated based on whether the UE supports PSS/SSS- or LP-SS-based LP-WUR due to differing measurement mechanisms.

Test cases should cover several specific scenarios related to NR cell reselection and measurement relaxation cases. These include intra-frequency, inter-frequency (covering lower, equal, and higher priority cells), and inter-RAT cell reselection scenarios when UEs operate with relaxed measurement criteria (e.g., 16x relaxation factor). Wake-up delay after LP-WUS reception is also highlighted as a key performance metric to be verified.

Observation 5: RAN4 should discuss the test coverage and if and how to include various cell reselection scenarios across intra-frequency, inter-frequency, and inter-RAT cases.

# Open Issues

In this part, we make our proposals based on the specific open issues. We do not expand the scope from what is stated below since during the last RAN4#115 meeting there was very little time to discuss these topics. In the interest of time, we belive companies should remain focused on the following open items before moving to additional proposals.

Issue 3-1-2 General aspects

- Proposals

  - P1: The focus of the test cases is the time and delay behaviour when a UE stays in a particular mode, particularly RRM offloading, RRM relaxation and LP-WUR monitoring mode. (vivo)

  - P2: Define test mode for LP-WUR functionality specified in the rel-19. (Nokia)

There is a general agreement that timing/delay behavior during operation in various modes is key to testing. However, Nokia’s focus on requiring a dedicated test mode.

Proposal 1: RAN4 to focus on time and delay behavior of RRM offloading, RRM relaxation and LP-WUR monitoring modes.

Proposal 2: Do not define a test mode for LP-WUS/WUR performance part.

With respect to specific test case scenarios, there are multiple proposals brought by multiple companies. These issues are summarized in Issue 3-2-1 below:

Issue 3-2-1: Test case categories/scenarios

- Proposals

  - P1: Consider test case for the following categories: Category 1: FR1 LP-WUR monitoring related test case; Category 2: FR1 RRM offloading related test case; Category 3: FR1 RRM relaxation related test case (vivo)

  - P2: Introduce a test case for to test LR Evaluation (delay) requirements. (Nokia)

  - P3: Design test cases for LP-WUR measurement and evaluation at RRC_IDLE/INACTIVE state; design test case for relaxed MR measurement for serving cell/neighbour cell measurements (CT)

  - P4: Define test cases for evaluation of exit conditions for the following cases. (Huawei)

    - Case 1: UE exist Case 1 to legacy

    - Case 2: UE exist Case 3 to legacy

    - Case 3: UE exist Case 1 to Case 3

  - P5: RAN4 to define the test cases to verify both MR RRM relaxation and MR RRM offloading scenarios. RAN4 to introduce the RRM relaxation test cases similar as Rel-16 cell reselection (Ericsson)

  - P6: RAN4 to discuss whether to introduce an applicability to the LP-WUR UE which supports both LP-SS and SSB. (Ericsson)

Issue 3-2-2: Test case for OOK based LR and OFDB based LR

- Proposals

  - P1: Introduce separate sets of test cases for UE supporting PSS/SSS based LP-WUR and UE supporting LP-SS based LP-WUR. (Nokia Huawei)

  - P2: RAN4 to introduce a single test for both LP-SS and SSB based LP-WUR UE (Ericsson)

Issue 3-2-3: Test case design for entry conditions for case 1/case 3

- Proposals

  - P1: Use test mode for LP-WUR entry requirements. FFS for the details of how to do this in RRM; Introduce MR only entry test case, and MR + LR entry test cases for RRM measurements; (Nokia)

  - P2: Do not define dedicated test cases for evaluation of entry conditions. (Huawei)

Issue 3-2-4: Test case design for RRM relaxation

- Proposals

  - P1: Introduce a test case for MR reselection with case 3 and with 16x relaxation. (Nokia)

  - P2: Define Cell reselection to FR1 intra-frequency NR case for UE fulfilling relaxed measurement criterion; Cell reselection to FR1 inter-frequency NR case for UE fulfilling relaxed measurement criterion. (vivo)

  - P3: For Case#3, at least to introduce following tests to guarantee the test coverage (CMCC)

    - intra-frequency cell re-selection

    - inter-frequency cell re-selection including lower and equal priority re-selection and higher priority re-selection

    - inter-RAT cell re-selection

  - P4: In case 3, the test cases for cell reselection shall be considered and the specific number of test cases could be discussed in this meeting. (ZTE)

  - P5: Introduce the test cases where the exit conditions can be tested: UE meets the cell reselection requirements with initial conditions being WUR operating in the relaxed measurement mode. (Qualcomm)

Issue 3-2-5: Test case design for offloading case

- Proposals

  - P1: For RRM offloading case, a joint procedures can be considered to verify both the LR measurement requirement and whether a MR is triggered in time. (vivo)

  - P2: RAN4 to discuss the test methodology about how to verify MR RRM offloading scenario, such as whether to introduce a new test mode. (Ericsson)

  - P3: Introduce the test cases where the exit conditions can be tested: UE meets the cell reselection requirements with initial conditions being WUR operating in the full offloading mode. (Qualcomm)

Issue 3-2-6: Test case design for higher priority frequency layer search

- Proposals

  - P1: Introduce test case for high priority frequency search with case 1 and case 3. (Nokia)

  - P2: Not to introduce test case considering the test feasibility and testing time effort for Case#1 relaxed higher frequency carrier cell re-selection requirement. (CMCC)

  - P3: Define 1 test case for higher priority cell reselection in Case 1. (Huawei ZTE)

Issue 3-2-6: Test case design for LP-WUS monitoring

- Proposals

  - P1: Introduce test cases for LP-WUS monitoring entry / exit with MR sync delays and paging reception. (Nokia)

  - P2: Define one test case to verify the MR wake up delay when UE receives LP-WUS. (Huawei Qualcomm)

  - P3: No need to discuss LP-WUS missed detection or false-alarm rate performance for RRM. (Qualcomm)

Issue 3-2-7: On test case configuration

- Proposals

  - P1: Due to the large scaling factor for MR relaxation requirement, DRX 320ms or 640ms shall be configured as test setup to minimize the testing time effort. (CMCC)

  - P2: RAN4 to introduce new LP-SS configuration for LP-WUS test case. (Ericsson)

Issue 3-2-8: On FR1/FR2 for test case

- Proposals

  - P1: RAN4 to only introduce FR1 based LP-WUS test case. (Ericsson)

In general there is a broad agreement that test cases must comprehensively cover monitoring, relaxation, and offloading scenarios. However, we believe that only exit conditions should be verified.

Proposal 3: RAN4 to consider only define test cases for exit conditions.

We have two proposals here about test specificity of “separate test sets” vs a more efficient “single test case” approach. We believe that separate tests ensure clarity and precision in understanding this new feature, especially because the two types use different signaling and measurement approaches.

Proposal 4: RAN4 to consider defining separate test cases based on PSS/SSS vs LP-SS.

In addition, we believe that high-priority layers should be verified, but the scope should be maintained brief.

Proposal 5: RAN4 to define a test case for higher priority cell reselection in Case 1.

Finally, we believe that the focus on LP-WUS/WUR should remain in FR1, since there is no verifiable traction of the feature in Frequency Range 2 at all.

Proposal 6: RAN4 to only introduce RRM performance requirements for FR1.

# Conclusion

In this contribution, we summarize our observations and proposals below:

Observation 1: Testing should verify timing and delay performance of the UE while operating in LP-WUR related modes. It is to be discussed the verification coverage in terms of which exit and entry conditions to test.

Observation 2: Verifying LP-WUR behavior is not unreliable or impractical without a test mode.

Observation 3: On the other hand, a test mode is quite invasive in terms of UE hardware/firmware implementation

Observation 4: Test cases need to be separated based on whether the UE supports PSS/SSS- or LP-SS-based LP-WUR due to differing measurement mechanisms.

Observation 5: RAN4 should discuss the test coverage and if and how to include various cell reselection scenarios across intra-frequency, inter-frequency, and inter-RAT cases.

Proposal 1: RAN4 to focus on time and delay behavior of RRM offloading, RRM relaxation and LP-WUR monitoring modes.

Proposal 2: Do not define a test mode for LP-WUS/WUR performance part.

Proposal 3: RAN4 to consider only define test cases for exit conditions.

Proposal 4: RAN4 to consider defining separate test cases based on PSS/SSS vs LP-SS.

Proposal 5: RAN4 to define a test case for higher priority cell reselection in Case 1.

Proposal 6: RAN4 to only introduce RRM performance requirements for FR1.

# References

[1] RP-241824, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), vivo / NTT DOCOMO, RAN #105, Melbourne, Australia, September 9-12, 2024