# 3GPP TSG-RAN WG4 Meeting #116                                   R4-2509577

# Bangalore, India, August 25 – August 29, 2025

Agenda item:	7.24.7

Source:	Apple

Title:	On the MDR Performance Requirements for LP-WUS

Document for:	Discussion

# Introduction

5G systems are designed and developed targeting for both mobile telephony and vertical use cases. Besides latency, reliability, and availability, UE energy efficiency is also critical to 5G. Currently, 5G devices may have to be recharged per week or day, depending on individual’s usage time. In general, 5G devices consume tens of milliwatts in RRC idle/inactive state and hundreds of milliwatts in RRC connected state. Designs to prolong battery life is a necessity for improving energy efficiency as well as for better user experience.

The Rel-18 study item for "Study on low-power wake up signal and receiver for NR" includes investigations to the followings.

- The low-power wake-up signal and receiver, including power saving benefit, coverage, system overhead impact, network energy impact and other related aspects.

- The receiver architecture for low-power wake-up receiver and provide analysis for power consumption, noise figure and etc.

- L1 design and procedure changes needed to support the low-power wake-up signal and evaluations for the link performances.

- Higher layer protocol changes needed to support the low-power wake-up signals

- Related RAN4 impacts

In RRC IDLE/INACTIVE modes, it’s observed that significant UE power saving gain (up to more than 90%) is obtained by using LP-WUS/WUR to trigger UE MR paging monitoring compared with existing I-DRX operation (with and without PEI), if sufficient relaxation to MR RRM measurement is applied. In RRC CONNECTED mode, it’s observed that moderate UE power saving gain (up to more than 10%) is obtained with marginal impact to capacity by using LP-WUS/WUR to trigger UE MR PDCCH monitoring compared with existing UE power saving techniques, across different types of XR traffic and system load scenarios.

# Background

RAN4 RRM and RF sesions have continued monitoring the progress of this WI on RAN1 and RAN2 sessions. RAN1 has mostly completed the signal design and UE operation on both Idle/Inactive and Connected Mode. RAN2 continues on LP-WUS configuration, LP-SS configuration and the Entry/Exit condition for LP-WUS monitoring and RRM relaxation related aspects. RAN4 RRM is focused on RRM Measurements relaxation aspects based on LP-SS, while RAN4 RF is focused on determining REFSENS and SNR for LP-WUS detection.

Observation 1: LP-WUS detection is revelant for both Idle/Inactive and Connected Mode.

For the LP-WUR, there are two types of receivers considered: an OOK receiver and a OFDM receiver. From LP-WUS design point of view, RAN1 has specified OOK (both OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK on-duration symbols, and with the possibility of this OFDM sequence to carry information the information as well.

Observation 2: For LP-WUS detection, the LP-WUR receiver can be of OOK type and of OFDM type.

The OOK receiver type performs envelop detection to decode the informaton carried over the OOK ON and OFF symbols. This receiver has lower power consumption but also lower link-level performance. It’s not sensitive to frequency error and it’s moderately sensitive to timing errors. See Figure 1.

Figure 1: OOK Receiver - homodyne/zero-IF architecture with baseband envelope-detection based LP-WUR

The OFDM receiver has higher complexity since it may perform time-domain or frequency-domain correlation with known sequences, decoding the information carried in the overlay sequences. This receiver has higher power consumption but also a better link-level performance. It can be sensitive to both time and frequency errors, but may reuse PSS/SSS for synchronization. See Figure 2.

Figure 2: OFDM Receiver - IQ based LP-WUR

In terms of the OOK waveform, RAN1 has designed based on OOK with Manchester Encoding as the baseline waveform for LP-WUS and LP-SS. The OOK-based receiver can be used to detect the OOK symbols, while the OFDM receiver can detect also the OFDM overlaid sequence. DFT-s-OFDM is used to generate OOK waveform to allow the reuse of the gNB hardware for generating this waveform while multiplexing this newly generated signal with legacy NR signals.

Observation 3: New OOK modulation and new OFDM overlaid sequence detection has been specified for LP-WUS detection, which are novel in the context of TS 38.101-4.

In terms of modulation, the parameter MMM can be 1, 2, or 4, representing the number of OOK symbols contained within a single OFDM symbol. Regarding the information payload, an LP-WUS can carry up to 5 information bits. These bits are mapped to one of 32 possible codepoints, each corresponding to a group of UEs. These codepoints indicate which UEs should begin PDCCH or Paging monitoring at a specific Paging Occasion (PO). This codepoint-based wake-up signaling mechanism is used for LP-WUS in both idle/inactive and connected modes.

In terms of coding, for # of information bits >=3 bits, Reed-Muller (RM) code is used with rate matching to match the number of OFDM symbols available. For more details, see TS 38.212 Section 5.3.3 where RM code is specified for Uplink Control Information (UCI). For 1 and 2 information bits, the procedure is given in the same Section 5.3.3.

Observation 4: For OOK envelope detection, LP-WUS coding aspects have both an information payload dimension and a code rate dimension.

In terms of the so-called Overlaid OFDM Sequences, these are specified to be on top of the OOK ON symbols, i.e. the ON duration of the OOK pattern transmitted for the LP-WUS. These overlaid sequences may or may not carry actual information. The receiver can perform sequence correlation in either time or frequency domain, and it is expected that an OFDM receiver can detect early the LP-WUS compared to an OOK receiver, depending on the number of candidate overlay sequences for each OOK ON symbol.

When using overlaid OFDM sequences, several details are important. First, the length of the overlaid OFDM sequences is given by 12 * X / M for all M values, where X = 11 is the number of PRBs for the LP-WUS, meaning the sequence length is 132, 66 and 33 for M = 1, 2 and 4, respectively. In addition, overlaid OFDM sequences use Zaddoff-Chu (ZC) sequences with cyclic extension. BZC is given by the largest prime number such that BZC < LZC, with the corresponding allowed root values and cyclic extensions, which are in turn obtained from RRC signalling. In addition, the maximum number of candidate overlaid sequences per OOK ON chip for one cell is 16 for M=1; 8 for M=2; and 4 for M=4. If the overlaid sequence does not carry information, a single overlaid sequence is on each OOK ‘ON’ symbol. Otherwise, the raw information bits are used to determine the overlaid OFDM sequences for each OOK ON chip. This is

- Assume L is the number of candidate overlaid OFDM sequences for one OOK ON chip.

- Assume N is the number of raw information bits. If N / log2 L is not an integer, bit 0 as MSB is used for padding.

- The raw information bits are divided into K segments from MSB to LSB, where K = ceil(N / log2 L).

In one OOK ON chip, a segment of information bits is mapped to one sequence sequentially. See Figure 3 below.

Figure 3: Overlay sequence to bit mapping in OOK waveform

Observation 5: For OFDM detection, LP-WUS can be transmitted in several ways based on the configured M-factor, which determines a maximum number of candidate overlaid sequences per OOK ON chip.

In general, it can be observed that several new aspects are introduced with this new LP-WUS waveform. Hence, RAN4 needs to discuss if and how to determine the relevant demodulation requirements based on the multiple novel aspects discussed above.

# Discussion

In terms of concrete parameters, we first need to recognize the different variations of the LP-WUR. During the last RAN4#115 meeting, it was agreed that we will consider both receivers and waveforms.

Observation 6: During RAN4#115 it was agreed to consider both OOK and OFDM receivers, and both waveforms OOK-1 and OOK-4.

In terms of the frequency range being considered for demodulation requirements, we believe we should take priority with FR1. We don’t see strong traction of deploying this feature, at least in Rel-19, on frequencies other than FR1. In addition, operators have strong interest in deploying the feature in a restricted number of frequencies, which justified even more that FR2 should be deproritized at least from requirements point of view. RAN4 can decide once FR1 requirements are fully in place whether more time should be allocated to work on the FR2 side.

Observation 7: There is stronger interest in deploying the LP-WUS feature on a limited number of frequencies, and seems more cost effective that frequencies are in FR1.

Proposal 1: Prioritize FR1 study. RAN4 to decide about FR2 after requirements after meaningful progress granted for FR1.

On the other hand, in terms of duplexing, it’s common practice in demodulation requirements to define for both TDD and FDD, even though the actual SNR requirements might be very close, the value of having both configurations is in testing the time domain scheduling of LP-WUS. In terms of bandwidth and number of RBs being considered, relevant configurations are quite clear. To maintain typical configurations cited in 38.101-4, we should maintain 10MHz bandwidth for FDD and 40MHz bandwidth for TDD as usual.

Observation 8: RAN1 design supports LP-WUS length in actual OFDM symbols used larger than one slot, hence also non-consecutive OFDM symbols at least across multiple slots.

Proposal 2: Consider a SCS of 15kHz for a channel bandwidth of 10MHz FDD, and a SCS of 30kHz for a channel bandwidth of 40MHz TDD. Do not consider FR2 120kHz for the moment.

In terms of propagation related characteristics, antenna configuration and channels are relevant. We believe that the antenna configuration should be assumed as 1Tx/1Rx since this should be in any case a low-complexity feature for both BS and UE. In terms of propagation, both TDLA and TDLC channels could be taken into consideration since there are valid operating points. However, the expected mobility of a UE operating with this feature is assumed to be low. Hence, a good approach would be to define requirements based only on TDLA30-10.

Observation 9: For best LP-WUS/WUR operation, expectations are that UE should operate in rather low mobility conditions.

Proposal 3: Consider 1Tx/1Rx as antenna configuration.

Proposal 4: Define TDLA30-10 as propagation channel.

In previous discussions about LP-WUS detection, several parameters were agreed upon. For OFDM receivers, a typical frequency offset of 5ppm was considered. For OOK receivers, a frequency offset of 10 to 20ppm was considered, but we believe a frequency offset of 20ppm should be considered to accommodate different receiver implementations.

Proposal 5: Consider defining requirements using a frequency offset of 5ppm for the OFDM LP-WUS receiver, and 20ppm for the OOK LP-WUS receiver.

RAN1 has agreed on parameters for residual time offset based on expected synchronisation performance with LP-SS. These parameters depend on the LP-WUS ON duration, which varies with the modulation order M = {1,2,4}.

Proposal 6: Consider defining requirements using a residual time offset T equal to 5us for OOK-1, T= 2us for OOK-4 with M=2, T= 1us for OOK-4 with M=4.

However, we can note that the current demodulation requirements specified in TS 38.101-4 are based on the assumption of ideal time and frequency synchronization, as the resulting impact on the required SNR has been quite minimal. Building on this, we propose that RAN4 should further investigate the influence of these practical deviations in frequency and timing compared to ideal synchronization conditions. Should this analysis confirm that the effects remain negligible, it would be reasonable to continue specifying demodulation requirements under ideal synchronization assumptions. This approach would help streamline testing procedures by reducing complexity without compromising the accuracy or relevance of the performance requirements.

Observation 10: Current demodulation requirements in TS 38.101-4 consider ideal time and frequency synchronization since the effect on required SNR is minimal.

Proposal 7: RAN4 to assess how practical frequency and time offsets affect LP-WUS miss-detection rate, and determine if demodulation requirements can be defined assuming ideal synchronization to simplify testing.

In addition, RAN1 has defined for the OFDM LP-WUR receiver the maximum number of candidate overlaid sequences. The longer the chip duration due to M, the larger the maximum number of candidate overlaid sequences, and the number of roots. For mapping between a sequence index cm and root value (qcm) & cyclic shift (CS) value (ncs, cm).

qcm=qi ,   i=floor(cmP),  P=NseqNroot

ncs, cm=ncs, k,     where k=mod(cm, P)

where cyclic shift value ncs, kis determined by
ncs, k=k*floor BZC P, where k=0,…, P-1

At least for FR1, RAN1 design supports a maximum 2 roots to be used for overlaid OFDM sequences for M=1/2/4, with a maximum of 2 roots for each M value, and different roots can be picked for different M values.

Observation 11: For the OFDM LP-WUR, RAN1 has defined the maximum number of candidates overlaid sequences to carry LP-WUS information per OOK ON chip for one cell, with 16 candidates for M=1, 8 candidates for M=2 and 4 candidates for M=4, with corresponding mapping between roots and cyclic shifts.

Proposal 8: For the OFDM LP-WUR, consider at most one test per modulation factor M. FFS on overlaid sequence configuration details.

The OOK LP-WUR receiver has a clear coding pattern based on the number of information OOK bits. For 1 and 2 information bits, coding is as described in the first row of TS 38.212, Section 5.3.3.1 and Section 5.3.3.2, respectively. For more than 3 information bits, Reed Muller coding is applied.

Observation 12: For the OOK LP-WUR, RAN1 has defined two types of bit encoding schemes.

Proposal 9: For the OOK LP-WUR, limit the coding scheme tests to two: one for 1 or 2 information bits, and one for the maximum number of information bits, which is 5.

Proposal 10: Consider a total of L >= 16 coded bits for 1-2 information bits, and a total of L=32 bits for 5 information bits.

With respect to previous proposals on Type-1 vs Type-2 UEs as defined in the RF room, we are only agreeing to define separate requirements if these are tied to OOK vs OFDM receiver waveforms. If the concern is only about the implied difference in noise figure, we believe we do not need to define separate requirements.

Observation 13: The noise figure difference implied by Type-1 and Type-2 UEs only affects the REFSENS test in the RF domain. It does not affect the target SNR perceived by the UE for demodulation requirements. Only a difference in the received waveforms are relevant.

Proposal 11: Do not define separate demodulation requirements for Type-1 and Type-2 LP-WUR capable UEs if those are not tied to Envelope vs Sequence Detection.

In addition, another proposal that remains from the previous meeting is whether to define LP-SS synchronization requirements. We believe that such kind of requirements are out of the scope of RAN4 demodulation, hence should not be considered.

Observation 14: RAN4 demodulation requirements are not related to synchronization performance, but only with LP-WUS detection rate.

Proposal 12: Do not define LP-SS based synchronization requirements.

To perform agreed test based on the parameters discuss above, we need to discuss again any concern about the testability of the new LP-WUS signal. Like PBCH, LP-WUS lacks a direct mechanism to determine its detection rate. However, we can indirectly detect LP-WUS by configuring an appropriate PDCCH detection condition such that at the target SNR for LP-WUS guarantees quasi-perfect detection rate in PDCCH. We can clearly tell if a LP-WUS decoding instance was triggered by the UE by simply detecting the associated PDCCH, since the MR monitored the configured PDCCH windows directly.

Observation 15: LP-WUS miss-detection rate can be evaluated indirectly via the associated PDCCH detection rate.

Proposal 13: RAN4 to focus on defining demodulation requirements and its associated parameters since testability is clear.

Finally, another aspect that has being brought during this work item is the LP-WUS false alarm rate (FAR), but we prefer to leave this up to UE implementation. First, FAR is not a new concept, since it already happens in PDCCH detection, and it’s not a phenomenon that can be tested by the network or by TE vendor. Second, it’s up to UE implementation to minimize FAR in order to maximize the power saving gains of this feature.

# Conclusion

In this contribution, we continue the discussion on the simulation assumption.

Observation 1: LP-WUS detection is revelant for both Idle/Inactive and Connected Mode.

Observation 2: For LP-WUS detection, the LP-WUR receiver can be of OOK type and of OFDM type.

Observation 3: New OOK modulation and new OFDM overlaid sequence detection has been specified for LP-WUS detection, which are novel in the context of TS 38.101-4.

Observation 4: For OOK envelope detection, LP-WUS coding aspects have both an information payload dimension and a code rate dimension.

Observation 5: For OFDM detection, LP-WUS can be transmitted in several ways based on the configured M-factor, which determines a maximum number of candidate overlaid sequences per OOK ON chip.

Observation 6: During RAN4#115 it was agreed to consider both OOK and OFDM receivers, and both waveforms OOK-1 and OOK-4.

Observation 7: There is stronger interest in deploying the LP-WUS feature on a limited number of frequencies, and seems more cost effective that frequencies are in FR1.

Proposal 1: Prioritize FR1 study. RAN4 to decide about FR2 after requirements after meaningful progress granted for FR1.

Observation 8: RAN1 design supports LP-WUS length in actual OFDM symbols used larger than one slot, hence also non-consecutive OFDM symbols at least across multiple slots.

Proposal 2: Consider a SCS of 15kHz for a channel bandwidth of 10MHz FDD, and a SCS of 30kHz for a channel bandwidth of 40MHz TDD. Do not consider FR2 120kHz for the moment.

Observation 9: For best LP-WUS/WUR operation, expectations are that UE should operate in rather low mobility conditions.

Proposal 3: Consider 1Tx/1Rx as antenna configuration.

Proposal 4: Define TDLA30-10 as propagation channel.

Proposal 5: Consider defining requirements using a frequency offset of 5ppm for the OFDM LP-WUS receiver, and 20ppm for the OOK LP-WUS receiver.

Proposal 6: Consider defining requirements using a residual time offset T equal to 5us for OOK-1, T= 2us for OOK-4 with M=2, T= 1us for OOK-4 with M=4.

Observation 10: Current demodulation requirements in TS 38.101-4 consider ideal time and frequency synchronization since the effect on required SNR is minimal.

Proposal 7: RAN4 to assess how practical frequency and time offsets affect LP-WUS miss-detection rate, and determine if demodulation requirements can be defined assuming ideal synchronization to simplify testing.

Observation 11: For the OFDM LP-WUR, RAN1 has defined the maximum number of candidates overlaid sequences to carry LP-WUS information per OOK ON chip for one cell, with 16 candidates for M=1, 8 candidates for M=2 and 4 candidates for M=4, with corresponding mapping between roots and cyclic shifts.

Proposal 8: For the OFDM LP-WUR, consider at most one test per modulation factor M. FFS on overlaid sequence configuration details.

Observation 12: For the OOK LP-WUR, RAN1 has defined two types of bit encoding schemes.

Proposal 9: For the OOK LP-WUR, limit the coding scheme tests to two: one for 1 or 2 information bits, and one for the maximum number of information bits, which is 5.

Proposal 10: Consider a total of L >= 16 coded bits for 1-2 information bits, and a total of L=32 bits for 5 information bits.

Observation 13: The noise figure difference implied by Type-1 and Type-2 UEs only affects the REFSENS test in the RF domain. It does not affect the target SNR perceived by the UE for demodulation requirements. Only a difference in the received waveforms are relevant.

Proposal 11: Do not define separate demodulation requirements for Type-1 and Type-2 LP-WUR capable UEs if those are not tied to Envelope vs Sequence Detection.

Observation 14: RAN4 demodulation requirements are not related to synchronization performance, but only with LP-WUS detection rate.

Proposal 12: Do not define LP-SS based synchronization requirements.

Observation 15: LP-WUS miss-detection rate can be evaluated indirectly via the associated PDCCH detection rate.

Proposal 13: RAN4 to focus on defining demodulation requirements and its associated parameters since testability is clear.

# References

[1] RP-241824, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), vivo / NTT DOCOMO, RAN #105, Melbourne, Australia, September 9-12, 2024