3GPP TSG-RAN4 Meeting #116	R4-2509580

# Bengaluru, India, August 25th – 29th, 2025

Source:	LG Electronics

Title: 	FWA MPR for intra-band non-contiguous CA

Agenda Item:	7.1.2.1.2

Document for:	Discussion and Approval

Introduction

RAN4 agreed PC1.5 MPR for the following cases (green colour) except for FWA of PC1.5 intra-band NC CA.

|  |  | Handheld | FWA |
| --- | --- | --- | --- |
| PC1.5 intra-band C CA (2Tx) | Contiguous RB allocation | Agreed | Agreed |
|  | Non-contiguous RB allocation | Agreed | Agreed |
| PC1.5 intra-band NC CA (dualPA) |  | Agreed | Not yet |

For FWA of PC1.5 intra-band NC CA, MPR should be defined.

Discussion

For PC1.5 intra-band NC CA of  FWA with indicating dualPA, based on [1],

Proposal 1: For PC1.5 intra-band NC CA of FWA with indicating dualPa, it is proposed to define MPR in Table 2-1 to meet -13dBm/MHz and -30dBm/MHz respectively.

Table 2-1 : PC1.5 FWA MPR for intra-band NC CA with 2x26dBm+2LO

| B | MPR (dB) |  |
| --- | --- | --- |
|  | -13dBm/MHz | -30dBm/MHz |
| 0 ≤ B <0.54 | ≤ 8.5 | ≤ 9.0 |
| 0.54 ≤ B < 2.16 | ≤ 8.0 | ≤ 9.0 |
| 2.16 ≤ B < 5.4 | ≤ 7.5 | ≤ 9.0 |
| 5.4 ≤ B | ≤ 7.0 | ≤ 9.0 |

Conclusions

This document discusses MPR of FWA device for PC1.5 intra-band NC CA.

Proposal 1: For PC1.5 intra-band NC CA of FWA with indicating dualPa, it is proposed to define MPR in Table 2-1 to meet -13dBm/MHz and -30dBm/MHz respectively.

Table 2-1 : PC1.5 FWA MPR for intra-band NC CA with 2x26dBm+2LO

| B | MPR (dB) |  |
| --- | --- | --- |
|  | -13dBm/MHz | -30dBm/MHz |
| 0 ≤ B <0.54 | ≤ 8.5 | ≤ 9.0 |
| 0.54 ≤ B < 2.16 | ≤ 8.0 | ≤ 9.0 |
| 2.16 ≤ B < 5.4 | ≤ 7.5 | ≤ 9.0 |
| 5.4 ≤ B | ≤ 7.0 | ≤ 9.0 |

References

[1] R4-2502091, HPUE for intra-band UL CA, LG Electronics, TSG-RAN4 Meeting #114, Feb 2025

Annex A: Draft CR

< START OF CHANGE #1 >

#### 6.2A.2.2.1	MPR to meet -30dBm/MHz

##### 6.2A.2.2.1.1		PC3 with indicating dualPA-Architecture supported

MPR in this clause is for intra-band non-contiguous CA power class 3 for UEs indicating IE dualPA-Architecture supported. The allowed maximum output power reduction is defined as:

MPR=MAWhere MA is defined as follows

MA = 	15; 	0 ≤ B < 1.08

14.5; 	1.08 ≤ B < 2.16

13.5; 	2.16 ≤ B < 3.24

12.5;       3.24 ≤ B < 5.04

11.5; 	5.04≤ B < 10.08

10.5; 	10.08 ≤ B < 16.38

10;        16.38 ≤ B < 21.78

9; 	      21.78 ≤ B

##### 6.2A.2.2.1.2		PC2 with indicating dualPA-Architecture supported

MPR in this clause is for intra-band non-contiguous CA power class 2 for UEs indicating IE dualPA-Architecture supported. The allowed maximum output power reduction is defined as:

MPR=MAWhere MA is defined as follows

MA = 	15.5; 	0 ≤ B < 1.44

15.0; 	1.44 ≤ B < 2.88

14.0;    2.88 ≤ B < 5.76

12.0; 	5.76 ≤ B < 10.8

10.5; 	10.8 ≤ B < 23.04

9.0; 	 23.04 ≤ B

##### 6.2A.2.2.1.3		PC3 without indicating dualPA-Architecture supported

MPR in this clause is for intra-band non-contiguous CA power class 3 for UEs without indicating IE dualPA-Architecture supported. The allowed maximum output power reduction is defined as:

MPR=MAWhere MA is defined as follows

MA = 	17.5; 	0 ≤ B < 1.08

17.0; 	1.08 ≤ B < 2.16

16.5; 	2.16 ≤ B < 3.24

16;      3.24 ≤ B < 5.04

15; 	5.04≤ B < 10.08

14.5; 	10.08 ≤ B < 36

10;      36 ≤ B < 56.88

9; 	56.88 ≤ B

##### 6.2A.2.2.1.4		PC2 without indicating dualPA-Architecture supported

MPR in this clause is for intra-band non-contiguous CA power class 2 for UEs without indicating IE dualPA-Architecture supported. The allowed maximum output power reduction is defined as:

MPR=MAWhere MA is defined as follows

MA = 	19.5; 	0 ≤ B < 1.08

19; 	1.08 ≤ B < 2.16

18; 	2.16 ≤ B < 5.04

16.5; 	5.04≤ B < 10.08

16; 	10.08 ≤ B < 36

12;      36 ≤ B < 56.88

10.5; 	56.88 ≤ B

##### 6.2A.2.2.1.5		PC1.5 with indicating dualPA-Architecture supported

MPR in this clause is for intra-band non-contiguous CA power class 1.5 for UEs with indicating IE dualPA-Architecture supported. The allowed maximum output power reduction is defined as:

MPR= MA,total , where MA,total is defined as follows

MA,total= MA + MA,delta , where MA equals to MA defined in sub clause 6.2A.2.2.1.2 and MA,delta is [1]dB for Handheld UE.

#### 6.2A.2.2.2	MPR to meet -13dBm/MHz

##### 6.2A.2.2.2.1		PC3 with indicating dualPA-Architecture supported

MPR in this clause is for intra-band non-contiguous CA power class 3 for UEs indicating IE dualPA-Architecture supported. The allowed maximum output power reduction is defined as:

MPR=MA

Where MA is defined as follows

MA = 	9	;	 0 ≤ B < 0.54

8	;	 0.54 ≤ B < 1.08

7	; 	1.08 ≤ B < 2.16

6.5	; 	2.16 ≤ B < 3.24

5.5	; 	3.24 ≤ B < 5.4

4	; 	5.4 ≤ B

##### 6.2A.2.2.2.2		PC2 with indicating dualPA-Architecture supported

MPR in this clause is for intra-band non-contiguous CA power class 2 for UEs indicating IE dualPA-Architecture supported. The allowed maximum output power reduction is defined as:

MPR=MA

Where MA is defined as follows

MA = 	9	;	 0 ≤ B < 0.54

8	;	 0.54 ≤ B < 1.08

7	; 	1.08 ≤ B < 2.16

6.5	; 	2.16 ≤ B < 3.24

6	; 	3.24 ≤ B < 5.4

5.5	; 	5.4 ≤ B ≤ 10.8

4	; 	10.8 < B

##### 6.2A.2.2.2.3		PC3 without indicating dualPA-Architecture supported

MPR in this clause is for intra-band non-contiguous CA power class 3 for UEs without indicating IE dualPA-Architecture supported. The allowed maximum output power reduction is defined as:

MPR=MAWhere MA is defined as follows

MA = 	11; 	0 ≤ B < 1.08

10.5; 	1.08 ≤ B < 2.16

10; 	2.16 ≤ B < 3.24

9.5; 	3.24≤ B < 5.04

8.5; 	5.04 ≤ B < 10.08

7.5;      10.08 ≤ B < 36

7; 	36 ≤ B

##### 6.2A.2.2.2.4		PC2 without indicating dualPA-Architecture supported

MPR in this clause is for intra-band non-contiguous CA power class 2 for UEs without indicating IE dualPA-Architecture supported. The allowed maximum output power reduction is defined as:

MPR=MAWhere MA is defined as follows

MA = 	14; 	0 ≤ B < 1.08

12; 	1.08 ≤ B < 2.16

11.5; 	2.16 ≤ B < 3.24

11; 	3.24≤ B < 5.04

9.5; 	5.04 ≤ B < 10.08

8.5;      10.08 ≤ B < 36

6.5; 	36 ≤ B

##### 6.2A.2.2.2.5		PC1.5 with indicating dualPA-Architecture supported

MPR in this clause is for intra-band non-contiguous CA power class 1.5 for UEs with indicating IE dualPA-Architecture supported. The allowed maximum output power reduction is defined as:

MPR= MA,total , where MA,total is defined as follows

MA,total= MA + MA,delta , where MA equals to MA defined in sub clause 6.2A.2.2.2.2 and MA,delta is [1]dB for Handheld UE.

< END OF CHANGE #1 >