3GPP TSG-RAN WG4 Meeting #116	R4-2509611

# Bengaluru, India, August 25 – 29, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 5774 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (Netw_Energy_NR-Perf) Correction of SSB-less SCell activation TC - R19 CatA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | Netw_Energy_NR-Perf |  |  |  |  |  | Date: |  |  | 2025-8-15 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the SSB-less SCell activation TC 6.5.3.16 and TC 4.5.3.12, we observed some mistakes on the SCell configuration based on RAN5 TT calculation: - Io (SCell) for Configurations 1 and 2: The spec mentions different values for both TCs: - TC 6.5.3.16 (Cell2): -60.02 dBm - TC 4.5.3.12(Cell3): -57.02 dBm - Based on our calculations, -57.02 dBm seems to be the correct value for both. - SS-RP Value (Configuration 3): The spec mentions different values for both TCs: - TC 6.5.3.16(Cell1): -93 dBm - TC 4.5.3.12(Cell2): -95 dBm - However, as per our analysis, the SS-RP value should be -93.99~-94 dBm in both cases. - The Noc value used differs between the two TCs: - TC 6.5.3.16: -103 - TC 4.5.3.12: -104 - Based on our calculations, Noc = -104 should also be used for TC 6.5.3.16 |  |  |  |  |  |  |  |  |
| Summary of change: |  | In TC 6.5.3.16, the following parameters for the SCell configuration are corrected: - Io (SCell) for Configurations 1 and 2: changed to -57.02 dBm. - SS-RSRP value (Configuration 3): changed to 94dBm. - Noc value: changed to -104. In TC 4.5.3.12, SS-RSRP value (Configuration 3) is changed to 94dBm. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test cases are not completely correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.6.5.3.16 and A.4.5.3.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### <Start of Change 1>

#### A.4.5.3.12	Inter-band SSB-less Scell activation using A-TRS

##### A.4.5.3.12.1	Test Purpose and Environment

The purpose of this test is to verify the SSB less SCell activation delay is within the requirements stated in clause 8.3.2.

The supported test configurations are shown in table A.4.5.3.12.1-1 below. The test parameters are given in tables A.4.5.3.12.1-2 and cell-specific parameters in A.4.5.3.12.1-3 below. The test consists of two successive time periods, with duration of T1 and T2, respectively. There are three carriers, E-UTRA has one cell, NR has two cells. All cells have constant signal levels throughout the test. Before the test starts the UE is connected to Cell 1 (PCell) on E-UTRA and Cell 2 (PSCell) on NR but is not aware of Cell 3 (SCell) on NR. The UE is monitoring the PCell and PSCell. The UE shall be continuously scheduled in the PCell and PSCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which the SCell (Cell 3) becomes added. The configuration for the SCell (Cell 3) is not provided with SSB configuration (absoluteFrequencySSB) in the SCell FrequencyInfoDL nor SMTC configuration for the SCell and PSCell (Cell 2) is indicated as reference cell by higherlayer parameter referenceCell-r18. Cell 3 is configured with aperiodic-TRS as shown in table A.4.5.3.12.1-3. The RS(s) of the Cell 3 is QCL-TypeA with TRS(s) of the Cell 3, and the TRS(s) of the Cell 3 is QCL-TypeC with SSB(s) of Cell 2.

The test equipment sends a MAC message for activation of the SCell and triggering the aperiodic CSI-RS for fast SCell activation. The point in time at which the MAC message is received at the UE antenna connector, in a slot # denoted m, defines the start of time period T2.

The UE shall be able to report valid CSI in PSCell for the activated SCell at latest in slot m+THARQ+Tactivation_time+TCSI_ReportingNR slot length, as defined in clause 8.3. The UE shall start reporting CSI in PSCell after at least one CSI-RS transmission occasion for channel measurement and reporting after slot (m+k) and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PSCell interruption due to activation of SCell shall occur in between slot m+1+THARQNR slot length to slot m+1+THARQ+3 ms+Tfirst_ATRSNR slot length+Ninterruptionas defined in clause 8.3. Any E-UTRA PCell interruption due to activation of SCell shall occur in between subframe m1+1+THARQEUTRA slot length to subframe m2+1+THARQ+3 ms+Tfirst_ATRSEUTRA slot length+Ninterruption  where m1 and m2 are the index of the first and last subframe of E-UTRA PCell which overlaps with slot m.

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PSCell during activation and deactivation of Scell, respectively.

The test equipment verifies the activation time by counting the slots from the time when the Scell activation command is sent until a CSI report with other than CQI index 0 is received.

Table A.4.5.3.12.1-1: Inter-band SSB less SCell SCell activation in FR1 supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD, NR 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode |
| 2 | LTE FDD, NR 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode |
| 3 | LTE FDD, NR 30 kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode |
| 4 | LTE TDD, NR 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode |
| 5 | LTE TDD, NR 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode |
| 6 | LTE TDD, NR 30 kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode |
| NOTE 1: The UE is only required to be tested in one of the supported test configurations NOTE 2: The UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration, |  |

Table A.4.5.3.12.1-2: General test parameters for Inter-band SSB less SCell SCell activation in FR1

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1,2,3 | One E-UTRAN radio channel (1) and two NR radio channel (2,3) are used for this test |
| Active PCell |  | Cell 1 | Primary cell on E-UTRAN RF channel number 1. As specified in clause A.3.7.2.1 |
| Active PSCell |  | Cell 2 | Primary secondary cell on NR RF channel number 2. |
| Configured deactivated SCell |  | Cell 3 | Configured deactivated secondary cell on NR RF channel number 3 |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| Cell-individual offset for cells on E-UTRA RF channel number | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on NR channel number | dB | 0 | Individual offset for cells on secondary component carrier. |
| Cell 3 timing offset to Cell 2 | s | Length of CP of Cell 3 |  |
| Time alignment error between Cell 3 and Cell 2 | s | 3 | The value of time alignment error depends upon the type of carrier aggregation. |
| T1 | s | 7 | During this time the PSCell shall be known and the SCell is configured and detected. |
| T2 | s | 1 | During this time the UE shall activate the SCell. |
| THARQ | ms | k1 NR slot length | k1 is a number of slots indicated by the PDSCH-to-HARQ_feedback timing indicator field in a corresponding DCI format or provided by dl-DataToUL-ACK if the PDSCH-to-HARQ feedback timing field is not present in the DCI format, the value is defined in TS 38.213 [3] |
| TCSI_Reporting | ms | 15 | the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting (clause 5.2.2.5 in TS 38.214 [26]) and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [2] |
| k | slot |  | As specified in clause 4.3 of TS 38.213 [3] |

Table A.4.5.3.12-3: Cell specific test parameters for Inter-band SSB less SCell activation in FR1

| Parameter |  | Unit | Cell 2 |  | Cell 3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 |  |  | T2 |
| SSB ARFCN |  |  | freq1 |  | Not applicable |  |  |  |
| Duplex mode | Config 1,4 |  | FDD |  |  |  |  |  |
|  | Config 2,3,5,6 |  | TDD |  |  |  |  |  |
| TDD configuration | Config 1,4 |  | Not Applicable |  |  |  |  |  |
|  | Config 2,5 |  | TDDConf.1.1 |  |  |  |  |  |
|  | Config 3,6 |  | TDDConf.2.1 |  |  |  |  |  |
| Reference Channel |  |  | N/A |  |  |  | Cell 2 |  |
| BWchannel | Config 1,4 | MHz | Note 7 |  |  |  |  |  |
|  | Config 2,5 |  | Note 7 |  |  |  |  |  |
|  | Config 3,6 |  | Note 7 |  |  |  |  |  |
| BWoccupied | Config 1,4 | RB | 52 Note 5 |  |  |  |  |  |
|  | Config 2,5 |  | 52 Note 5 |  |  |  |  |  |
|  | Config 3,6 |  | 106 Note 6 |  |  |  |  |  |
| DL initial BWP configuration | Config 1, 2, 3, 4, 5, 6 |  | DLBWP.0.1 |  |  |  |  |  |
| DL dedicated BWP configuration | Config 1, 2, 3, 4, 5, 6 |  | DLBWP.1.1 |  |  |  |  |  |
| UL initial BWP configuration | Config 1, 2, 3, 4, 5, 6 |  | ULBWP.0.1 |  |  |  |  |  |
| UL dedicated BWP configuration | Config 1, 2, 3, 4, 5, 6 |  | ULBWP.1.1 |  |  |  |  |  |
| TCI state |  |  | TCI.State.0 |  |  |  | TCI.State.0 Note 8 |  |
| Aperiodic CSI-RS for SCell activation | Config 1,4 |  | N/A |  |  | TRS.1.3 FDD |  |  |
|  | Config 2,5 |  | N/A |  |  | TRS.1.3 TDD |  |  |
|  | Config 3,6 |  | N/A |  |  | TRS.1.4 TDD |  |  |
| gapBetweenBursts |  | Slot | N/A |  |  |  |  |  |
| DRX Cycle |  | ms | Not Applicable |  |  |  |  |  |
| PDSCH Reference | Config 1,4 |  | SR.1.1 FDD |  | SR.1.1 FDD |  |  |  |
| measurement channel | Config 2,5 |  | SR.1.1 TDD |  | SR.1.1 TDD |  |  |  |
|  | Config 3,6 |  | SR.2.1 TDD |  | SR.2.1 TDD |  |  |  |
| RMSI CORESET | Config 1,4 |  | CR.1.1 FDD |  | CR.1.1 FDD |  |  |  |
| Reference Channel | Config 2,5 |  | CR.1.1 TDD |  | CR.1.1 TDD |  |  |  |
|  | Config 3,6 |  | CR.2.1 TDD |  | CR.2.1 TDD |  |  |  |
| RMC CORESET | Config 1,4 |  | CCR.1.1 FDD |  | CCR.1.1 FDD |  |  |  |
| Reference Channel | Config 2,5 |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  |  |  |
|  | Config 3,6 |  | CCR.2.1 TDD |  | CCR.2.1 TDD |  |  |  |
| TRS configuration | Config 1,4 |  | TRS.1.1 FDD |  | TRS.1.1 FDD |  |  |  |
|  | Config 2,5 |  | TRS.1.1 TDD |  | TRS.1.1 TDD |  |  |  |
|  | Config 3,6 |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  |  |  |
| OCNG Patterns | Config 1,2,4,5 |  | OP.1 Note 5 |  |  |  |  |  |
|  | Config 3,6 |  | OP.1 Note 6 |  |  |  |  |  |
| SMTC configuration |  |  | SMTC.1 |  |  |  | Not Applicable |  |
| SSB configuration | Config 1,2,4,5 |  | SSB.1 FR1 |  |  |  | Not Applicable |  |
|  | Config 3,6 |  | SSB.2 FR1 |  |  |  | Not Applicable |  |
| CSI-RS configuration for CSI reporting | Config 1,4 |  | CSI-RS.1.1 FDD |  |  |  |  |  |
|  | Config 2,5 |  | CSI-RS.1.1 TDD |  |  |  |  |  |
|  | Config 3,6 |  | CSI-RS.2.1 TDD |  |  |  |  |  |
| PDSCH/PDCCH | Config 1,2,4,5 | kHz | 15 |  |  |  |  |  |
| subcarrier spacing | Config 3,6 |  | 30 |  |  |  |  |  |
| reportConfigType | Config 1-6 |  | periodic |  |  |  |  |  |
| reportQuantity | Config 1-6 |  | cri-RI-PMI-CQI |  |  |  |  |  |
| CSI reporting periodicity | Config 1,2,4,5 | slot | 5 |  |  | N/A |  |  |
| CSI reporting offset | Config 3,6 | slot | 10 |  |  | N/A |  |  |
|  | Config 1,2,4,5 |  | 2 |  |  | N/A |  |  |
|  | Config 3,6 |  | 4 |  |  | N/A |  |  |
| EPRE ratio of PSS to SSS |  |  | 0 |  |  |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2,4,5 | dBm/SCS | -107 |  |  |  |  |  |
|  | Config 3,6 |  | -104 |  |  |  |  |  |
|  |  | dB | 10 |  |  |  | 22 |  |
|  |  | dB | 10 |  |  |  | 22 |  |
| SS-RSRPNote3 | Config 1,2,4,5 | dBm/SCS | -97 |  |  | NA |  |  |
|  | Config 3,6 |  | -94 |  |  | NA |  |  |
| SCH_RP Note 3 |  | dBm/15 kHz | -97 |  |  |  | NA |  |
| Propagation condition |  | - | AWGN |  |  |  |  |  |
| IoNote3 | Config 1,2,4,5 | dBm/ 9.36 MHz | -68.63 |  |  |  | -57.02 |  |
|  | Config 3,6 | dBm/ 38.16 MHz | -62.54 |  |  |  | -50.92 |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled within BWoccupied. NOTE 3: SS-RSRP, Io and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2.] NOTE 5: All UL/DL transmission shall be confined within BWoccupied (i.e. 10 MHz, 52 RBs) from FC,low, and Io is independent of the BWchannel configured. NOTE 6: All UL/DL transmission shall be confined within BWoccupied (i.e. 40 MHz, 106 RBs) from FC,low, and Io is independent of the BWchannel configured. NOTE 7: NRB,c. is derived from Table 5.3.2-1 in TS 38.101-1[18] with configured BWchannel. NOTE 8: The SSB in referenceSignal in the TCI state is configured as the SSB in Cell 2. |  |  |  |  |  |  |  |  |

##### A.4.5.3.12.2	Test Requirements

During T2 the UE shall send the first CSI report for SCell in the first available uplink resource after slot (m+k). UE is allowed to postpone CSI report to next available uplink resource if an available uplink resource is subject to interruption.  Whether CSI report in slot (m+k) was interrupted is checked by monitoring ACK/NACK sent in PCell in slot (m+k).

During T2 the UE shall start sending CSI reports for SCell with non-zero CQI index at latest in a slot m+THARQ+Tactivtion_time+TCSI_ReportingNR slot length, Tactivation_time = Tfirst_ATRS + Tgap + TATRS + 5 ms, as defined in clause 8.3.

During T2 interruption of PSCell during SCell activation shall not happen outside the slot m+1+THARQNR slot length to  m+1+THARQ+3 ms+TXNR slot length+Ninterruption, and interruption of E-UTRA PCell during SCell activation shall not happen outside the subframe m1+1+THARQEUTRA slot length to subframe m2+1+THARQ+3 ms+TXEUTRA slot length+Ninterruption, as defined in clause 8.3.

The interruption of PSCell shall not be more than the values specified for EN-DC in clause 8.2.1.2.19.

All of the above test requirements shall be fulfilled in order for the observed SCell activation delay and SCell deactivation delay to be counted as correct. The rate of correct observed SCell activation delay during repeated tests shall be at least 90%.

NOTE:	During T2 if there are no uplink resources for reporting the valid CSI in a slot m+THARQ+Tactivtion_time+TCSI_ReportingNR slot length as defined in clause 8.3 then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

### <End of Change 1>

### <Start of Change 2>

#### A.6.5.3.16	Inter-band SSB-less SCell Activation based on A-TRS

##### A.6.5.3.16.1	Test Purpose and Environment

The purpose of this test is to verify that the inter-band SSB-less SCell activation times are within the requirements stated in clause 8.3.2.

The supported test configurations are shown in table A.6.5.3.16.1-1A and A.6.5.3.16.1-1B below. The test parameters are given in Tables A.6.5.3.16.1-2 and cell-specific parameters in tables A.6.5.3.16.1-3 below. The test consists of two successive time periods, with duration of T1and T2, respectively. There are two NR carriers, each with one cell. Both cells have constant signal levels throughout the test. Before the test starts the UE is connected to PCell Cell 1, but is not aware of Cell 2. The UE is only monitoring the PCC. The UE shall be continuously scheduled in the PCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which the SCell (Cell 2) becomes configured on radio channel 2. The test equipment sends a MAC message for activation of the SCell and triggering the aperiodic CSI-RS for SCell activation.

The point in time at which the MAC message is received at the UE antenna connector, in slot # denoted n (where n mod 20=1), defines the start of time period T2. The UE shall be able to report valid CSI in PCell for the activated SCell at latest in slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot length, as defined in clause 8.3. The UE shall start reporting CSI in PCell after at least one CSI-RS transmission occasion for channel measurement and reporting after slot n+THARQ+3 msNR slot length and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PCell interruption due to activation of SCell shall occur in the slot n+1+THARQNR slot length to n+1+THARQ+3 ms+TXNR slot length+Ninterruption, as defined in clause 8.3, where Ninterruption is the interruption length given in clause 8.2.

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during activation and deactivation of SCell, respectively.

The test equipment verifies the activation time by counting the slots from the time when the SCell activation command is sent until a CSI report with other than CQI index 0 is received.

Table A.6.5.3.16.1-1A: Inter-band SSB-less SCell Activation based on A-TRS for NR PCell

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode |
| NOTE 1: The UE is only required to be tested in one of the supported test configurations NOTE 2: The UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration, |  |

Table A.6.5.3.16.1-1B: Inter-band SSB-less SCell Activation based on A-TRS for NR SCell

| ConfigSCell | Description |
| --- | --- |
| 1 | NR 15 kHz SCS, ≥10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SCS, ≥10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SCS, ≥40 MHz bandwidth, TDD duplex mode |
| NOTE 1: The UE is only required to be tested in one of the supported test configurations NOTE 2: The UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration, |  |

Table A.6.5.3.16.1-2: General test parameters for Inter-band SSB-less SCell Activation based on A-TRS

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1,2 | Two NR radio channel (1, 2) are used for this test, where NR radio chanel 1 and 2 are in different NR bands. |
| Active PCell |  | Cell 1 | Primary cell on NR RF channel number 1. |
| Configured deactivated SCell |  | Cell 2 | Configured deactivated secondary cell on NR RF channel number 2 |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| Cell 2 timing offset to cell1 | s | Length of CP of Cell 2 |  |
| T1 | s | 7 | During this time the PCell shall be known and the SCell configured |
| T2 | s | 1 | During this time the UE shall activate the SCell. |
| THARQ | ms | Config 1: 2 Config 2: 3 Config 3: 2.5 | k1×NR slot length k1 is a number of slots and is indicated by the PDSCH-to-HARQ-timing-indicator field in the DCI format, if present, or provided by dl-DataToUL-ACK, the value of k should be the minimum value defined in TS 38.213 [3] that will meet the timing constraints of this test case. |
| TCSI_Reporting | ms | 15 | the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting (clause 5.2.2.5 in TS 38.214 [26]) and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [2] |

Table A.6.5.3.16.1-3: PCell test parameters for Inter-band SSB-less SCell Activation based on A-TRS

| Parameter |  | Unit | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| Duplex mode | Config 1 |  | FDD |  |
|  | Config 2,3 |  | TDD |  |
| TDD configuration | Config 1 |  | Not applicable |  |
|  | Config 2 |  | TDDConf.1.1 |  |
|  | Config 3 |  | TDDConf.2.1 |  |
| BWchannel | Config 1,2 | MHz | Note 7 |  |
|  | Config 3 |  | Note 7 |  |
| BWoccupied | Config 1,2 | RB | 52 Note 5 |  |
|  | Config 3 |  | 106 Note 6 |  |
| Initial BWP configuration |  |  | DLBWP.0.1 |  |
| TCI state |  |  | TCI.State.0 |  |
| TRS Configuration | Config 1 |  | TRS.1.1 FDD |  |
|  | Config 2 |  | TRS.1.1 TDD |  |
|  | Config 3 |  | TRS.1.2 TDD |  |
| PDSCH Reference measurement channel | Config 1 |  | SR.1.1 FDD |  |
|  | Config 2 |  | SR.1.1 TDD |  |
|  | Config 3 |  | SR.2.1 TDD |  |
| Dedicated CORESET parameters | Config 1 |  | CCR.1.1 FDD |  |
|  | Config 2 |  | CCR.1.1 TDD |  |
|  | Config 3 |  | CCR.2.1 TDD |  |
| RMSI CORESET parameters | Config 1 |  | CR.1.1 FDD |  |
|  | Config 2 |  | CR.1.1 TDD |  |
|  | Config 3 |  | CR.2.1 TDD |  |
| OCNG Patterns | Config 1,2 |  | OP.1Note 5 |  |
|  | Config 3, |  | OP.1 Note 6 |  |
| SSB Configuration | Config 1,2 |  | SSB.1 FR1 |  |
|  | Config 3 |  | SSB.2 FR1 |  |
| CSI-RS configuration for CSI reporting (Note 8) | Config 1 |  | CSI-RS.1.1 FDD |  |
|  | Config 2 |  | CSI-RS.1.1 TDD |  |
|  | Config 3 |  | CSI-RS.2.1 TDD |  |
| SMTC configuration |  |  | SMTC.1 |  |
| reportConfigType |  |  | periodic |  |
| reportQuantity |  |  | cri-RI-PMI-CQI |  |
| CSI reporting periodicity | Config 1,2 | slot | 5 |  |
|  | Config 3 |  | 10 |  |
| CSI reporting offset | Config 1,2 | slot | 3 |  |
|  | Config 3 |  | 5 |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |
| Note2 | Config 1,2 | dBm/SCS | -107 |  |
|  | Config 3 |  | -104 |  |
|  |  | dB | 10 |  |
|  |  | dB | 10 |  |
| SS-RSRPNote3 | Config 1,2 | dBm/SCS | -97 |  |
|  | Config 3 |  | -94 |  |
| SCH_RP Note 3 |  | dBm/15 kHz | -93 |  |
| Io Note3 | Config 1,2 | dBm/ 9.36 MHz | -68.63 |  |
|  | Config 3 | dBm/ 38.16 MHz | -62.54 |  |
| Propagation condition |  | - | AWGN |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled within BWoccupied. NOTE 3: Io, SS-RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. NOTE 5: All UL/DL transmission shall be confined within BWoccupied (i.e. 10 MHz, 52 PRBs) from FC,low, and Io is independent of the BWchannel configured. NOTE 6: All UL/DL transmission shall be confined within BWoccupied (i.e. 40 MHz, 106 PRBs) from FC,low, and Io is independent of the BWchannel configured. NOTE 7: NPRB,c. is derived from Table 5.3.2-1 in TS38.101-1[18] with configured BWchannel. NOTE 8: On top of the reference configurations, CSI-RS offset should be set to meet the CSI reference resource timing definition in Clause 5.2.2.5 in TS 38.214 [26]. |  |  |  |  |

Table A.6.5.3.16.1-4: SCell parameters for Inter-band SSB-less SCell Activation based on A-TRS

| Parameter |  | Unit | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| Duplex mode | ConfigSCell 1 |  | FDD |  |
|  | ConfigSCell 2,3 |  | TDD |  |
| TDD configuration | ConfigSCell 1 |  | Not applicable |  |
|  | ConfigSCell 2 |  | TDDConf.1.1 |  |
|  | ConfigSCell 3 |  | TDDConf.2.1 |  |
| referenceCell | ConfigSCell 1,2,3 |  | Cell 1 |  |
| BWchannel | ConfigSCell 1,2 | MHz | Note 7 |  |
|  | ConfigSCell 3 |  | Note 7 |  |
| BWoccupied | ConfigSCell 1,2 | RB | 52 Note 5 |  |
|  | ConfigSCell 3 |  | 106 Note 6 |  |
| Initial BWP configuration |  |  | DLBWP.0.1 |  |
| TCI state |  |  | TCI.State.0 Note 9 |  |
| TRS Configuration | ConfigSCell 1 |  | TRS.1.1 FDD |  |
|  | ConfigSCell 2 |  | TRS.1.1 TDD |  |
|  | ConfigSCell 3 |  | TRS.1.2 TDD |  |
| PDSCH Reference measurement channel | ConfigSCell 1 |  | - |  |
|  | ConfigSCell 2 |  | - |  |
|  | ConfigSCell 3 |  | - |  |
| Dedicated CORESET parameters | ConfigSCell 1 |  | - |  |
|  | ConfigSCell 2 |  | - |  |
|  | ConfigSCell 3 |  | - |  |
| RMSI CORESET parameters | ConfigSCell 1 |  | - |  |
|  | ConfigSCell 2 |  | - |  |
|  | ConfigSCell 3 |  | - |  |
| OCNG Patterns | ConfigSCell 1,2 |  | OP.1Note 5 |  |
|  | ConfigSCell 3 |  | OP.1 Note 6 |  |
| SSB Configuration | ConfigSCell 1,2,3 |  | Not provided |  |
| Aperiodic CSI-RS for Scell activation | ConfigSCell 1 |  | TRS.1.3 FDD |  |
|  | ConfigSCell 2 |  | TRS.1.3 TDD |  |
|  | ConfigSCell 3 |  | TRS.1.4 TDD |  |
| gapBetweenBursts | ConfigSCell 1,2 | slot | 2 |  |
|  | ConfigSCell 3 | slot | 3 |  |
| CSI-RS configuration for CSI reporting (Note 8) | ConfigSCell 1 |  | CSI-RS.1.1 FDD |  |
|  | ConfigSCell 2 |  | CSI-RS.1.1 TDD |  |
|  | ConfigSCell 3 |  | CSI-RS.2.1 TDD |  |
| SMTC configuration |  |  | Not provided |  |
| reportConfigType |  |  | periodic |  |
| reportQuantity |  |  | cri-RI-PMI-CQI |  |
| CSI reporting periodicity | ConfigSCell 1,2 | slot | - |  |
|  | ConfigSCell 3 |  | - |  |
| CSI reporting offset | ConfigSCell 1,2 | slot | - |  |
|  | ConfigSCell 3 |  | - |  |
| EPRE ratio of PSS to SSS |  | dB | N/A |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |
| Note2 | Config 1,2 | dBm/SCS | -107 |  |
|  | Config 3 |  | -104 |  |
|  |  | dB | 22 |  |
|  |  | dB | 22 |  |
| SS-RSRPNote3 | Config 1,2 | dBm/SCS | - |  |
|  | Config 3 |  | - |  |
| SCH_RP Note 3 |  | dBm/15 kHz | N/A |  |
| Io Note3 | Config 1,2 | dBm/ 9.36 MHz | -57.02 |  |
|  | Config 3 | dBm/ 38.16 MHz | -50.92 |  |
| Propagation condition |  | - | AWGN |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled within BWoccupied. NOTE 3: Io, SS-RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. NOTE 5: All UL/DL transmission shall be confined within BWoccupied (i.e. 10 MHz, 52 PRBs) from FC,low, and Io is independent of the BWchannel configured. NOTE 6: All UL/DL transmission shall be confined within BWoccupied (i.e. 40 MHz, 106 PRBs) from FC,low, and Io is independent of the BWchannel configured. NOTE 7: NPRB,c. is derived from Table 5.3.2-1 in TS38.101-1[18] with configured BWchannel. NOTE 8: On top of the reference configurations, CSI-RS offset should be set to meet the CSI reference resource timing definition in Clause 5.2.2.5 in TS 38.214 [26]. NOTE 9: The SSB in referenceSignal in the TCI state is configured as the SSB in Cell 2. |  |  |  |  |

##### A.6.5.3.16.2	Test Requirements

During T2 the UE shall send the first CSI report for SCell in the first available uplink resource after at least one CSI-RS transmission occasion for channel measurement and reporting after slot (n+1+THARQ+3 msNR slot length). UE is allowed to postpone CSI report to next available UL resource if an available uplink resource is subject to interruption.

During T2 the UE shall start sending CSI reports for SCell with non-zero CQI index at latest in a slot n+THARQ+Tactivtion_time+TCSI_ReportingNR slot length, Tactivation_time = Tfirst_ATRS + Tgap + TATRS + 5 ms, as defined in clause 8.3.2.

During T2 interruption of PCell during SCell activation shall not happen outside the slot n+1+THARQNR slot length to n+1+THARQ+3 ms+TXNR slot length+Ninterruption, as defined in clause 8.3.2.

The interruption on any activated serving cell shall not be more than the values specified for SA in clause 8.2.2.2.2.

All of the above test requirements shall be fulfilled in order for the observed SCell activation delay. The rate of correct observed SCell activation delay during repeated tests shall be at least 90 %.

NOTE:	During T2 if there are no uplink resources for reporting the valid CSI in a slot THARQ+Tactivtion_time+TCSI_ReportingNR slot length as defined in clause 8.3 then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

### <End of Change 2>