3GPP TSG-RAN WG4 Meeting #116	R4-2509614

Bangalore, India, 25-29 August, 2025

Source:	Nokia, Nokia Shanghai Bell

Title:	On performance part of L3 measurement delay reduction

Agenda item:	7.14.5

Document for:	Discussion

Introduction

In this contribution we discuss the RRM performance requirements for FR2-1 SSB based L3 measurement delay reduction, considering CSSF optimization.

Test cases for CSSF optimization

At RAN4#115 meeting, the list of test cases was agreed as below. In this paper, we are discussing the detailed test configurations taking SSB-based intra-frequency measurement without gaps for solution 1 as an example.

In legacy test cases, the measurement delay is verified by configuring event triggered measurement reporting on a neighbour cell. By setting the neighbor cell on the same frequency or different frequency with the active cell, the intra-frequency or inter-frequency measurement delay can be tested e.g. in TS 38.133 clause A.7.6 for intra-frequency measurement on FR2 cell. In these test cases, single carrier frequency is assumed hence no CSSF is applied.

Observation 1: In legacy test cases, the measurement delay is verified by configuring event triggered measurement reporting on a neighbour cell and no CSSF is applied.

When UE is configured to measure one serving CC per FR2 band, the enhanced CSSF shall be derived by counting one serving CC per band. Hence a scenario with at least two carrier frequencies in one band is required to verify the measurement delay with enhanced CSSF. The test case shall include at least one PSCell/PCell, one SCell and a neighbor cell in the same band. Following the legacy test cases, the test case for enhanced CSSF can be designed by configuring event triggered measurement reporting on a neighbour cell on the same frequency with PSCell (in EN-DC) or PCell (in FR2 SA).

Issue 2-2-2: TC list for CSSF optimization solution 1

- Agreement

| TC index | Scenario | CA/DC Operation mode | Configurations and scopes | Company |
| --- | --- | --- | --- | --- |
| 1 | Aspect 1: SSB based Intra-frequency measurement without MG | 1 TC per operation mode: FR1+FR2 CA and EN-DC, UE is only required to pass one of them | - Non-DRX, - Without SSB time index detection - CA mode: FR1 PCC+ 2 FR2 intra-band SCCs, network configures MOs on both FR2 SCCs and indicates to measure one FR2 SCC - EN-DC: LTE PCC + FR2 PSCC + 1 FR2 intra-band SCCs, network configures MOs on both FR2 SCCs and UE by-default to measure one FR2 PSCC | Nokia |
| 2 | Aspect 2: SSB based Inter-frequency measurement without MG | 1 TC per operation mode: FR1+FR2 CA and EN-DC, UE is only required to pass one of them | - Non-DRX, - Without SSB time index detection - CA mode: FR1 PCC+ 2 FR2 intra-band SCCs, network configures MOs on both FR2 SCCs and indicates to measure one FR2 SCC - EN-DC: LTE PCC + FR2 PSCC + 1 FR2 intra-band SCCs, network configures MOs on both FR2 SCCs and UE by-default to measure one FR2 PSCC | Ericsson |
| 3 | Aspect 3: Inter-RAT SSB measurement without MG | 1 TC per operation mode: FR1+FR2 CA and EN-DC, UE is only required to pass one of them | - Non-DRX, - Without SSB time index detection - CA mode: FR1 PCC+ 2 FR2 intra-band SCCs, network configures MOs on both FR2 SCCs and indicates to measure one FR2 SCC - EN-DC: LTE PCC + FR2 PSCC + 1 FR2 intra-band SCCs, network configures MOs on both FR2 SCCs and UE by-default to measure one FR2 PSCC | ZTE |

Note: network indication is up to the other RAN2/4 core part discussion. In case network indication for a specific SCC measurement is replaced by other new solution(e.g., SCC selected by UE implementation), use the new solution for CA mode test.

A scenario with at least two carrier frequencies in one FR2 band is required to verify the measurement delay with enhanced CSSF.

The test cases for enhanced CSSF are designed by configuring event triggered measurement reporting on a neighbour cell on the same frequency with PSCell (in EN-DC) or PCell (in FR2 SA).

If single FR2 band is assumed above, UE will measure PSCell/PCell based on the predefined rule for selecting the serving carrier to be measured on one band. Besides, we further agreed network may indicate the carrier to be measured per band in some cases. A test case is needed to verify the measurement delay with enhanced CSSF in case network indicates the carrier to be measured per band.

As the network indicated carrier only applies when there is no PCC/PSCC on the band. The test case can be designed in FR1+FR2 or FR2 inter-band scenario. The event triggered measurement reporting can be configured to verify the measurement on a neighbor cell on the same frequency of one FR2 SCell.

To define sub-test to verify the measurement delay with enhanced CSSF in case network indicates the carrier to be measured per FR2 band.

In addition, it is now under discussion whether the CSSF solution 1 can be applied to FR1 intra-band scenarios. If this could be agreed, corresponding test cases shall be defined to verify the measurement delay by measuring one SCC per FR1 band.

To define the test cases to verify the measurement delay by measuring one SCC per FR1 band, if this would be agreed in core requirement discussion.

Conclusion

The following Observations and Proposals were made:

Observation 1: In legacy test cases, the measurement delay is verified by configuring event triggered measurement reporting on a neighbour cell and no CSSF is applied.

- A scenario with at least two carrier frequencies in one FR2 band is required to verify the measurement delay with enhanced CSSF.

The test cases for enhanced CSSF are designed by configuring event triggered measurement reporting on a neighbour cell on the same frequency with PSCell (in EN-DC) or PCell (in FR2 SA).

To define sub-test to verify the measurement delay with enhanced CSSF in case network indicates the carrier to be measured per FR2 band.

To define the test cases to verify the measurement delay by measuring one SCC per FR1 band, if this would be agreed in core requirement discussion.