3GPP TSG-RAN WG4 Meeting #116	R4-2509618

Bengaluru, India, 25 August 2025 – 29 August 2025

Source:	Nokia, Nokia Shanghai Bell

Title:	RRM requirements for SBFD operation

Agenda item:	7.21.4.2

Document for:	Discussion

Introduction

In the last RAN4 meeting, the following agreements were made [1]:

Sub-topic 2-1: RRM impacts of SBFD operation Issue 2-1-1b: CSI-RS L1 measurement – puncturing in frequency domain - Agreement - Define relaxed accuracy requirements based on 24 RB for Case 3 following RAN4#114-bis agreement, based on the simulation assumption in Annex. - Further discuss the upper bound of the measurement period due to processing time and its applicability. Issue 2-1-1c: CSI-RS L1 measurement – different symbol types - Agreements - For L1-RSRP and L1-SINR - Measurement period is extended by using R16 NR-U approach. - For RLM/BFD/CBD - Confirm the following for defining the requirements - RAN4 to define requirements based on the assumption that occasions of a single CSI-RS resource are all in the same symbol type. - Existing measurement period apply. Issue 2-1-2: Requirements for CSI-RS based L3 measurement - Agreements - For DUD case - UE measurement requirements would be same as in legacy with the condition that all CSI-RS resources configured for measurement are available in one DL subband of SBFD symbols and/or on non-SBFD DL symbols of the cell transmitting the CSI-RS. - Agreements (Monday online session) - For collision between CSI-RS resource and dynamic UL transmission - existing scheduling restriction apply, i.e. CSI-RS based L3 measurement is prioritized Issue 2-1-3: Requirements for scheduling restriction - Agreements - Do not define new scheduling restriction requirements for transition between UL and DL. - RAN4 to use the existing scheduling restriction requirements for inter-frequency SSB measurement without gap as a starting point, i.e. no new restriction is defined.

Discussion

Requirements for CSI-RS based L1 measurement

CSI-RS puncturing in frequency domain in SBFD symbols

The L1 measurement requirements for the case in which the CSI-RS resource is split across two subbands (DUD) have been discussed in RAN4 for the cases below:

- Case 1: X1 >= X3 and X2 >= X3

- Case 2: X1 >= X3 and X2 < X3

- Case 3: X1 < X3 and X2 < X3, (X1 + X2) >= X3

where X1 and X2 are CSI-RS BW in DL sub-band #1 and sub-band #2, and X3 is the min BW with which the existing accuracy requirements apply (X3=48 for L1-RSRP/L1-SINR and X3=24 for RLM/BFD/CBD).

The following issues related to CSI-RS based L1 measurement were discussed and agreed in RAN4 #114bis and RAN4#115 meetings:

- RAN4#114bis Agreements (adhoc session on Thursday with additions) - Note: the following are for UE that can support simultaneous PDSCH reception on the two DL subbands for DUD - Existing delay (number of samples) requirements and accuracy requirements apply for Case 1 and Case 2 - If UE indicates that CSI processing time is scaled by 2, further discuss whether and how to update the delay requirements - Option 1: Add a lower bound of 8ms on top of existing delay requirements. - Option 2: other - For Case 3 - For L1-RSRP and L1-SINR, existing delay requirements apply and accuracy requirements are relaxed provided that X1 > X4 or X2 > X4, where the accuracy requirements depend on the X4 = 24 PRBs - In the test case, not model the interfering UE. - RAN4#115 Agreement - Define relaxed accuracy requirements based on 24 RB for Case 3 following RAN4#114-bis agreement, based on the simulation assumption in Annex. - Further discuss the upper bound of the measurement period due to processing time and its applicability.

The support for PDSCH reception on two subbands for DUD has been discussed in past RAN1 meetings. Below we include the list of relevant agreements from RAN1. The following has been agreed in R1-2504673:

| 60. NR_duplex_evo | 60-1 | Basic feature for SBFD-aware UE TX/RX/measurement behaviour and procedures | 1. Support of cell-common configuration of time and frequency location of SBFD subbands 2. Support of UL transmission in one UL subband and DL reception in one[/two] DL subband in SBFD symbols 3. Support of link direction determination based on configured/scheduled transmissions/receptions and collision handling 4. Support of UL transmissions or DL receptions across SBFD symbols and non-SBFD symbols in different slots, where the transmissions or receptions are restricted to SBFD symbols or non-SBFD symbols (Configuration 1) 5. Support of separate CSI reports for SBFD and non-SBFD symbols 6. Support of separate PUCCH frequency resource configuration in SBFD symbols and non-SBFD symbols 7. Support of separate UL power control parameters based on unified TCI framework in SBFD symbols and non-SBFD symbols 8. Support of separate PUSCH frequency hopping offset in SBFD symbols and non-SBFD symbols 9. Support of separate SRS resource set configuration in SBFD symbols and non-SBFD symbols FFS: Whether to include two DL subband support in component 2 FFS: other component (which may be separate FG) |
| --- | --- | --- | --- |

For CSI-RS reception processing, RAN1 agreed that:

Agreement (RAN1#117) For a contiguous CSI-RS resource which overlaps with SBFD subband boundaries, only CSI-RS frequency resources within DL usable PRBs are valid for SBFD-aware. - No impact on CSI-RS sequence generation

CSI-RS sequence mapping is applied to CSI-RS resources within DL usable PRBs only (effectively, this is same as the case when the CSI-RS sequence mapped to the RBs outside the DL usable PRBs are punctured)

Agreement RAN1#118b For CSI processing timeline in SBFD symbols to process the CSI-RS across two DL subbands, - Option 3: It is up to UE capability to indicate one of the following: - CSI processing timeline and CPU counting are the same as legacy. Scale the CSI processing timeline Z/Z’ by factor of 2 and keep the same CPU counting as legacy.

In the past RAN4 meetings, we agreed to define relaxed accuracy requirements based on 24PRB for Case 3. It is FFS how to handle the processing time and its application. In our understanding, the accuracy performance was relaxed because some UE vendors were not able to simultaneously measure the CSI-RS resource across 2 DL subbands. According to RAN1 discussion on UE capability, we believe a UE supporting DL reception on two DL subbands shall be able to measure CSI-RS resources on two DL subbands. Hence the relaxed accuracy is under the condition that UE does not support DL reception in two DL subband in SBFD symbols, or in other words UE only supports DL reception in one DL subband in SBFD symbol. We should make it clear that the relaxed accuracy requirements based on 24 RB for Case 3 applies only if UE does not support simultaneous DL reception in two DL subbands in SBFD symbols.

For Case 3, the relaxed accuracy based on 24 PRB was adopted because some UEs were not able to measure and decode the CSI-RS resource in 2 DL subbands.

According to RAN1 discussion on UE capability, a UE supporting DL reception on two DL subbands shall be able to measure CSI-RS resources on two DL subbands.

The relaxed accuracy requirements based on 24 RB for Case 3 applies only if UE does not support DL reception in two DL subbands in SBFD symbols.

Accordingly, if UE only measures the CSI-RS resource in single DL subband, the UE does not need additional CSI processing time for measuring 2 DL subbands hence the lower bound due to CSI processing time is not needed. Therefore, if UE does not support DL reception in two DL subbands in SBFD symbols, the existing measurement delay shall apply i.e. lower bound is not needed.

For Case 3, if UE does not support DL reception in two DL subbands in SBFD symbols, the existing measurement delay shall apply i.e. lower bound is not needed.

On the other hand, some UEs may be able to support simultaneous DL reception in 2 DL subbands. We understood this comprises simultaneous reception of both data and signals i.e. simultaneous measurement shall also be supported. In this case, we can consider the CSI processing time which may be scaled by 2 up to UE capability indication. And a lower bound may be introduced to the existing measurement delay but the accuracy performance shall not be relaxed as the UE is measuring 2 DL subbands and the sum of the usable PRBs exceeds X3 in Case 3. Hence we propose for Case 3, if UE supports simultaneous DL reception in 2 DL subbands, a lower bound (if UE indicates the capability) can be introduced to the existing measurement delay but the accuracy performance shall be based on 48 PRBs.

For Case 3, if UE supports DL reception in 2 DL subbands, a lower bound (if UE indicates the capability) can be introduced to the existing measurement delay but the accuracy performance shall be based on 48 PRBs.

CSI-RS distributed over SBFD and non-SBFD symbols

The RAN4 measurement requirements for CSI-RS L1-RSRP and L1-SINR, and for CSI-RS based RLM/BFD/CBD are based on the periodicity of the CSI-RS resources. In the RAN1 LS response, RAN1 replied that the agreement below is applicable to CSI-RS based L1-RSRP/SINR:

Agreement (RAN1#118bis) For a CSI report associated with periodic/semi-persistent CSI-RS, the valid symbol type for CSI derivation for periodic/semi-persistent CSI-RS resources for the CSI report is explicitly configured. Only CSI-RS transmission occasions within the valid symbol types are used for CSI derivation.

In RAN4 #114b, it has been agreed:

#### Issue 2-1-1a: CSI-RS L1 measurement – collision with UL - Agreements (adhoc session on Thursday) - Dynamically scheduled UL transmission is prioritized over periodic/semi-persistent CSI-RS resource for L1-RSRP/SINR and RLM/BFD/CBD measurement. - Measurement period for periodic/semi-persistent CSI-RS based L1-RSRP/SINR and RLM/BFD/CBD is extended for collision with dynamically scheduled UL transmission following the R16 NR-U approach.

In RAN4 #115, it has been agreed:

- Agreements - For L1-RSRP and L1-SINR - Measurement period is extended by using R16 NR-U approach. - For RLM/BFD/CBD - Confirm the following for defining the requirements - RAN4 to define requirements based on the assumption that occasions of a single CSI-RS resource are all in the same symbol type. - Existing measurement period apply.

Lmax when using NR-U approach for L1 CSI-RS measurement:

In the past RAN4 meeting, it has been agreed to extend the measurement period of CSI-RS based L1-RSRP and L1-SINR based on the NR-U approach.

The NR-U approach consists of extending the measurement period by the number of samples in which the reference signal is not available at the UE. In this case, the clear channel assessment (CCA) is performed by the gNB, and on the UE side, there is an uncertainty of whether the reference signal has been sent and received with very low power, or whether the reference signal has not been transmitted due to CCA failure. In order to avoid extending the measurement period indefinitely, RAN4 has agreed that the measurement period would be increased up to a maximum number of failures, referred to as Lmax in different agreements and tables in TS 38.133.

In NR-U, the UE is not aware, in advance, of the number CCA failure at the gNB side during a measurement period. Lmax has been introduced to avoid that the UE extends the measurement period indefinitely.

In SBFD, RAN4 has agreed that the L1-RSRP and L1-SINR would also be extended to consider that the reference signals could overlap with symbols of different type. However, the situation is different from the NR-U case, because in SBFD, both the UE and the gNB are aware of the reference signal and measurement configurations, and the time location of the SBFD subbands. In SBFD, there is no uncertainty about the reference signal occasions that overlap with symbols of different types.

In SBFD, both the UE and the gNB are aware of the reference signal configuration and aware of the frame structure, considering the time location of SBFD subbands. There is no uncertainty about the reference signal occasions that overlap with symbols of different types.

Therefore, our proposal is to extend the measurement period of L1-RSRP and L1-SINR based on the number of samples that overlap with symbols other than the valid symbol type. However, we do not see a reason for defining a maximum number of overlapping symbols.

Do not define Lmax values in the extension of the L1-RSRP and L1-SINR measurement periods.

RACH requirements

In RAN4 meeting #114b, the following was agreed:

Issue 2-1-4: Requirements for RACH requirements - Agreements - Discuss whether and how RACH requirements are to be updated for SBFD operation based on RAN2 agreement.

In TS 38.133, the correct behaviour when not receiving Random Access Response is:

The UE shall again perform the Random Access Resource selection procedure defined in clause 5.1.2 in TS 38.321 [7] for the next available PRACH occasion, and transmit the preamble with the calculated PRACH transmission power, if no Random Access Response is received within the RA Response window configured in RACH-ConfigCommon or if no PDCCH addressed to UE’s C-RNTI is received within the RA Response window configured in BeamFailureRecoveryConfig, as defined in clause 5.1.4 in TS 38.321 [7].

Currently, the correct UE behavior is to update the PRACH transmission power if the Random Access Response is not received. In the RAN2 #127bis[3], RAN2 has agreed that:

For the PRACH transmission re-attempt in one RACH procedure, after certain (configured) number of times of RACH attempt in SBFD RACH occasions, UE is allowed to switch to legacy RACH occasions. FFS about the case when UE select legacy ROs first.

In RAN2 #128bis, RAN2 agreed that:

- RO-Type change procedure on RO type selection from legacy RO to additional RO in SBFD symbols is supported when the number of PRACH transmission attempts exceed a threshold (we assume it is the same threshold with the fallback from additional RO to legacy RO). If fallback from legacy RO to additional RO occurs, no further fallback to legacy RO is supported.

According to the agreements above, the UE is allowed to switch from additional RACH occasions to legacy RACH occasions, and from legacy RACH occasions to additional RACH occasions. This is not captured in the RAN4 requirements so far.

The requirements for 4-step RA type in clause 6.2.2.2 are updated to take into account the RO-type change procedure on RO type selection from legacy RO to additional RO in SBFD symbols, and the RO type selection from additional RO to legacy RO, as agreed in RAN2. The exact update can be discussed during the CR drafting.

SCell activation

In February meeting, RAN1 agreed to support SBFD operation in CA scenario as cited below. It also indicates the SBFD can be configured on SCell. RAN4 should discuss how to define the SCell activation delay for activating an SBFD SCell.

The RAN1 agreement from Feb meeting: Agreement Support SBFD operation on one TDD carrier in multi-carrier scenario. - Note: If the TDD carrier for SBFD operation is an Scell, support UE dedicated signaling for cell-specific configuration of time and frequency location of SBFD subbands for the Scell. - FFS whether/how to handle half-duplex CA case

RAN4 to define the SCell activation delay for activating a SBFD SCell.

In existing specification, we have defined the SCell activation delay for activating a downlink only SCell in TS 38.133 clause 8.3.2 and the SCell activation for activating a PUCCH SCell in TS 38.133 clause 8.3.7. The difference between the two is the UE needs to additionally activate the uplink for transmitting PUCCH on the PUCCH SCell hence additional procedures may be needed including random access procedure, PL-RS measurement etc.

When activating a SBFD SCell, the SBFD SCell is configured with both DL and UL subbands. Even PUCCH is not configured on the SBFD SCell, the network is expecting to schedule both DL and UL transmission on the SBFD SCell once it is activated. Otherwise, if SBFD SCell is activated as a downlink only SCell, the resource on the UL subband would be wasted. Therefore, we are expecting the SBFD SCell can be activated on both DL and UL subbands in the SCell activation procedure. RAN4 shall discuss if and how to activate an SBFD SCell on the UL subbands. An example draft CR is provided in [4] in line with the proposal.

RAN4 to agree that SBFD SCell shall be activated on both DL and UL subbands during SCell activation procedure.

Conclusion

The following Observations and Proposals were made:

Observation 1: For Case 3, the relaxed accuracy based on 24 PRB was adopted because some UEs were not able to measure and decode the CSI-RS resource in 2 DL subbands.

Observation 2: According to RAN1 discussion on UE capability, a UE supporting DL reception on two DL subbands shall be able to measure CSI-RS resources on two DL subbands.

Proposal 1: The relaxed accuracy requirements based on 24 RB for Case 3 applies only if UE does not support DL reception in two DL subbands in SBFD symbols.

Proposal 2: For Case 3, if UE does not support DL reception in two DL subbands in SBFD symbols, the existing measurement delay shall apply i.e. lower bound is not needed.

Proposal 3: For Case 3, if UE supports DL reception in 2 DL subbands, a lower bound (if UE indicates the capability) can be introduced to the existing measurement delay but the accuracy performance shall be based on 48 PRBs.

Observation 3: In NR-U, the UE is not aware, in advance, of the number CCA failure at the gNB side during a measurement period. Lmax has been introduced to avoid that the UE extends the measurement period indefinitely.

Observation 4: In SBFD, both the UE and the gNB are aware of the reference signal configuration and aware of the frame structure, considering the time location of SBFD subbands. There is no uncertainty about the reference signal occasions that overlap with symbols of different types.

Proposal 4: Do not define Lmax values in the extension of the L1-RSRP and L1-SINR measurement periods.

Proposal 5: The requirements for 4-step RA type in clause 6.2.2.2 are updated to take into account the RO-type change procedure on RO type selection from legacy RO to additional RO in SBFD symbols, and the RO type selection from additional RO to legacy RO, as agreed in RAN2. The exact update can be discussed during the CR drafting.

Proposal 6: RAN4 to define the SCell activation delay for activating a SBFD SCell.

Proposal 7: RAN4 to agree that SBFD SCell shall be activated on both DL and UL subbands during SCell activation procedure.

- References

- R4-2508274, WF on RRM requirements for NR_duplex_evo, RAN4#115 Huawei

- R4-2504904, WF on SBFD RRM, Huawei

- R4-2503623, summary [114][222] NR_duplex_evo, Huawei

- R4-2509619, draftCR on core requirements for SBFD operation, Nokia, Nokia Shanghai Bell