3GPP TSG-RAN4 Meeting #116	R4-2509619

# Bengaluru, India, 25-29 August, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draftCR | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR on SCell activation for SBFD SCell |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_duplex_evo-Core |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | RAN1 agreed to support SBFD operation in CA scenario. As discussed in our paper R4-2509618, the SCell activation delay for activating an SBFD SCell shall be specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR introduces the SCell activation delay for activating an SBFD SCell assumign both DL and UL shall be activated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RRM core requirements for SBFD SCell activation are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.X new clause, with title: SCell Activation Delay Requirement for Deactivated SBFD SCell |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change >

### 8.3.X	SCell Activation Delay Requirement for Deactivated SBFD SCell

The requirements in this clause shall apply for the UE configured with one SBFD SCell.

If the UE has a valid TA for transmitting on an SBFD SCell then the UE shall be capable to transmit valid CSI report and apply actions related to the activation command for the SCell being activated on the uplink of SBFD SCell no later than in slot n+THARQ+Tactivation_time+max ((TFirst_available_CSI +TCSI_processing),   3*Ttarget_PL-RS)+TCSI_Reporting_afterNR slot length,

Where:

-	A TA is considered to be valid provided that the TimeAlignmentTimer [2] associated with the TAG containing the SBFD SCell is running.

-	THARQ (in ms) is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3].

-	Tactivation_time is the SCell activation delay in milliseconds.

For the UE capable of l3-MeasUnknownSCellActivation-r18, if the UE is provided with ReportOnScellActivation and it reports valid L3 measurement results after receiving the SCell activation command for unknown SCell, the Tactivation_time is the SCell activation delay with L3 reporting in milliseconds as specified in clause 8.3.17 except the definition of Tuncertainty_MAC is replaced with:

-	Tuncertainty_MAC is the time period between reception of the last activation command for PDCCH TCI, PDSCH TCI (when applicable), UL spatial relation (for FR2) relative to

-	SCell activation command for known case;

-	First valid L3 report for unknown case, if UE reports valid L3-RSRP before receiving TCI activation command;

-	First valid L1-RSRP reporting for unknown case, if UE reports valid L3-RSRP after receiving TCI activation command.

Otherwise, Tactivation_time is the SCell activation delay in milliseconds as specified in clause 8.3.2 except the definition of Tuncertainty_MAC is replaced with:

-	Tuncertainty_MAC is the time period between reception of the last activation command for PDCCH TCI, PDSCH TCI (when applicable), UL spatial relation (for FR2) relative to

-	SCell activation command for known case;

-	First valid L1-RSRP reporting for unknown case.

-	Ttarget_PL-RS is the periodicity of the target pathloss reference signal determined during SBFD SCell activation.

-	TFirst_available_CSI is the delay uncertainty (in ms) in acquiring the first available downlink CSI reference resource.

-	TCSI_processing is the UE processing time for CSI reporting.

-	TCSI_reporting_after is the delay uncertainty (in ms) in acquiring the first available CSI reporting resources after end of max ((TFirst_available_CSI + TCSI_processing), 3*Ttarget_PL-RS)

If the UE does not have a valid TA for transmitting on an SBFD SCell then the UE shall be capable to perform downlink actions related to the SCell activation command as specified in [7] for the SCell being activated on the SBFD SCell no later than in slot n+THARQ+Tactivation_timeNR slot lengthTHARQ+Tactivation_time+TCSI_ReportingNR slot length, and shall be capable to perform uplink actions related to the SCell activation command as specified in [7] for the SCell being activated on the SBFD SCell no later than in slot n+THARQ+Tdelay_SBFD_SCellNR slot length n+THARQ+Tdelay_PUCCH_SCell+TCSI_Reporting_PUCCH_SCellNR slot lengthand shall transmit valid CSI report for the SCell being activated on the uplink of SBFD SCell no later than in slot n+THARQ+Tdelay_SBFD_SCellNR slot length, where:

Tdelay_PUCCH_SCell = Tactivation_time + max ((TFirst_available_CSI + TCSI_processing), (T1+T2+T3), 3*Ttarget_PL-RS) + TCSI_reporting_after

Where:

-	Tactivation_time is the SCell activation delay in milliseconds.

For the UE capable of l3-MeasUnknownSCellActivation-r18, if the UE is provided with ReportOnScellActivation and it reports valid L3 measurement results after receiving the SCell activation command for unknown SCell, the Tactivation_time is the SCell activation delay with L3 reporting in milliseconds as specified in clause 8.3.17 except the definition of Tuncertainty_MAC is replaced with:

-	Tuncertainty_MAC is the time period between reception of the last activation command for PDCCH TCI, PDSCH TCI (when applicable), UL spatial relation (for FR2) relative to

-	SCell activation command for known case;

-	First valid L3 report for unknown case, if UE reports valid L3-RSRP before receiving TCI activation command;

-	First valid L1-RSRP reporting for unknown case, if UE reports valid L3-RSRP after receiving TCI activation command;

Otherwise, Tactivation_time is the SCell activation delay in milliseconds as specified in clause 8.3.2 except the definition of Tuncertainty_MAC is replaced with:

-	Tuncertainty_MAC is the time period between reception of the last activation command for PDCCH TCI, PDSCH TCI (when applicable), UL spatial relation (when applicable in FR2) relative to

-	SCell activation command for known case;

-	First valid L1-RSRP reporting for unknown case.

-	Ttarget_PL-RS is the periodicity of the target pathloss reference signal determined during SBFD SCell activation.

-	TFirst_available_CSI is the delay uncertainty (in ms) in acquiring the first available downlink CSI reference resource.

-	TCSI_processing is the UE processing time for CSI reporting.

-	TCSI_reporting_after is the delay uncertainty (in ms) in acquiring the first available CSI reporting resources after end of max ((TFirst_available_CSI + TCSI_processing), (T1+T2+T3), 3*Ttarget_PL-RS)

-	T1 is the delay uncertainty in acquiring the first available PDCCH triggered PRACH occasion in the SBFD SCell after the slot n+THARQ+Tactivation_timeNR slot length.

-	T1 is up to the summation of a delay uncertainty for reception of PDCCH order, SSB to PRACH occasion association period and 10 ms, where SSB to PRACH occasion association period is defined in table 8.1-1 of TS 38.213 [3]

-	T2 is the delay from slot n+THARQ+Tactivation_time+T1NR slot lengthuntil UE has obtained a valid TA command for the target SBFD SCell being activated. Slot n is the slot where the UE receives SCell activation command.

-	T3 is the delay for applying the received TA for uplink transmission on target SBFD SCell being activated, as specified in clause 4.2 in TS 38.213 [3].

The pathloss reference signal is known for known SBFD SCell during activation if the following conditions are met during the period between the last transmission of the RS resource used for L3 measurement reporting and the completion of SBFD SCell activation, where the RS resource is the target pathloss reference signal or QCLed (with Type D) to the target pathloss reference signal:

-	The SCell activation command is received within 1280 ms upon the last transmission of the RS resource used for L3 measurement reporting

-	The target pathloss reference signal determination is based on the latest L3 measurement reporting

-	The target pathloss reference signal remains detectable during the SBFD SCell activation period

-	SNR of the target pathloss reference signal≥-3 dB

-	The associated SSBs with the target pathloss reference signal remain detectable during the SBFD SCell activation period

-	SNR of the associated SSB ≥-3 dB

For the UE capable of l3-MeasUnknownSCellActivation-r18, if the UE is provided with ReportOnScellActivation and it reports valid L3 measurement results after receiving the SCell activation command for unknown SCell, the pathloss reference signal is known for unknown SBFD SCell during activation if the following conditions are met during the period between the last transmission of the RS resource used for L3 reporting after SCell activation command and the completion of SBFD SCell activation, where the RS resource is the target pathloss reference signal or QCLed (with Type D) to the target pathloss reference signal.

-	The SCell activation command is received within 1280 ms upon the last transmission of the RS resource used for the L3 reporting after SCell activation command

--	The target pathloss reference signal determination is based on either the latest L1-RSRP measurement reporting or the L3 reporting after SCell activation command, if UE reports both before receiving TCI activation command

-	The target pathloss reference signal remains detectable during the SBFD SCell activation period

-	SNR of the target pathloss reference signal≥-3 dB

-	The associated SSBs with the target pathloss reference signal remain detectable during the SBFD SCell activation period

-	SNR of the associated SSB ≥-3 dB

Otherwise, the pathloss reference signal is known for unknown SBFD SCell during activation if the following conditions are met during the period between the last transmission of the RS resource used for L1-RSRP measurement reporting and the completion of SBFD SCell activation, where the RS resource is the target pathloss reference signal or QCLed (with Type D) to the target pathloss reference signal.

-	The SCell activation command is received within 1280 ms upon the last transmission of the RS resource used for L1-RSRP measurement reporting

-	The target pathloss reference signal determination is based on the latest L1-RSRP measurement reporting

-	The target pathloss reference signal remains detectable during the SBFD SCell activation period

-	SNR of the target pathloss reference signal≥-3 dB

-	The associated SSBs with the target pathloss reference signal remain detectable during the SBFD SCell activation period

-	SNR of the associated SSB ≥-3 dB

Otherwise, the pathloss reference signal is unknown.

The above delay requirement shall apply provided that:

-	The target pathloss reference signal determined during SBFD SCell activation is known otherwise longer activation time is expected if the pathloss reference signal is unknown; and

-	The RA on SBFD SCell is not interrupted by the RA on PCell otherwise additional delay to activate the SCell is expected; and

-	No SRS carrier based switching or SRS antenna port switching occurs during the SCell activation procedure otherwise the SBFD SCell activation delay can be extended.

The starting point and the end-point of an interruption window on PCell or any activated SCell in MCG for NR standalone mode, or on PSCell or any activated SCell in SCG for EN-DC mode is the same as the interruption in single SCell activation requirement in clause 8.3.2.

The requirement for unknown SBFD SCell applies provided that the PDCCH order (when applicable) and the activation commands for TCI, UL spatial relation and PL-RS (when applicable) are based on the latest valid L1-RSRP reporting or the L3 reporting after SCell activation command via uplink of SBFD SCell.

<End of Change >