3GPP TSG-RAN WG4 Meeting # 116	R4-2509626

Bengaluru, India, 25th – 29th August, 2025

Source:	LG Electronics

Title:	Discussion on RRM requirements of on-demand SSB Scell operation for Rel-19 NES

Agenda item:	7.23.3.1

Document for:	Discussion

Introduction

In agreed WID for Rel-19 NES operation [1], following objective was captured for RAN4 RRM core requirements:

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum. - Specify the corresponding core requirements, for the above features [RAN4].

In this contribution, we provide our views on RRM core requirements for RRM requirements of on-demand SSB Scell operation for Rel-19 NES.

Discussion

General On-demand SSB requirements

Additional processing time for OD-SSB parameter update

In the previous meeting, RAN4 achieved following agreement.

Issue 1-4-3: Additional Processing time for OD-SSB activation Agreement - RAN4 should introduce UE capability with values of 2 ms and 5 ms. Issue 1-4-4: Additional Processing time for OD-SSB parameter update Agreement - Option 1: RAN4 to apply the same timeline for MAC CE based OD-SSB activation and MAC CE based OD-SSB transmission parameter updates. - The additional processing time is 5ms or 2ms based on UE capability reporting. - Note 1: The additional processing time is only applied to L1 and L3 measurement transition period [after the SCell is activated]. - Option 2: When NW updates the OD-SSB parameters for an active Scell by MAC-CE, additional 5ms processing time is applied on top of THARQ+3ms after UE receiving the MAC-CE. - The additional processing time is 5ms - Note 1: The additional processing time is only applied to L1 and L3 measurement transition period [after the SCell is activated]. - Note 2: No capability is needed

In the previous meeting, RAN4 agreed to introduce 2ms and 5ms for additional processing time for OD-SSB activation in deactivated SCell. Some companies raised an issue whether to follow the same additional processing time for OD-SSB parameter update in activated SCell. From signalling perspective, there are no signalling difference between before and after SCell activation. From UE complexity perspective, if one think activated SCell increases complexity, we think there are not much difference, since there could be more than one activated SCell before and after OD-SSB SCell activation. Therefore, it would be better to introduce same processing time for simplicity unless well justified.

Proposal 1: RAN4 to apply the same timeline for MAC CE based OD-SSB activation and MAC CE based OD-SSB transmission parameter updates

Additional processing time for OD-SSB deactivation

In the previous meeting, RAN4 achieved following agreement.

Issue 1-4-5: RAN1 LS - OD-SSB deactivation timeline Agreement - RAN4 to confirm RAN1’s working assumption about OD-SSB deactivation processing time. Further discuss the wording in the reply LS. - Further discuss in RAN4 the UE additional processing time for OD-SSB deactivation. - Option 1: No UE additional processing time for OD-SSB deactivation - Option 2: Follow the same values as UE additional processing time for OD-SSB activation

During the discussion for processing time for OD-SSB deactivation, proposing company argued that additional deactivation processing time is required for re-scheduling of resources. We are not sure whether additional processing time is necessary due to some reason. Firstly, legacy specification has already included features causing re-scheduling, e.g. semi-persistent CSI-RS resource deactivation, which applies MAC-CE processing time. Secondly, current specification specifies the UE behaviour after processing time, however, there is no UE behaviour after deactivaton. Lastly, if the intention of additional deactivation processing time is to continuously transmit OD-SSB after Time instance B as illustrated in Figure 1, it is not acceptable, since RAN1 impact is expected , and they have already completed WI. In conclusion, propose not to introduce additional processing time for OD-SSB deactivation.

Proposal 2: No UE additional processing time for OD-SSB deactivation

Figure 1 Illustration of OD-SSB deactivation

On-demand based deactivated SCell L3 requirements (Case 1)

UE’s behaviour when OD-SSB further activation/reconfiguration within T2

In previous meeting, following agreement regarding minimum interval between multiple fast measurement window has been achieved:

Issue 1-1-2: OD-SSB based deactivated SCell measurement – Procedure Sub-issue 1: The start point of T2 Agreement: - The time point of OD-SSB deactivation. Sub-issue 2: The minimum interval(T2) between two FMWs Agreement: - For FR1, 5*max (measCycleSCell, DRX cycles) - For FR2, 4s for power class 1/5 and 3s for power class 2/3/4 Sub-issue 3 –UE’s behaviour when OD-SSB further activation/reconfiguration within T2 Agreement: - The UE is not required to perform “fast measurement” but follows legacy deactivated SCell measurement requirement within T2. Sub-issue 4 – UE’s behaviour after T2 when OD-SSB further activation/reconfiguration occurs within T2 between two FMWs The issue can be further discussed based on contribution driven, and the discussion will not impact the WI completion. - Option 1: When OD-SSB further activation/reconfiguration occurs within T2, the UE should perform ‘fast measurement’ immediately after T2. - Option 2: when OD-SSB further activation/reconfiguration occurs within T2, UE should follow legacy deactivated SCell measurement after T2. - Other option is not precluded.

In RAN4#112bis, RAN4 introduced the time window in which a UE is required to measure OD-SSB. The motivation of the time window is to reduce the UE power consumption, since NW may not guarantee OD-SSB deactivation. According to the agreement, the UE performs ‘fast L3 measurement’ during the time window, otherwise ‘slow L3 measurement’ is expected. In RAN4#113, slow L3 measurement was defined as using legacy deactivated SCell measurement, which means that the UE shall meet the requirements using measCycleSCell for measurement periodicity. In RAN4#114, the length of fast measurement window was defined. In RAN4#114bis, there was discussion regarding the starting point of T2, however, agreement was not achieved. In RAN4#115, the start point and the minimum interval between two FMWs are defined.

The remaining issue is to define UE’s behaviour after T2 when OD-SSB further activation/reconfiguration occurs within T2 between two FMWs. When NW indicates OD-SSB re-activation within T2, it means that the NW needs deactivated SCell measurement as fast as possible. Thus, we think Option 1, which proposes to initiate FMW immediately after T2 is valid. On the other hand, we also think Option 2 is simple solution. In conclusion, we would like to propose compromised solution between Option 1 and Option 2.

For example, when the NW needs fresh measurements after OD-SSB deactivation, NW can re-activate OD-SSB within T2. As agreed in the previous meeting, the UE follows legacy deactivated SCell measurement requirement within T2. If the UE has sent valid reports within T2, UE should follow legacy deactivated SCell measurement requirement after T2. Otherwise, the UE should follow deactivated SCell measurement requirement based on OD-SSB periodicity after T2.

Proposal 3: For UE’s behaviour after T2 when OD-SSB further activation/reconfiguration occurs within T2 between two FMWs:

    - If the UE provides valid measurement report within T2, the UE should follow legacy deactivated SCell measurement after T2,

      - Otherwise, the UE should perform ‘fast measurement’ immediately after T2

OD-SSB in FMW collided with MG

In the pervious meeting, following wayforward was captured in [2].

Sub-issue 2: OD-SSB in FMW collided with MG - Option 1: UE is allowed to skip the OD-SSB measurement if FMW collides with MG occasion. - Option 2: UE to prioritize the OD-SSB carrier/layer within FWM

In the current specification, Kp, which is the scaling factor for an SSB frequency layer to be measured without GAP, is defined. The principle is to skip the SSB measurement, if SMTC is partially overlapping with MG occasion. The principle can be reused for OD-SSB measurement within FMW.

Proposal 4: UE is allowed to skip the OD-SSB measurement if FMW collides with MG occasion

On-demand based deactivated SCell L3 requirements (Case 2)

Requirement for RAN1 Alt Time-C2

Agreement(RAN1#114) Regarding the relation in terms of time location between the always-on SSB and on-demand SSB, - For the case when the center frequency locations of always-on SSB and on-demand SSB are same, - Alt Time-C: RAN1 specification has no restriction with regards to overlapping - From RAN1 perspective, - Alt Time-C1: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a periodic time domain pattern is supported (the interval between SSB bursts is even and supported in legacy specification). - Alt Time-C2: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a non-periodic time domain pattern is supported. - It is up to RAN4 to define requirements, if any, corresponding to both or either of Alt Time-C1 or Alt Time-C2 - At least the following is supported: PBCH payload for the same SSB index (other than SFN index, half frame index) is the same for AO-SSB and OD-SSB - FFS: Whether half frame index is the same or different for AO-SSB and OD-SSB - For the case when the center frequency locations of always-on SSB and on-demand SSB are different, - Alt Time-C: RAN1 specification has no restriction with regards to overlapping - UE assumes that frequency resources of always-on SSB are not overlapped with those of on-demand SSB in frequency domain. - AO-SSB and OD-SSB are located in the same BWP - FFS: PBCH payload for the same SSB index (other than SFN index, half frame index) should be the same for AO-SSB and OD-SSB - NOTE: AO-SSB periodicity is not adapted Issue 1-2-1: Requirement for RAN1 Alt Time-C1 Agreement Update RAN4 agreement to apply EMP also to L1 measurement in the active SCell after SCell activation. Issue 1-2-2: Requirement for RAN1 Alt Time-C2 Agreement - Option 1: RAN4 to define the delay requirement only based on the OD-SSB periodicity. - Option 2: RAN4 to define the delay requirement only based on the OD-SSB periodicity only if - The periodicity of AO-SSB and OD-SSB are same - It doesn’t impact the WI’s completion.

In the previous meeting, RAN4 has achieved agreement to introduce measurement requirement on Alt Time-C1 from RAN1 agreement. The remaining issue is how to define Alt Time-C2, which is non-periodic pattern. For Time-C2, the principle of options on the table is that it is the UE implementation to perform measurement on AO-SSB in addition to OD-SSB. The majority view for Time-C2 is the requirement based on OD-SSB periodicity, which is Option 1. Regarding Option 2, it limits the scenario when the periodicity of AO-SSB and OD-SSB are same. We think Option 2 can be practical, since the sample of AO-SSB is located between every OD-SSB sample, it is easy to utilize it for the measurement. Furthermore, we think this approach is more suitable at least for L3 measurement.

Proposal 5: RAN4 to define L3 measurement requirements for Time-C2 (non-periodic time pattern) only based on OD-SSB periodicity, only if the periodicity of AO-SSB and OD-SSB are same

For L1 measurement, we have different views than L3 measurements. Typically, L1 measurements are suitable for time-critical procesures like beam management. In this sense, it is beneficial to define L1 measurement requirement based on using both AO-SSB and OD-SSB to reduce the measurement period.

In Figure 5, the example of Time-C2 when the same periodicity of AO-SSB and OD-SSB is illustrated. The red box shows the measurement period based on using both AO-SSB and OD-SSB. Since, both SSBs can be measured, the measurement period can be shorten. For instance, the measurement period of L1-RSRP can be easily derived as:

  - Max(TReport, Ceil(MⅹP/2)ⅹOD-SSB-period)

In conclusion, we think it is beneficial to define measurements requirements using both AO-SSB and OD-SSB for non-periodic Time-C2 at least for L1 measurement.

Figure 5 Time-C2 for L1 measurements when the periodicity of AO-SSB and OD-SSB are the same

Proposal 6: RAN4 to define L1 measurement requirements for Time-C2 (non-periodic time pattern) based on both AO-SSB and OD-SSB, only if the periodicity of AO-SSB and OD-SSB are same

  - For instance, L1-RSRP can be Max(TReport, Ceil(MⅹP/2)ⅹOD-SSB-period)

Requirement for scenario 3 (OD-SSB and always-on SSB are within different frequencies)

Issue 1-2-3: Requirement for scenario 3 (OD-SSB and AO-SSB are within different frequencies) Sub issue 1: RAN2 LS for scenario 3 Agreement: - RAN4 to confirm RAN2’s WS to introduce a new servingCellMO. Sub issue 2: Requirement for scenario 3 Agreement: RAN4 will further discuss whether and how to define the requirements for scenario 3, and meanwhile the discussion will not impact the WI completion.

The remaining issue is whether to define requirements for scenario 3, which is OD-SSB and AO-SSB are within different frequencies especially when AO-SSB is CD-SSB on sync raster and OD-SSB is NCD-SSB not on sync raster. We think scenario 3 is not a corner case, but it can be configured in the field, because typically AO-SSB is CD-SSB on sync raster. Therefore, we think requirement of scenario 3 should be defined.

As RAN4 confirmed RAN2’s Working Assumption in the previous meeting, AO-SSB and OD-SSB can have different MO. In general, the UE is required to perform L3 measurement based on MO. Hence, it is natural to measure both AO-SSB and OD-SSB in parallel, if there is enough time for RF retuning time. However, in case when the time offset of AO-SSB and OD-SSB is too close, there might be no time for RF retuning time. In this case, OD-SSB should be prioritized.

Proposal 7: RAN4 to define requirement when OD-SSB and AO-SSB are within different frequencies

  - UE can measure both AO-SSB and OD-SSB in parallel, if there is enough time for RF retuning time

  - Otherwise, prioiritze OD-SSB

On-demand based SCell activation

Following Wayforward have been achieved:

Issue 1-3-1: The known cell condition in Case 2 Agreement - The current known SCell activation delay methodology can be reused in Case 2 at least for Alt Time-C1 with the following combination. - When the SSB measured for known cell condition is AO-SSB or OD-SSB or the combine of AO-SSB and OD-SSB, and the SSB used in SCell activation procedure is AO-SSB, OD-SSB or the combine of AO-SSB and OD-SSB. - FFS whether apply to Alt Time-C2, if the requirement is defined. - FFS whether apply to scenario 3, if the requirement is defined. - FFS whether additional AGC time should be included in the known SCell activation delay requirement for scenario 3, if defined.

The remaining issue is whether to apply Time-C2 and Scenario 3 for known cell condition of SCell activation. This issue can be aligned with the measurement requirement of Time-C2 and Scenario 3.

Proposal 8: For the known cell condition for SCell activation in Case 2, the requirement can be aligned with the measurement requirement of Time-C2 and Scenario 3

Conclusion

In this contribution, we provide views on RRM requireiments for on-demand SSB SCell operation, and we propose

Proposal 1: RAN4 to apply the same timeline for MAC CE based OD-SSB activation and MAC CE based OD-SSB transmission parameter updates

Proposal 2: No UE additional processing time for OD-SSB deactivation

Proposal 3: For UE’s behaviour after T2 when OD-SSB further activation/reconfiguration occurs within T2 between two FMWs:

    - If the UE provides valid measurement report within T2, the UE should follow legacy deactivated SCell measurement after T2,

      - Otherwise, the UE should perform ‘fast measurement’ immediately after T2

Proposal 4: UE is allowed to skip the OD-SSB measurement if FMW collides with MG occasion

Proposal 5: RAN4 to define L3 measurement requirements for Time-C2 (non-periodic time pattern) only based on OD-SSB periodicity, only if the periodicity of AO-SSB and OD-SSB are same

Proposal 6: RAN4 to define L1 measurement requirements for Time-C2 (non-periodic time pattern) based on both AO-SSB and OD-SSB, only if the periodicity of AO-SSB and OD-SSB are same

  - For instance, L1-RSRP can be Max(TReport, Ceil(MⅹP/2)ⅹOD-SSB-period)

Proposal 7: RAN4 to define requirement when OD-SSB and AO-SSB are within different frequencies

  - UE can measure both AO-SSB and OD-SSB in parallel, if there is enough time for RF retuning time

  - Otherwise, prioiritze OD-SSB

Proposal 8: For the known cell condition for SCell activation in Case 2, the requirement can be aligned with the measurement requirement of Time-C2 and Scenario 3

Reference

[1] RP-242354, “Revised WID: Enhancements of network energy savings for NR,” Ericsson

[2] R4-2508277, “WF on RRM requirements for Netw_Energy_NR_enh_Part1,” Ericsson