3GPP TSG-RAN WG4 Meeting # 116	R4-2509627

Bengaluru, India, 25th – 29th August, 2025

Source:	LG Electronics

Title:	Discussion on RRM requirements of SSB adaptation for Rel-19 NES

Agenda item:	7.23.3.2

Document for:	Discussion

Introduction

In agreed WID for Rel-19 NES operation [1], following objective was captured for RAN4 RRM core requirements:

- Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown - Specify the corresponding core requirements, for the above features [RAN4].

In this contribution, we provide our views on RRM core requirements for RRM requirements of adaptation of SSB for Rel-19 NES.

Discussion

L3 requirements for SSB adaptation

Issue 1-3-1: L3 requirements for SSB adaptation – transition for serving cell Agreement RAN4 to define L3 requirements at transition for SSB adaptation same as legacy L3 transition period requirement. - The cell identification and measurement period requirements apply based on the longer period before or after the transition. Issue 1-3-2: L3 requirements for SSB adaptation – after transition for serving cell Agreement - Subsequent to transition, serving cell identification and measurement period requirements are based on the additional SMTC configuration according to SSB adaptation mapped with SSB periodicity. Issue 1-3-4: Colliding with MG - Proposals - Option 1: If the SMTC occasion of the selected additional SMTC configuration collides with MG occasion, the UE is allowed to skip measurements in the overlapping SMTC occasion of the serving SCell.

Colliding with MG

In the current specification, Kp, which is the scaling factor for an SSB frequency layer to be measured without GAP, is defined. The principle is to skip the SSB measurement, if SMTC is partially overlapping with MG occasion. The principle can be reused for SSB adaptaion.

Proposal 1: UE is allowed to skip the SSB adaptation measurement if SMTC is parialliy overlapped with MG occasion

Impacts due to additional SMTC for intra-frequency with gap

In the previous meeting, RAN4 defined L3 measurement requirements based on additional SMTC configuration according to SSB adapatiaon for intra-freuqency without gap requirements. Unlike intra-freuqency without gap requirements, particulary for intra-frequency without gap requirement, there can be an issue with measurement gap. In general, intra-frequency measurement with gap is applied to enable UE to perform intra-frequency measurement when the SSB to be measured is not within the active bandwidth part. To measure within measurement gap, current requirement take maximum of MGRP and SMTC periodicity. However, when SSB adaptation is indicated, the periodicity of SSB or SMTC can be reduced, which may lead to partially overlapped with measurement gap. In consequence, since a UE may not find SSB to be measured outside of measurement gap, the UE may not leverage the frequent adapted SSB in this situation. It is observed some modification regarding measurement gap is required, otherwise switching SMTC periodicity is not reflected in intra-frequency with gap requriements.

Figure 1 Intra-frequency measurement with gap for SSB adpatation

Regarding inter-frequency measurement, RAN2 concludes that the adapted SSB on neighbor cell is measured based on legacy SMTC, thus, reusing legacy requirement is enough.

Proposal 2: RAN4 to define intra-frequency with gap requirement for SSB adpataion

Conclusion

In this contribution, we provide views on RRM requireiments for SSB adaptation, and we propose

Proposal 1: UE is allowed to skip the SSB adaptation measurement if SMTC is parialliy overlapped with MG occasion

Proposal 2: RAN4 to define intra-frequency with gap requirement for SSB adpataion

Reference

[1] RP-242354, “Revised WID: Enhancements of network energy savings for NR,” Ericsson

[2] R4-2508441, “WF on RRM requirements for Netw_Energy_NR_enh_Part2,” Huawei