3GPP TSG-RAN WG4 Meeting # 116	R4-2509629

Bengaluru, India, 25th – 29th August, 2025

Source:	LG Electronics

Title:	Discussion on RRM requirements of UE-initiated/event-driven beam management

Agenda item:	7.20.4.1

Document for:	Discussion

Introduction

In agreed WID for Rel-19 MIMO operation [1], following objective was captured:

- Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting

In this contribution, we provide our views on RRM core requirements for UE-initiated/event-driven beam reproting

Discussion

Time window based or cover both no time window based and with time window based

#### Issue 1-3: Whether RAN4 to specify requirements for no time window based or cover both no time window based and with time window based with eventInstanceCount-r19? <Way forward > - Option 1: (Qualcomm, Apple, LGE) - Do not define requirements for eventInstanceCount-r19 > 1 - Option 2: (China Telecom, Nokia, Xiaomi, Samsung, CMCC, Huawei, Ericsson, ZTE, MTK) - Specify requirements to cover both cases of no time window based and with time window based with configurable eventInstanceCount-r19

Remaining issue is whether to specify requirements when eventDetectionTimeWindowLength-r19 and eventInstanceCount-r19 is configured. From RAN4 perspecive, RAN4 already achieved agreement on the definition of event triggerd L1-RSRP measurement reporting delay, which is defined as the time between an event that will trigger a measurement report and the point when the UE transmits first PUCCH over the air interface. Regarding a event that will trigger a measurement report, the event could be a single event instance, which increases the number of event instances as greater than or equal to certain count. For example, if eventDetectionTimeWindowLength-r19 is configured, and the number of event instances is greater than or equal to eventInstanceCount-r19, the UE will trigger the measurement report. The last event instance would be the event that will trigger the measurement report. Thus, L1-RSRP measurement reporting delay can be reused for window based event.

Proposal 1: Regarding window-based event when eventDetectionTimeWindowLength-r19 and eventInstanceCount-r19 is configured , event triggered reporting delay can be reused

In our view, we think the requirement for minimum interval between event instance indiction is missing. If there is no requirement, the UE is allowed to count multiple event indication without updating the measurement, which is not the intention of introducing window based event. Similar requirement has already been specified in RLM and BFR in RAN1 and RAN4, however, it was not discussed or specified for UE triggered beam reporting in RAN1 specification. Thus RAN4 needs to define the minimum requirement.

Interval between two event instance can be the periodicity of reference signal of new or current beam. The minimum requirement for event instance indication is similar to 8.5.4 -Minimum requirement for L1 indication for BFR, hence small specification work is expected. Furthemore, considering this meeting is the last meeting of WI, draft CR[2] has been submitted in our companion paper.

Proposal 2: RAN4 to specify minimum interval requirement between two event instance, when eventDetectionTimeWindowLength-r19 and eventInstanceCount-r19 is configured

Figure 1 Event instance when eventDetectionTimeWindowLength is configured

In the draft CR[2], we added modification on top of 8.5.4 (Minimum requirement for L1 indication for BFR).

Firstly, lower bound of the minimum interval of event indication is changed to 1ms. This modification takes into account FR2 SCS of 120 kHz. In the legacy requirement, lower bound is 2ms, which is 2 slot for 15kHz, 4 slots for 30kHz, and 16 slots for 120kHz. For 120KHz, 16 slot is too long for lower bound for minimum interval in FR2. We think the number slot of 30kHz SCS can be reference for minimum interval. Thus, Ceil(4*0.125 ms)=1ms can be calculated, which is the half of current lower bound.

Secondly, some scaling factor can be introduced when the current beam and new beam is overlapped on the same symbol. A UE may not be able to receive current beam and new beam simultaneously. Furthermore, according to agreement in the previous meeting, event evaluation should be based on the most recent result from both new beam and current beam when timeRestrictionForChannelMeasurement is configured, which means that both current and new beam should be updated to determine event instance.

Proposal 3: Regarding minimum interval requirement between two event instance, RAN4 to consider as follows:

- 1ms as lower bound of minimum interval for FR2

- Scaling factor to handle the case when the current beam and new beam is overlapped on the same symbol

Whether to specify new unified TCI switching delay requirements?

- Issue 1-5: Whether to specify new unified TCI switching delay requirements? Agreement: - The existing requirements for TCI state switch delay are applicable for UE-initiated/even-triggered beam management - Further discuss additional enhancements in the next meeting, and the discussion does not impact the WI core part completion.

In RAN1#119 meeting, reducing beam application latency after a UE-initiated/event-driven beam reporting was discussed. The issue is to whether to reduce activated TCI state switching delay by removing additional SSB reception (Tfirst-SSB) for reducing the unified TCI state beam application time. In RAN1#120bis meeting, RAN1 try to draw conclusion, but there was no concensus. Although the agreement was not made, the basic assumption is that the UE stores the QCL properties of the SSB associated with the refrecne signal reported in the UE initiated beam report. In RAN1#121, no agreement or conclusion was achieved for the issue. Furthermore, we are not sure enhancement of unified TCI stsate switching delay is within the scope of WI (no broken spec). Given the fact that it has not been actively discussed in RAN4 due to limited time, no further enahcnement for unified TCI stsate switching delay is needed in this release.

Proposal 4: For unified TCI switching delay requirements, no further enhancements in Rel-19

Whether/How to specify RAN4 requirements if indicated TCI states is changed during the L1 measurement time?

#### Issue 1-6: Whether/How to specify RAN4 requirements if indicated TCI states is changed during the L1 measurement time? <Way forward > - Option 1: (Nokia, Ericsson, MTK) - No need to capture this scenario in RAN4 requirements - Option 2: (Xiaomi, Samsung) - The valid L1 evaluation of event-triggered L1-RSRP should be after the MAC-CE command or DCI. If the event has been triggered, the L1 evaluation time should be restart. - Option 3: (Apple) - Add the applicability rule of event triggered L1 measurement: - The event trigger condition shall be maintained from when condition is met over the air until end of measurement period at the UE. - The reporting delay requirements are applicable when indicated TCI state remains the same through the measurement period

If the indicated TCI stae is changed durding the L1 measurement time, the current beam is changed. From UE perspective, in this case the L1 evaluation time should be restarted. In order to meet the requirement, it should be guaranteed that the current beam is always same during measurement period. In this sense, we prefer Option 3.

Proposal 5: For the case when indicated TCI states is changed during the L1 measurement time, prefer Option3 (add the applicablility rule of event triggered L1 measurement)

Conclusion

In this contribution, we provide views on RRM requireiments for UE-initiated/event-driven beam reproting, and we propose

Proposal 1: Regarding window-based event when eventDetectionTimeWindowLength-r19 and eventInstanceCount-r19 is configured , event triggered reporting delay can be reused

Proposal 2: RAN4 to specify minimum interval requirement between two event instance, when eventDetectionTimeWindowLength-r19 and eventInstanceCount-r19 is configured

Proposal 3: Regarding minimum interval requirement between two event instance, RAN4 to consider as follows:

- 1ms as lower bound of minimum interval for FR2

- Scaling factor to handle the case when the current beam and new beam is overlapped on the same symbol

Proposal 4: For unified TCI switching delay requirements, no further enhancements in Rel-19

Proposal 5: For the case when indicated TCI states is changed during the L1 measurement time, prefer Option3 (add the applicablility rule of event triggered L1 measurement)

Reference

[1] RP-242394, “WID revision: NR MIMO Phase 5,” Samsung

[2] R4-2509631, “Draft CR for minimum requirement for L1 indication for event trigger Reporting based on L1-RSRP measurements,” LG Electronics