# 3GPP TSG-RAN WG4 Meeting #116                                                            R4-2509639

Bengaluru, India, August 25th – 29th, 2025

Source:                Tejas Networks

Title:                      Views on modification of existing Rx requirements for SBFD

Agenda Item:       7.21.3.3

Document for:     Discussion

# Introduction

In the RAN4 #115 meeting, RAN4 agreed to further study, SBFD in-band blocking.

# Discussion

## In-band blocking for FR1 WA BS

In the RAN4 #115 agreed to further study BS receiver in-band blocking with following agreement and WF [1].

- Way forward - For in-band blocking FR1 WA BS, FFS on the value by taking the below range as starting point: - [-28, -16.7] dBm, in which the min and max are coming from co-ex simulations based on 100% grid shift. - For requirements assumption as deployment guidance, FFS on the following options - Option 1: The in-band blocking requirements assume a [TBD] dB coupling loss between interfering NR BS transmitter operating in and NR SBFD BS receive - Option 2: Distance, need to solve the issue that different frequency bands/scenarios would have different values - Option 3: One note can be added to generally describe the deployment restriction. - For requirement definition, FFS on the following aspects - Interference source: interference from BS operating at adjacent channel(s) - Interference signal CBW: TBD - Interference signal mean power: TBD - Interference signal frequency: - For DU configuration: [TBD] offset from the lower Base Station RF Bandwidth edge - For UD configuration: [TBD] offset from the upper Base Station RF Bandwidth edge - For DUD configuration: [TBD] offset from the lower/upper Base Station RF Bandwidth edge - For ACS/blocking requirement and test, own Tx is ON and the corresponding REFSENS degradation will be considered

Our goal is to balance deployment feasibility, coexistence requirements, and practical BS capabilities, while ensuring reliable performance in SBFD-TDD adjacent operator scenarios. Our viewpoint is based on extensive coexistence simulations in UMa scenarios, focusing on realistic GS assumptions and their impact on UL SBFD wideband received power (CDFs). In our previous contributions [2]–[4], we provided in-band blocking requirements for the SBFD-TDD adjacent operator scenario, including results for 10%, 20%, and 100% GS as presented in Table 1. Based on our UMa simulations and consensus, we recommend -25 dBm as the in-band blocking level for FR1 WA BS in SBFD-TDD adjacent operator scenarios. This value falls centrally within the 100% GS-derived range of [-28, -16.7] dBm, offering a conservative yet achievable threshold.

Table 1: FR1 UMa- UMa conducted in-band blocking simulation results

| Scenario | Aggressor | Grid Shift | 99% CDF |
| --- | --- | --- | --- |
| UMa | TDD | 10% | -0.78 |
|  |  | 20% | -14.10 |
|  |  | 100% | -28.05 |

Proposal 1: A -25 dBm in-band blocking level is proposed for adoption in the SBFD-TDD adjacent operator scenario to ensure adequate interference mitigation and coexistence performance.

- For requirements assumption as deployment guidance

Option 1 assumes a constant coupling loss, which greatly simplifies certification by having labs use a single attenuator setting and eliminating per-scenario configuration. However, a one-size-fits-all loss cannot reflect the diversity of urban, suburban, and indoor environments or frequency dependent propagation characteristics in FR1 bands. Determining the TBD value demands a rigorous worst-case path loss analysis that factors in antenna isolation, cable attenuation, and mounting tolerances. To stay relevant, this fixed loss metric must be revisited periodically as new field measurements become available.

Option 2 ties coupling loss to separation distance and environment using established propagation models, enabling operators to set isolation requirements that match LOS urban macros, NLOS suburbs, or indoor small-cell layouts across different frequency bands. This realism supports site-specific planning but adds complexity for operators and test labs, which must emulate or physically reproduce multiple distances and model parameters. It also requires clear definitions of environment categories and standard distance points such as 10 m, 50 m, and 100 m with an added safety margin for installation variances.

Option 3 replaces numeric coupling or distance brackets with a concise deployment restriction note instructing operators to ensure “sufficient” separation, shielding, or analog filtering to meet interference mitigation best practices. This high-level guidance maximizes flexibility by letting network planners apply local site knowledge and avoids over engineering for improbable worst-case scenarios. The trade-off is potential ambiguity in certification, so the note should require simple proofs site-survey reports, on-site coupling measurements, or documented filter performance without locking in a single dB threshold.

Observation 1: Options 1 and 2 provide precise, repeatable metrics but lack flexibility for real-world variability without extensive modelling. Option 3 offers adaptability but sacrifices prescriptive clarity. While test labs prefer fixed values for efficiency, operators require flexibility to address diverse deployment conditions. A purely numeric approach may misalign with field realities, whereas advisory guidance risks inconsistent interpretation.

Proposal 2: Option 3 is adopted, accompanied by a deployment restriction note to guide implementation and ensure consistent interpretation across varied scenarios.

## In-band blocking for FR1 Micro BS

- Way forward - For simulation RAN4 to use the same grid shift assumption as FR1 Macro BS as starting point - Companies to check the blocking level in the range of [-20, -32] dBm

Building upon the simulation results presented in our previous paper [5], we make the following observations regarding the FR1 MR BS.

Table 2: FR1 UMi- Umi conducted in-band blocking simulation results

| Scenario | Aggressor | Grid Shift | 99% CDF |
| --- | --- | --- | --- |
| UMi | TDD | 10% | -16.03 |
|  |  | 100% | -32.30 |

Observation 2: In the FR1 MR BS scenario with a medium-area BS and a 10% GS, the current in-band blocking requirement of -38 dBm needs to be increased.

Proposal 3 : For the FR1 MR BS scenario with a 10% GS, the current in-band blocking requirement of -38 dBm should be increased to account for the elevated levels of in-band blocking power.

# Conclusion

Proposal 1: A -25 dBm in-band blocking level is proposed for adoption in the SBFD-TDD adjacent operator scenario to ensure adequate interference mitigation and coexistence performance.

Observation 1: Options 1 and 2 provide precise, repeatable metrics but lack flexibility for real-world variability without extensive modelling. Option 3 offers adaptability but sacrifices prescriptive clarity. While test labs prefer fixed values for efficiency, operators require flexibility to address diverse deployment conditions. A purely numeric approach may misalign with field realities, whereas advisory guidance risks inconsistent interpretation.

Proposal 2: Option 3 is adopted, accompanied by a deployment restriction note to guide implementation and ensure consistent interpretation across varied scenarios.

Observation 2: In the FR1 MR BS scenario with a medium-area BS and a 10% GS, the current in-band blocking requirement of -38 dBm needs to be increased.

Proposal 3 : For the FR1 MR BS scenario with a 10% GS, the current in-band blocking requirement of -38 dBm should be increased to account for the elevated levels of in-band blocking power.

# Reference

- R4-2508713, Way Forward for [115][307] NR_duplex_evo_BSRF.

- R4-2503299, Views on modification of existing Rx requirements for SBFD, Tejas Networks Ltd.

- R4-2415046, Views on modification of existing Rx requirements for SBFD, Tejas Networks Ltd.

- R4-2505469, Views on modification of existing Rx requirements for SBFD, Tejas Networks Ltd.

- R4-2417789, Views on modification of existing Rx requirements for SBFD, Tejas Networks Ltd.

- TR 38.858, “Study on evolution of NR duplex operation,” 3GPP.