3GPP TSG-RAN WG4 Meeting #116                                                                R4-2509640

# Bengaluru, India, August 25th – 29th, 2025

Agenda item:			7.7.5

Source:	Sharp

Title:	NTN VSAT Type-6 Max TRP and EIRP values

Document for:	Discussion

# Introduction

Table 9.2.1.0-1 of TS 38.101-5 is proposed to be updated at RAN4#116 to include the definition for of NTN VSAT Type 6 in CR [1].  Additionally, tables 9.2.1.2-1 and 9.2.1.2-2 of TS 38.101-5 are proposed to be updated in the same CR to include the definitions of the Minimum peak EIRP and Maximum output power limits for Mobile VSAT for the new Type 6 device.

# Discussion

In table 9.2.1.2-2, new Operating Bands are proposed for n509, n508, for n248, n247 and associated with those bands is the new UE Type-6 of the CR in [1]. Furthermore, the values for TRPmax and ERIPmax associated with the new UE Type-6 are provided as 40 dBm and 76.6 dBm respectively. However, those values are in brackets.

The value for TRPmax and ERIPmax for Ku band NTN VSAT coexistence simulations assumption work (i.e. 10Watts (=>40 dBm) and 76.6 dBm respectively) have been agreed and used since RAN4#115 as captured in [2]

Proposal 1: Agree the value for TRPmax for Type-6 UE in table 9.2.1.2-2 as 40dBm

Proposal 2: Agree value for EIRPmax for Type-6 UE in table 9.2.1.2-2 as be 76.6dBm

# Reference

- R4-2510861: CR to TS 38.101-5 for adding 9.2.1 requirements for NTN Ku bands

- R4-2508774: Ku band co-existence simulation assumptions (running document)