3GPP TSG-RAN WG4 Meeting #116	R4-2509641

# Bengaluru, India, 25-29 August, 2025

Source: 	KDDI

Title: 	(NR_newRAT-Core) WF on NR A-MPR reevaluation scenarios on n1 for Japan

Agenda Item:	4.2.1

Document for:	Discussion

- 1. Introduction

In the previous meeting, RAN4 triggered NR A-MPR reevaluation scenarios and to specify new NS_05/NS_05U for n1 PC3 and PC2 on n1 for Japan [1]. In last meeting, WF [2] was agreed. We propose ways forward for NS_05U and NS_05 A-MPR in band n1 for PC2 and PC3.

- 2. Discussion

  - NS_05 PC3

A-MPR region definition and values

Firstly, in the last meeting, the WF[2] on NS_05 PC3 A-MPR region definition and values for 15MHz and 20MHz cases was agreed as follows.

RAN4#115 WF[2] < Issue 1-1-1: NS_05 PC3 A-MPR region split and values for PUCCH > Way forward: - A-MPR region definition and values for NS_05 PC3 region split for 20MHz are as follows. Table 1: A-MPR regions for NS_05 region split (Power Class 3, 20MHz) Channel Bandwidth (MHz); Carrier Centre Frequency, Fc (MHz); Region A1; Region A2; Region A3. LCRB; RBstart; LCRB; RBstart; LCRB; RBstart. 20; < 1945.7; ≥ RBstart*1.5 + 4.14MHz/12/scs;; ≥ 0; ≤ 5.76MHz/12/scs; ≤ RBstart*1.5 + 4.14MHz/12/scs;; ≤ 1.8MHz/12/scs; ≤ 5.76MHz/12/scs; > 1.8MHz/12/scs ≤ 4.68MHz/12/scs. Table 5: A-MPR values for NS_05 region split (Power Class 3, 20MHz, Fc<1945.7MHz) Modulation/Waveform; A1 (dB); A2 (dB); A3 (dB). Outer/Inner; Outer/Inner; Outer/Inner. DFT-s-OFDM; Pi/2 BPSK; ≤ 3.5; ≤ 3; ≤ 2.0. QPSK; ≤ 4; ≤ 3.5; ≤ 2.5. 16 QAM; ≤ 5; ≤ 4; ≤ 2.5. 64 QAM; ≤ 5; ≤ 4; ≤ 3.0. 256 QAM; ≤ 5; ≤ 4.5; ≤ 4.5. CP-OFDM; QPSK; ≤ 6; ≤ 5.5; ≤ 4.0. 16 QAM; ≤ 6; ≤ 5.5; ≤ 4.0. 64 QAM; ≤ 6; ≤ 5.5; ≤ 4.5. 256 QAM; ≤ 7; ≤ 6.5; ≤ 6.5. Way forward: - FFS A-MPR region definition and values for NS_05 PC3 region split for 15MHz are as follows. Table 2: A-MPR regions for NS_05 region split (Power Class 3, 15MHz) Channel Bandwidth (MHz); Carrier Centre Frequency, Fc (MHz); Region A1; Region A2; Region A3. LCRB; RBstart; LCRB; RBstart; LCRB; RBstart. 15; 1927.5 ≤ fc < 1932.5; ≥ RBstart*1.2 + 4.68MHz/12/scs; ≥ 0; ≤ 3.96MHz/12/scs; 0; ≤ 3.96MHz/12/scs; ≤ 3.6MHz/12/scs; > 0. 15; 1932.5 ≤ fc < 1938.2; ≥ 8.28/12/scs; ≤ 0.54/12/scs; ≤ 2.16MHz/12/scs; ≤ 1.62MHz/12/scs; ≤ 2.16MHz/12/scs; > 1.62MHz/12/scs ≤ 2.16MHz/12/scs.

Next, in the previous meeting, the WF[3] on NS_05 PC3 A-MPR region definition was agreed as follows.

RAN4#114bis WF[3] < Issue 2-6-1: A-MPR region and value for NS_05 PC3 > Agreement: - Update the last meeting agreements for NS_05 PC3 as follows. Table 1: A-MPR regions for NS_05 (Power Class 3) Channel Bandwidth (MHz); Carrier Centre Frequency, Fc (MHz); Region A1; Region A2. LCRB; RBstart; LCRB; RBstart. 5; N/A; N/A; N/A; N/A. 10; < 1930.7; ≥ 1.35*RBstart + 4.32MHz/12/scs; ≥ 0; ≤ 2.16MHz/12/scs; ≤ 1.8MHz/12/scs. 15; 1927.5 ≤ fc < 1932.5; ≥ RBstart*1.2 + 4.68MHz/12/scs; ≥ 0; ≤ 3.96MHz/12/scs; ≤ 3.6MHz/12/scs. 15; 1932.5 ≤ fc < 1938.2; ≥ 8.28/12/scs; ≤ 0.54/12/scs; ≤ 2.16MHz/12/scs; ≤ 2.16MHz/12/scs. 20; < 1945.7; ≥ RBstart*1.5 + 4.14MHz/12/scs;; ≥ 0; ≤ 5.76MHz/12/scs; ≤ RBstart*1.5 + 4.14MHz/12/scs;; ≤ 4.68MHz/12/scs.

And then, in the previous meeting, the WF[4] on NS_05 PC3 A-MPR region definition and values was agreed as follows.

RAN4#114 WF[4] < Issue 1-1-1: A-MPR region and value for NS_05 PC3 > Way forward: - A-MPR region definition and values for NS_05 PC3 are as follows. Table 1: A-MPR regions for NS_05 (Power Class 3) Channel Bandwidth (MHz); Carrier Centre Frequency, Fc (MHz); Region A1; Region A2. LCRB; RBstart; LCRB; RBstart. 5; N/A; N/A; N/A; N/A. 10; < 1930.7; ≥ 1.35*RBstart + 4.32MHz/12/scs; >0; ≤ 2.16MHz/12/scs; ≤ 1.8MHz/12/scs. 15; 1927.5 ≤ fc < 1932.5; ≥ RBstart*1.2 + 4.68MHz/12/scs; >0; ≤ 3.96MHz/12/scs; ≤ 3.6MHz/12/scs. 15; 1932.5 ≤ fc < 1938.2; ≥ 8.28/12/scs; ≤ 0.54/12/scs; ≤ 2.16MHz/12/scs; ≤ 2.16MHz/12/scs. 20; < 1945.7; ≥ RBstart*1.5 + 4.14MHz/12/scs;; >0; ≤ 5.76MHz/12/scs; ≤ RBstart*1.5 + 4.14MHz/12/scs;; ≤ 4.68MHz/12/scs. Table 2: A-MPR values for NS_05 (Power Class 3, 10MHz, Fc<1930.7MHz) Modulation/Waveform; A1 (dB); A2 (dB). Outer/Inner; Outer/Inner. DFT-s-OFDM; Pi/2 BPSK; ≤ 3; ≤ 2. QPSK; ≤ 4; ≤ 3. 16 QAM; ≤ 4; ≤ 3. 64 QAM; ≤ 4.5; ≤ 3.5. 256 QAM; ≤ 5; ≤ 4.5. CP-OFDM; QPSK; ≤ 5; ≤ 4.5. 16 QAM; ≤ 5; ≤ 4.5. 64 QAM; ≤ 5; ≤ 4.5. 256 QAM; ≤ 6; ≤ 6. Table 3: A-MPR values for NS_05 (Power Class 3, 15MHz, 1927.5≤fc<1932.5) Modulation/Waveform; A1 (dB); A2 (dB). Outer/Inner; Outer/Inner. DFT-s-OFDM; Pi/2 BPSK; ≤ 3; ≤ 2.5. QPSK; ≤ 4; ≤ 3. 16 QAM; ≤ 4.5; ≤ 3.5. 64 QAM; ≤ 4.5; ≤ 4. 256 QAM; ≤ 5; ≤ 4.5. CP-OFDM; QPSK; ≤ 6; ≤ 5. 16 QAM; ≤ 6; ≤ 5. 64 QAM; ≤ 6; ≤ 5. 256 QAM; ≤ 6.5; ≤ 6. Table 4: A-MPR values for NS_05 (Power Class 3, 15MHz, 1932.5≤Fc<1938.2MHz) Modulation/Waveform; A1 (dB); A2 (dB). Outer/Inner; Outer/Inner. DFT-s-OFDM; Pi/2 BPSK; ≤ 2; ≤ 2. QPSK; ≤ 3; ≤ 2.5. 16 QAM; ≤ 3; ≤ 3. 64 QAM; ≤ 3; ≤ 3. 256 QAM; ≤ 4.5; ≤ 4. CP-OFDM; QPSK; ≤ 4.5; ≤ 4.5. 16 QAM; ≤ 4.5; ≤ 4.5. 64 QAM; ≤ 4.5; ≤ 4.5. 256 QAM; ≤ 6; ≤ 6. Table 5: A-MPR values for NS_05 (Power Class 3, 20MHz, Fc<1945.7MHz) Modulation/Waveform; A1 (dB); A2 (dB). Outer/Inner; Outer/Inner. DFT-s-OFDM; Pi/2 BPSK; ≤ 3.5; ≤ 3. QPSK; ≤ 4; ≤ 3.5. 16 QAM; ≤ 5; ≤ 4. 64 QAM; ≤ 5; ≤ 4. 256 QAM; ≤ 5; ≤ 4.5. CP-OFDM; QPSK; ≤ 6; ≤ 5.5. 16 QAM; ≤ 6; ≤ 5.5. 64 QAM; ≤ 6; ≤ 5.5. 256 QAM; ≤ 7; ≤ 6.

Next, the WF[2] on the table layout for A-MPR region definition was agreed as follows.

RAN4#115 WF[2] < Issue 1-1-3: A-MPR table format> Agreement: - Use the more “modern” table layout when writing the CR as proposed in Table 2.

Additionally, we are aware of that some A-MPR values are smaller than MPR in Table 6.2.2-1 of 38.101-1 as shown with yellow highlighted and red texts in the following Table 1. A-MPR values should be max(A-MPR, MPR). Then, we propose to consider the following A-MPR region and values for NS_05 PC3.

Proposal 1: Consider the following Table 1 and 2 as PC3 A-MPR for NS_05.

Table 1: A-MPR regions for NS_05 region split (Power Class 3)

| Channel Bandwidth (MHz) | Carrier Centre Frequency, Fc (MHz) | Region A |  |  | Region B |  |  | Region C |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | LCRB | RBstart | A-MPR | LCRB | RBstart | A-MPR | LCRB | RBstart | A-MPR |
| 5 | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| 10 | < 1930.7 | ≥ 1.35*RBstart + 4.32MHz/12/scs | ≥ 0 | A1 | ≤ 2.16MHz/12/scs | ≤ 1.8MHz/12/scs | A2 | N/A | N/A | N/A |
| 15 | 1927.5 ≤ fc < 1932.5 | ≥ RBstart*1.2 + 4.68MHz/12/scs | ≥ 0 | A3 | ≤ 3.96MHz/12/scs | 0 | A4 | ≤ 3.96MHz/12/scs | ≤ 3.6MHz/12/scs; > 0 | A5 |
| 15 | 1932.5 ≤ fc < 1938.2 | ≥ 8.28/12/scs | ≤ 0.54/12/scs | A6 | ≤ 2.16MHz/12/scs | ≤ 1.62MHz/12/scs | A7 | ≤ 2.16MHz/12/scs | > 1.62MHz/12/scs ≤ 2.16MHz/12/scs | A8 |
| 20 | < 1945.7 | ≥ RBstart*1.5 + 4.14MHz/12/scs; | ≥ 0 | A9 | ≤ 5.76MHz/12/scs; ≤ RBstart*1.5 + 4.14MHz/12/scs; | ≤ 1.8MHz/12/scs | A10 | ≤ 5.76MHz/12/scs | > 1.8MHz/12/scs ≤ 4.68MHz/12/scs | A11 |

■IMD5_1906.6-1915.7 MHz: -25 dBm/MHz, ■IMD5_ 1884.5-1906.6 MHz: -30 dBm/MHz, ■TBD(will be proposed in Aug.)

■Could be harmonized as common value

Table 2: A-MPR values for NS_05 region split (Power Class 3, 20MHz)

| Modulation/Waveform |  | A1 (dB) | A2 (dB) | A3 (dB) | A4 (dB) | A5 (dB) | A6 (dB) | A7 (dB) | A8 (dB) | A9 (dB) | A10 (dB) | A11 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3 | ≤ 2 | ≤ 3 | [xx] | [xx] | ≤ 2 | [xx] | [xx] | ≤ 3.5 | ≤ 3 | ≤ 2.0 |
|  | QPSK | ≤ 4 | ≤ 3 | ≤ 4 | [xx] | [xx] | ≤ 3 | [xx] | [xx] | ≤ 4 | ≤ 3.5 | ≤ 2.5 |
|  | 16 QAM | ≤ 4 | ≤ 3 | ≤ 4.5 | [xx] | [xx] | ≤ 3 | [xx] | [xx] | ≤ 5 | ≤ 4 | ≤ 2.5 |
|  | 64 QAM | ≤ 4.5 | ≤ 3.5 | ≤ 4.5 | [xx] | [xx] | ≤ 3 | [xx] | [xx] | ≤ 5 | ≤ 4 | ≤ 3.0 |
|  | 256 QAM | ≤ 5 | ≤ 4.5 | ≤ 5 | [xx] | [xx] | ≤ 4.5 | [xx] | [xx] | ≤ 5 | ≤ 4.5 | ≤ 4.5 |
| CP-OFDM | QPSK | ≤ 5 | ≤ 4.5 | ≤ 6 | [xx] | [xx] | ≤ 4.5 | [xx] | [xx] | ≤ 6 | ≤ 5.5 | ≤ 4.0 |
|  | 16 QAM | ≤ 5 | ≤ 4.5 | ≤ 6 | [xx] | [xx] | ≤ 4.5 | [xx] | [xx] | ≤ 6 | ≤ 5.5 | ≤ 4.0 |
|  | 64 QAM | ≤ 5 | ≤ 4.5 | ≤ 6 | [xx] | [xx] | ≤ 4.5 | [xx] | [xx] | ≤ 6 | ≤ 5.5 | ≤ 4.5 |
|  | 256 QAM | ≤ 6 6.5 | ≤ 6 6.5 | ≤ 6.5 | [xx] | [xx] | ≤ 6 6.5 | [xx] | [xx] | ≤ 7 | ≤ 6.5 | ≤ 6.5 |

Proposal 2: Merge region A1, A3 and A9 into one unified region with using their biggest A-MPR values for NS_05 PC3.

Proposal 3: Merge region A4 and A10 into one unified region with using their biggest A-MPR values for NS_05 PC3.

Proposal 4: Merge region A2, A5, A7 and A11 into one unified region with using their biggest A-MPR values for NS_05 PC3.

  - NS_05 PC2

A-MPR region definition and values

Firstly, in the last meeting, the WF[2] on NS_05 PC2 A-MPR region definition and values was agreed as follows.

RAN4#115 WF[2] < Issue 1-1-4: A-MPR region for NS_05 PC2 > < Issue 1-1-5: A-MPR values for NS_05 PC2 > Agreement: - Define one set A-MPR for n1 PC2 NS_05 covering both 1T and 2T. - Adding 1dB to 1Tx A-MPR values for 2Tx in all configurations for PC2 NS_05. - Agree with the following A-MPR region and values for NS_05 PC2.

Table 1: A-MPR regions for NS_05 (Power Class 2) Channel Bandwidth (MHz); Carrier Centre Frequency, Fc (MHz); Region A1; Region A2; Region A3. LCRB; RBstart; LCRB; RBstart; LCRB; RBstart. 5; < 1925; ≥ 3.24(18RB); ≤ 0.18; N/A; N/A; N/A; N/A. 10; ≤ 1925; ≥ 1.35*RBstart + 4.32MHz/12/scs; ≥ 0; ≤ 2.16MHz/12/scs; ≤ 1.8MHz/12/scs; < 1.35* RBstart*12*SCS + 4.32 ≥ 1.2* RBstart*12*SCS + 3.6; ≥ 0. 10; 1925 < fc < 1930.7; ≥ 1.35*RBstart + 4.32MHz/12/scs; ≥ 0; ≤ 2.16MHz/12/scs; ≤ 1.8MHz/12/scs; N/A; N/A. 15; 1927.5 ≤ fc <1932.5; ≥ RBstart*1.2 + 3.96MHz/12/scs; ≥ 0; ≤ RBstart*1.2 + 3.96MHz/12/scs ≤ 4.5MHz/12/scs; ≤ 3.6MHz/12/scs; ≤ RBstart*1.2 + 3.96MHz/12/scs ≥ RBstart*12*SCS * + 3.6 > 4.5MHz/12/scs; ≥ 0. 15; 1932.5 ≤ fc < 1938.2; ≥ 8.28MHz/12/ scs; ≤ 1.8MHz/12/scs; ≤ 2.16MHz/12/scs; ≤ 2.16MHz/12/scs; N/A; N/A. 20; < 1945.7; ≥ 1.0*12*SCS*RBstart + 4.14MHz/12/scs; ≥ 0; ≤ 6.48MHz/12/scs; < RBstart*1.0 + 4.14MHz/12/scs; ≤ 4.68MHz/12/scs; N/A; N/A.

aa

Table 2: A-MPR values for NS_05 (Power Class 2, 5MHz, fc<1925MHz) Modulation/Waveform; A1 (dB). Outer/Inner. DFT-s-OFDM; Pi/2 BPSK; ≤ 3.5. QPSK; ≤ 4.0. 16 QAM; ≤ 4.5. 64 QAM; ≤ 4.5. 256 QAM; ≤ 5.5. CP-OFDM; QPSK; ≤ 5.5. 16 QAM; ≤ 5.5. 64 QAM; ≤ 5.5. 256 QAM; ≤ 7.5. Table 3: A-MPR values for NS_05 (Power Class 2, 10MHz, fc≤ 1925) Modulation/Waveform; A1 (dB); A2 (dB); A3 (dB). Outer/Inner; Outer/Inner; Outer/Inner. DFT-s-OFDM; Pi/2 BPSK; ≤ 4.5; ≤ 3.0; ≤ 3.0. QPSK; ≤ 5.5; ≤ 4.0; ≤ 4.0. 16 QAM; ≤ 6.0; ≤ 4.5; ≤ 4.5. 64 QAM; ≤ 6.0; ≤ 4.5; ≤ 4.5. 256 QAM; ≤ 6.0; ≤ 5.0; ≤ 5.0. CP-OFDM; QPSK; ≤ 6.5; ≤ 5.5; ≤ 5.5. 16 QAM; ≤ 6.5; ≤ 5.5; ≤ 5.5. 64 QAM; ≤ 6.5; ≤ 5.5; ≤ 5.5. 256 QAM; ≤ 7.0; ≤ 7.0; ≤ 7.0. Table 4: A-MPR values for NS_05 (Power Class 2, 10MHz, 1925 < fc < 1930.7) Modulation/Waveform; A1 (dB); A2 (dB). Outer/Inner; Outer/Inner. DFT-s-OFDM; Pi/2 BPSK; ≤ 4.5; ≤ 3.0. QPSK; ≤ 5.5; ≤ 4.0. 16 QAM; ≤ 6.0; ≤ 4.5. 64 QAM; ≤ 6.0; ≤ 4.5. 256 QAM; ≤ 6.0; ≤ 5.0. CP-OFDM; QPSK; ≤ 6.5; ≤ 5.5. 16 QAM; ≤ 6.5; ≤ 5.5. 64 QAM; ≤ 6.5; ≤ 5.5. 256 QAM; ≤ 7.0; ≤ 7.0. Table 5: A-MPR values for NS_05 (Power Class 2, 15MHz, 1927.5 ≤ fc <1932.5) Modulation/Waveform; A1 (dB); A2 (dB); A3 (dB). Outer/Inner; Outer/Inner; Outer/Inner. DFT-s-OFDM; Pi/2 BPSK; ≤ 4.5; ≤ 4.0; ≤ 3.5. QPSK; ≤ 5.5; ≤ 4.5; ≤ 4.0. 16 QAM; ≤ 6.0; ≤ 5.0; ≤ 4.5. 64 QAM; ≤ 6.0; ≤ 5.0; ≤ 4.5. 256 QAM; ≤ 6.0; ≤ 5.5; ≤ 5.5. CP-OFDM; QPSK; ≤ 7.5; ≤ 6.5; ≤ 5.5. 16 QAM; ≤ 7.5; ≤ 6.5; ≤ 5.5. 64 QAM; ≤ 7.5; ≤ 6.5; ≤ 5.5. 256 QAM; ≤ 7.5; ≤ 7.0; ≤ 6.5. Table 6: A-MPR values for NS_05 (Power Class 2, 15MHz, 1932.5 ≤ fc < 1938.2) Modulation/Waveform; A1 (dB); A2 (dB). Outer/Inner; Outer/Inner. DFT-s-OFDM; Pi/2 BPSK; ≤ 3.5; ≤ 3.5. QPSK; ≤ 4.5; ≤ 4.0. 16 QAM; ≤ 4.5; ≤ 4.5. 64 QAM; ≤ 5.0; ≤ 4.5. 256 QAM; ≤ 5.5; ≤ 5.0. CP-OFDM; QPSK; ≤ 5.5; ≤ 6.0. 16 QAM; ≤ 5.5; ≤ 6.0. 64 QAM; ≤ 5.5; ≤ 6.0. 256 QAM; ≤ 7.0; ≤ 7.0. Table 7: A-MPR values for NS_05 (Power Class 2, 20MHz, fc < 1945.7) Modulation/Waveform; A1 (dB); A2 (dB). Outer/Inner; Outer/Inner. DFT-s-OFDM; Pi/2 BPSK; ≤ 4.5; ≤ 4.0. QPSK; ≤ 5.5; ≤ 4.5. 16 QAM; ≤ 6.5; ≤ 5.0. 64 QAM; ≤ 6.5; ≤ 5.5. 256 QAM; ≤ 6.5; ≤ 5.5. CP-OFDM; QPSK; ≤ 7.5; ≤ 6.5. 16 QAM; ≤ 7.5; ≤ 6.5. 64 QAM; ≤ 7.5; ≤ 6.5. 256 QAM; ≤ 7.5; ≤ 7.0.

Next, same as PC3, the WF[2] on the table layout for A-MPR region definition was agreed. Then, we propose to consider the following A-MPR region and values for NS_05 PC2.

Proposal 5: Consider the following Table 3 and 4 as PC2 A-MPR for NS_05.

Table 3: A-MPR regions for NS_05 (Power Class 2)

| Channel Bandwidth (MHz) | Carrier Centre Frequency, Fc (MHz) | Region A |  |  | Region B |  |  | Region C |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | LCRB | RBstart | A-MPR | LCRB | RBstart | A-MPR | LCRB | RBstart | A-MPR |
| 5 | < 1925 | ≥ 3.24(18RB) | ≤ 0.18 | A1 | N/A | N/A | N/A | N/A | N/A | N/A |
| 10 | ≤ 1925 | ≥ 1.35*RBstart + 4.32MHz/12/scs | ≥ 0 | A2 | ≤ 2.16MHz/12/scs | ≤ 1.8MHz/12/scs | A3 | < 1.35* RBstart*12*SCS + 4.32 ≥ 1.2* RBstart*12*SCS + 3.6 | ≥ 0 | A4 |
| 10 | 1925 < fc < 1930.7 | ≥ 1.35*RBstart + 4.32MHz/12/scs | ≥ 0 | A5 | ≤ 2.16MHz/12/scs | ≤ 1.8MHz/12/scs | A6 | N/A | N/A | N/A |
| 15 | 1927.5 ≤ fc <1932.5 | ≥ RBstart*1.2 + 3.96MHz/12/scs | ≥ 0 | A7 | ≤ RBstart*1.2 + 3.96MHz/12/scs ≤ 4.5MHz/12/scs | ≤ 3.6MHz/12/scs | A8 | ≤ RBstart*1.2 + 3.96MHz/12/scs ≥ RBstart*12*SCS * + 3.6 > 4.5MHz/12/scs | ≥ 0 | A9 |
| 15 | 1932.5 ≤ fc < 1938.2 | ≥ 8.28MHz/12/ scs | ≤ 1.8MHz/12/scs | A10 | ≤ 2.16MHz/12/scs | ≤ 2.16MHz/12/scs | A11 | N/A | N/A | N/A |
| 20 | < 1945.7 | ≥ 1.0*12*SCS*RBstart + 4.14MHz/12/scs | ≥ 0 | A12 | ≤ 6.48MHz/12/scs; < RBstart*1.0 + 4.14MHz/12/scs | ≤ 4.68MHz/12/scs | A13 | N/A | N/A | N/A |

■IMD5_1906.6-1915.7 MHz: -25 dBm/MHz, ■IMD5_ 1884.5-1906.6 MHz: -30 dBm/MHz, ■TBD(will be proposed in Aug.)

■,,■Could be harmonized as common value

Table 4: A-MPR values for NS_05 (Power Class 2)

| Modulation/Waveform |  | A1 (dB) | A2 (dB) | A3 (dB) | A4 (dB) | A5 (dB) | A6 (dB) | A7 (dB) | A8 (dB) | A9 (dB) | A10 (dB) | A11 (dB) | A12 (dB) | A13 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3.5 | ≤ 4.5 | ≤ 3.0 | ≤ 3.0 | ≤ 4.5 | ≤ 3.0 | ≤ 4.5 | ≤ 4.0 | ≤ 3.5 | ≤ 3.5 | ≤ 3.5 | ≤ 4.5 | ≤ 4.0 |
|  | QPSK | ≤ 4.0 | ≤ 5.5 | ≤ 4.0 | ≤ 4.0 | ≤ 5.5 | ≤ 4.0 | ≤ 5.5 | ≤ 4.5 | ≤ 4.0 | ≤ 4.5 | ≤ 4.0 | ≤ 5.5 | ≤ 4.5 |
|  | 16 QAM | ≤ 4.5 | ≤ 6.0 | ≤ 4.5 | ≤ 4.5 | ≤ 6.0 | ≤ 4.5 | ≤ 6.0 | ≤ 5.0 | ≤ 4.5 | ≤ 4.5 | ≤ 4.5 | ≤ 6.5 | ≤ 5.0 |
|  | 64 QAM | ≤ 4.5 | ≤ 6.0 | ≤ 4.5 | ≤ 4.5 | ≤ 6.0 | ≤ 4.5 | ≤ 6.0 | ≤ 5.0 | ≤ 4.5 | ≤ 5.0 | ≤ 4.5 | ≤ 6.5 | ≤ 5.5 |
|  | 256 QAM | ≤ 5.5 | ≤ 6.0 | ≤ 5.0 | ≤ 5.0 | ≤ 6.0 | ≤ 5.0 | ≤ 6.0 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.0 | ≤ 6.5 | ≤ 5.5 |
| CP-OFDM | QPSK | ≤ 5.5 | ≤ 6.5 | ≤ 5.5 | ≤ 5.5 | ≤ 6.5 | ≤ 5.5 | ≤ 7.5 | ≤ 6.5 | ≤ 5.5 | ≤ 5.5 | ≤ 6.0 | ≤ 7.5 | ≤ 6.5 |
|  | 16 QAM | ≤ 5.5 | ≤ 6.5 | ≤ 5.5 | ≤ 5.5 | ≤ 6.5 | ≤ 5.5 | ≤ 7.5 | ≤ 6.5 | ≤ 5.5 | ≤ 5.5 | ≤ 6.0 | ≤ 7.5 | ≤ 6.5 |
|  | 64 QAM | ≤ 5.5 | ≤ 6.5 | ≤ 5.5 | ≤ 5.5 | ≤ 6.5 | ≤ 5.5 | ≤ 7.5 | ≤ 6.5 | ≤ 5.5 | ≤ 5.5 | ≤ 6.0 | ≤ 7.5 | ≤ 6.5 |
|  | 256 QAM | ≤ 7.5 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.5 | ≤ 7.0 | ≤ 6.5 | ≤ 7.0 | ≤ 7.0 | ≤ 7.5 | ≤ 7.0 |

Proposal 6: Merge region A1, A2, A5 and A10 into one unified region with using their biggest A-MPR values for NS_05 PC2.

Proposal 7: Merge region A3, A6 and A11 into one unified region with using their biggest A-MPR values for NS_05 PC2.

Proposal 8: Merge region A7 and A12 into one unified region with using their biggest A-MPR values for NS_05 PC2.

Proposal 9: Merge region A8 and A13 into one unified region with using their biggest A-MPR values for NS_05 PC2.

- 3. Conclusion

Proposal 1: Consider the following Table 1 and 2 as PC3 A-MPR for NS_05.

Table 1: A-MPR regions for NS_05 region split (Power Class 3)

| Channel Bandwidth (MHz) | Carrier Centre Frequency, Fc (MHz) | Region A |  |  | Region B |  |  | Region C |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | LCRB | RBstart | A-MPR | LCRB | RBstart | A-MPR | LCRB | RBstart | A-MPR |
| 5 | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| 10 | < 1930.7 | ≥ 1.35*RBstart + 4.32MHz/12/scs | ≥ 0 | A1 | ≤ 2.16MHz/12/scs | ≤ 1.8MHz/12/scs | A2 | N/A | N/A | N/A |
| 15 | 1927.5 ≤ fc < 1932.5 | ≥ RBstart*1.2 + 4.68MHz/12/scs | ≥ 0 | A3 | ≤ 3.96MHz/12/scs | 0 | A4 | ≤ 3.96MHz/12/scs | ≤ 3.6MHz/12/scs; > 0 | A5 |
| 15 | 1932.5 ≤ fc < 1938.2 | ≥ 8.28/12/scs | ≤ 0.54/12/scs | A6 | ≤ 2.16MHz/12/scs | ≤ 1.62MHz/12/scs | A7 | ≤ 2.16MHz/12/scs | > 1.62MHz/12/scs ≤ 2.16MHz/12/scs | A8 |
| 20 | < 1945.7 | ≥ RBstart*1.5 + 4.14MHz/12/scs; | ≥ 0 | A9 | ≤ 5.76MHz/12/scs; ≤ RBstart*1.5 + 4.14MHz/12/scs; | ≤ 1.8MHz/12/scs | A10 | ≤ 5.76MHz/12/scs | > 1.8MHz/12/scs ≤ 4.68MHz/12/scs | A11 |

■IMD5_1906.6-1915.7 MHz: -25 dBm/MHz, ■IMD5_ 1884.5-1906.6 MHz: -30 dBm/MHz, ■TBD(will be proposed in Aug.)

■Could be harmonized as common value

Table 2: A-MPR values for NS_05 region split (Power Class 3, 20MHz)

| Modulation/Waveform |  | A1 (dB) | A2 (dB) | A3 (dB) | A4 (dB) | A5 (dB) | A6 (dB) | A7 (dB) | A8 (dB) | A9 (dB) | A10 (dB) | A11 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3 | ≤ 2 | ≤ 3 | [xx] | [xx] | ≤ 2 | [xx] | [xx] | ≤ 3.5 | ≤ 3 | ≤ 2.0 |
|  | QPSK | ≤ 4 | ≤ 3 | ≤ 4 | [xx] | [xx] | ≤ 3 | [xx] | [xx] | ≤ 4 | ≤ 3.5 | ≤ 2.5 |
|  | 16 QAM | ≤ 4 | ≤ 3 | ≤ 4.5 | [xx] | [xx] | ≤ 3 | [xx] | [xx] | ≤ 5 | ≤ 4 | ≤ 2.5 |
|  | 64 QAM | ≤ 4.5 | ≤ 3.5 | ≤ 4.5 | [xx] | [xx] | ≤ 3 | [xx] | [xx] | ≤ 5 | ≤ 4 | ≤ 3.0 |
|  | 256 QAM | ≤ 5 | ≤ 4.5 | ≤ 5 | [xx] | [xx] | ≤ 4.5 | [xx] | [xx] | ≤ 5 | ≤ 4.5 | ≤ 4.5 |
| CP-OFDM | QPSK | ≤ 5 | ≤ 4.5 | ≤ 6 | [xx] | [xx] | ≤ 4.5 | [xx] | [xx] | ≤ 6 | ≤ 5.5 | ≤ 4.0 |
|  | 16 QAM | ≤ 5 | ≤ 4.5 | ≤ 6 | [xx] | [xx] | ≤ 4.5 | [xx] | [xx] | ≤ 6 | ≤ 5.5 | ≤ 4.0 |
|  | 64 QAM | ≤ 5 | ≤ 4.5 | ≤ 6 | [xx] | [xx] | ≤ 4.5 | [xx] | [xx] | ≤ 6 | ≤ 5.5 | ≤ 4.5 |
|  | 256 QAM | ≤ 6.5 | ≤ 6.5 | ≤ 6.5 | [xx] | [xx] | ≤ 6.5 | [xx] | [xx] | ≤ 7 | ≤ 6.5 | ≤ 6.5 |

Proposal 2: Merge region A1, A3 and A9 into one unified region with using their biggest A-MPR values for NS_05 PC3.

Proposal 3: Merge region A4 and A10 into one unified region with using their biggest A-MPR values for NS_05 PC3.

Proposal 4: Merge region A2, A5, A7 and A11 into one unified region with using their biggest A-MPR values for NS_05 PC3.

Proposal 5: Consider the following Table 3 and 4 as PC2 A-MPR for NS_05.

Table 3: A-MPR regions for NS_05 (Power Class 2)

| Channel Bandwidth (MHz) | Carrier Centre Frequency, Fc (MHz) | Region A |  |  | Region B |  |  | Region C |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | LCRB | RBstart | A-MPR | LCRB | RBstart | A-MPR | LCRB | RBstart | A-MPR |
| 5 | < 1925 | ≥ 3.24(18RB) | ≤ 0.18 | A1 | N/A | N/A | N/A | N/A | N/A | N/A |
| 10 | ≤ 1925 | ≥ 1.35*RBstart + 4.32MHz/12/scs | ≥ 0 | A2 | ≤ 2.16MHz/12/scs | ≤ 1.8MHz/12/scs | A3 | < 1.35* RBstart*12*SCS + 4.32 ≥ 1.2* RBstart*12*SCS + 3.6 | ≥ 0 | A4 |
| 10 | 1925 < fc < 1930.7 | ≥ 1.35*RBstart + 4.32MHz/12/scs | ≥ 0 | A5 | ≤ 2.16MHz/12/scs | ≤ 1.8MHz/12/scs | A6 | N/A | N/A | N/A |
| 15 | 1927.5 ≤ fc <1932.5 | ≥ RBstart*1.2 + 3.96MHz/12/scs | ≥ 0 | A7 | ≤ RBstart*1.2 + 3.96MHz/12/scs ≤ 4.5MHz/12/scs | ≤ 3.6MHz/12/scs | A8 | ≤ RBstart*1.2 + 3.96MHz/12/scs ≥ RBstart*12*SCS * + 3.6 > 4.5MHz/12/scs | ≥ 0 | A9 |
| 15 | 1932.5 ≤ fc < 1938.2 | ≥ 8.28MHz/12/ scs | ≤ 1.8MHz/12/scs | A10 | ≤ 2.16MHz/12/scs | ≤ 2.16MHz/12/scs | A11 | N/A | N/A | N/A |
| 20 | < 1945.7 | ≥ 1.0*12*SCS*RBstart + 4.14MHz/12/scs | ≥ 0 | A12 | ≤ 6.48MHz/12/scs; < RBstart*1.0 + 4.14MHz/12/scs | ≤ 4.68MHz/12/scs | A13 | N/A | N/A | N/A |

■IMD5_1906.6-1915.7 MHz: -25 dBm/MHz, ■IMD5_ 1884.5-1906.6 MHz: -30 dBm/MHz, ■TBD(will be proposed in Aug.)

■,,■Could be harmonized as common value

Table 4: A-MPR values for NS_05 (Power Class 2)

| Modulation/Waveform |  | A1 (dB) | A2 (dB) | A3 (dB) | A4 (dB) | A5 (dB) | A6 (dB) | A7 (dB) | A8 (dB) | A9 (dB) | A10 (dB) | A11 (dB) | A12 (dB) | A13 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3.5 | ≤ 4.5 | ≤ 3.0 | ≤ 3.0 | ≤ 4.5 | ≤ 3.0 | ≤ 4.5 | ≤ 4.0 | ≤ 3.5 | ≤ 3.5 | ≤ 3.5 | ≤ 4.5 | ≤ 4.0 |
|  | QPSK | ≤ 4.0 | ≤ 5.5 | ≤ 4.0 | ≤ 4.0 | ≤ 5.5 | ≤ 4.0 | ≤ 5.5 | ≤ 4.5 | ≤ 4.0 | ≤ 4.5 | ≤ 4.0 | ≤ 5.5 | ≤ 4.5 |
|  | 16 QAM | ≤ 4.5 | ≤ 6.0 | ≤ 4.5 | ≤ 4.5 | ≤ 6.0 | ≤ 4.5 | ≤ 6.0 | ≤ 5.0 | ≤ 4.5 | ≤ 4.5 | ≤ 4.5 | ≤ 6.5 | ≤ 5.0 |
|  | 64 QAM | ≤ 4.5 | ≤ 6.0 | ≤ 4.5 | ≤ 4.5 | ≤ 6.0 | ≤ 4.5 | ≤ 6.0 | ≤ 5.0 | ≤ 4.5 | ≤ 5.0 | ≤ 4.5 | ≤ 6.5 | ≤ 5.5 |
|  | 256 QAM | ≤ 5.5 | ≤ 6.0 | ≤ 5.0 | ≤ 5.0 | ≤ 6.0 | ≤ 5.0 | ≤ 6.0 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.0 | ≤ 6.5 | ≤ 5.5 |
| CP-OFDM | QPSK | ≤ 5.5 | ≤ 6.5 | ≤ 5.5 | ≤ 5.5 | ≤ 6.5 | ≤ 5.5 | ≤ 7.5 | ≤ 6.5 | ≤ 5.5 | ≤ 5.5 | ≤ 6.0 | ≤ 7.5 | ≤ 6.5 |
|  | 16 QAM | ≤ 5.5 | ≤ 6.5 | ≤ 5.5 | ≤ 5.5 | ≤ 6.5 | ≤ 5.5 | ≤ 7.5 | ≤ 6.5 | ≤ 5.5 | ≤ 5.5 | ≤ 6.0 | ≤ 7.5 | ≤ 6.5 |
|  | 64 QAM | ≤ 5.5 | ≤ 6.5 | ≤ 5.5 | ≤ 5.5 | ≤ 6.5 | ≤ 5.5 | ≤ 7.5 | ≤ 6.5 | ≤ 5.5 | ≤ 5.5 | ≤ 6.0 | ≤ 7.5 | ≤ 6.5 |
|  | 256 QAM | ≤ 7.5 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.5 | ≤ 7.0 | ≤ 6.5 | ≤ 7.0 | ≤ 7.0 | ≤ 7.5 | ≤ 7.0 |

Proposal 6: Merge region A1, A2, A5 and A10 into one unified region with using their biggest A-MPR values for NS_05 PC2.

Proposal 7: Merge region A3, A6 and A11 into one unified region with using their biggest A-MPR values for NS_05 PC2.

Proposal 8: Merge region A7 and A12 into one unified region with using their biggest A-MPR values for NS_05 PC2.

Proposal 9: Merge region A8 and A13 into one unified region with using their biggest A-MPR values for NS_05 PC2.

- 4. Reference

[1] R4-2418439 (NR_newRAT-Core) NR A-MPR reevaluation scenarios on n1 for Japan, KDDI, RAN4#113

[2] R4-2508023 WF on PHS protection update, KDDI, RAN4#115

[3] R4-2505222 WF on PHS protection update, KDDI, RAN4#114bis

[4] R4-2502980 WF on UE requirements for PHS protection, KDDI, RAN4#114

- 5. Annex