3GPP TSG-RAN WG4 Meeting #116	R4-2509646

Bengaluru, India, August 25th – 29th, 2025

Source:	Tejas Networks

Title:	Study of RAN4 impacts for RRM measurement prediction for AIML mobility in NR

Agenda item:	7.19.3.1

Document for:	Discussion

Introduction

The discussions on AI/ML mobility in NR were continued in RAN4#115 in the scope of Rel-19 SI. Agreements achieved during this meeting are summarized in the WF [1].

Discussion

During the RAN4 #112bis meeting, RAN4 agreed to study aspects related to NR AIML mobility requirements. In this paper, we provide our views on issues related to factors that potentially impact RAN4 requirements.

Relative RSRP accuracy definition

Agreement: - For intra-frequency L3-RSRP, - Relative accuracy of predicted L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), - cell 1 and cell 2 are on the same frequency - the reported RSRP of cell2 can be measured or predicted. - FFS on whether and how to consider time difference between the two reported RSRPs when applying the definition of the relative RSRP accuracy - Relative RSRP accuracy for Beam level measurements is FFS during WI phase depending upon RAN2 progress. - For inter-frequency L3-RSRP, - Relative accuracy of predicted L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), - cell 2 is on a different frequency than cell 1 but in the same FR as cell 1 - the reported RSRP of cell2 can be measured or predicted. - FFS on whether and how to consider time difference between the two reported RSRPs when applying the definition of the relative RSRP accuracy

For FR2 testing, since it involves a radiated test setup, RAN4 has agreed that the reported measurements will be used for ground truth extraction. This implies that in the current definition of relative RSRP accuracy:

Relative accuracy of predicted L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2)

the reported and ground truth values would be the same. As a result, the formula effectively reduces to an absolute RSRP definition, making the relative RSRP definition meaningless for FR2. Therefore, RAN4 should clarify the applicability of the relative RSRP definition for FR2.

Observation 1: For FR2, the reported measurements will be used for ground truth extraction.

Proposal 1: RAN4 to clarify the applicability of relative RSRP definition in the case FR2 testing.

Similarly, for FR1, the method for ground truth extraction is still under discussion. The possible options considered are:

Alt 1: Ground truth for FR1 is based on the transmitted or received power.

Alt 2: Ground truth for FR1 is based on the reported measurement value under certain conditions.

If RAN4 agrees on Alt 2, the same issue identified for FR2 will also apply to FR1. The reported and ground truth values will be identical, and the relative RSRP definition will effectively become obsolete. RAN4 should clarify the applicability of the relative RSRP definition for FR1 as well.

Proposal 2: RAN4 to clarify the applicability of relative RSRP definition in the case FR1 testing.

Inter-frequency prediction scenario

Agreement: - RAN4 to study the factors potentially impacting RSRP prediction accuracy for inter-frequency prediction scenario considering at least the following aspects: - the side condition of frequency prediction (e.g., EPRE difference) - the impact of correlation coefficient between measured and predicted frequency layers - impact of cluster approach, e.g., - When measurement from single cell in one carrier frequency is used by the UE as an input to predict the RRM measurement for the intra-FR and co-located cell in another carrier frequency. - When measurement from a group of cells in one carrier frequency is used by the UE as an input to predict the RRM measurement for the intra-FR and co-located cell in another carrier frequency.

FR1 to FR1 inter frequency measurement prediction based on AIML refers to predicting the measurements of a target cell at a different frequency based on the measurements from another cell. RAN4 has agreed that the inputs used to predict the measurements of Cell may themselves be either directly measured or predicted.

However, relying on a predicted result to predict another cell’s measurements may lead to reduced accuracy and should not compromise the prediction accuracy requirements. Therefore, it is important to identify the specific prediction method used for input measurements. If the prediction method used is based on domains other than inter-frequency prediction, such as temporal domain prediction or spatial domain prediction, this approach effectively constitutes a combined domain prediction. In such cases, it is worth considering whether other domain predictions, such as time-domain or spatial-domain techniques, can be effectively leveraged. If so, the combined domain prediction approach should be evaluated, as it has the potential to enhance prediction accuracy by integrating complementary prediction domains.

Proposal 3: RAN4 should explore potential enhancements in inter-frequency and spatial domain measurement prediction by combining them with temporal domain prediction.

As per the RAN2 study, using a group of cells in one carrier frequency as input for prediction can improve RRM measurement accuracy compared to using a single cell. However, defining separate performance requirements for cluster-based and non-cluster-based approaches may not be practical or beneficial. Standardizing the number of cells in a cluster is challenging, as intra-frequency neighbour cells may not always be available due to varying network deployments, UE mobility, and real-time channel conditions.

Additionally, the decision to use a cluster-based or non-cluster-based prediction model should be left to the implementation. RAN4 should focus on defining the minimum requirements necessary to ensure accurate mobility event predictions.

Proposal 4: RAN4 not study the impact of cluster-based and non-cluster-based inter-frequency prediction scenarios.

Proposal 5: No need to define separate requirement for clustered and non-clustered approach in inter frequency prediction scenario.

Conclusion

This section summarizes our discussion by listing all the observations and proposals discussed in this contribution.

Observation 1: For FR2, the reported measurements will be used for ground truth extraction.

Proposal 1: RAN4 to clarify the applicability of relative RSRP definition in the case FR2 testing.

Proposal 2: RAN4 to clarify the applicability of relative RSRP definition in the case FR1 testing.

Proposal 3: RAN4 should explore potential enhancements in inter-frequency and spatial domain measurement prediction by combining them with temporal domain prediction.

Proposal 4: RAN4 not study the impact of cluster-based and non-cluster-based inter-frequency prediction scenarios.

Proposal 5: No need to define separate requirement for clustered and non-clustered approach in inter frequency prediction scenario.

- References

- R4-2508268, WF on AI/ML Mobility Part 1 (Nokia), Malta, Malta, May 19th – May 23rd, 2025.

- RP-250970, Status Report to TSG, Prague, Czech Republic, June 9-13, 2025.

- R4-2415968, Work Plan for RAN4 Discussions on AI/ML for Mobility in NR, Nokia, OPPO, MediaTek, Hefei, China, Oct 14 – Oct 18, 2024