3GPP TSG-RAN WG4 # 116	R4-2509654

Bengaluru, India, 25th August 2025 – 29th August 2025

Agenda item:	7.12.3

Source:	MediaTek Inc.

Title:	TP to TR38.753: TDL approaches and related Annex

Document for:	Approval

# Introduction

This TP handles sections of “TDL approaches” and “Annex B TDL approaches”.

# Reference

[1]	RP-241610 SID: Study on spatial channel model for demodulation performance requirements

# Text proposal to TR 38.753

The following TP to TR 38.753 is proposed for approval.

-------------------------------------------------- Start of text proposal --------------------------------------------------

## 5.2	TDL Approaches

The novel multi-cluster TDL approach builds on top of the widely used and well tested legacy RAN4 TDL channel models. In the legacy models, spatial selectivity is introduced by applying MIMO correlation matrices on top of uncorrelated fading MIMO channel. However, the legacy correlation models (TS 38.101-4: Annex B.2.3) amplify the signal only in the broadside direction (AOD/AOA) of the TX and RX antenna arrays. As a result, the spatial degrees of freedom remain limited, and only a relatively low number of spatial layers can be supported. In order to support a higher number of layers, multiple separate spatial signal directions should exist. This can be achieved by introducing spatial clusters, where each cluster has its own separable mean AOD/AOA.

The multi-cluster TDL models aim to be an easier-to-use and easier-to-configure alternative to CDL models. There is just a handful of high-level parameters that can be chosen to achieve desired channel properties, and to match the scope of different performance tests. To further improve the test coverage, the spatial properties of the clusters (in terms of AOA/AOD) could be defined to be time-varying.

### 5.2.1	MIMO correlation and angular spread

For both LTE and NR, RAN4 demodulation and CSI reporting tests until 3GPP Release 18 employ TDL fading channel models. In many of those tests, spatial selectivity is introduced by applying MIMO correlation matrices on top of an uncorrelated fading MIMO channel. Correlation models are defined for two different antenna array assumptions: Uniform Linear Array (ULA) and Cross-polarized Antennas (X-pol).

MIMO correlation (TDL) and angular spread (CDL) approaches are alternative ways to generate spatial selectivity. High angular spread corresponds to low MIMO correlation and vice versa. As can be seen in Figure 5.2.1-1, a typical 3GPP correlation model generates spatial selectivity that resembles that of a single-cluster CDL channel model. As a result of the real-valued spatial correlation matrix, the strongest beam direction is always at 0⁰ AOD/AOA angle (broadside) of each linear array dimension.

The spatial selectivity of a MIMO channel can be depicted as power angular distribution (PAD) that represents the average channel power response as function of TX or RX direction. Figure 5.2.1-1(a) illustrates the (PAD) measured from an 8TX-8RX X-pol medium correlated TDL channel model (α1=0.3, β=0.6). As can be seen, this channel favors the AODs and AOAs close to zero degrees. The PAD at the TX side is slightly wider than at the RX side because the TX-correlation is lower than the RX-correlation (α < β). Similarly, Figure 5.2.1-1(b) shows the PAD measured from a single-cluster 8TX-8RX CDL channel model with mean AOA = 0 degrees. Here, the angular spread is at the TX side CASD = 1.3710 and at the RX side CASA = 15.632 degrees, and the assumed antenna element radiation patterns is flat.

(a)                                                                                          (b)

Figure 5.2.1-1: Spatial selectivity of X-pol correlated TDL channel (a) and single-cluster CDL channel (b).

### 5.2.2	Multi-cluster TDL channel via TX-RX beam steering

The multi-cluster TDL channel model re-uses and extends the concept of beam steering that has earlier been specified for RAN4 PMI reporting tests ([X] TS 38.101-4: B.2.3.2.3 and B.2.3.2.3A). In TX-RX beam steering, the average AOA and AOD present in a correlated TDL MIMO channel are shifted to desired directions by post-processing. Note that beam steering is a part of the channel model, it is not an action taken by the transmitter or the receiver.

An example of multi-cluster channel is illustrated in Figure 5-2.

Figure 5.2.2-1: TX-RX beam steering channel with two spatial clusters.

The general channel model employs one or more statistically independent time-varying legacy TDL channel instances using some existing spatial correlation model. For the purpose of TX and RX beam steering, the delay taps of each TDL channel instance are split into spatial clusters. The delay taps in a spatial cluster are steered corresponding to common cluster-specific AOD/AOA angles. For mathematical notation, let n(k,m) denote the cluster index corresponding to delay tap m of TDL channel instance k.

The MIMO channel impulse response matrix with dimensions Nrx× Ntx at some time instant is

Hτ=k=1Km=1MPk∙pm∙Dn(k,m)(rx)Hk,mDn(k,m)(tx)∙δτ-τm                                (5.2.2-1)

where

k is the TDL channel instance index, m is the delay tap index, and τ is the delay variable

K is the number of legacy TDL channel instances, and M is the number of delay taps in the TDL model

pm is the power of delay tap m from the TDL power-delay profile, and Pk the power weight of TDL model instance k so that kmPk∙pm=1

τm is the propagation delay of tap m

Hk,m is the spatially correlated fading MIMO channel matrix of delay tap m of TDL channel instance k with unit average power response and dimensions Nrx× Ntx

Dn(k,m)(rx) is the diagonal RX steering matrix of cluster n(k,m) with dimensions Nrx× Nrx

Dn(k,m)(tx) is the diagonal TX steering matrix of cluster n(k,m) with dimensions Ntx× Ntx

δτ is the Dirac delta function.

The implementation complexity of the channel model grows with the number of underlying TDL channel instances K. It is anticipated that one or two TDL channels is adequate for most testing purposes. Due to the diagonal structure of the steering matrices, the matrix multiplications in equation 5.2.2-1 can be implemented with low complexity as element-wise complex phase rotations of matrix Hk,m.

#### 5.2.2.1	Steering matrices for cross-polarized antenna arrays

Steering parameters with cross-polarized (X-pol) antenna array assumption:

- gNB TX-array dimensions:

- Ntx1 and Ntx2 are the number of antenna elements with the same polarization in the first and the second array dimension. The total number of TX antennas is

- Ntx=2∙Ntx1∙Ntx2.

- UE RX-array dimensions:

- Nrx1 and Nrx2 are the number of antenna elements with the same polarization in the first and the second array dimension. Nrx2=1The total number of TX antennas is Nrx=2∙Nrx1∙Nrx2. Note that 3GPP X-pol model (TS 38.101-4 Annex B.2.3.2) supports only 1D RX-arrays so that Nrx2=1.

- TX phases: θtx1(n) and θtx2(n) per cluster n

- RX phases: θrx1(n) and θrx2(n) per cluster n

Nrx=2∙Nrx1∙Nrx2

TX phases: θtx1(n) and θtx2(n) per cluster n

RX phases: θrx1(n) and θrx2(n) per cluster n

The steering matrices are defined as:

Dn(tx)=1001⊗Dθtx1(n),Ntx1⊗Dθtx2(n),Ntx2

Dn(rx)=1001⊗Dθrx1(n),Nrx1⊗Dθrx2(n),Nrx2

where the steering matrix of size N× N as function of phase θ is defined as

Dθ,N=diag1ejθ⋯ejθ(N-1)

Assuming the antenna arrays employ uniform distance between elements in each linear dimension, the phase shift θ corresponds to AOA or AOD angle φ so that θφ,dλ=2π∙dλ∙sinφ, where d is the distance between adjacent antenna elements and λ is the carrier wavelength.

The steering phases θtx1(n), θtx2(n), θrx1(n), θrx2(n) may be specified either directly or by specifying the corresponding directional angles φtx1(n), φtx2(n), φrx1(n), φrx2(n).

The beam steering phases can be either constant or (slowly) time-varying. In general, the model parameters can be chosen to match the scope of each test case. For example, the number of clusters can be selected so that the desired number of layers (rank) is supported. Optionally, cluster-specific Doppler shifts can also be specified.

### 5.2.3	Example cluster models

In this section, example parameterizations of four-cluster channels employing a varying number of TDLC channel instances are given. Model B1 with four TDLC instances is presented in Table 5.2.3-1, model B2 with one TDLC instance is given in Table 5.2.3-2, and model B3 with two TDLC instances is given in Table 5.2.3-3. The three models share the same cluster-specific beam steering phases, and they also have similar cluster power distributions where the first two clusters are strong and the last two clusters are weak. Therefore, all three models have similar spatial properties.

In model B1, each cluster employs its own TDLC channel instance. In model B2, the delay taps of a single TDLC instance are split into 4 clusters. In model B3, the two TDLC instances are each further split into two clusters so that delay taps 1-4 are grouped into one and taps 5-12 into another cluster.

Assuming 8TX-8RX array setup with (Ntx1, Ntx2, Nrx1, Nrx2)= (4, 1, 4, 1), the steering phases both at TX and RX side are chosen so that the TX and RX beams become mutually orthogonal between clusters. Orthogonality between the beams maximizes the spatial multi-layer capacity of the channel for a given set of power weights.

The cluster power distribution also affects the spatial properties of the channel. For a set of orthogonal beams, assigning equal power for all clusters results in the highest channel capacity. The total power of a cluster depends on the power weight of the corresponding TDL channel instance, and on the sum power of the delay taps included in the cluster. Thus, the cluster power distribution can be adjusted by two different methods: by setting the power weights and by splitting the delay taps into clusters so that a desired distribution is achieved.

Table 5.2.3-1: Cluster model B1: 4 TDLC instances, 4 clusters

| Delay profile / correlation | Cluster index | Power (dB) | θtx1 | θrx1 |
| --- | --- | --- | --- | --- |
| TDLC-300 / XP-high | 1 | 0 | 0 | 0 |
| TDLC-300 / XP-high | 2 | 0 | -π/2 | -π/2 |
| TDLC-300 / XP-high | 3 | -9 | π | π |
| TDLC-300 / XP-high | 4 | -9 | π/2 | π/2 |

Table 5.2.3-2: Cluster model B2: 1 TDLC channel instance, 4 clusters

| TDLC-300 / XP-high |  |  | TX-RX beam steering |  |  |
| --- | --- | --- | --- | --- | --- |
| Tap | Delay (ns) | Tap power (dB) | Cluster index | θtx1 | θrx1 |
| 1 | 0 | -6.9 | 1 | 0 | 0 |
| 2 | 65 | 0 |  |  |  |
| 3 | 70 | -7.7 | 2 | -π/2 | -π/2 |
| 4 | 190 | -2.5 |  |  |  |
| 5 | 195 | -2.4 |  |  |  |
| 6 | 200 | -9.9 |  |  |  |
| 7 | 240 | -8.0 |  |  |  |
| 8 | 325 | -6.6 |  |  |  |
| 9 | 520 | -7.1 | 3 | π | π |
| 10 | 1045 | -13.0 |  |  |  |
| 11 | 1510 | -14.2 | 4 | π/2 | π/2 |
| 12 | 2595 | -16.0 |  |  |  |

Table 5.2.3-3: Cluster model B3: 2 TDLC channel instances, 4 clusters

| TDLC-300 / XP-high |  |  | TX-RX beam steering |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tap | Delay (ns) | Tap power (dB) | TDL channel 1 Power: 0 dB |  |  | TDL channel 2 Power: -9 dB |  |  |
|  |  |  | Cluster index | θtx1 | θrx1 | Cluster index | θtx1 | θrx1 |
| 1 | 0 | -6.9 | 1 | 0 | 0 | 3 | π | π |
| 2 | 65 | 0 |  |  |  |  |  |  |
| 3 | 70 | -7.7 |  |  |  |  |  |  |
| 4 | 190 | -2.5 |  |  |  |  |  |  |
| 5 | 195 | -2.4 | 2 | -π/2 | -π/2 | 4 | π/2 | π/2 |
| 6 | 200 | -9.9 |  |  |  |  |  |  |
| 7 | 240 | -8.0 |  |  |  |  |  |  |
| 8 | 325 | -6.6 |  |  |  |  |  |  |
| 9 | 520 | -7.1 |  |  |  |  |  |  |
| 10 | 1045 | -13.0 |  |  |  |  |  |  |
| 11 | 1510 | -14.2 |  |  |  |  |  |  |
| 12 | 2595 | -16.0 |  |  |  |  |  |  |

# C.1	TDL Approaches

## C.1.1	Multi-cluster TDL channel for 2-CW fixed-MCS testing

As one example, testing of two-codeword (up to 8 layers) MIMO demodulation – so that consistently different channel quality per CW is maintained – has turned out to be challenging with current RAN4 correlation models. This section presents examples how to construct PDSCH demodulation tests with fixed rank, fixed Type I PMI, and different fixed MCS per CW.

### C.1.1.1	Example 1

For fixed PMI testing, the per-cluster phase shifts: θtx1(n) and θtx2(n) can be chosen to be fixed and to match the beams of the transmit PMI. In Type I codebook, each CW often maps to a subset of the orthogonal PMI-beams. This is illustrated below for the 5-layer precoder (TS 38.214: 5.2.2.2.1). Consequently, per-CW channel quality levels can be adjusted by adjusting the cluster-specific channel powers.

To minimize cross-layer/CW interference at the UE, the RX-phases θrx1(n) can be chosen so that the corresponding steering vectors are orthogonal. Alternatively, by choosing non-orthogonal RX steering vectors, cross-layer/CW interference can be increased if so desired. Finally, the selection of the X-pol spatial correlation model parameters (α1, α2, β) determines how strictly the mean spatial directions are maintained. Higher correlation is analogous to narrower angular spread per cluster.

Figure C.1.1.1-1: Illustration of a codebook for 5-layer CSI reporting

To demonstrate the concept, a rank 5 PDSCH setup as detailed in Table C.1.1.1-1 was simulated in the channel model defined in Table C.1.1.1-2. The results are depicted in Figure C.1.1.1-1. Here, CW0 employs 16QAM while CW1 has QPSK, but as clusters corresponding to CW0 are 6dB stronger, the BLER performances of the two CWs are almost on par. Thus, it can be concluded that CW-specific TX-RX beam steering clusters can effectively be used to scale CW-specific demodulation and decoding performance.

Table C.1.1.1-1: Simulation setup

| Antenna setup: Ntx1, Ntx2, Nrx1, Nrx2 | 2, 2, 4, 1 |
| --- | --- |
| Rank 5 Type1 TX-PMI: i1,1, i1,2, i2 1st dim PMI beam indexes: l, l’, l” 2nd dim PMI beam indexes: m, m’, m” | 0, 0, 0 0, O1, O1 => phase 2πl/O1N1 = 0, π, π 0, 0, O2 => phase 2πm/O2N2 = 0, 0, π |
| MCS / CW0 MCS / CW1 | MCS13, 16-QAM, code rate 0.48 MCS7, QPSK, code rate 0.51 |

Table C.1.1.1-2: Cluster model with 3 TDL-A channel instances and 3 clusters

| TDLA-30 delay profile 10Hz Doppler XP-high correlation | TX-RX beam steering |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TDL channel 1 / cluster 1 Power: 0 dB |  |  | TDL channel 2 / cluster 2 Power: -6 dB |  |  | TDL channel 3 / cluster 3 Power: -6 dB |  |  |
|  | θtx1 | θtx2 | θrx1 | θtx1 | θtx2 | θrx1 | θtx1 | θtx2 | θrx1 |
|  | 0 | 0 | 0 | -π | 0 | π/2 | -π | -π | π |

Figure C-.1.1.1-2: 2: CW fixed MCS case in three-cluster TDL-A channel.

### C.1.1.2	Example 2

A model with just one TDL channel instance can also satisfy the 2-CW fixed MCS testing requirements by setting mapping relation between clusters and taps to control the 2CWs power and delay difference in the reasonable level. Take channel model in Table C.1.1.2-1 as an example, where cluster 3 and cluster 4 correspond to last four taps, leading to large power and delay difference with cluster 1 and 2. Specific precoder mapping the 1st CW to first two clusters and 2nd CW to second two clusters leads to large SINR difference between two CWs. In the case, different MCSs could be configured to match performance difference.

Table C.1.1.2-1: Cluster model with one TDL-C channel instance and 4 clusters

| TDLC-300 / XP-high / 100Hz Doppler |  |  | TX-RX beam steering |  |  |
| --- | --- | --- | --- | --- | --- |
| Tap | Delay (ns) | Tap power (dB) | Cluster index | θtx1 | θrx1 |
| 1 | 0 | -6.9 | 1 | 0 | 0 |
| 2 | 65 | 0 |  |  |  |
| 3 | 70 | -7.7 | 2 | -π/2 | -π/2 |
| 4 | 190 | -2.5 |  |  |  |
| 5 | 195 | -2.4 |  |  |  |
| 6 | 200 | -9.9 |  |  |  |
| 7 | 240 | -8.0 |  |  |  |
| 8 | 325 | -6.6 |  |  |  |
| 9 | 520 | -7.1 | 3 | -π | -π |
| 10 | 1045 | -13.0 |  |  |  |
| 11 | 1510 | -14.2 | 4 | π/2 | π/2 |
| 12 | 2595 | -16.0 |  |  |  |

A simulation is performed to verify the characteristic with simulation assumptions captured in Table C.1.1.2-2. For rank 8 with fixed Type I Single panel precoder i1,1=i1,2=i2=0, the 2nd and the 3rd cluster are mapped to the last four channel taps with low power and large delay. This leads to a big performance difference between two codewords as shown in Figure C.1.1.2-1.

Table C1.1.2-2: Simulation setup

| Parameters | Values |
| --- | --- |
| Antenna setup: Ntx1, Ntx2, Nrx1, Nrx2 | (4,1,4,1) for 8TX-8RX (1-dimensional antenna arrays) |
| Precoder | Type I Single panel, Fixed precoder i1,1=i1,2=i2=0 |
| Rank | 8 |
| MCS | 13 |

Figure C.1.1.-2-1: 2-CW fixed MCS case in four-cluster TDL-C channel

-------------------------------------------------- End of text proposal --------------------------------------------------