3GPP TSG-RAN WG4 Meeting #116	R4-2509655

Bengaluru, India, 25th August 2025 – 29th August 2025

Agenda item:	7.17.7

Source:	MediaTek Inc.

Title:	Discussion on CSI reporting requirements for CSI prediction

Document for:	Discussion

# Introduction

In RAN4#115, RAN4 agreed to continue discussion on CSI reporting requirements for CSI prediction issues [1]. Also, updated simulation assumptions for AI/ML CSI prediction were agreed [2]. In this contribution, we provide our views and simulation results on CSI reporting requirement framework for CSI prediction.

# Discussion

In this Chapter, we provide our views on CSI prediction requirement issues and share simulation results.

Way forward on simulation assumptions for CSI prediction was agreed in the previous meeting [2]. In Table 1 we list used simulation parameters. Compared to assumptions, we have limited our simulation scope to FDD 10MHz bandwidth with 15kHz SCS. For Step-1 and Step-2, we provide simulation results for all previously agreed Dopplers (20Hz, 50Hz, and 100Hz) and both agreed TX configurations (32 (N1, N2) = (4, 4), and 16 (N1, N2) = (4, 2)). Step-1 and Step-2 datasets are generated with link level simulator using ideal channel estimation. For Step-3, we provide simulation results for both agreed Dopplers (20Hz and 50Hz) and one of agreed TX configurations (16 (N1, N2) = (4, 2)). Step-3 datasets are generated with link level simulator using practical channel estimation. Training datasets consist of channel realizations of all agreed Dopplers separately or mixed. Testing datasets consist of independent channel realizations of all agreed Dopplers (i.e. channel realizations are different from those used for model training).

Companies were encouraged to provide the model complexity information of their AI/ML CSI prediction models to help alignment. For this evaluation purpose we have used AI models that have <10k parameters and require <2 MFLOPs per inference for both 16Tx and 32Tx model.

Table 1: Simulation parameters for AI/ML based CSI prediction

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | FDD, OFDM |
| Carrier frequency | 2GHz for FDD |
| Bandwidth | 10MHz for FDD |
| Symbol | 1 symbol |
| Number of subbands | 13 subbands, 4RB as a bundle for FDD |
| Subcarrier spacing | 15kHz for 2GHz |
| gNB TX antennas | 32 (N1, N2) = (4, 4) as defined in Rel-15 type1 16 (N1, N2) = (4, 2) as defined in Rel-18 CSI prediction requirements |
| UE RX antennas | 4 |
| Channel model | TDLA30 as baseline with XP medium correlation |
| Doppler spread | 20Hz, 50Hz, 100Hz |
| Delay spread | 30ns |
| Channel estimation | Ideal DL channel estimation (step-1, step-2) / Practical channel estimation (step-3) |
| Rank per UE | Rank 1 and 2 |
| CSI feedback assumption | Check the CSI prediction performance of raw channel per layer Check the CSI prediction performance with Rel-18 codebook quantization |
| KPI for Step-1 | Average of SGCS for intermediate results over all subbands per layer SGCS1: Compares the SVD of model output (before feedback) with the SVD of the ground truth Raw channel |
| KPI for Step-2 | Average of SGCS for intermediate results over all subbands per layer SGCS 2: Compares the PMI feedback-derived based on Rel-18 eType II-Doppler CB with the model output against the ground-truth raw channel’s SVD SGCS 3: Compares the PMI feedback-derived based on Rel-18 eType II-Doppler CB with the model output against the PMI -derived based on Rel-18 eType II-Doppler CB with the ground truth |
| Model input type | Raw channel matrix |
| Model out type | Raw channel matrix |
| Observation window | Observation window (number/distance): 5/5ms |
| Prediction window | Prediction window (number/distance between prediction instances/distance from the last observation instance to the 1st prediction instance): 1/5ms/5ms |
| Training and validation datasets for Step-1 and Step-2 | Set1: 16TX containing 20Hz Doppler ideal estimates Set2: 16TX containing 50Hz Doppler ideal estimates Set3: 16TX containing 100Hz Doppler ideal estimates Set4: 16TX containing 20Hz, 50Hz, and 100Hz Doppler ideal estimates (mixed) Set5: 32TX containing 20Hz Doppler ideal estimates Set6: 32TX containing 50Hz Doppler ideal estimates Set7: 32TX containing 100Hz Doppler ideal estimates Set8: 32TX containing 20Hz, 50Hz, and 100Hz Doppler ideal estimates (mixed) |
| Testing datasets for Step-1 and Step-2 | Set1: 16TX containing only 20Hz Doppler ideal estimates Set2: 16TX containing only 50Hz Doppler ideal estimates Set3: 16TX containing only 100Hz Doppler ideal estimates Set4: 32TX containing only 20Hz Doppler ideal estimates Set5: 32TX containing only 50Hz Doppler ideal estimates Set6: 32TX containing only 100Hz Doppler ideal estimates |
| Training and validation datasets for Step-3 | Set1: 16TX containing 20Hz Doppler practical estimates Set2: 16TX containing 50Hz Doppler practical estimates Set3: 16TX containing 20Hz and 50Hz Doppler practical estimates (mixed) |
| Testing datasets for Step-3 | Set1: 16TX containing only 20Hz Doppler practical estimates Set2: 16TX containing only 50Hz Doppler practical estimates |
| MCS for Step-3 | MCS13, MCS17, MCS20, MCS22 |

In the following Table 2 we share our average of SGCS for intermediate results over all subbands per layer for 4 different measures. Firstly, we have non-predicted (sample and hold) raw channel SGCS. Secondly, we have predicted raw channel SGCS. Thirdly, we have predicted Rel-18 quantized precoder SGCS2 where comparison is done to ground truth of unquantized SVD precoder. Finally, we have predicted Rel-18 quantized precoder SGCS3 where comparison is done to ground truth Rel-18 quantized precoder. Note, that only SGCS2 results are new results compared to our previous discussion paper [3].

Table 2: Step-1 and Step-2 results of average of SGCS over all subbands per layer

| Test | Training Doppler [Hz] | Testing Doppler [Hz] | Channel sample and hold [SGCS per layer] | Channel prediction [SGCS per layer] | Rel-18 precoder prediction SGCS2 [SGCS per layer] | Rel-18 precoder prediction SGCS3 [SGCS per layer] |
| --- | --- | --- | --- | --- | --- | --- |
| FDD 16TX | 20 | 20 | [0.795, 0.650] | [0.999, 0.999] | [0.907, 0.825] | [0.992, 0.985] |
|  | 50 | 50 | [0.325, 0.187] | [0.969, 0.944] | [0.892, 0.801] | [0.955, 0.922] |
|  | 100 | 100 | [0.301, 0.166] | [0.750, 0.572] | [0.774, 0.597] | [0.821, 0.675] |
|  | Mixed (20, 50, 100) | 20 | [0.795, 0.650] | [0.957, 0.923] | [0.879, 0.778] | [0.941, 0.898] |
|  |  | 50 | [0.325, 0.187] | [0.872, 0.777] | [0.822, 0.688] | [0.873, 0.781] |
|  |  | 100 | [0.301, 0.166] | [0.666, 0.456] | [0.677, 0.471] | [0.717, 0.533] |
| FDD 32TX | 20 | 20 | [0.814, 0.670] | [0.999, 0.998] | [0.869, 0.735] | [0.990, 0.982] |
|  | 50 | 50 | [0.391, 0.201] | [0.972, 0.946] | [0.853, 0.708] | [0.961, 0.924] |
|  | 100 | 100 | [0.309, 0.160] | [0.743, 0.561] | [0.774, 0.597] | [0.836, 0.690] |
|  | Mixed (20, 50, 100) | 20 | [0.814, 0.670] | [0.954, 0.911] | [0.844, 0.693] | [0.944, 0.894] |
|  |  | 50 | [0.391, 0.201] | [0.881, 0.770] | [0.797, 0.611] | [0.891, 0.787] |
|  |  | 100 | [0.309, 0.160] | [0.664, 0.444] | [0.673, 0.438] | [0.749, 0.563] |

From these results we can make the following observations.

Observation #1: FDD 16TX and FDD 32TX results with the same Doppler perform quite similarly.

Observation #2: In lower Doppler, accuracy is better than in higher Doppler.

Observation #3: Rel-18 codebook quantization performance is close to raw channel prediction performance.

In the following Figures 1 to 4 and Table 3 we share our PDSCH Rank 2 throughput simulation results using various MCS. We have simulated MCS13, MCS17, MCS20, and MCS22 to find suitable and feasible operation point for testing.

We report SNR needed to achieve 90% relative throughput using predicted PMI and sample & hold (legacy) PMI. Also, we report throughput ratio γ between follow PMI and random PMI, that would be used as actual PMI reporting requirement for both predicted PMI and sample & hold (legacy) PMI options.

Figure 1: Step-3 throughput results of PDSCH Rank2 MCS13 in Dopplers 20Hz and 50Hz.

Figure 2: Step-3 throughput results of PDSCH Rank2 MCS17 in Dopplers 20Hz and 50Hz.

Figure 3: Step-3 throughput results of PDSCH Rank2 MCS20 in Dopplers 20Hz and 50Hz.

Figure 4: Step-3 throughput results of PDSCH Rank2 MCS22 in Dopplers 20Hz and 50Hz.

Table 3: Step-3 PDSCH Rank 2 results collection of SNR and γ

| Doppler | MCS | Prediction SNR [dB] | Sample&Hold SNR [dB] | Prediction γ | Sample&Hold γ |
| --- | --- | --- | --- | --- | --- |
| 20Hz | 13 | 0.8dB | 2.4dB | 4.3 | 3.0 |
|  | 17 | 4.1dB | 5.9dB | 4.0 | 2.7 |
|  | 20 | 6.7dB | 8.5dB | 3.3 | 2.4 |
|  | 22 | 9.0dB | 11.0dB | 2.8 | 2.2 |
| 50Hz | 13 | 4.0dB | 3.9dB | 2.3 | 2.4 |
|  | 17 | 8.0dB | 7.8dB | 2.1 | 2.1 |
|  | 20 | 10.9dB | 10.5dB | 1.9 | 2.0 |
|  | 22 | 13.4dB | 13.0dB | 1.8 | 1.9 |

From these results we can make the following observations.

Observation #4: Prediction gain measured in SNR varies between 1.6 to 2.0dB in Doppler 20Hz.

Observation #5: Prediction gain measured in γ varies between 0.6 to 1.3 (27% to 48%) in Doppler 20Hz.

Observation #6: There is no prediction gain in Doppler 50Hz.

Observation #7: Operation point of MCS13 in Doppler 20Hz is close to SNR 0dB.

As observed from the results, there is no prediction gain in Doppler 50Hz, and therefore we suggest ignoring it in further studies. Our assumption is, also based on previous Rel-18 non-AI prediction studies, that Dopplers too low and too high are not well suited for prediction but there is range of Dopplers that are best suited for PMI prediction. We are open to study more Dopplers closer to 20Hz if there is enough interest from other companies, or we can continue to work only with Doppler 20Hz that seems to enable measurable prediction gains in performance.

Proposal #1: Exclude Doppler 50Hz in further studies.

Proposal #2: Focus on Doppler 20Hz in further studies.

We prefer to use high enough MCS when defining requirements to guarantee robust testing SNR. Also, we prefer to focus on more flexible FDD configurations first to find feasible test configuration before trying to implement more complex TDD configurations.

Proposal #3: Focus on MCS20 or higher for robust testing SNR.

Proposal #4: Focus on FDD to find feasible test configuration before using effort on more complex TDD configurations.

# Conclusion

In this contribution we provide the view on the CSI reporting requirement framework for CSI prediction issues. The following observations and proposals are made:

Observation #1: FDD 16TX and FDD 32TX results with the same Doppler perform quite similarly.

Observation #2: In lower Doppler accuracy is better than in higher Doppler.

Observation #3: Rel-18 codebook quantization performance is close to raw channel prediction performance.

Observation #4: In simulation assumptions there exists sufficient test setup for FDD with Periodic CSI-RS configuration.

Observation #4: Prediction gain measured in SNR varies between 1.6 to 2.0dB in Doppler 20Hz.

Observation #5: Prediction gain measured in γ varies between 0.6 to 1.3 (27% to 48%) in Doppler 20Hz.

Observation #6: There is no prediction gain in Doppler 50Hz.

Observation #7: Operation point of MCS13 in Doppler 20Hz is close to SNR 0dB.

Proposal #1: Exclude Doppler 50Hz in further studies.

Proposal #2: Focus on Doppler 20Hz in further studies.

Proposal #3: Focus on MCS20 or higher for robust testing SNR.

Proposal #4: Focus on FDD to find feasible test configuration before using effort on more complex TDD configurations.

# Reference

- R4-2508080, WF on NR_AIML_air_part1, Qualcomm inc.

- R4-2508102, WF on updated simulation assumptions for AI based CSI prediction, OPPO, CAICT, vivo, Nokia, Ericsson, Qualcomm, CMCC, CTC, Huawei, Hisilicon, Mediatek, CATT, Samsung, ZTE Corporation, Apple

- R4-2505596, Discussion on CSI reporting requirement framework for CSI prediction, MediaTek inc.