3GPP TSG-RAN WG4 Meeting #116	R4-2509656

Bengaluru, India, 25th August 2025 – 29th August 2025

Agenda item:	7.18.3

Source:	MediaTek Inc.

Title:	Discussion on Testing issues for CSI compression two-sided models

Document for:	Discussion

# Introduction

In RAN4#115, RAN4 concluded that simulation results are aligned enough to proceed in two study tracks of Option 3, and Option 4a to further check the feasibility of testing for AI/ML CSI compression [1]. In the second track of Option 3 encoder and decoder are trained with mixed dataset from contributing companies. In Option 4 labeled dataset is generated using frozen pretrained encoder. The labelled dataset is then used to train decoder, and this decoder is furthermore used to train encoder. In this contribution, we provide our simulation results based on the agreed data generation assumptions and AI/ML model [2].

# Discussion

For training, validation, and testing data, we follow RAN4’s agreement to reuse system level parameters in TR38.843. Here, we consider the baseline case and ideal channel estimation.

Table 1. Parameters for system level simulation

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD, OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) |
| Frequency Range |  | FR1 only, [2GHz, 4GHz] |
| Inter-BS distance |  | 200m |
| Channel model |  | According to TR 38.901 |
| Antenna setup and port layouts at gNB |  | Companies need to report which option(s) are used between - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE |  | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for (rank 1-4) |
| BS Tx power |  | 44dBm for 20MHz |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | Baseline: 15kHz for 2GHz; |
| Simulation bandwidth |  | Baseline: 10 MHz for 15kHz |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | SU-MIMO |
| MIMO layers |  | Baseline: 1 |
| CSI feedback |  | Feedback assumption at least for baseline scheme -CSI feedback periodicity (full CSI feedback): 5 ms (baseline) -Scheduling delay (from CSI feedback to time to apply in scheduling): 4 ms |
| Traffic load (Resource utilization) |  | Baseline: 50% Optional: 20/70% |
| UE distribution |  | CSI compression: 80% indoor (3 km/h), 20% outdoor (30 km/h) |
| UE receiver |  | MMSE-IRC as the baseline receiver |
| Feedback assumption |  | Realistic |
| Channel estimation |  | Realistic or ideal channel estimation |

The detailed structures of ResCNN encoder (Encoder-1) and decoder for two-sided CSI compression AI/ML model are shown in Figure 1 and 2. The detailed structures of low complexity Encoders are shown in Figure 3, 4, and 5. More detailed implementations are described in simulation assumptions [2].

| Figure 1. Detailed model diagrams of CNN based lower-complexity encoder (Encoder 1) | Figure 2. Detailed model diagrams of CNN based Decoder |
| --- | --- |

| Figure 3. Detailed model diagram of Transformer based lower-complexity encoder (Encoder 2) | Figure 4. Detailed model diagrams of CNN based lower-complexity encoder (Encoder 3) | Figure 5. Detailed model diagram of MLP based lower-complexity encoder (Encoder 4) |
| --- | --- | --- |

We generate training, validation and testing data based on assumptions in Table 1 and use the main hyperparameters listed in Table 2 for training. We also use ideal channel estimation.

Table 2. Hyperparameters used for all models

| Hyperparameters | MediaTek dataset MTK0R4_113dsei01.zip | Mixed dataset (updated) MIXEDR4_114dsei00.zip |
| --- | --- | --- |
| Number of samples in training dataset (80%) | 480k | 400k |
| Number of samples in validation dataset (10%) | 60k | 50k |
| Number of samples in testing dataset (10%) | 60k | 50k |
| Quantization (for quantized models) | (1/8, 3/8, 5/8, 7/8) | (1/8, 3/8, 5/8, 7/8) |
| Optimizer | Adam | Adam |
| Learning rate | 0.0001 | 0.0001 |
| Batch size | 256 | 256 |
| Max number of epochs | 150 | 150 |
| Stopping criteria | No validation loss improvement over 25 epochs | No validation loss improvement over 25 epochs |
| Loss function | 1 - SGCS | 1 - SGCS |
| Drop rate | No drop out | No drop out |

## 2.1 Option 3 Track 2 (training of encoder using other companies frozen pretrained decoder and mixed dataset)

In RAN4#115 companies were encouraged to train encoder, using other companies frozen pretrained decoders, using updated mixed dataset for training and testing, and report the average CDF. As a second priority, if time allows, companies are encouraged to also use companies’ own dataset for training and updated mixed dataset for testing. Furthermore, as a third priority, companies are encouraged to also use companies’ own dataset for training and testing.

Additionally, companies are encouraged to provide joint training results using updated mixed dataset for training and testing and using companies’ own dataset for training and updated mixed dataset for testing.

There were 4 proposed Encoder implementations available: high complexity CNN, low complexity Transformer, low complexity CNN, and low complexity MLP. We have implemented all Encoder options.

We provide our results for test set performance in Table 3.

Table 3. SGCS of quantized models

|  | MediaTek Encoder and Decoder joint training |  | MediaTek Encoder with frozen pretrained Samsung Decoder |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Testing on Mixed dataset |  | Testing on Mixed dataset |  | Testing on MediaTek dataset |  |
|  | Training on Mixed dataset | Training on MediaTek dataset | Training on Mixed dataset | Training on MediaTek dataset | Training on Mixed dataset | Training on MediaTek dataset |
| Encoder-1 (Agreed encoder, CNN with 47M FLOPs) | 0.6950 | 0.7163 | 0.7006 | 0.6767 | 0.7117 | 0.7217 |
| Encoder-2 (Transformer with 3.63M FLOPs) | 0.7064 | 0.6267 | 0.6907 | 0.6034 | 0.6972 | 0.7129 |
| Encoder-3 (CNN with 4.14M FLOPs) | 0.6744 | 0.7054 | 0.6820 | 0.6497 | 0.6939 | 0.7047 |
| Encoder-4 (MLP with 0.22M FLOPs) | 0.6705 | 0.5369 | 0.6348 | 0.4144 | 0.6327 | 0.6623 |

Testing high complexity CNN Encoder (Encoder-1) with frozen pretrained CNN based decoder that is trained with mixed dataset can achieve performance close to joint training of Encoder and Decoder.

Testing results using lower complexity CNN Encoder (Encoder-3) are approximately 0.02, measured in SGCS, worse compared to high complexity Encoder when training and testing with mixed dataset, and can be considered as feasible.

Testing results using lower complexity Transformer Encoder (Encoder-2) are approximately on par, measured in SGCS, compared to high complexity CNN Encoder, when trained with Mixed dataset. When training is done with MediaTek dataset and testing is done with Mixed dataset, testing results are clearly worse compared to high complexity CNN Encoder.

Testing results using lower complexity MLP Encoder (Encoder-4) are approximately 0.02, measured in SGCS, worse compared to high complexity CNN Encoder, when jointly trained with Mixed dataset. When training is done with MediaTek dataset and testing is done with Mixed dataset, testing results are clearly worse compared to high complexity CNN Encoder, and worse that low complexity Transformer Encoder. Also, when frozen pretrained decoder is used, the testing results are the worst of all studied Encoders.

In general, the principle of using a frozen pretrained decoder can be considered a feasible methodology when paired with well-matched encoder. However, as can be seen then the generalization performance seems to be sensitive to the training dataset if the encoder and decoder backbones are different.

Observation #1: Option 3 Track 2 SGCS testing results with frozen pretrained decoder (trained with mixed dataset) using own or mixed dataset can in most cases closely match reference SGCS results where Encoder and Decoder are trained together (joint training).

Observation #2: Testing results with frozen pretrained decoder (trained with mixed dataset) using lower complexity CNN Encoder are slightly worse compared to high complexity Encoder when trained with Mixed dataset.

Observation #3: Testing results with frozen pretrained decoder (trained with mixed dataset) using lower complexity Transformer Encoder are slightly worse compared to high complexity Encoder when trained with Mixed dataset.

Observation #4: Testing results with frozen pretrained decoder (trained with mixed dataset) using lower complexity MLP Encoder are clearly worse compared to high complexity Encoder when trained with Mixed dataset.

Observation #5: Testing results with frozen pretrained decoder (trained with mixed dataset) using lower complexity Transformer and MLP Encoders are clearly worse compared to high complexity Encoder when trained with MediaTek dataset and tested with Mixed dataset.

## 2.2 Option 4a (training of decoder with labelled dataset)

In RAN4#115 companies were encouraged to train decoder, using labelled dataset generated using selected company’s (Samsung) frozen pretrained encoder as a Step 1. These decoders are encouraged to be shared for further testing. We have used Samsung encoder for dataset labelling as suggested in WF. With trained decoder from Step 1 companies were encouraged to train encoder with their own dataset and mixed dataset as a Step 2. There were 4 proposed Encoder implementations available: high complexity CNN, low complexity Transformer, low complexity CNN, and low complexity MLP. We have implemented all Encoder options.

For the Step 3, companies are encouraged to use their frozen pretrained Encoder from Step 2 and take all available shared frozen pretrained Decoders from other companies from Step 1. In total 7 Decoders were shared for evaluation. We were able to execute testing with all Decoders. In the following Tables 4-6 we provide test results with different requested combinations of training and testing datasets.

Table 4. SGCS of Quantized models trained with Mixed dataset and tested with Mixed dataset.
Encoder is trained with frozen pretrained decoder from the Step-2.

|  | Ericsson | Huawei | MediaTek | Nokia | Qualcomm | Samsung | Vivo |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Encoder-1 (Agreed encoder, CNN with 47M FLOPs) | 0.6898 | 0.7007 | 0.6860 | 0.6848 | 0.6954 | 0.6933 | 0.6914 |
| Encoder-2 (Transformer with 3.63M FLOPs) | 0.6839 | 0.6927 | 0.6814 | 0.6791 | 0.6881 | 0.6861 | 0.6850 |
| Encoder-3 (CNN with 4.14M FLOPs) | 0.6625 | 0.6737 | 0.6565 | 0.6545 | 0.6678 | 0.6642 | 0.6627 |
| Encoder-4 (MLP with 0.22M FLOPs) | 0.6256 | 0.6344 | 0.6215 | 0.6199 | 0.6295 | 0.6277 | 0.6264 |

Table 5. SGCS of Quantized models trained with MediaTek dataset and tested with Mixed dataset.
Encoder is trained with frozen pretrained decoder from the Step-2.

|  | Ericsson | Huawei | MediaTek | Nokia | Qualcomm | Samsung | Vivo |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Encoder-1 (Agreed encoder, CNN with 47M FLOPs) | 0.6813 | 0.6901 | 0.6721 | 0.6723 | 0.6847 | 0.6811 | 0.6813 |
| Encoder-2 (Transformer with 3.63M FLOPs) | 0.6016 | 0.6298 | 0.6118 | 0.6136 | 0.6037 | 0.6019 | 0.6213 |
| Encoder-3 (CNN with 4.14M FLOPs) | 0.6337 | 0.6474 | 0.6274 | 0.6237 | 0.6399 | 0.6346 | 0.6336 |
| Encoder-4 (MLP with 0.22M FLOPs) | 0.4235 | 0.4346 | 0.4129 | 0.4179 | 0.4258 | 0.4259 | 0.4228 |

Table 6. SGCS of Quantized models trained with MediaTek dataset and tested with MediaTek dataset.
Encoder is trained with frozen pretrained decoder from the Step-2.

|  | Ericsson | Huawei | MediaTek | Nokia | Qualcomm | Samsung | Vivo |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Encoder-1 (Agreed encoder, CNN with 47M FLOPs) | 0.7032 | 0.7161 | 0.6919 | 0.6908 | 0.7088 | 0.7031 | 0.7031 |
| Encoder-2 (Transformer with 3.63M FLOPs) | 0.6993 | 0.7054 | 0.6853 | 0.6844 | 0.7042 | 0.6998 | 0.6949 |
| Encoder-3 (CNN with 4.14M FLOPs) | 0.6928 | 0.7034 | 0.6900 | 0.6886 | 0.6981 | 0.6960 | 0.6944 |
| Encoder-4 (MLP with 0.22M FLOPs) | 0.6439 | 0.6552 | 0.6350 | 0.6345 | 0.6481 | 0.6447 | 0.6445 |

In general Option 4 results seem to be comparable to Option 3 Track 2 results, meaning this methodology seems to be as feasible as Option 3 Track 2.

Testing results using lower complexity CNN Encoder (Encoder-3) are slightly worse compared to high complexity CNN Encoder (Encoder-1), but it depends on used training and testing datasets. When our own dataset is used for training and mixed dataset is used for testing, performance difference between high and low complexity CNN Encoders is approximately 0.05. When mixed dataset is used for training and testing, performance difference between high and low complexity CNN Encoders is approximately 0.03. When our own dataset is used for training and testing, performance difference between high and low complexity CNN Encoders is approximately 0.01.

Testing results using lower complexity Transformer Encoder (Encoder-2) are slightly worse compared to high complexity CNN Encoder (Encoder-1), but it depends on used training and testing datasets. When our own dataset is used for training and mixed dataset is used for testing, performance difference between high and low complexity Encoders is approximately 0.07. When mixed dataset is used for training and testing, performance difference between high and low complexity Encoders is approximately 0.01. When our own dataset is used for training and testing, performance difference between high and low complexity Encoders is approximately 0.01.

Testing results using lower complexity MLP Encoder (Encoder-4) are slightly worse compared to high complexity CNN Encoder (Encoder-1), but it depends on used training and testing datasets. When our own dataset is used for training and mixed dataset is used for testing, performance difference between high and low complexity Encoders is approximately 0.25. When mixed dataset is used for training and testing, performance difference between high and low complexity Encoders is approximately 0.07. When our own dataset is used for training and testing, performance difference between high and low complexity Encoders is approximately 0.06.

Observation #6: 7 decoders were successfully used for evaluation.

Observation #7: Option 4 SGCS results are comparable to SGCS results in Option 3 Track 2.

Observation #8: Performance difference between high and low complexity CNN Encoders depends on used training and testing datasets (varies between 0.01 to 0.05).

Observation #9: Performance difference between high complexity CNN Encoder and low complexity Transformer Encoder depends on used training and testing datasets (varies between 0.01 to 0.07).

Observation #10: Performance difference between high complexity CNN Encoder and low complexity MLP Encoder depends on used training and testing datasets (varies between 0.06 to 0.25).

# Conclusion

In this contribution, simulation results for alignment are provided. Considering studied AI/ML models, we have the following observations:

Observation #1: Option 3 Track 2 SGCS results with frozen pretrained decoder (trained with mixed dataset) using own or mixed dataset can in most cases closely match reference SGCS results where Encoder and Decoder are trained together (joint training).

Observation #2: Training results using lower complexity CNN Encoder are slightly worse compared to high complexity Encoder.

Observation #3: Training results using lower complexity Transformer Encoder are slightly worse compared to high complexity Encoder when trained with Mixed dataset.

Observation #4: Training results using lower complexity MLP Encoder are slightly worse compared to high complexity Encoder when trained with Mixed dataset.

Observation #5: Training results using lower complexity Transformer and MLP Encoders are clearly worse compared to high complexity Encoder when trained with MediaTek dataset and tested with Mixed dataset.

Observation #6: 7 decoders were successfully used for evaluation.

Observation #7: Option 4 SGCS results are comparable to SGCS results in Option 3 Track 2.

Observation #8: Performance difference between high and low complexity CNN Encoders depends on used training and testing datasets (varies between 0.01 to 0.05).

Observation #9: Performance difference between high complexity CNN Encoder and low complexity Transformer Encoder depends on used training and testing datasets (varies between 0.01 to 0.07).

Observation #10: Performance difference between high complexity CNN Encoder and low complexity MLP Encoder depends on used training and testing datasets (varies between 0.06 to 0.25).

# Reference

- R4-2508051, WF on NR_AIML_air_part2, Ericsson

- R4-2508084, Simulation assumptions for CSI compression, Huawei, Hisilicon, OPPO, CATT, Vivo, NTU, Nokia, Ericsson, Qualcomm, Xiaomi, Mediatek, APPLE, Rohde & Schwarz, Samsung, Intel, ZTE Corporation, Sanechips, CAICT