3GPP TSG-RAN WG4 Meeting # 116	R4-2509659

Bengaluru, India, 25th August 2025 – 29th August 2025

Agenda item:	7.21.6

Source:	MediaTek Inc.

Title:	Discussion on SBFD UE demodulation and CSI requirements

Document for:	Discussion

# Introduction

In Rel-19 subband non-overlapping full duplex (SBFD) operation at gNB side within a TDD carrier has been introduced targeting to better UL coverage compared to TDD and improved UL/DL latency. In Rel-19 solution gNB can perform simultaneous Tx and Rx on the DL and UL subbands while UE remains half duplex. RAN4 needs to specify necessary BS and/or UE demodulation performance requirements to support SBFD operation [1].

# Discussion

In this discussion paper we will focus on UE side impacts of the SBFD feature. In Rel-19 SBFD solution gNB can perform simultaneous Tx and Rx on the DL and UL subbands while UE remains half duplex. We can have some UE demodulation related assumptions from TR 38.858 [2] excluding Cross link interference (CLI) aspects.

- FR1 and FR2-1 are both applicable.

- Both time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs.

In Rel-19 SBFD operation the uplink subband can be located at one side of the carrier or can be located at the middle part of the carrier (Figure 1, Left). Uplink subband is defined per symbol granularity in time domain where SBFD symbols are configured in consecutive manner within a TDD-UL-DL pattern period and exact start and end symbols are defined. Therefore, a slot can consist of SBFD symbols and non-SBFD symbols (Figure 1, Right). For single PDSCH occasion it is defined that if a UE is configured with SBFD symbols, the UE does not receive a PDSCH that is mapped to both SBFD symbols and non-SBFD symbols within a slot.

Observation #1: In Rel-19 SBFD operation the uplink subband can be located at one side of the carrier or can be located at the middle part of the carrier.

Observation #2: In Rel-19 SBFD operation start and end symbols of SBFD symbols are defined per symbol granularity.

Observation #3: In Rel-19 SBFD operation UE is not assumed to receive PDSCH that is mapped to both SBFD symbols and non-SBFD symbols within a slot.

|  | StartSlot EndSlot StartSymbol EndSymbol |
| --- | --- |

Figure 1: Frequency domain allocation of downlink and uplink in Rel-19 SBFD.

In Rel-19 SBFD operation the uplink subband cannot be used for downlink purpose. PDSCH frequency domain allocation is signalled for every slot and in the usual scenario gNB should not indicate PDSCH allocation in uplink subband part. However, if PDSCH allocation would overlap with uplink subband part, assigned PRBs that fall outside DL usable PRBs are invalid and should not be used for PDSCH resource mapping. Also, the number of PRBs for TBS determination is based assigned PRBs within DL usable PRBs only, see Figure 2.

Observation #4: In Rel-19 SBFD operation PDSCH frequency domain should be signalled to DL usable PRBs in typical scenario.

Figure 2: Example of PDSCH frequency domain allocation

Issue 2-1-3 whether to introduce UE demodulation with SBFD carrier (with DL sub-band and UL sub-suband configured) - Way forward - Option 1: Define limited number of additional SBFD cases - Other options are not precluded

In the previous meeting, it was proposed to introduce UE demodulation requirements for the SBFD carrier. We still consider this test to be strictly functional, where the UE is evaluated on its ability to correctly ignore portions of the PDSCH RE allocation and accurately calculate the TBS in slots configured with SBFD symbols. In the time domain, there is limited scope for testing, as the UE is not expected to receive a single PDSCH occasion mapped to both SBFD and non-SBFD symbols within a slot. In the frequency domain, the test is limited to verifying that the UE can correctly ignore PRBs overlapping with the SBFD UL part. Therefore, we do not see significant value in the proposed UE requirements and propose not to introduce such requirements.

Observation #5: UE demodulation requirements with SBFD carrier would be strictly functional tests.

Proposal #1: We propose not to introduce UE demodulation requirements with SBFD carrier.

# Conclusion

In this contribution we provide the view on the SBFD UE demodulation and CSI requirements. The following observations and proposals are made:

Observation #1: In Rel-19 SBFD operation the uplink subband can be located at one side of the carrier or can be located at the middle part of the carrier.

Observation #2: In Rel-19 SBFD operation start and end symbols of SBFD symbols are defined per symbol granularity.

Observation #3: In Rel-19 SBFD operation UE is not assumed to receive PDSCH that is mapped to both SBFD symbols and non-SBFD symbols within a slot.

Observation #4: In Rel-19 SBFD operation PDSCH frequency domain should be signalled to DL usable PRBs in typical scenario.

Proposal #1: We propose not to introduce UE demodulation requirements with SBFD carrier.

# Reference

- RP-250328, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Charter Communications, Qualcomm, Fraunhofer HHI, Fraunhofer IIS

- 3GPP TR 38.858 V18.2.0 (2024-12), Study on Evolution of NR Duplex Operation (Release 18)

- R4-2508723, Way Forward for [115][330] NR_duplex_evo_demod, Samsung