3GPP TSG-RAN WG4 Meeting #116											R4-2509663

Bengaluru, IN, 25th – 29th August, 2025

Agenda item:	6.14.2.1

Source: 	Fujitsu

Title: 	Discussion on the coexistence study for US 4.9GHz new band

Document for:	Approval

# 1. Introduction

A one quarter study phase to identify and document coexistence requirements for the new band:

- Study coexistence 	.

    - Identify adjacent spectrum protection requirements

      - Identify any potential emission limits more stringent than the normal BS and UE emission masks

    - Identify other technologies operating in the 4940-4990 MHz band

      - Identify potential coexistence mechanism(s)

        - Frequency division sharing

        - Geographic separation

        - TDD synchronization

      - Identify any potential emission limits more stringent than the normal BS and UE emission masks

At the RAN#108 meeting, a spectrum-related work item [1] was approved to specify a new NR TDD 4.9GHz band for US operation. The objective of [1] states a one-quarter study phase below is planned before the detailed discussion on the minimum requirements. In this paper, we would like to provide our view on the coexistence study for US 4.9GHz new band.

# 2. Discussion

According to the Seventh Report and Order and Ninth Further Notice of Proposed Rulemaking [2], the following is stated.

The commission also sought comment on whether emission mask M is sufficient, or whether a tighter emission mask should be imposed for high power operations. No party commented on high power emission masks, and we take no further action on the emission mask M at this time.

Considering the above, our understanding is that the requirements for the emission mask have already been discussed in the US, and it was concluded that emission mask M is sufficient. The emission mask M has been captured as the applicable emission masks for 4940-4990 MHz in the 47 CFR [3] Part 90.210 (m). As far as we have checked, there appears to be no official documents regarding adjacent spectrum protection requirements other than the above emission mask in the US currently.

Observation 1: The requirements for the emission mask have already been discussed in the US, and it was concluded that emission mask M is sufficient.

Observation 2: The emission mask M has been stated to be the applicable emission mask for 4940-4990 MHz in the 47 CFR Part 90.210 (m).

Regarding the other perspective, the Seventh Report and Order and Ninth Further Notice of Proposed Rulemaking [2] states that there is a nationwide Band manager, which will coordinate all operations in the band to ensure that any non-public safety use remains fully secondary to, and preemptible by, public safety operations. The 47 CFR [3] Part 90.1217 also states that the 4.9 GHz Band Manager will have the primary responsibility of the frequency coordination and interference protection for the 4.9 GHz band incumbent public safety operations. It means that it is the Band Manager’s responsibility to determine how to address the coordination issue within the same frequency range, including frequency division sharing, geographic separation and TDD synchronization. Given this, we think RAN4 does not need to have any further discussion regarding the coexistence issue within the same frequency range.

Observation 3: It is the Band Manager’s responsibility to determine how to address the coordination issue within the same frequency range. Given this, RAN4 does not need to have any further discussion regarding the coexistence issue within the same frequency range.

Considering the above observations, it seems we already have enough information regarding the coexistence requirements for starting the RAN4 discussion. Then our proposal is below.

Proposal: RAN4 should complete the discussion regarding the coexistence study at the August meeting and start the discussion on the details of minimum requirements for the US 4.9GHz new band from the October meeting.

# 3. Conclusions

In this contribution, we provided our view on the coexistence study for US 4.9GHz new band as follows.

Observation 1: The requirements for the emission mask have already been discussed in the US, and it was concluded that emission mask M is sufficient.

Observation 2: The emission mask M has been stated to be the applicable emission mask for 4940-4990 MHz in the 47 CFR Part 90.210 (m).

Observation 3: It is the Band Manager’s responsibility to determine how to address the coordination issue within the same frequency range. Given this, RAN4 does not need to have any further discussion regarding the coexistence issue within the same frequency range.

Proposal: RAN4 should complete the discussion regarding the coexistence study at the August meeting and start the discussion on the details of minimum requirements for the US 4.9GHz new band from the October meeting.

# 4. Reference

- RP-251887, New WID on Introduction of NR TDD 4.9GHz Band for US Operation, Fujitsu

- Seventh Report and Order and Ninth Further Notice of Proposed Rulemaking, Docket 07-100, January 18, 2023

- 47 CFR Part 90 – Private Land Mobile Radio Service, https://www.ecfr.gov/current/title-47/chapter-I/subchapter-D/part-90