3GPP TSG-RAN WG4 Meeting #116				                                                                      R4-2509673

Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	7.19.3.1

Source: 	OPPO

Title: 	Study of impacts on RAN4 requirements for RRM measurement prediction

Document for:	Approval

- Introduction

In last meeting, the RRM requirements for AI mobility were discussed. The agreements and open issues were captured in the WF [1] and topic summary [2] respectively.

In this contribution, we will further evaluate the impacts on RRM requirements for measurement prediction based on RAN2 progress and current RAN4 discussions, and some background information derived from RAN2 SI is provided in Annex.

- Discussion

Performance Metrics/KPIs for Measurement Prediction use case

Regarding the definition of relative prediction accuracy, the following agreements were further reached in RAN4#115 meeting:

Issue 2-1: Relative RSRP accuracy definition Agreement: - For intra-frequency L3-RSRP, - Relative accuracy of predicted L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), - cell 1 and cell 2 are on the same frequency - the reported RSRP of cell2 can be measured or predicted. - FFS on whether and how to consider time difference between the two reported RSRPs when applying the definition of the relative RSRP accuracy - Relative RSRP accuracy for Beam level measurements is FFS during WI phase depending upon RAN2 progress. - For inter-frequency L3-RSRP, - Relative accuracy of predicted L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), - cell 2 is on a different frequency than cell 1 but in the same FR as cell 1 - the reported RSRP of cell2 can be measured or predicted. - FFS on whether and how to consider time difference between the two reported RSRPs when applying the definition of the relative RSRP accuracy

Based on the agreements in last meeting, the relative accuracy is defined as the accuracy of RSRP difference between two cells. The remaining parts to be clarified is beam level measurement and whether and how to consider the time difference between two reported RSRPs. We think this can be further discussed in work item stage when defining the detailed requirements.

In last meeting, it was also discussed whether to define relative accuracy for two measurements within the same cell. From our understanding, this includes two aspects, the first one is the difference between two beams from the same cell and the second one is the difference between two measurements from the same cell at different time point. For the two aspects, we understand they are already covered by the two FFS part in the above agreement and no separate definition is needed.

Proposal 1: Keep the existing relative RSRP accuracy definition in the TP and leave the two FFS parts to WI stage.

Based on the discussions in previous meeting, there are two approaches to define the ground truth, where alt1 is to use the expected measurement results calculated based on transmission power and channel modelling and alt2 is to use the reported measurement results under certain conditions.

  - Alt 1: Ground truth for FR1 will be based on the transmitted or reception power

  - Alt 2: Ground truth for FR1 will be based on the reported measurement value under certain conditions

For alt1, the advantage is that it is easily calculated and TE side can clearly know the expected results at each timing point, but the issue is that TE does not know when the measurement is performed or predicted. Therefore, TE doesn’t know which one to be compared which may request additional report from UE on the time stamp. The other issue is that the performance requirements may be used in model monitoring in LCM procedure, we understand it may cause problem since UE cannot acquire the expected measurement results in real LCM implementation. In addition, we understand the model input and training data is based on the actual measurement results, thus the output is usually close to the UE measurement.

For alt2, according to the existing measurement model, from inputs at point A, UE needs to experience L1 filter, beam consolidation/selection and L3 filter to get L3 measurement results at point C in which the latter two parts are RRC configured. Based on the legacy procedure, when the SINR is high enough, the UE measurement results are approximate to the ideal values. The advantage is that this can accommodate different UE implementation e.g., L1 filtering, and can be aligned with model training and inference procedure. To define the prediction accuracy and testing, certain channel modelling, beam consolidation configuration and L3 filter configuration can be considered as possible side conditions. The issue for alt2 is that UE may not perform actual measurement on certain occasions in some of the prediction cases, but we think this can be resolved by setting additional procedure in the testing. The other concern is that both ground truth and report are from UE which makes UE possible to cheat in testing. But we understand the actual measurement used as ground truth still needs to meet the legacy requirements, and cheating in the testing is not benefit for UE since it may lead bad performance in real practice. So we do not need to assume that UE will cheat in the testing.

From the analysis above, we prefer to use alt2 as baseline to define the ground truth. But considering this is the last meeting for study item and we need to consider the detailed testing procedure and the TE implementation, we are also OK to keep both options and leave the decision to work item stage.

Proposal 2: Use Alt2 (i.e., reported measurement value under certain conditions) as baseline to define the ground truth for prediction accuracy in FR1.

Impacts of measurement errors on the prediction accuracy

According to the discussion in AI/ML BM and the simulation assumption in [3], the following assumption is used for RF error and baseband error:

- Simulation assumptions for RF error The impact of RF errors should be considered. Following assumptions can be used to generate RF errors. - Truncated Gaussian distribution under ±4.5 dB RF error (μ=0, σ2=4.5) is used - Each TX beam are Independent - Use the LLS simulation assumptions defined in Table 1-5 to generate L1-RSRP difference as the baseband errors. Note: When analog beamforming is used in LLS, SNR value of above row represents the post beamforming SNR of the strongest Tx-Rx beam pair. For other Tx-Rx beam pairs, there are two options. Option 1: Apply error distribution from the post beamforming SNR of the strongest Tx-Rx beam pair to all Tx-Rx beam pair’s measurements in the SLS Option 2: Generate error distribution separately for each individual Tx-Rx beam pair’s SNRs and apply to the corresponding Tx-Rx beam pair measurement in the SLS. When analog beamforming is not used in LLS, Apply error distribution based on the SNR of above row to the strongest Tx-Rx beam pair’s measurements in the SLS Option 1: Apply error distribution from the SNR of above row to all Tx-Rx beam pairs’ measurements in the SLS Option 2: Generate error distribution separately for each individual Tx-Rx beam pairs’ SNR values and apply to the corresponding Tx-Rx beam pair measurements in the SLS. SNR of the strongest Tx-Rx beam pair can be fixed or not fixed.

For AI mobility, based on the proposals in last meeting, there are the following options for baseband and RF error model:

- Option 1: - BB error: - Alt1: Use link level simulation assumptions to generate L3-RSRP difference as baseband error. - FFS: Added independently per Tx beam per cell per measurement occasion according to the corresponding SINR; assuming one shot measurement - Alt2: Modelled as a certain distribution. - RF error: - Alt1: Modelled as uniform distribution - Alt2: Modelled as [Truncated] Gaussian distribution Under ±[X] dB RF error (μ=0, σ^2=[X]). - FFS: For RF error application, each of Tx beam is independent - FFS: A random RF error is generated for all the beams of the same frequency layer per SMTC. - Option 2: - Modelled as Gaussian distribution for the total error including RF error and BB error - Option 3: - Modelling measurement errors as a function of SINR accounting for variations in propagation environments across serving and neighboring cells.

According to the above proposals, we performed some simulations on the measurement prediction accuracy based on different error models and the results are summarized as below:

| Error model Scenarios | Without error | Normal/ Gaussian distribution (μ=0, σ2=16) | Uniform distribution [-4, +4] | Segmentation error based on SINR |
| --- | --- | --- | --- | --- |
| UE speed: 30km/h | 0.26 | 1.92 | 1.86 | 0.70 |
| UE speed: 60km/h | 0.51 | 2.29 | 2.07 | 0.95 |

Some assumptions used in the simulation are as below for reference:

- Case B, MRRT = 50%

- Measurement period: FR1: 200ms

- Observation Window: 600ms

- Prediction Window：200ms

For the last column, segmentation error based on SINR means to add different errors for different SINR ranges, i.e.,

- when SINR ≤ -9dB, add a random error within [-8, +8]dB

- when -9dB＜SINR ≤ -6dB, add a random error within [-6, +6]dB

- when -6dB＜SINR ≤ -3dB, add a random error within [-4, +4]dB

- when -3dB＜SINR ≤ 0dB, add a random error within [-2, +2]dB

- when SINR ＞0dB add a random error within [-1, +1]dB

According to the results, we can see that the performance is a little better when considering the different errors relevant to SINR which is aligned with theoretical analysis since it is closer to the real situation. But the performance when modelling the error as a certain distribution is also acceptable. Of course, the error value is just an example, during WI stage, we can use link level simulation to get more accurate values.

Based on the analysis and simulation results above, we think the similar approaches as beam management can be used as baseline, i.e., use the truncated Gaussian distribution for RF error and use link level simulation to generate the baseband error. But for baseband error, there may be some difference for mobility. E.g., for beam management, the SNR assumption only applies to the strongest Tx-Rx beam pair, for other beam pairs, the assumption is not aligned and depends on companies’ implementation. While for AI mobility, we think the error can be added per cell rather than per beam and the error is relevant to SINR rather than SNR since the interference from neighbor cell needs to be considered. And we can discuss how to differentiate different SINR values and how to add the different errors on the ideal dataset during WI stage.

Proposal 3: Take the following option 1 as baseline to model the measurement error and the details can be left to WI when discussing the simulation:

- Option 1:

  - BB error:

    - Use link level simulation assumptions to generate L3-RSRP difference as baseband error.

  - RF error:

    - Alt1: Modelled as uniform distribution

    - Alt2: Modelled as truncated Gaussian distribution Under ±[X] dB RF error (μ=0, σ^2=[X]).

Proposal 4: Further discuss during WI stage:

- The detailed assumptions for system level and link level simulation

- Whether and how to simulate different SINR values for deriving baseband error

- Whether and how to align the procedure for adding different errors on the ideal dataset

Impacts on Core requirements

Based on the scenario definition in RAN2, UE measurement behaviour can be different from legacy, e.g., UE can skip some measurement based on different MRRT values or OW/PW values for intra-frequency measurement prediction or UE can skip some measurement on certain frequency for inter-frequency measurement prediction.

From RAN4 perspective, the measurement period requirements can be different according to the skipped measurements, e.g., the skipped SMTC occasions may not cause scheduling restrictions and the skipped frequencies may not need to be counted in the CSSF calculation.  The impact of skipped measurement instances and the corresponding measurement period requirements can be discussed in WI stage.

Proposal 5: Discuss the measurement period requirements in WI stage considering:

- the impact of OW/PW length and the skipped measurement instances (e.g., based on MRRT) for intra-frequency measurement.

- the impact of skipped measurement frequencies or instances on CSSF calculation for inter-frequency measurement.

LCM related requirements

In RAN2#129bis meeting, the following agreements are reached:

General LCM: Agreements 1. The general LCM framework for beam management can be the baseline (where applicable) for AI mobility, such as the following aspects: - Data collection for model training - UE capability - Applicability reporting - Inference configuration and reporting - Performance monitoring and management 2 Only functionality-based LCM is considered for AI/ML mobility (i.e. we don’t support model based LCM).

Based on the agreements, the functionality-based LCM is considered for AI mobility and the general LCM framework for beam management can be the baseline. Considering the LCM related requirements for AI BM is still under discussion and the detailed LCM procedure for AI mobility is not clear, we think we can wait for more progress from RAN1/2 AI mobility and RAN4 AI BM discussion to decide the LCM related requirements.

Proposal 6: Leave the LCM related requirements discussion to WI stage and wait for more progress from RAN2 AI mobility.

Based on the above discussions, we provide the updated TP in our companion paper [4]. It can be used as baseline and further refined based on the discussions and conclusions in this meeting.

Proposal 7: Take the TP in [4 R4-2509672] as baseline and further refine based on the conclusions in this meeting.

- Conclusions

In this contribution, we discussed the possible RAN4 requirements for RRM measurement prediction and got the following observations and proposals:

Proposal 1: Keep the existing relative RSRP accuracy definition in the TP and leave the two FFS parts to WI stage.

Proposal 2: Use Alt2 (i.e., reported measurement value under certain conditions) as baseline to define the ground truth for prediction accuracy in FR1.

Proposal 3: Take the following option 1 as baseline to model the measurement error and the details can be left to WI when discussing the simulation:

- Option 1:

  - BB error:

    - Use link level simulation assumptions to generate L3-RSRP difference as baseband error.

  - RF error:

    - Alt1: Modelled as uniform distribution

    - Alt2: Modelled as truncated Gaussian distribution Under ±[X] dB RF error (μ=0, σ^2=[X]).

Proposal 4: Further discuss during WI stage:

- The detailed assumptions for system level and link level simulation

- Whether and how to simulate different SINR values for deriving baseband error

- Whether and how to align the procedure for adding different errors on the ideal dataset

Proposal 5: Discuss the measurement period requirements in WI stage considering:

- the impact of OW/PW length and the skipped measurement instances (e.g., based on MRRT) for intra-frequency measurement.

- the impact of skipped measurement frequencies or instances on CSSF calculation for inter-frequency measurement.

Proposal 6: Leave the LCM related requirements discussion to WI stage and wait for more progress from RAN2 AI mobility.

Proposal 7: Take the TP in [4 R4-2509672] as baseline and further refine based on the conclusions in this meeting.

- References

R4-2508268 WF on FS_NR_AIML_Mob_Part1

R4-2505564 Topic summary for [115][215] FS_NR_AIML_Mob_Part1

R4-2505152 Updated simulation assumptions for AI/ML BM

R4-2509672 TP on RAN4 aspects for TR 38.744