3GPP TSG-RAN WG4 Meeting #116				                                                                      R4-2509674

Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	7.19.3.2

Source: 	OPPO

Title: 	Study of impacts on RAN4 requirements for measurement event prediction

Document for:	Approval

- Introduction

In last meeting, the RRM requirements for AI mobility were discussed and the following agreement was reached for measurement event prediction.

Issue 3-1: Performance metric for event prediction use case Agreement: In case of indirect measurement event prediction use case, if the report includes the predicted RSRPs [corresponding to the predicted event occurrence time], RAN4 will define requirement for the absolute and/or relative accuracy of predicted RSRP.

In this contribution, we will evaluate the impacts on RRM requirements for measurement event prediction based on RAN2 further progress.

- Discussion

In last meeting, it was agreed that RAN4 will define requirement for the absolute and/or relative accuracy of predicted RSRP if the report includes the predicted RSRPs [corresponding to the predicted event occurrence time]. According to RAN2 discussion, the spec impact of measurement prediction can be applicable for measurement event prediction unless otherwise described explicitly. Based on this design, we think the requirements for measurement prediction can be also reused for the predicted RSRP in measurement event prediction.

Proposal 1: For indirect prediction, if the report includes the predicted RSRPs, the requirements for measurement prediction can be reused.

In last meeting, it was mentioned that RAN4 may need to discuss the delay requirements for predicted event triggered reporting. The possible delay may be the predicted RSRP reporting delay which can include the measurement delay and the inference delay. It would be similar as the discussion for measurement prediction and AI BM which has no conclusions so far. We can wait for more progress. For the event triggered reporting delay, we can further discuss whether delay requirements are needed in work item based on RAN2 design.

Proposal 2: For indirect prediction, discuss whether the predicted RSRP reporting delay and the event triggered reporting delay is needed in work item based on RAN2 design.

For the metric/KPI for indirect and direct event prediction, RAN2 has used the following for evaluation:

### 5.3.1 Evaluation methodology, metrics and assumptions The performance metric F1 score is defined as following: F1 score = 2*Precision*Recall/(Precision + Recall) Where: Precision = n3/(n1+n3) Recall =n3/(n2+n3) For indirect prediction, the counter n1, n2 and n3 in the formula are defined as following: - Counter n3(true event prediction): it increases by 1 when a ground-truth event occurs around a predicted event with ETD, whose range is [0, maximum ETD] or vice versa; - Counter n1(false event detection): it increases by 1 when no ground-truth event occurs around a predicted event with ETD, whose range is [0, maximum ETD]; - Counter n2(missed event detection): it increases by 1 when no event is predicted around a ground-truth event with ETD, whose range is [0, maximum ETD]. The ETD, i.e. timing difference between ground-truth event and predicted event, is illustrated in Figure 5.3.1-1: Figure 5.3.1-1: illustration of ETD As illustrated in Figure 5.3.1-1, only if the ETD between a predicted event and a ground-truth event e.g. ground-truth event 2 is less than or equal to maximum ETD, the ETD can still be tolerated. Otherwise, both false event and missed event are detected. For direct prediction, the counter n1, n2 and n3 in the formula are defined as following: - Counter n3 (true event prediction): it increases by 1 when a ground-truth event occurs within the occurrence window of predicted event whose possibility is higher than a predefined threshold; - Counter n1 (false event detection): it increases by 1 when no ground-truth event occurs within the occurrence window of predicted event whose possibility is higher than a predefined threshold; - Counter n2 (missed event detection): it increases by 1 when a ground-truth event occurs, but it doesn’t fall in the occurrence window of any predicted event whose possibility is higher than a predefined threshold. For direct prediction method, a measurement event could be predicted within an occurrence window starting from current time instance, i.e. t0, and future time instance t1 with a probability as illustrated in Figure 5.3.1-1. Figure 5.3.1-2: occurrence window of direct prediction method

From RAN4 perspective, the above KPIs can be considered to define the requirements for prediction accuracy, e.g., the performance metrics can be defined as certain ETD or certain F1 score under some conditions.

For both direct and indirect event prediction, the HO failure number are also used in RAN2 evaluation which we think can also be used for performance definition.

Proposal 3: For indirect prediction, the following approaches can be considered as possible KPI to evaluate the performance:

- Alt 1: The requirements are defined as [maximum] ETD under certain conditions (e.g., certain probability of true event prediction, false event detection or F1 score etc.)

- Alt 2: The requirements are defined as probability of true event prediction or F1 score under certain conditions (e.g., ETD ranges)

- Alt 3: The handover failure rate per UE under certain time durations

- Other alternatives are not precluded.

But based on the latest discussion in RAN2, they are reconsidering the performance metrics for monitoring and some companies think the F1 score may have problem to derive since the ground truth is not available. So they may not use such metrics for performance monitoring and may not support direct event prediction. Considering the detailed design will be further discussed in work item, we think we can wait for more RAN2 progress to discuss how to define the performance metrics and requirements in work item.

Proposal 4: For direct and indirect prediction, RAN4 to discuss how to define the performance metrics and requirements in work item based on further RAN2 progress.

- Conclusions

In this contribution, we discussed the possible RAN4 requirements for measurement event prediction and got the following proposals:

Proposal 1: For indirect prediction, if the report includes the predicted RSRPs, the requirements for measurement prediction can be reused.

Proposal 2: For indirect prediction, discuss whether the predicted RSRP reporting delay and the event triggered reporting delay is needed in work item based on RAN2 design.

Proposal 3: For indirect prediction, the following approaches can be considered as possible KPI to evaluate the performance:

- Alt 1: The requirements are defined as [maximum] ETD under certain conditions (e.g., certain probability of true event prediction, false event detection or F1 score etc.)

- Alt 2: The requirements are defined as probability of true event prediction or F1 score under certain conditions (e.g., ETD ranges)

- Alt 3: The handover failure rate per UE under certain time durations

- Other alternatives are not precluded.

Proposal 4: For direct and indirect prediction, RAN4 to discuss how to define the performance metrics and requirements in work item based on further RAN2 progress.

- References

R4-2508268 WF on FS_NR_AIML_Mob_Part1

R4-2505564 Topic summary for [115][215] FS_NR_AIML_Mob_Part1