3GPP TSG-RAN WG4 Meeting #116				                                                                      R4-2509675

Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	7.19.4

Source: 	OPPO

Title: 	Study of testability and interoperability for AI mobility

Document for:	Approval

- Introduction

In RAN#102 meeting, the SID on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR is approved. In RAN#105 meeting, the related work plan in RAN4 has been further checked and updated as follows:

Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4] - Study the impacts on requirements based on RAN2 assumptions, and coordinate with RAN2 if needed - Study the testability and interoperability based on RAN2 framework (e.g., number of cells to measure, beams etc.) - NOTE 4: Leverage the work from “AI/ML for NR air interface” led by RAN1 and avoid the duplicate study for testability and interoperability. - NOTE 5: Avoid the overlaps with RAN2 work for evaluation

In last meeting, the testability issues for AI mobility were discussed and the agreements were captured in WF [1]. In this contribution, we will further discuss the RAN4 impacts on testability based on the open issues captured in [2].

- Discussion

Prediction consistency in time domain

### Issue 1-1-1: Prediction consistency in time domain <Way forward >: - Consider the propagation condition for the validation of the RRM measurement prediction requirements before modeling different time-varying characteristics per cell/site in FR1. - Further discuss the following options on how to model different time-varying characteristics per cell/site in FR1. - Option 1: Change Tx power. - FFS: whether it is possible to change Tx power every symbol at TE using conductive test - Option 2: Change channel condition with fixed Tx power. - Option 3: Computer simulations alike test setup can be adopted. TE can transmit the signals of cell 1, 2, 3 with time-variance following a certain channel model. - Option 4: Select one reference UE trajectory model (e.g., from TR 38.843) to study prediction consistency impact

For the prediction consistency in time domain, it was agreed to discuss how to model different time-varying characteristics per cell/site. We understand it is hard to make the final decision in SI stage but further clarification can be discussed to align the understanding. Based on the proposals in last meeting, there are three aspects which need to be considered, Tx power (i.e., generate power of signals), channel condition and UE trajectory model.

In legacy testing, the generate power of signals is constant over time and AWGN channel is used. But for AI/ML based mobility, to verify the prediction accuracy, the UE received power over time need to be time-varying and consistent. Two approaches can be considered: the first approach is to change the generate power of signals over time based on a certain rule and still use AWGN channel (option 1). The second approach is to use fixed generate power of signals over time and assume the signal pass a fading channel (option 2/3). But since conducted testing is used for FR1, even for the second approach, the fading channel need to be modelled from Tx side of TE. So generally the above options are similar and not preclusive to each other. To simply the discussion, we can use one of options as baseline to refine the wording and check whether and how TE can implement that.

On the other hand, when setting the Tx power of TE side, the dynamic range of UE received power needs to be considered, e.g., to meet some certain side conditions to guarantee the performance.

For channel model, TDL and simplified CDL can be considered and further evaluated in work item when discussing the simulation and test procedure.

Regarding the UE trajectory model, it has already been used in RAN2 evaluation. We are fine to consider it in RAN4 simulation, and we understand it should be used in system level simulation when deriving training and inference dataset. But how to use it in testing procedure is not clear, and we prefer to leave it to work item after the simulation assumption and procedure are decided.

Proposal 1: To verify the measurement prediction accuracy, the following approaches can be considered to model the time-varying characteristics per cell/site:

- Option 1: TE to change Tx power over time based on a certain rule.

- Option 2: TE to transmit the signals of cells with time-variance following a certain channel model.

  - FFS the channel model:

    - Option A: TDL channel

    - Option B: Simplified CDL

Proposal 2: RAN4 to discuss whether and how to add UE trajectory model in the simulation and testing.

On the other hand, the above discussion is for a signal cell, and for inter-frequency inter-cell prediction, the relation between different carriers needs to be considered.

Proposal 3: For inter-frequency prediction, RAN4 to discuss whether and how to construct the relation of configurations between the two carries.

Testing setup

### Issue 1-2-1: Testing Setup in FR2 <Way forward >: - For the testing setup for AI/ML mobility in FR1 and FR2, discuss the following configurations in the next meeting: - Number of SSBs per cell - Channel model - FFS the same or different for temporal/frequency domain prediction - Impact of UE trajectory - FFS the same or different for temporal/frequency domain prediction - Dynamic range of received power (consistency of channel) - Measurement prediction specific parameters - Emulation of noise and EPRE - L3 filter coefficient

In precious meeting, several parameters are mentioned that need to be configured in the testing, e.g., number of SSB, measurement prediction specific parameters, L3 filter coefficient, the dynamic range of received power which depends on the SINR condition etc. But the above configurations may have impacts on the core and performance requirements. How to setup these configurations can be discussed in work item based on the requirements definition.

For channel model, UE trajectory impact and emulation of noise and EPRE, the discussions can be covered by prediction consistency issue.

Proposal 4: The testing setup of channel model/UE trajectory/Emulation of noise and EPRE can be covered by prediction consistency issue.

Proposal 5: The other testing setup can be left to WI based on the requirements definition.

- Received power pends on SINR side condition of requirements

- Measurement prediction specific parameters pend on the sets of requirements

Regarding the number of cells, for inter-frequency measurement, if single cell approach is considered, 2 or 3 cells are enough. And if cluster approach is used, more cells is needed which depends on the requirements definition and can be discussed in WI stage.

Proposal 6: For inter frequency measurement prediction, more than 3 cells are needed if cluster approach is assumed and the exact number can be decided in WI stage according to the requirements discussion.

Ground Truth derivation

The ground truth definition is still discussing in the requirements impact of AI mobility and two approaches are considered. According to our companion paper on the requirements discussion, we think alt2 (i.e., based on actual measurement report) can be used. And we think the same conclusion can be used here for testing.

Proposal 7: Regarding the ground truth derivation in tests, the actual measurement report based on legacy procedure can be used.

Testing goal

Regarding the testing goal, the following agreement was reached in previous meeting.

- Agreement - As baseline, the testing goal is to verify whether the minimum performance of AI/ML functionality/feature can be achieved.

For intra and inter measurement prediction, similar as legacy, the testing goal is to verify the measurement reported in OW meets the new requirements if defined and to verify the predicted measurement in PW meets the accuracy requirements under certain side conditions.

Proposal 8: The testing goal for measurement prediction is to verify:

- the measurement reported in OW meets the new requirements if defined;

- the predicted measurement in PW meets the accuracy requirements under certain side conditions.

Testing of Generalization aspects

### Issue 1-1-4: Testing of Generalization aspects No conclusions on generalization aspects have been achieved so far. Note: No conclusion for this issue will not stop the completion of this study item.

For model generalization, RAN2 has evaluated the performance over UE speeds, frequency and different cell configurations and shows the generalization performs well or can be resolved by certain approaches. And RAN2 has the following agreements in last meeting for inference configurations and applicability report.

Agreements - Use the beam management agreements as the baseline, including the following aspects (details can be updated after further progress is made in AI/ML PHY BM use case) • Consider Option A and Option B like scheme (if/when AI/ML PHY makes further progress) • Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability. • Upon receiving one or more full inference configuration(s) via RRCReconfiguration message, UE shall maintain all the full inference configuration(s) no matter the full inference configuration is applicable or inapplicable until the network releases it explicitly. • Support the explicit reporting of applicability/inapplicability in the initial report and subsequent reporting when the applicability changes. • Together with inapplicability reporting, UE further indicates a simple cause value of inapplicability (FFS pending AI/ML PHY progress). • No prohibit timer is introduced. - Associated ID should be optionally configurable for training and inference (i.e. it may not mandatorily required for all training/inference configurations). FFS for WI phase further details (what absence means, which use case, whether it is per cell or multiple cells/frequency, terminology).

Agreements - The following potential “inference parameters” can be considered for inference/measurement configuration, which provides the necessary information for UE to examine the applicability, it may contain • PW length (for temporal case A) • Skipping pattern (e.g. SSB config that indicates SSBs transmitted) (optional). MRRT (for temporal case B ) not discussed in study item, but if RAN4 says otherwise can be considered in WI • Measured and predicted beam pattern (optional). MRRS (for spatial domain prediction) not discussed in study item, but if RAN4 says otherwise can be considered in WI • Measured and predicted frequency (For inter-freq domain prediction) - Model related choices (i.e., cluster-based vs. cell-based, L1 filtering, RRM sub-use cases, OW length, direct vs. indirect) can be up to UE implementation unless a requirement is identified to specify them.

From RAN4 side, we think our work is to discuss how to test the generalization and which parameters/configurations need to be test for generalization. It is a common issue for AI based use cases and is still discussing in AI/ML for NR Air Interface without consensus. But there is a general guideline for this issue as below and we think the methodologies can be reused for mobility use case as baseline.

- 1st step: Define baseline testing for a certain scenario.

- 2nd step: Study and define testing for generalization.

Since the baseline testing is pending on the requirements definition, for AI mobility, we think the discussion can be left to WI stage. During WI, RAN4 need to discuss the baseline testing configurations based on requirements and discuss what parameters are needed for generalization testing. And then also discuss how to set the different configurations, e.g., dynamic configurations in one test or multiple tests with different configurations.

Proposal 9: For generalization testing, leverage the methodology in NR_AIML_Air work item:

- 1st step: Define baseline testing for a certain scenario.

- 2nd step: Study and define testing for generalization.

Proposal 10: Discuss the details of generalization testing in WI stage, e.g.,

- what parameters are needed for generalization testing

- how to set the different configurations, e.g., dynamic configurations in one test or multiple tests

LCM related testability aspects

### Issue 1-1-6: LCM related testability aspects No conclusions on LCM related testability aspects have been achieved so far. Note: No conclusion for this issue will not stop the completion of this study item.

For LCM related testing, we need to firstly discuss the LCM related requirements. According to our companion paper on requirements, the LCM is still discussing in RAN2 and LCM framework for beam management is used as baseline. But we understand the detailed procedure is not clear, to define LCM related requirements and testing, RAN4 need to wait for more progress from RAN2 and the discussion can be left to WI stage.

Proposal 11: Leave the LCM related testing discussion to WI stage after the requirements are clear.

Test environment mismatch

### Issue 1-2-6: Test environment mismatch <Way forward>: - Discuss whether and how the following aspects should be considered for AI/ML mobility testing: - Data consistency between training and inference - Data consistency between serving cell configuration (e.g. TCI state update, TA updates, etc) and dynamic radio environment

From testing perspective, the issue is how to guarantee the testing dataset generated by testing environment is aligned with UE training dataset. From our side, we understand when discussing the testing during WI stage, we need to consider how much alignment is needed for training dataset and inference dataset which is similar as the discussion in AI/ML air. But since it is not clear now how to derive the dataset and what kinds of configurations are needed for AI mobility, we think it can be further discussed during WI stage after the requirements are clear.

Proposal 12: Discuss in work item how to derive the dataset and how much alignment is needed for training and inference dataset.

Interoperability

In last meeting when drafting TP, the issue for interoperability was discussed, but there is no clear definition for interoperability among companies. From our understanding, interoperability means the interface alignment between UE and NW to accommodate different UE or NW implementation. During AI/ML air discussion, the issue is only for two sided model since the models are appearing in pair, i.e., the model implementation in one side may be impacted by the model/reference provided in the other side. And the different reference will cause different implementation which make the UE and NW need to cooperate well to guarantee the performance.

But for single sided model, there is no such issue since all the AI/ML model/function implementation is relying on UE/NW itself and the interaction between UE and NW is same as legacy. On the other hand, the issue is common for all the single sided model use case and no such issue is identified for beam management. We think it is same for AI mobility. So we prefer to capture that no issues are identified for interoperability. If companies think the description is too strong, we are also fine to remove the section from TP which is same as AI/ML air work item.

During the discussion, some companies mentioned that when UE has different model implementation, NW may need to have different management methodology. But we understand after we define the requirements for AI/ML, as long as UE can meet the minimum requirements, NW can give equal treatment to them and whether and how to improve the management methodology is up to NW implementation.

Proposal 13: Capture in the TP no issues are identified for interoperability or remove the whole section for interoperability which is same as NR_AIML_Air.

Proposal 14: As long as UE can meet the minimum requirements, whether and how to improve the management methodology is up to NW implementation.

Testing for event prediction

In our companion paper, we discussed the possible requirements and performance metrics for measurement event prediction. Correspondingly, we need to consider the testability for the performance metrics. Based on RAN2 discussion, some KPIs are related to the real event, e.g., for the calculation of F1 score, the true event prediction is when a real event occurs around a predicted event with ETD, whose range is [0, maximum ETD] or vice versa. But it seems hard to get the real event in the test if the handover is based on the predicted event. This aspect needs to be considered for testing.

Proposal 15: For the testing of event prediction, RAN4 to study how to know the real event occurs.

- Conclusions

In this contribution, we discussed RAN4 impacts on testability for AI mobility and got the following proposals:

Proposal 1: To verify the measurement prediction accuracy, the following approaches can be considered to model the time-varying characteristics per cell/site:

- Option 1: TE to change Tx power over time based on a certain rule.

- Option 2: TE to transmit the signals of cells with time-variance following a certain channel model.

  - FFS the channel model:

    - Option A: TDL channel

    - Option B: Simplified CDL

Proposal 2: RAN4 to discuss whether and how to add UE trajectory model in the simulation and testing.

Proposal 3: For inter-frequency prediction, RAN4 to discuss whether and how to construct the relation of configurations between the two carries.

Proposal 4: The testing setup of channel model/UE trajectory/Emulation of noise and EPRE can be covered by prediction consistency issue.

Proposal 5: The other testing setup can be left to WI based on the requirements definition.

- Received power pends on SINR side condition of requirements

- Measurement prediction specific parameters pend on the sets of requirements

Proposal 6: For inter frequency measurement prediction, more than 3 cells are needed if cluster approach is assumed and the exact number can be decided in WI stage according to the requirements discussion.

Proposal 7: Regarding the ground truth derivation in tests, the actual measurement report based on legacy procedure can be used.

Proposal 8: The testing goal for measurement prediction is to verify:

- the measurement reported in OW meets the new requirements if defined;

- the predicted measurement in PW meets the accuracy requirements under certain side conditions.

Proposal 9: For generalization testing, leverage the methodology in NR_AIML_Air work item:

- 1st step: Define baseline testing for a certain scenario.

- 2nd step: Study and define testing for generalization.

Proposal 10: Discuss the details of generalization testing in WI stage, e.g.,

- what parameters are needed for generalization testing

- how to set the different configurations, e.g., dynamic configurations in one test or multiple tests

Proposal 11: Leave the LCM related testing discussion to WI stage after the requirements are clear.

Proposal 12: Discuss in work item how to derive the dataset and how much alignment is needed for training and inference dataset.

Proposal 13: Capture in the TP no issues are identified for interoperability or remove the whole section for interoperability which is same as NR_AIML_Air.

Proposal 14: As long as UE can meet the minimum requirements, whether and how to improve the management methodology is up to NW implementation.

Proposal 15: For the testing of event prediction, RAN4 to study how to know the real event occurs.

- References

R4-2508269 WF on FS_NR_AIML_Mob_Part2

R4-2505565 Topic summary for [115][216] FS_NR_AIML_Mob_Part2

R4-2509672 TP on RAN4 aspects for TR 38.744