3GPP TSG-RAN WG4 Meeting #116	                                           R4-2509677

Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.24.5.2

Source:	OPPO

Title:	Discussion on RRM core requirements for LP-WUSWUR

Document for:	Approval

- Introduction

Previous meeting discussed the RRM requirements regarding LP-WUS/WUR and reached some agreements in [1]. This contribution will provide our further considerations on the open issues captured in [2].

- Discussion

Issue 1-1-9: LP-WUR status at legacy case (not at LP-WUS monitoring case/fully offloading(case 1) case/RRM relaxation (case 3) case) Agreement: At legacy state, when and how to turn on LR for serving cell measurement is up to UE implementation. Issue 1-1-9: LP-WUR status at legacy case (not at LP-WUS monitoring case/fully offloading(case 1) case/RRM relaxation (case 3) case) - Proposals - P1: No LR measurement and evaluation requirements apply at the legacy state, i.e., for the following cases: from legacy case to LP-WUR monitoring, from legacy case to RRM measurement fully offloading (case 1), and from legacy case to RRM measurement relaxation (case 3). (vivo xiaomi Apple CT Huawei ZTE) - P1-1: The requirements in clause apply when the LP-WUR is in ON state. Before entering LP-WUS, RRM offloading and RRM relaxation mode and after exiting LP-WUS, RRM offloading and RRM relaxation mode, it is up to LP-WUS UE implementation when and how to turn the LP-WUR to ON state for serving cell measurement. (Nokia)

For the LP-WUS status at legacy case, it was already agreed in precious meeting that when and how to turn on LR for serving cell measurement is up to UE implementation. And considering the LR based threshold is optional, we think no LR measurement and evaluation requirements are needed at the legacy state.

Proposal 1: No LR measurement and evaluation requirements apply at the legacy state.

Issue 1-1-11: RRM requirements for FR2 - Proposals - P1: Prioritize defining RRM requirements for FR1 in this WI (Apple Nokia) - P2: There is not any concrete proposal on FR2 requirements so far. Suggest RAN4 to confirm whether FR2 RRM requirement needs be defined in this release, or confirm the status of FR2 requirement will not impact the completion of WI (vivo, Nokia)

So far, all the requirements discussion and simulation are for FR1 and no technical proposals for FR2. If we want to define LR measurement requirements for FR2, we understand the beam sweeping assumption is needed and the simulation may also be needed. Considering this is the last meeting for core part, we prefer to only focus on FR1 requirements in this release.

Proposal 2: Prioritize defining RRM requirements for FR1 in this WI.

Issue 1-1-15 RedCap related - Proposals - P1: The discussion on whether Redcap UE should support LP-WUS and how to support if necessary should not have any impact on WI completion (vivo) - P2: RAN4 to define LP-WUR related requirements for RedCap UE (Huawei) - P3: RedCap UE shall meet LP-WUS related requirement based on UE’s capability (Ericsson)

For RedCap related requirements, we understand it is not clearly within the WI scope, and as we know, both RAN2 and RAN4 RF didn’t discuss this issue. If we want to define the requirements, we think the signaling support need to be confirmed and all requirements including RF and RRM should be included. Considering the timeline, we are not sure whether the time is enough to add this requirement and we think the requirements for normal UE should be completed and guaranteed firstly.

Proposal 3: The requirements for normal UE should be completed in high priority in Rel-19. After that, RAN4 can further discuss whether to apply for Redcap UE considering the RRM workload, RAN2 signaling support and UE RF impact.

Issue 1-2-2: Periodicity for LP-WUR RRM requirements Agreement: Regardless of whether LP-SS is configured or not, periodicity for SSB based LP-WUR measurement delay requirements is based on LO periodicity. Issue 1-2-2-2: Upper bound on SSB-based LP-WUR measurement periodicity - Proposals - P1: No need to define upper bound for SSB-based LP-WUR measurement periodicity. (xiaomi oppo CATT CT Nokia) - P2: Prefer the periodicity for measurement and evaluation delay requirements for LP-WUR based on SSB in IDLE/Inactive mode is 1.28s if the LO periodicity is 2.56s. (vivo ZTE) - P3: Postpone the discussion for the case where LP-SS periodicity > LO periodicity until RAN1 has more agreements for LP-SS periodicity (Huawei)

In previous meeting, it was agreed that periodicity for SSB based LP-WUR measurement delay requirements is based on LO periodicity regardless of whether LP-SS is configured or not, but whether to define upper bound is still FFS. If the upper bound is defined, the requirements would be defined as min(N*LO periodicity, Tupper bound). And it means when DRX cycle is large, UE is required to perform measurement based on an interval smaller than DRX cycle. We think it is not reasonable or necessary considering UE powering saving, and there were not any such requirements in legacy.

Proposal 4: No need to define upper bound for SSB based LP-WUR measurement requirements.

Issue 1-2-4-2-3: On how to define LR evaluation requirements Working assumption: Option 1: using x1=2*x and y1=2*y for the evaluation requirement. Option 2: The value x1 for LP-SS based and y1 for SSB based LR evaluation requirement could be defined by adding extra [1 or 2 ] samples

For defining the LR evaluation requirements, the baseline sample number x and y which are used to satisfy accuracy requirement are needed based on the working assumption in last meeting. And based on our simulation results in companion paper, to achieve 3.5dB accuracy, 1 sample is needed for AWGN channel and 2 samples are needed for TDL channel, except one case that when M=1, L=6 and the interference LP-SS sequence is opposite. So the measurement requirements can be defined based on 2 samples for both SSB based and LP-SS based. For evaluation requirements, we prefer to use option 1 which is safer and can guarantee at least two measurements to do filter.

For RF margin, we think at least 2.5dB is needed and we are open to consider the larger value.

Proposal 5: Use x1=2*x and y1=2*y for the LR evaluation requirement where x = y = 2 samples.

Proposal 6: Define requirements based on [6] dB accuracy and reflect the margin into entry/exit condition.

Issue 1-5-1: LR based RRM with EMR in IDLE/inactive mode - Proposals - P1: For a UE which supports idleInactiveNR-MeasReport-r16 or idleInactiveEUTRA-MeasReport-r16, and serving cell configures carriers for idle mode CA/DC measurement reporting with T331 running, UE shall keep the MR ON for EMR measurement regardless of the MR offloading condition is met or not. (Apple Nokia) - P1-1: UE shall not enter relaxation / offloading if configured simultaneously with EMR. (Nokia) - P2: Postpone the discussion on the EMR issue (vivo) - P3: RAN4 to discuss the UE’s behaviour of MR relaxation with EMR impact (Ericsson Nokia)

In legacy requirements, the inter-frequency, inter-RAT and configured IDLE model CA/DC measurements share the same scaling factor, and when the relaxed measurement criterion is fulfilled, we need to consider whether to relax the IDLE mode CA/DC measurement.

In Rel-16 powering saving, the IDLE mode CA/DC measurement is relaxed only when T331 is not running. We think the same approach can be reused for case 3.

And for high priority layers, it is allowed to perform relaxed measurement according to the highPriorityMeasRelax indication in Rel-16 power saving. But for this work item, when case 1 condition is fulfilled, we understand the high priority layers measurement can be relaxed without indication and when T331 is running, the carriers configured for IDLE mode CA/DC measurements can also be included.

Proposal 7: When case 3 (relaxed measurement) criterion is fulfilled, the configured IDLE mode CA/DC measurements can be relaxed only when T331 is not running.

Proposal 8: When case 1 (serving cell measurement offloading) criterion is fulfilled, UE is expected to perform relaxed high priority layers measurement without NW indication. And when T331 is running, the carriers configured for IDLE mode CA/DC measurements can also be included.

Issue 1-5-3: MR measurements after wake-up - Proposals: - P1: After MR wake up, MR follows SSB periodicity for serving and neighbouring cell evaluation. (Nokia)

There are two cases that UE need to wake up MR. The first one is when UE is at case 1 and need to perform high priority layers measurement. In such case, it is clear that MR only need to perform relaxed neighbor cell measurement. The second one is when UE exits case 1. For such case, RAN2 has defined the entry and exit conditions for serving cell offloading as below.

##### 5.2.4.x.3 Serving cell measurement offloading rules for LP-WUS UE LP-WUS UE may choose to perform serving cell measurement offloading (i.e., serving cell measurement fully offloaded to LR and no serving cell measurement via MR is required) according to requirements specified in TS 38.133 [8] if the entry condition for serving cell measurement offloading in clause 5.2.4.x.4 is fulfilled. LP-WUS UE is not required to perform serving cell measurement offloading according to requirements specified in TS 38.133 [8] if the exit condition for serving cell measurement offloading in clause 5.2.4.x.4 is fulfilled. ##### 5.2.4.x.4 Serving cell measurement offloading criterion for LP-WUS UE The entry condition for serving cell measurement offloading is fulfilled when: - Srxlev > SLP_WUS_offloadingEntryThresholdP_MR, and, - Qrxlevmeas_lr > QLP_WUS_offloadingEntryThresholdP_LR, if QLP_WUS_offloadingxEntryThresholdP_LR is configured, and, - Squal > SLP_WUS_offloadingEntryThresholdQ_MR, if SLP_WUS_offloadingEntryThresholdQ_MR is configured, and - Qqualmeas_lr > QLP_WUS_offloadingEntryThresholdQ_LR, if QLP_WUS_offloadingEntryThresholdQ_LR is configured. The exit condition for serving cell measurement offloading is fulfilled when: - Qrxlevmeas_lr < QLP_WUS_offloadingExitThresholdP_LR, or, - Qqualmeas_lr < QLP_WUS_offloadingExitThresholdQ_LR, if QLP_WUS_offloadingExitThresholdQ_LR is configured. The variables in the formula are defined as follows: - Srxlev = current Srxlev value of the serving cell (dB). - Squal = current Squal value of the serving cell (dB). - Qrxlevmeas_lr= current measured cell RX level value of the serving cell based on LR (RSRP). - Qqualmeas _lr = current measured cell quality value of the serving cell based on LR (RSRQ). Editor’s NOTE: The detailed parameters for serving cell measurement offloading will be aligned with RRC specification. Editor’s NOTE: FFS how to capture separate thresholds for different UE types (to be aligned with RRC specification).

Based on the behavior description, LP-WUS UE is not required to perform serving cell measurement offloading according to requirements specified in TS 38.133 [8] if the exit condition for serving cell measurement offloading is fulfilled. But which requirements to be met after exiting, relaxed measurement or legacy measurement, is not clear. From our understanding, the default behavior should be legacy if no corresponding criterion is evaluated and fulfilled. But we can also see that only one evaluation period will be impacted and after this evaluation, the UE behavior will be clear. So we are also OK to use relaxed measurement requirements considering powering saving and the less difference between the two approaches.

Proposal 9: When UE exits the serving cell offloading case, the following approaches can be considered for UE behavior:

- Option 1: UE follow relaxed measurement and meet the corresponding requirements.

- Option 2: UE follow legacy measurement and meet the corresponding requirements.

- Option 3: Up to UE implementation and no requirements apply.

One additional issue to be clarified is the following time for cell selection initiation which is 10s for legacy. But for LP-WUS UE, the evaluation time of cell selection criteria has already exceeded 10s, the time for cell selection initiation may need to be updated.

If the LP-WUS UE in RRC_IDLE has not found any new suitable cell based on searches and measurements using the intra-frequency, inter-frequency and inter-RAT information indicated in the system information during the time T, the LP-WUS UE shall initiate cell selection procedures for the selected PLMN as defined in TS 38.304 [1], where T= 10s. Table 4.X.2.3.1-1: Nserv DRX cycle length [s]; Nserv [number of 16*DRX cycles]. 0.32; 4. 0.64; 4. 1.28; 2. 2.56; 2.

Proposal 10: RAN4 to discuss whether to update the time for cell selection initiation for LP-WUS UE.

When we review the draft CR, we found that the spec structure is not aligned, and we think it is better to align them firstly and the following can be used as baseline:

## 4.x IDLE mode measurement for UE with LP-WUR ### 4.x.1 Introduction ### 4.x.2 Requirements ### 4.x.2.1 UE measurement capability ### 4.x.2.2 LP-WUR serving cell measurement and evaluation requirements ##### 4.x.2.2.1 LP-WUR measurement and evaluation requirements for PSS/SSS ##### 4.x.2.2.2 LP-WUR measurement and evaluation requirements for LP-SS #### 4.X.2.3 MR Measurement and evaluation of serving cell #### 4.X.2.4 MR measurements of intra-frequency NR cells #### 4.X.2.5 MR Measurements of inter-frequency NR cells ##### [4.X.2.5.2 Measurements for UE with LP-WUR fulfilling [case3/relaxed MR measurement] criterion] ##### [4.X.2.5.3 Measurements for UE with LP-WUR fulfilling [MR fully offloading] criterion] #### 4.X.2.6 MR Measurements of inter-RAT E-UTRAN cells ##### [4.X.2.6.2 Measurements for UE fulfilling relaxed measurement criteria] ##### [4.X.2.6.3 Measurements for UE fulfilling serving cell measurement offloading entry criteria] 4.X.2.7 Maximum interruption in paging reception 4.X.2.8 MR measurements of high priority layers

Proposal 11: Use the above structure as baseline for drafting CR.

- Conclusion

This contribution gave the following proposals on the core requirements for LP-WUR/WUS.

Proposal 1: No LR measurement and evaluation requirements apply at the legacy state.

Proposal 2: Prioritize defining RRM requirements for FR1 in this WI.

Proposal 3: The requirements for normal UE should be completed in high priority in Rel-19. After that, RAN4 can further discuss whether to apply for Redcap UE considering the RRM workload, RAN2 signaling support and UE RF impact.

Proposal 4: No need to define upper bound for SSB based LP-WUR measurement requirements.

Proposal 5: Use x1=2*x and y1=2*y for the LR evaluation requirement where x = y = 2 samples.

Proposal 6: Define requirements based on [6] dB accuracy and reflect the margin into entry/exit condition.

Proposal 7: When case 3 (relaxed measurement) criterion is fulfilled, the configured IDLE mode CA/DC measurements can be relaxed only when T331 is not running.

Proposal 8: When case 1 (serving cell measurement offloading) criterion is fulfilled, UE is expected to perform relaxed high priority layers measurement without NW indication. And when T331 is running, the carriers configured for IDLE mode CA/DC measurements can also be included.

Proposal 9: When UE exits the serving cell offloading case, the following approaches can be considered for UE behavior:

- Option 1: UE follow relaxed measurement and meet the corresponding requirements.

- Option 2: UE follow legacy measurement and meet the corresponding requirements.

- Option 3: Up to UE implementation and no requirements apply.

Proposal 10: RAN4 to discuss whether to update the time for cell selection initiation for LP-WUS UE.

Proposal 11: Use the following structure as baseline for drafting CR.

## 4.x IDLE mode measurement for UE with LP-WUR ### 4.x.1 Introduction ### 4.x.2 Requirements ### 4.x.2.1 UE measurement capability ### 4.x.2.2 LP-WUR serving cell measurement and evaluation requirements ##### 4.x.2.2.1 LP-WUR measurement and evaluation requirements for PSS/SSS ##### 4.x.2.2.2 LP-WUR measurement and evaluation requirements for LP-SS #### 4.X.2.3 MR Measurement and evaluation of serving cell #### 4.X.2.4 MR measurements of intra-frequency NR cells #### 4.X.2.5 MR Measurements of inter-frequency NR cells ##### [4.X.2.5.2 Measurements for UE with LP-WUR fulfilling [case3/relaxed MR measurement] criterion] ##### [4.X.2.5.3 Measurements for UE with LP-WUR fulfilling [MR fully offloading] criterion] #### 4.X.2.6 MR Measurements of inter-RAT E-UTRAN cells ##### [4.X.2.6.2 Measurements for UE fulfilling relaxed measurement criteria] ##### [4.X.2.6.3 Measurements for UE fulfilling serving cell measurement offloading entry criteria] 4.X.2.7 Maximum interruption in paging reception 4.X.2.8 MR measurements of high priority layers

References

- R4-2508280 WF on RRM requirements for NR_LPWUS, vivo, RAN4#115

- R4-2505572 Topic summary for [115][223] NR_LPWUS, vivo, RAN4#115