3GPP TSG-RAN WG4 Meeting #116									     R4-2509697

Bengaluru, India, August 25th – 29th, 2025

Source: 	OPPO

Title: 	On RRM performance requirements of R19 NES

Agenda Item:	7.23.4

Document for:	Discussion

Introduction

In this document, we provide our views on the RRM performance and test case requirements in R19 NES.

Discussion

- OD-SSB

In core part, RAN4 has agreed to define requirements for OD-SSB case 1 and OD-SSB+AO-SSB case 2, including OD-SSB based deactivated SCell measurement and Scell activation. And for scenario 3B, it is not discussed too much in RAN4 and we don’t think the measurement requirements would be quite different due to OD-SSB and RAN4 does not need to test these.

In our view, RAN4 needs to test the new cases which are brought due to OD-SSB, like FMW for deactivated Scell measurement, OD-SSB based Scell activation for scenario 2 and 2a. For case 1 and 2, most procedure and requirements are similar except the effective periodicity of OD-SSB and AO-SSB in certain scenarios. We prefer to only choose one test case for case 2 to justify this.

Observation 1: RAN4 needs to test the new cases which are brought due to OD-SSB, like FMW for deactivated Scell measurement, OD-SSB based Scell activation.

Proposal 1: RAN4 not need to test the requirements for scenario 3B.

Observation 2: For case 1 and 2, most procedure and requirements are similar except the effective periodicity of OD-SSB and AO-SSB in certain scenarios.

Proposal 2: RAN4 to only choose one test case for case 2 to justify the effective periodicity of OD-SSB and AO-SSB.

- OD-SIB1

The OD-SIB1 acquisition procedure includes new steps like UL-WUS transmission and RAR and new Paging interruption requirements are going to be defined in core part. But RAN4 only defined test cases for cell-reselection in Idle mode before, and no test case for paging interruption. Correspondingly, we prefer RAN4 not to define new test cases to verify paging interruption due to OD-SIB1in IDLE mode.

Proposal 3: As for no test case for paging interruption in current spec, RAN4 not to introduce any new test case due to OD-SIB1 acquisition procedure in IDLE mode.

- SSB adaptation

For SSB adaptation, L1 and L3 impact are identified in core requirements. But compared to legacy UE behaviour, the main different parts due to SSB adaptation are about DCI triggering and new SMTC mapping to SSB periodicity. From procedure wise, we think the impact of SSB adaptation for either L1 or L3 measurements could be similar. To reduce the amount of test cases, we prefer to define one TC for L3 measurement.

For example, RAN4 tests SSB adaptation with normal Scell activation or for L3 measurement, assuming adapted SSB periodicity and updated SMTC periodicity applies after DCI 2_9 indication.

Proposal 4: To reduce the amount of test cases, RAN4 to test SSB adaptation for either Scell activation or for L3 measurement, not to define new test case for L1 measurement.

Conclusions

Based on above analysis, we have following observations and proposals.

Observation 1: RAN4 needs to test the new cases which are brought due to OD-SSB, like FMW for deactivated Scell measurement, OD-SSB based Scell activation.

Proposal 1: RAN4 not need to test the requirements for scenario 3B.

Observation 2: For case 1 and 2, most procedure and requirements are similar except the effective periodicity of OD-SSB and AO-SSB in certain scenarios.

Proposal 2: RAN4 to only choose one test case for case 2 to justify the effective periodicity of OD-SSB and AO-SSB.

Proposal 3: As for no test case for paging interruption in current spec, RAN4 not to introduce any new test case due to OD-SIB1 acquisition procedure in IDLE mode.

Proposal 4: To reduce the amount of test cases, RAN4 to test SSB adaptation for either Scell activation or for L3 measurement, not to define new test case for L1 measurement.