# 3GPP TSG-RAN WG4 Meeting #116	R4-2509700

# Bengaluru, India, August 25th – 29th, 2025

Title: 	On CSI-RS based L1 measurements

Source: 	OPPO

Agenda item:	7.25.3.2

Document for:	Discussion

Introduction

In last meeting, a WF was approved on RRM requirements for R19 mobility [1].

In this contribution, we provide our views on remaining issues of CSI-RS based L1 measurements.

Discussion

Issue 3-2: whether step 2 is skipped for serving cell? Candidate options: - Option A: RAN4 requirements regarding ‘skipping step 2’ do not apply for the case CSI-RS based L1 measurement for LTM candidate cell is configured in legacy CSI-RS based L1 measurement in serving cell in FR2. (QC, HW) - Option A1: Requirement regarding ‘skipping step 2’ is not applicable if an FR2 candidate cell, from which CSI-RS is configured with repetition set to ‘off’, is one of the FR2 serving cells. (QC, OPPO) - Option A2: The CSI-RS based L1-RSRP measurement requirement is not applicable if an FR2 candidate cell, from which CSI-RS is configured with repetition set to 'off', is one of the FR2 serving cells (QC). - Option B: keep the current measurement procedure unchanged for serving cell when it is in the candidate cell for LTM. (MTK, Apple, E///)

RAN4 has agreed that whether step 2 can be skipped for neighboring cell or not based on new UE capability reporting, but needs to further discuss whether step 2 can be skipped for serving cell or not.

Agreement:

- For a UE not supporting the capability, Step 2 is necessary.

  - The CSI-RS resources in Step 3 should be at least Type-D QCL’ed with the SSB in Step 2.

  - UE Rx beam sweeping factor is not included in the CSI-RS based L1-RSRP measurement period assuming the CSI-RS resources are in the CSI-RS resource set with ‘repetition = off.’

- For a UE supporting the capability, Step 2 can be skipped, and if the Step 2 is skipped:

  - The CSI-RS resources in Step 3 should be Type-D QCL’ed with the SSB in Step 1.

  - The CSI-RS resources should be periodic or semi-persistent, and the CSI-RS resources are in the CSI-RS resource set with ‘repetition = off.’

  - UE Rx beam sweeping factor needs to be included in the CSI-RS based L1-RSRP measurement period, and further discuss the detail of beam sweeping factor.

If the candidate LTM cell is a serving cell, the UE capability of skipping SSB based L1 measurement in step 2 can also apply. But the intermediate SSB-based L1-RSRP measurement on serving cell cannot skipped in step 2. For gNB-UE beam management on a serving cell, the UE could still use SSB based L1-RSRP measurement for UE Rx beam refinement for the serving cell. UE should follow 3 components for L1 measurement on serving cell as follows, where P-2 and P-3 cannot be changed for serving cell.

- P-1 (wide gNB Tx & wide UE Rx): SSB based BM (implicit/explicit SSB-ID report to gNB)

- P-2 (narrow gNB Tx & wide UE Rx): On CSI-RS resources with ‘repetition = off’, UE applies a fixed Rx beam obtained from P1 and measures/reports CRI and L1-RSRP.

- P-3(narrow gNB Tx & narrow UE Rx): On CSI-RS resources with ‘repetition = on’, UE sweeps its Rx beam with a finer beamwidth, desirably confined within the P1/P2 Rx beamwidth. No report is required.

Thus, we support to introduce a unified UE capability of skipping SSB based L1 measurement for CSI-RS based LTM, but requirement regarding ‘skipping step 2’ is not applicable if an FR2 candidate cell, from which CSI-RS is configured with repetition set to ‘off’, is one of the FR2 serving cells.

Proposal 1: RAN4 Requirement regarding ‘skipping step 2’ is not applicable if an FR2 candidate cell, from which CSI-RS is configured with repetition set to ‘off’, is one of the FR2 serving cells.

- Issue 3-3: Sharing factor for resource collision between serving and neighbour cell Candidate options: - For FR2 CSI-RS L1 measurement requirement, - if the CSI-RS resources configured for L1 RSRP measurement in serving cell and neighbour cell are colliding in time domain,. - Option 1: sharing factor shall be introduced. (HW, QC, E///) - Option 2: introduce measurement restriction as legacy. (MTK, vivo)

We think either option can be implemented, but to be simplified, option 2 needs less changes to current spec, and also UE can perform L1 measurement more flexible. We support to introduce measurement restriction for UE to select one of the two CSI-RS resources configured for L1 RSRP measurement in serving cell and neighbour cell in case of colliding in time domain.

Proposal 2: Support to introduce measurement restriction for UE to select one of the two CSI-RS resources configured for L1 RSRP measurement in serving cell and neighbour cell in case of colliding in time domain.

Conclusion

In the contribution we provide the following proposals:

Proposal 1: RAN4 Requirement regarding ‘skipping step 2’ is not applicable if an FR2 candidate cell, from which CSI-RS is configured with repetition set to ‘off’, is one of the FR2 serving cells.

Proposal 2: Support to introduce measurement restriction for UE to select one of the two CSI-RS resources configured for L1 RSRP measurement in serving cell and neighbour cell in case of colliding in time domain.

Reference

- R4-2508282, WF on NR mobility enhancements Phase 4_part1