# GPP TSG-RAN WG4 Meeting #116	R4-2509703

# Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.26.4

Source:	OPPO

Title:	Discussion on RRM performance requirements for XR

Document for:	Approval

- Introduction

Based on the agreements for XR core requirements reached in previous RAN4 meetings, this contribution will provide our considerations on test case design due to the MG skipping.

- Discussion

## Issue 5-2: Skipping indication on frequency domain <Agreement>: - It is RAN4 common understanding that: If UE is configured with per-FR gap, the MG skipping only applies to the same FR and has no impacts on UE behavior on the other FR. If UE is configured with per-UE gap, the MG skipping applies to both FRs. - Further discuss whether it has already been specified in RAN1 CR, and whether/how to capture in RAN4 spec.

As agreed in the last RAN4 meeting, UE behaviour for the MG skipping is different for per-FR gap and per-UE gap. For per-FR gap, UE can only skip the gap in the same FR and can enable Tx/Rx during the gap occasion in the same FR. However, UE behaviour on the other FR will not be impacted. When the MG skipping is applicable to a per-UE gap, UE can enable Tx/Rx for both FRs. These two cases should be verified in the test cases.

Proposal 1: Consider both per-FR gap and per-UE gap in the test case.

## Issue 2-1: Type 2 gaps <Agreement> - MG cancelling DCI applies to concurrent MGs and cancels the MG surviving the collision handling rules. - The timeline X ms for the MG cancelling the colliding MGs is based on the one that survives collision handling rules. - DCI skipping cannot be applied if any gap is configured with gapAssociationPRS - FFS the applicability of the gap cancellation if two non colliding concurrent gaps are separated by less than X ms ## Issue 2-2: NCSG <Agreement> - MG cancelling DCI doesn’t apply to MGs configured via GapConfig-r17 with ncsgInd-r17. ## Issue 2-3: Preconfigured measurement gaps <Agreement> - MG cancelling DCI doesn’t apply to MGs configured via GapConfig-r17 with preConfigInd-r17. ## Issue 2-4: Rel-18 concurrent gaps <Agreement> - MG cancelling DCI doesn’t apply to Rel-18 concurrent gaps.

Besides, the MG skipping scheme applies to Rel-17 concurrent gaps, and the gap occasion surviving the Rel-17 gap collision handling rules will be cancelled in XR. This scenario should also be considered in the test case.

Proposal 2: Consider Rel-17 concurrent gaps in the test case.

When the MG occasion is cancelled by DCI, the delay requirements for the MO which is supposed to be measured in the MG will be extended and RAN4 have identified the impacted clauses as shown above. We think the extended delay requirements for the measurement above should be verified.

Proposal 3: Define test cases for:

- Intra-frequency measurements with gaps

- Inter-frequency measurements with gaps

- Inter-RAT E-UTRAN FDD measurements

- Inter-RAT E-UTRAN TDD measurements

- Inter-frequency L1-RSRP with MG

## Issue 3-1: Formulas for measurement delay extension without DRX <Way forward> - RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - Option 1: Max(200 ms × CSSFinter, 8 × Max(MGRP , SMTC period) × CSSFinter + Nskip × Max(MGRP , SMTC period)), - Option 2: Max( 200 ms, ( 8 + Nskip ) ´ Max(MGRP , SMTC period) ) ´ CSSFinter - Option 3: - Tidentify_inter_without_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter+ Tskip) ms - Tidentify_inter_with_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter + TSSB_time_index_inter+ Tskip) ms - where Tskip = Nskip ´ Max(MGRP , SMTC period) - Other options are not precluded - where Nskip is the number of skipped MG occasions in the measurement period. - FFS: CSSFinter ## Issue 3-2: Formulas for measurement delay extension with DRX <Way forward>: - Option 1: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - DRX cycle ≤ 320 ms - Max(200 ms CSSFinter, Ceil(8 1.5) Max(MGRP, SMTC period, DRX cycle) CSSFinter + Nskip × Max(MGRP, SMTC period, DRX cycle) ) - DRX cycle > 320 ms - 8 DRX cycle CSSFinter + Nskip DRX cycle - Option 2: Inter-frequency cell detection period requirements with MG cancellation due to XR can be as in the table below for FR1 and FR2-1, respectively: - DRX cycle ≤ 320 ms - Max(200 ms, Ceil((8+XPSS/SSS,gaps) 1.5) Max(MGRP, SMTC period, DRX cycle)) CSSFinter - DRX cycle > 320 ms - Ceil(8+XPSS/SSS,gaps) DRX cycle CSSFinter - Option 3: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - Tidentify_inter_without_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter+ Tskip) ms - Tidentify_inter_with_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter + TSSB_time_index_inter+ Tskip) ms - Where - DRX cycle ≤ 320 ms, Tskip = Nskip ´ Max(MGRP , SMTC period, DRX cycle) - DRX cycle > 320 ms, Tskip = Nskip ´ DRX cycle - Other options are not precluded

Although the exact delay extension requirements are not determined yet, it seem different formulas may be used without DRX and with DRX. We propose to consider both cases when defining test cases.

Proposal 4: Define the test cases for the cases without DRX and with DRX.

To reduce the workload on test cases and meanwhile maintain the test coverage, it is better to verify both UE behaviour of Tx/Rx on the skipping MG occasion and the extended delay in the same test cases.

Proposal 5: UE Tx/Rx behavior in the skipping MG occasions and the extended delay requirements should be verified in the same test case.

- Conclusion

This contribution gave the following proposals for test cases considering the MG skipping in XR:

Proposal 1: Consider both per-FR gap and per-UE gap in the test case.

Proposal 2: Consider Rel-17 concurrent gaps in the test case.

Proposal 3: Define test cases for:

- Intra-frequency measurements with gaps

- Inter-frequency measurements with gaps

- Inter-RAT E-UTRAN FDD measurements

- Inter-RAT E-UTRAN TDD measurements

- Inter-frequency L1-RSRP with MG

Proposal 4: Define the test cases for the cases without DRX and with DRX.

Proposal 5: UE Tx/Rx behavior in the skipping MG occasions and the extended delay requirements should be verified in the same test case.

References

- R4-2508285, WF on RRM requirements for NR_XR_Ph3, RAN4 #115