# 3GPP TSG-RAN WG4 Meeting #116	R4-2509706

# Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	7.27.6

Source: 	OPPO

Title: 	On RRM performance requirements of (e)RedCap UEs with NR FR1-NTN

Document for:	Approval

Introduction

In last RAN4 meeting, a WF on RRM requirements for Rel-19 NR NTN phase3 was approved [1].

In this paper, we will provide our views on test cases of (e)RedCap UEs with NR FR1-NTN.

Discussion on support of (e)RedCap UE with NTN

<Way Forward>: - Proposal 1 (CMCC): - For 2Rx RedCap UE, reuse the measurement accuracy requirement of NTN. - For 1Rx RedCap UE, define new measurement accuracy requirement on NTN operating bands with reusing the RSRP accuracy and side conditions from legacy 1Rx RedCap UE. - Proposal 2 (vivo): For the performance requirement in NTN for NR Phase 3, measurement accuracy requirements need to be defined for (e)Redcap UEs with FR1-NTN. - Proposal 3 (HW): RAN4 to define accuracy requirements for following measurements for RedCap in NTN. - Intra- and inter-frequency RSRP - Intra- and inter-frequency RSRQ - Intra- and inter-frequency SINR - L1-RSRP - Rx-Tx for NW verified location

For measurement accuracy, we support to keep the agreements in RAN4#114bis.

RAN4#114-bis:

#### Issue 2-2-1: L1/L3 measurement accuracy for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

Agreement:

    - For 2Rx (e)RedCap UEs with FR1-NTN bands, RAN4 to reuse the measurement accuracy requirements for normal UEs defined in NTN.

    - For 1Rx (e)RedCap UEs with FR1-NTN bands, RAN4 to relax the L1/L3 measurement accuracy for SS-RSRP, SS-RSRQ, SS-SINR and L1-RSRP compared to those defined for 2Rx (e)RedCap UEs.

      - The same relaxation defined in TN for 1Rx (e)RedCap UEs can be reused.

- SS-RSRP, SS-RSRQ, SS-SINR: relax by 1dB.

- L1-RSRP: relax by 3dB.

Proposal 1: RAN4 to reuse the measurement accuracy requirement of 2Rx UEs and relaxed 1 Rx UE for RedCap over NTN.

##### Issue 2-1-4-2: General test scope <Way Forward>: The following Table is based on the input from companies, and it can be a baseline. Test category; Company comments. RRC_IDLE state mobility; Cell reselection; Support: CATT, Ericsson, vivo, ZTE, CMCC, HW. RRC_IDLE state mobility; Configured Grant based Small Data Transmissions (CG-SDT); Support: CATT, Ericsson, vivo. RRC_CONNECTED state mobility; Handover - RACH-based - RACH-less; Support: CATT, vivo, ZTE, CMCC, HW. CHO - time-based - location-based; Support: CATT, Ericsson, vivo, ZTE, CMCC, HW. satellite switching with re-synchronization; Support: CATT, vivo, ZTE, CMCC, HW. RRC Connection Mobility Control; Support: CATT, Ericsson, vivo, ZTE, CMCC, HW. Timing; UE transmit timing; Support: CATT, Ericsson, vivo, CMCC, HW. UE timer accuracy; Support: vivo. Timing Advance; Support: CATT, Ericsson, vivo, CMCC, HW. Signalling characteristics; Radio Link Monitoring; Support: CATT, Ericsson, vivo, ZTE, CMCC, HW. Beam Failure Detection and Link recovery; Support: CATT, Ericsson, vivo, ZTE, CMCC, HW. Active BWP switch delay; Support: CATT, Ericsson, vivo, CMCC, HW. Active TCI state switching delay; Support: vivo. UE-specific CBW change; Support: CATT, Ericsson, vivo, CMCC, HW. Pathloss reference signal switching delay; Support: CATT, Ericsson, vivo, HW. Measurement procedure; NR intra-frequency measurements; Support: CATT, Ericsson, vivo, ZTE, CMCC, HW. NR inter-frequency measurements; Support: CATT, Ericsson, vivo, ZTE, CMCC, HW. L1-RSRP measurements for Reporting; Support: CATT, Ericsson, vivo, ZTE, CMCC, HW. NR measurements for positioning; Support: CATT, Ericsson. Measurement Performance; SS-RSRP/SS-RSRQ/SS-SINR Measurement Performance; Support: Ericsson, CMCC, HW. L1-RSRP measurement Performance for beam reporting; Support: Ericsson, CMCC, HW. - RAN4 to further discuss the specific test case list with configurations.

In general, RAN4 should avoid redundant test cases for redcap UE with NTN, compared to NTN normal UE or redcap UE. The delta parts are to be additionally tested in new test cases.

The following list in proposal 2 is our preference.

Proposal 2: Consider the test case list as below:

| Test category |  | Comments |
| --- | --- | --- |
| RRC_IDLE state mobility | Cell reselection | Only test the delta part |
| RRC_IDLE state mobility | Configured Grant based Small Data Transmissions (CG-SDT) | Not needed, due to similar core requirements |
| RRC_CONNECTED state mobility | Handover - RACH-based - RACH-less | Not needed, due to similar core requirements |
|  | CHO - time-based - location-based | Choose one |
|  | satellite switching with re-synchronization |  |
|  | RRC Connection Mobility Control | Only test the delta part |
| Timing | UE transmit timing | Only test the delta part |
|  | UE timer accuracy |  |
|  | Timing Advance |  |
| Signalling characteristics | Radio Link Monitoring | Only test the delta part |
|  | Beam Failure Detection and Link recovery | Only test the delta part |
|  | Active BWP switch delay |  |
|  | Active TCI state switching delay |  |
|  | UE-specific CBW change |  |
|  | Pathloss reference signal switching delay |  |
| Measurement procedure | NR intra-frequency measurements | Only test the delta part |
|  | NR inter-frequency measurements | Only test the delta part |
|  | L1-RSRP measurements for Reporting | Only test the delta part |
|  | NR measurements for positioning |  |
| Measurement Performance | SS-RSRP/SS-RSRQ/SS-SINR Measurement Performance | Only test the delta part |
|  | L1-RSRP measurement Performance for beam reporting | Only test the delta part |

Conclusions

In this paper we provided our views on RRM perf requirements for support of (e)RedCap UEs with NR FR1-NTN.

Proposal 1: RAN4 to reuse the measurement accuracy requirement of 2Rx UEs and relaxed 1 Rx UE for RedCap over NTN.

Proposal 2: Consider the test case list as below:

| Test category |  | Comments |
| --- | --- | --- |
| RRC_IDLE state mobility | Cell reselection | Only test the delta part |
| RRC_IDLE state mobility | Configured Grant based Small Data Transmissions (CG-SDT) | Not needed, due to similar core requirements |
| RRC_CONNECTED state mobility | Handover - RACH-based - RACH-less | Not needed, due to similar core requirements |
|  | CHO - time-based - location-based | Choose one |
|  | satellite switching with re-synchronization |  |
|  | RRC Connection Mobility Control | Only test the delta part |
| Timing | UE transmit timing | Only test the delta part |
|  | UE timer accuracy |  |
|  | Timing Advance |  |
| Signalling characteristics | Radio Link Monitoring | Only test the delta part |
|  | Beam Failure Detection and Link recovery | Only test the delta part |
|  | Active BWP switch delay |  |
|  | Active TCI state switching delay |  |
|  | UE-specific CBW change |  |
|  | Pathloss reference signal switching delay |  |
| Measurement procedure | NR intra-frequency measurements | Only test the delta part |
|  | NR inter-frequency measurements | Only test the delta part |
|  | L1-RSRP measurements for Reporting | Only test the delta part |
|  | NR measurements for positioning |  |
| Measurement Performance | SS-RSRP/SS-RSRQ/SS-SINR Measurement Performance | Only test the delta part |
|  | L1-RSRP measurement Performance for beam reporting | Only test the delta part |

Reference

- R4-2508287, WF on RRM requirements for NR_NTN_Ph3_Part1, CATT