# 3GPP TSG-RAN WG4 Meeting #116	R4-2509707

# Bengaluru, India, August 25th – 29th, 2025

Agenda item:	9.3

Source: 	OPPO

Title: 	Discussion on neighboring cell measurement for Rel-18 SSB-less

Document for:	Discussion

Introduction

An LS on neighbouring cell measurement for R18 SSB-less was received in this RAN4 meeting. It could have impact on the definition of intra-frequency measurement.

In this contribution paper, we provide our view on reply LS of neighboring cell measurement for Rel-18 SSB-less.

Discussion

RAN2 have discussed the presence of servingCellMO for SSB-less SCell and the impact on neighbour cell measurements and reached the following agreements: - Introduce per UE capability to indicate whether the UE supports the configuration of servingCellMO for SCell that does not transmit SS/PBCH block. A UE supporting this feature shall also support NR intra-frequency measurements on neighbour cells based on servingCellMO associated with SCell that does not transmit SS/PBCH block. NW can optionally configure servingCellMO for SCell that does not transmit SS/PBCH block if UE indicate such capability. ACTION: RAN2 respectfully asks RAN4 to take the above agreements and agreed CRs into account.

In current RAN4 spec, a measurement is defined as a SSB based intra-frequency measurement provided the centre frequency of the SSB of the serving cell and the centre frequency of the SSB of the neighbour cell indicated for measurement are the same, and the subcarrier spacing of the two SSBs are also the same.

If the UE supports ncd-SSB-BWP-Wor-r18, a measurement is defined as a SSB based intra-frequency measurement provided the centre frequency of the reference SSB of the serving cell and the centre frequency of the SSB of the neighbour cell are the same, and the subcarrier spacing of the two SSBs are also the same. The reference SSB is the SSB defined in BWP-specific servingCellMO under BWP-DownlinkDedicated of active DL BWP. If the field is absent, the reference SSB is the SSB defined in servingCellMO under ServingCellConfig [2].

A measurement is defined as an SSB based inter-frequency measurement provided it is not defined as an intra-frequency measurement.

What the reference SSB is for the case of the SSB-less SCell needs to be clarified. Based on RAN2 agreements, per UE capability to indicate whether the UE supports the configuration of servingCellMO for SCell that does not transmit SS/PBCH block was introduced.

If UE supports and indicates this capability, UE shall also support NR intra-frequency measurements on neighbour cells based on servingCellMO associated with SCell even if such SCell does not transmit SSB. Thus, the reference SSB for intra-frequency is the SSB defined in servingCellMO associated with SCell. When the centre frequency of the SSB of the neighbour cell and the reference SSB are the same, such measurement is defined as SSB based intra-frenquency measurement, otherwise it is inter-frequency measurement.

However, if UE does not support this capability of configuration of servingCellMO for SCell that does not transmit SS/PBCH block, the reference SSB for intra-frequency becomes the centre frequency of the SSB of the serving cell. Provided that the associated SCell does not transmit SSB, any SSB of the neighbour cell which falls into the Cell bandwidth of the SCell is inter-frequency SSB.

Whether the intra-frequency SSB or inter-frequency SSB based measurement needs gap depends on whether such SSB is fully within the active BWP of the SCell.

Proposal 1: If UE indicates the support of configuration of servingCellMO for SCell that does not transmit SS/PBCH block, the reference SSB for intra-frequency measurement is the SSB defined in servingCellMO associated with SCell.

Proposal 2: If UE does not support this capability, any SSB of the neighbour cell which falls into the cell bandwidth of the SCell is defined as inter-frequency SSB, provided that the associated SCell does not transmit SSB.

A revised CR for the definition of intra-frequency measurement is needed.

Proposal 3: Whether the intra-frequency SSB or inter-frequency SSB based measurement needs gap depends on whether such SSB is fully within the active BWP of the SCell.

Conclusion

We provide our views as below:

Proposal 1: If UE indicates the support of configuration of servingCellMO for SCell that does not transmit SS/PBCH block, the reference SSB for intra-frequency measurement is the SSB defined in servingCellMO associated with SCell.

Proposal 2: If UE does not support this capability, any SSB of the neighbour cell which falls into the cell bandwidth of the SCell is defined as inter-frequency SSB, provided that the associated SCell does not transmit SSB.

A revised CR for the definition of intra-frequency measurement is needed.

Proposal 3: Whether the intra-frequency SSB or inter-frequency SSB based measurement needs gap depends on whether such SSB is fully within the active BWP of the SCell.

- References

[1] R4-2509021,	Reply LS on neighboring cell measurement for Rel-18 SSB-less