# 3GPP TSG-RAN WG4 Meeting #116	R4-2509710

# Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.22.4

Source:	OPPO

Title:	Discussion on RRM core requirements for A-IoT

Document for:	Approval

- Introduction

This contribution will provide our considerations on RRM core requirements for A-IoT.

- Discussion

TD2R_min: the minimum time between a D2R transmission and the corresponding R2D transmission following it.

Agreement: It is up to RAN4 to define the value(s) for TD2R_min. - Note: RAN1 expects that the value(s) for TD2R_min will be defined in RAN4 specifications

In RAN1 #121 meeting, RAN1 expects that the value(s) for TD2R_min will be defined in RAN4. TD2R_min is the minimum time gap between a D2R transmission and the corresponding R2D transmission. During this time gap, the reader is expected to process D2R transmission and prepare R2D response, and meanwhile the device needs to switch from Tx state to Rx state.

Observation 1: TD2R_min should consider processing and preparing time for reader and Tx-Rx switch time for device.

From functionality perspective, TD2R_min is quite similar as T2 in RFID system, where minimum value of T2 is defined as 3.0*Tpri. This principle can be used as the starting point to define TD2R_min.

Proposal 1: TD2R_min = 3* TD2R_chip can be used as the starting point, where TD2R_chip is the chip length of D2R transmission.

Issue 1-2-2: Device receive timing FFS: - Option 1 (ZTE): RAN4 shall consider accuracy requirements on TD2R_min if the range of it depends on RAN1.

The next open issue in RRM is whether to consider accuracy requirements on TD2R_min. Our preference is not. The accuracy, if introduced in RAN4, is also dependent on the 10% SFO in our understanding. There is no need to define multiple types of accuracy requirements for SFO. On the other hand, accuracy on TD2R_min is not testable.

Proposal 2: Not consider accuracy requirements on TD2R_min.

- Conclusion

This contribution gave our considerations on test cases for D2R transmit timing requirements and the following proposals.

Observation 1: TD2R_min should consider processing and preparing time for reader and Tx-Rx switch time for device.

Proposal 1: TD2R_min = 3* TD2R_chip can be used as the starting point, where TD2R_chip is the chip length of D2R transmission.

Proposal 2: Not consider accuracy requirements on TD2R_min.

References

- R4-2508275, WF on RRM requirements for Ambient_IoT_Solutions, CMCC, RAN4 #115