# 3GPP TSG-RAN WG4 Meeting #116	R4-2509712

# Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.22.3.2

Source: 	CMCC

Title: 	Discussion on A-IoT device RF requirements

Document for:	Discussion

# 1. Introduction

In RAN plenary #106 meeting, a new WI[1] of A-IoT is approved with following RAN4 lead RF part objectives:

- RAN4 scope:

  - Specify RF requirements for Ambient-IoT BS, device 1, and CW

    - RF requirements for Type 1-C Ambient-IoT BS

    - RF requirements for device 1

    - RF requirements for CW

  - Specify RRM core requirements for device 1, if necessary

  - Study and develop OTA test methodology for A-IoT device 1

    - Consider test methods specified in TR 38.870 as starting point. Take test system reuse, test system complexity and test time into account, when developing test methods suitable for Ambient IoT.

    - Develop the preliminary Measurement Uncertainty (MU) assessment for the test system

  - Use band n8 as an example band

In this contribution, we discuss the RF requirements A-IOT devices.

- Discussion

2.1 Tx requirements

- Backscattering power

Last meeting agreements are listed as below:

Issue 3-1-1: Transmit output power Agreement in RAN4#115: - Performance metric: EIRP - FFS on how to determine device’s peak antenna gain direction - FFS on whether to measure multiple directions - Take [10] dB backscatter loss for OOK as starting point, based on the assumption that the incoming CW signal and the backscatter signal are aligned with the peak gain direction of the device antenna. - FFS for BPSK - The power of the backscattered signal shall include only the 1st lower sideband and the 1st upper sideband, excluding the carrier itself.

One issue is the gain orientation for Tx requirements. Define requirements like the spherical coverage will give more flexibility for final device deployment, e.g. no limiting the device orientation. It’s suggested to define spherical coverage related requirements. Besides, when defining Rx requirements, we also consider orientation related requirements. Both Tx and Rx are used to define orientation related requirements, to reduce testing time, we can only define one orientation requirements.

For backscatter power direction, it depends on the detailed CW input level direction. To simplify testing, it’s suggested to only define multiple direction requirements for Rx. As for the device peak antenna gain direction, it depends on how to design/place antenna and the direction of input CW level. One suggestion is that peak antenna gain direction is based on declaration.

Proposal 1: it’s suggested to only define device’s peak antenna gain direction which is based on declaration.

Some papers show the backscatter power is dependent on the input CW level. Therefore a two step model is suggested as below. Based on the survey from tag vendors, following two step model is suggested. We can define the backscatter power with input CW level in spec. As for the input CW level range for step one, the lower bound is suggested to be R2D refsense used for demode and the upper bound is suggested to be R2D refsense used for energy harvest +10dB.

Pbs≥Pcw−8, −34dBm≤Pcw≤−17dBm−25,  Pcw≥−17

For requirement definition, it’s suggested to leave more space for device implementation. It’s suggested to only define two points for backscatter power.

Proposal 2: it’s suggested to define two points for backscatter power into spec as below.

- When input CW level is at -27dBm, the backscatter loss is 8dB and backscatter power is -35dBm

- The maximum backscatter power is suggested to be larger than -25dBm

In theory, OOK’s backscatter is 6dB worse than BPSK, among which 3dB come from the 0 signal and the other 3dB comre from the DC part. For BPSK if there is phase difference error, power from “1” will leakge to “0” which equals to additional backscatter loss. So the loss for BPSK is suggested to be 4dB less than that of OOK.

Proposal 3: 8dB backscatter loss for OOK and 4dB for BPSK.

- SFO requirements

Issue 3-2-1: SFO requirement Agreement in RAN4#115: - No need to directly test SFO if SFO can be indirectly verified via other test. - FFS whether to explicitly define SFO requirements or not.

- As discussed in D2R channel bandwidth, SFO already been reflected into CBW definition which will be considerred during all other RF requirements testing/definition. At reader side, reader could know device’s SFO performance by using start/midable preamble. So even device has worst SFO performance, it doesn’t impact reader’s receiver performance. So no need for explicit SFO requirements.

- Proposal 4: no needs for explicit SFO requirements.

- Emission requirements

Issue 3-3-1: SEM requirements Agreement in RAN4#115: - Use EIRP as performance metric - Device’s peak antenna gain direction is based on declaration, i.e. same peak gain direction for backscattering loss testing - The applicable frequency range for SEM: from the CBW edge to [OOB boundary] - Define flat requirements for SEM as starting point, [9dBc~14dBc] based on achievable harmonic performance - Define requirements applicable to all CBWs, FFS on how to reduce the test burden. - Use the [maximum] CW input power level, FFS on the value.

- Different from NR system, the device RF part may have no impact on SEM performance and SEM requirements only based on signal characteristics, i.e. the harmonic performance of device.

- Observation 1: harmonic performance will determine final SEM requirements of device.

From this point of view, it seems only flat requirement is enough which is derived by the worst 3th harmonic performance with 9dB less level compared with fundamental signal. Considering sideband shift with SFS, the reference bandwidth is suggested as 12dBc compared with wanted power within DSB. For the testing, it seems no need to test the 3th harmonic impact for all CBWs, instead, the minimum and max are enough.

Proposal 5: 12dBc/1SB flat requirement is suggested for SEM. And can only test minimum and max CBW to reduce the test burden.

- Spurious requirements

Issue 3-3-1: spurious emission requirements Agreement in RAN4#115: - Use EIRP as performance metric - Device’s peak antenna gain direction is based on declaration, i.e. same peak gain direction for backscattering loss testing - OOB boundary - Option 1: D2R channel bandwidth plus minimum D2R channel bandwidth - Option 2: max of 500kHz and 10 times NB where NB equals to D2R CBW - Other options are not precluded - Define requirements applicable to all CBWs, FFS on how to reduce the test burden. - Use the [maximum] CW input power level, FFS on the value.

For spurious emission, legacy OOB of UE requriements is mainly used to leave transient bandwidth for RF band filter. For device, there is no transmit RF band filter. As discussed for occupied bandwidth, device has larger occupied bandwidth than NB which will be more easily to create interference. In theory, it would be much stricter for device to meet ITU spurious emission requirements. But spurious emission requirments come from ITu, device should still meet such requirements. So the OOB boundary should be larger than 500kHz or 10*NB, here the NB is suggested to equal to D2R CBW.

Observation 2: the OOB is defined  as Fb as in ITU, i.e. max of 500kHz and 10 times NB where NB equals to D2R CBW.

The spurious emission limits apply for the frequency ranges that are more than FOOB (MHz) from the edge of the channel bandwidth. If we follow the same logic as SEM that unwanted emission is contributed only from harmonic performance. Following table list the FOOB value from CBW edge which equals to nth harmonic that achieve -36dBm/100kHz.

| Tb (μs) | DSB | PSD | harmonic order | FOOB/kHz |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | -25 |  | Tchip (μs) |  |  |  |  |  |  |  |  |
|  |  |  |  | 133.33 | 66.67 | 33.33 | 16.67 | 8.33 | 4.17 | 2.08 | 1.04 | 0.69 |
| 266.67 | 15 | -36.7608583 | 11.00 | 79 | 154 | 304 | 604 | 1204 | 2402 | 4811 | 9619 |  |
| 133.33 | 30 | -39.77132112 | 7.00 |  | 97 | 188 | 367 | 728 | 1446 | 2892 | 5777 |  |
| 66.67 | 60 | -42.78129536 | 5.00 |  |  | 135 | 255 | 495 | 974 | 1938 | 3861 |  |
| 33.33 | 120 | -45.79224678 | 5.00 |  |  |  | 270 | 510 | 989 | 1953 | 3876 |  |
| 16.67 | 240 | -48.80124391 | 3.00 |  |  |  |  | 300 | 540 | 1022 | 1983 |  |
| 8.33 | 480 | -51.8141499 | 3.00 |  |  |  |  |  | 600 | 1082 | 2043 |  |
| 4.17 | 959 | -54.81923936 | 3.00 |  |  |  |  |  |  | 1201 | 2163 |  |
| 1.39 | 2878 | -59.59045191 |  |  |  |  |  |  |  |  |  |  |
| SFS |  |  |  | 3.75 | 7.50 | 15.00 | 29.99 | 60.02 | 119.90 | 240.38 | 480.77 | 724.64 |

The same logic as for SEM discussion, we have following proposals.

Proposal 6: only test minimum and max CBW to reduce the test burden.

Proposal 7: use the -10dBm maximum input power requirements(i.e. 1m distant CW assumption) as the maximum CW input power level for spurious emission testing.

2.1 Rx requirements

- REFSENSE

Issue 3-4-1: Reference sensitivity Agreement in RAN4#115: - Using [-34dBm] as starting point to define REFSENS requirements, based on the peak gain direction of device antenna. - Consider to define orientation requirements with following options: - Option 1: min sensitivity at peak antenna gain direction and EIS spherical coverage requirements, XdBm at Xth percentage - Option 2: spatial averaging EIS - Option 3: EIS in the peak direction - Testing metric: 10% miss detection rate, FFS on whether to consider false alarm - FFS for the FRC, consider following as starting point: SCS; 15; 15; 15; 15. PRB; 1; 2; 3; 4. TBS; 20; 20; 20; 20. CRC; 6; 6; 6; 6. M; 6; 12; 24; 24.

- The REFSENSE analysis is listed as below

| reader power | 33 |
| --- | --- |
| Coupling loss | 59 |
| REFSENSE | -30 |

- Proposal 8: REFSENSE is suggested as -34dBm

In actual deployment environment, the tag will randomly rotate in all directions. Device usually has omni-directional or using dipole antenna (omi-directional in ɵ plane). So EIS spherical coverage requriement is suggested, XdBm at Ydegree. Here the X is suggested to be 3dB worse than REFSENSE and Y is suggested to be +-60degree.

Proposal 9: define min sensitivity at peak antenna gain direction and EIS spherical coverage requirements, XdBm at Y degree. Y is suggested to be +-60degree, X is suggested to be 3dB worse than REFSENSE.

For the FRC, usually the REFSENSE is defined at cell edge which means smaller data rate, in such case, M value is suggested only for 2 and 6.

- Max input power

Issue 3-5-1: Maximum input power Agreement in RAN4#115: - Define maximum input level at peak antenna direction, using [-4dBm] as starting point.

If we assume 1m distance, the max input level is 38dBm+3dB(2Tx assumption) - 31.59=9.41dBm, if assume -34dBm REFSENSE which means 43.41dB dynamic range. Typical dynamic range for tag is in the range from 30dB to 40dB, if we choose 40dB dynamic range, max input level is -34+40=6dBm. If we choose 30dB dynamic range, max input level is -34+30= -4dBm.

Proposal 10: -4dBm max input power with 30dB dynamic range assumption.

# 3. Conclusions

In this contribution, RF requirements for device 1 are discussed with following observations and proposals.

Proposal 1: it’s suggested to only define device’s peak antenna gain direction which is based on declaration.

Proposal 2: it’s suggested to define two points for backscatter power into spec as below.

- When input CW level is at -27dBm, the backscatter loss is 8dB and backscatter power is -35dBm

- The maximum backscatter power is suggested to be larger than -25dBm

Proposal 3: 8dB backscatter loss for OOK and 4dB for BPSK.

- Proposal 4: no needs for explicit SFO requirements.

- Observation 1: harmonic performance will determine final SEM requirements of device.

Proposal 5: 12dBc/1SB flat requirement is suggested for SEM. And can only test minimum and max CBW to reduce the test burden.

Observation 2: the OOB is defined  as Fb as in ITU, i.e. max of 500kHz and 10 times NB where NB equals to D2R CBW.

Proposal 6: only test minimum and max CBW to reduce the test burden.

Proposal 7: use the -10dBm maximum input power requirements(i.e. 1m distant CW assumption) as the maximum CW input power level for spurious emission testing.

- Proposal 8: REFSENSE is suggested as -34dBm

Proposal 9: define min sensitivity at peak antenna gain direction and EIS spherical coverage requirements, XdBm at Y degree. Y is suggested to be +-60degree, X is suggested to be 3dB worse than REFSENSE.

Proposal 10: -4dBm max input power with 30dB dynamic range assumption.

# 4. Reference

- RP-243326, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, RAN1 Vice-chair (Huawei), 3GPP TSG RAN Meeting #106, Dec. 2024.

- R4-2420470, WF on RF requirements for A-IOT, Moderator (Huawei).