# 3GPP TSG-RAN WG4 Meeting #116	R4-2509713			R4-250xxxx

# Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.22.2

Source: 	CMCC

Title: 	Discussion on A-IoT System parameters

Document for:	Discussion

# 1. Introduction

In RAN plenary #106 meeting, a new WI [1] of A-IoT is approved with following RAN4 lead RF part objectives:

- RAN4 scope:

  - Specify RF requirements for Ambient-IoT BS, device 1, and CW

    - RF requirements for Type 1-C Ambient-IoT BS

    - RF requirements for device 1

    - RF requirements for CW

  - Specify RRM core requirements for device 1, if necessary

  - Study and develop OTA test methodology for A-IoT device 1

    - Consider test methods specified in TR 38.870 as starting point. Take test system reuse, test system complexity and test time into account, when developing test methods suitable for Ambient IoT.

    - Develop the preliminary Measurement Uncertainty (MU) assessment for the test system

  - Use band n8 as an example band

In this contribution, we discuss the system parameters for A-IOT.

- Discussion

Last meeting agreements for A-IoT system parameters have been listed as below:

Issue 2-2-1: D2R bandwidth Agreement in RAN4#115: - Define D2R CBW based on all Tb, Tc, R combinations as finally approved in RAN1. - Assumed 10% SFO for device - For Reader, [90%] filter spectrum utility ([10%] guard band) is considered. - The value of transmission BW and channel BW will be explicitly listed. - Using following equation for BS as starting point: D2R CBW for BS (KHz)=(2SB Transmission BW*(1/2)+2* Small frequency shift)/0.9 =(2000*(1+R)/Tb)* (1+∣SFO∣)/0.9 =(1000*(R+1)/(Tc*R)) * (1+∣SFO∣)/0.9 - Define [200kHz]，[3.54MHz] D2R CBW for AIoT BS. Other D2R CBWs for BS are FFS - Using following equation for device as starting point: D2R CBW for device (KHz)=2SB Transmission BW*(1/2)+2* Small frequency shift =(2000*(1+R)/Tb)* (1+∣SFO∣) =(1000*(R+1)/(Tc*R)) * (1+∣SFO∣) - Topic 2-3: Channel raster Issue 2-3-1: R2D channel raster Agreement in RAN4#115: - Define 10KHz channel raster - FFS on channel raster offset

- D2R channel bandwidth

In NR UE spec, there are three kind of bandwidth related definition, the channel bandwidth, the maximum transmission bandwidth configuration, the transmission bandwidth. The former two are the definition in RAN4. for A-IoT D2R channel bandwidth, it should be noted there is no maximum transmission bandwidth configuration definition. The same logic apply for R2D.

Proposal 1: there is no maximum transmission bandwidth configuration defintion of D2R and R2D in RAN4.

- Last meeting only list two candidate D2R CBW values for AIoT BS. To reduce testing burden, it’s OK to only use the minimum and maximum CBW value for testing. But for core requirements, all values should be defined in the spec. Besides, it’s suggested to use table based methdology for final CBW definition. The detailed value for D2R CBW for AIoT BS and device are listed as below.

|  | DSB /kHz | D2R CBW at BS side |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tb (μs) |  | Tchip (μs) |  |  |  |  |  |  |  |  |
|  |  | 133.33 | 66.67 | 33.33 | 16.67 | 8.33 | 4.17 | 2.08 | 1.04 | 0.69 |
| 266.67 | 15 | 18 | 27 | 46 | 82 | 156 | 302 | 597 | 1184 |  |
| 133.33 | 30 |  | 37 | 55 | 92 | 165 | 311 | 606 | 1194 |  |
| 66.67 | 60 |  |  | 73 | 110 | 183 | 330 | 624 | 1212 |  |
| 33.33 | 120 |  |  |  | 147 | 220 | 366 | 661 | 1249 |  |
| 16.67 | 240 |  |  |  |  | 293 | 440 | 734 | 1322 |  |
| 8.33 | 480 |  |  |  |  |  | 587 | 881 | 1469 |  |
| 4.17 | 960 |  |  |  |  |  |  | 1174 | 1761 |  |
| 1.39 | 2880 |  |  |  |  |  |  |  |  | 3530 |

|  | DSB /kHz | D2R CBW at device side |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tb (μs) |  | Tchip (μs) |  |  |  |  |  |  |  |  |
|  |  | 133.33 | 66.67 | 33.33 | 16.67 | 8.33 | 4.17 | 2.08 | 1.04 | 0.69 |
| 266.67 | 15 | 17 | 25 | 41 | 74 | 140 | 272 | 537 | 1066 |  |
| 133.33 | 30 |  | 33 | 50 | 82 | 149 | 280 | 545 | 1074 |  |
| 66.67 | 60 |  |  | 66 | 99 | 165 | 297 | 562 | 1091 |  |
| 33.33 | 120 |  |  |  | 132 | 198 | 330 | 595 | 1124 |  |
| 16.67 | 240 |  |  |  |  | 264 | 396 | 661 | 1190 |  |
| 8.33 | 480 |  |  |  |  |  | 528 | 793 | 1322 |  |
| 4.17 | 960 |  |  |  |  |  |  | 1056 | 1585 |  |
| 1.39 | 2880 |  |  |  |  |  |  |  |  | 3177 |

- Proposal 2: it’s suggested to define all D2R CBW values for AIoT BS and device as above. For BS, only using minimum 200kHz and maximum 3530MHz for testing.

- Channel raster

From RAN1 point of view, carrier frequency is at the center of centric RE. This means the FIR shaping filter is not symetric. For wide CBW such asymetric guard band will not impact FIR shaping filter design but for narrow CBW, e.g. 180kHz, such asymetric guard band will decrease filter performance. One solution is to shift the carrier frequency before IFFT to get symetric shaping filter and re-shift the carrier frequency using RF LO. For such operation, we don’t need to shift the channel raster. Therefore, no shift of channel raster.

For channel raster, to maximize filter performance, symetric guard band is required especially for AIoT smaller CBW, e.g. 3.75kHz. For such case the carrier frequency which is located on channel raster should be at the center of transmission bandwidth which require -7.5kHz or +2.5kHz shift of carrier frequency but such shift should be achieved before IFFT, instead of shift channel raster. Therefore, no shift for channel raster.

Proposal 3: no offset are required for channel raster to support symetric guard band.

# 3. Conclusions

In this contribution, system parameters are discussed with following proposals.

Proposal 1: there is no maximum transmission bandwidth configuration defintion of D2R in RAN4.

- Proposal 2: it’s suggested to define all D2R CBW values for AIoT BS and device as above. For BS, only using minimum 200kHz and maximum 3530MHz for testing.

Proposal 3: no offset are required for channel raster to support symetric guard band.

# 4. Reference

- RP-243326, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, RAN1 Vice-chair (Huawei), 3GPP TSG RAN Meeting #106, Dec. 2024.

- R4-2420470, WF on RF requirements for A-IOT, Moderator (Huawei).

- R4-2417088, WF on RF requirement impact of A-IoT, Huawei.

- R4-2414305, WF on RF requirements for A-IoT, Huawei

- EPC™ Radio-Frequency Identity Protocols Generation-2 UHF RFID, Specification for RFID Air Interface Protocol for Communications at 860 MHz - 960 MHz, Version 2.0.0 Ratified