3GPP TSG RAN WG4 Meeting #116	R4-2509724

Bengaluru, India 25-29 August, 2025

# Source:	MediaTek Inc.

# Title:	Discussion on the detail scope of LBCA via switching in Rel-20

# Agenda Item: 	11

# Document for:	Information

# Introduction

In last Prague RAN #108 meeting, the WF on RAN4 R20 package was endorsed [1]. However, RAN did not decide detail work scope for Cross RF and baseband topics in Rel-20. RAN only decided a few broad objectives such as extension of LBCA, UAV and either topic between DL fragmented carrier and intra-band non-collocated CA/DC in 5G-A as shown in below agreement in [1]. In this paper, we share our preferred detail objectives on the low band DL CA via switching topics in Rel-20 timeline.

## Agreements:

The following two WI are agreed to be introduced in R20.

- Extension of LBCA (the exact scope and WID draft will be discussed and decided in RAN#109):

  - Further discussion on the possible scoping in RAN#109 based on the following two objectives

    - Study and if feasible, specify non-RRC based switching solution, e.g. dynamic switching, by taking the RAN1 impacts into consideration

    - Dual DL solution

    - RAN1/2 impact is expected and will be further discussed in RAN#109

  - Target 0.5 TU (0.25 RF and 0.25 RRM) in Q4’25 and 0.75 TU allocation (0.25 RF and 0.5 RRM) from Q1’26

- UAV (the exact scope and WID draft will be discussed and decided in RAN#109):

  - UAV UE with antenna array with up to 33dBm maximum output power on TDD spectrum

  - RAN1/2 scope is not expected

  - Target 0.5 TU allocation (0.25 RF and 0.25 RRM)

The following two proposals will be discussed at RAN#109, where it will be decided whether or not to include either of the proposals

  - Fragmented DL:

    - Introducing RF/RRM requirements related to new UE behavior for reducing the number of UE Rx RF chains

    - RAN1/2 scope is TBC

  - Intra-band non-collocated EN-DC/NR-CA deployment

    - Type 3 UE requirements assuming MRTD<CP and power imbalance < [xx] dB for handheld

    - RAN1/2 scope is TBC

# Discussions on low band DL CA via switching in Rel-20

In Rel-20 for 5G-A feature discussion, RAN4 already had many discussions time to concrete detail work objectives for Dual DL solution as shown in Figure 1. However, RAN4 did not make consensus on the dynamic switching mechanism in Rel-20.

Figure 1. RAN4 Switching mechanism between FDD band and Dual DL operation (FDD DL + SDL DL)

To make consensus on the dynamic switching mechanism, RAN4 need to agree some restrictions points for dynamic switching operation (non-semi static RRC based switching) to reduce RAN4 work scope and minimize other WGs workload for RAN4 leading topics in Rel-20.

- Consideration points to reduce Workload in RAN4 and other WGs

- Consider semi-static switching mechanism or only allow Dynamic switching without DCI change/update between FDD band and SDL in Rel-20. LBCA via Switching between FDD and FDD is not supported in Rel-20.

- Consider wide Rx filter to receive Dual DL (both FDD DL and SDL) operation at the same time as shown in Figure 2.

- Only self-carrier scheduling shall be assumed to reduce workload. For dual DL with dynamic switching, cross-carrier scheduling is not necessary and hence shall not be considered.

- The same time mask as for the Rel-19 semi-static pattern can be reused.

- For CA_n5A-n29A and CA_n29A-n71A in Figure 3, RAN4 only reuse Rel-19 low-band DL CA via switching mechanism, as the wider frequency band operation for dual DL carrier reception may degrade the antenna performance due to the wider frequency operation.

Figure 2. Wide channel filter to cover both FDD DL and SDL DL

Figure 3. low-band DL CA combinations with wider channel operation (> 100MHz frequency span) for Dual DL

- Other RAN WGs impacts for LBCA via switching with dynamic switching mechanism in Rel-20

- To support dynamic switching pattern in Rel-20, RAN1 may need to define symbol-based switching pattern instead of slot-based switching pattern in TS38.214  Need to clear understanding for the other WGs impact to discuss and define the new switching pattern for dynamic switching in RAN1.

- Collison handling issues between scheduled UL on FDD UL carrier and configured DL (e.g. SPS, CSI-RS, etc) on SDL carrier  Need to discuss and specify the collision handling mechanism in TS38.213.

- For FDD+FDD band CA via switching (i.e.., n29+n106 and other FDD combos), RAN4 need to support cross carrier scheduling if RAN4 agree to support cross carrier scheduling for FDD+FDD band CA via switching. It will be impact to RAN1/RAN2 workload.

- Evaluate whether to satisfy the legacy UE Preparation Procedure time in TS38.214 is enough for dynamic switching mechanism or extend the UE processing time considering with some timing related parameters.

Based on the above observation and analysis, we propose the detail objectives Low-band DL CA via switching features as follow:

Proposal 1: For Release 20 LBCA via switching, RAN4 can consider semi-static switching mechanism with Dual DL or RAN1 should be involved in early stage how much RAN1 impacts will be estimated to support Dynamic switching mechanism without DCI change/update between FDD band and Dual DL. Based on the RAN1 conclusion, RAN will decide whether to support dynamic switching mechanism or not.

Proposal 2: To reduce workload and minimize other WGs impacts, RAN4 do not consider the cross-carrier scheduling between each carrier/band (only self-carrier scheduling is assumed) in Rel-20.

- Work Item Objectives for Low-band DL CA via switching

- Specify UE RF requirements for semi-static switching mechanism or Study phase for dynamic switching mechanism without DCI updates/changes, including at least switching gap, and corresponding physical layer procedures to switch between {Case 1, Case 3} [RAN4, RAN1]

  - Case 1: 1 Tx/2 Rx on FDD carrier 1 and 0 Tx/0 Rx on SDL carrier 2

  - Case 3: 0 Tx/2 Rx on FDD carrier 1 and 0 Tx/2 Rx on SDL carrier 2

    - Example Bands for LBDL CA: CA_n12A-29A, CA_n14A-n29A and CA_n28A-n67A.

* Note: RAN4 do not consider cross-carrier scheduling in Rel-20 to minimize other WGs impacts.

- Investigate on the UE preparation procedure time when RAN4 allow to support dynamic switching mechanism. [RAN1, RAN4]

# Conclusion

In this RAN4 #116 meeting, we provided our preferred objectives on low-band DL CA via switching scope in Rel-20 based on the endorsed WF [1] at last RAN #108 Meeting.

To minimize other WGs and reduce scope in RAN4, we propose detail work objectives in 5G-A as follow:

Proposal 1: For Release 20 LBCA via switching, RAN4 can consider semi-static switching mechanism with Dual DL or RAN1 should be involved in early stage how much RAN1 impacts will be estimated to support Dynamic switching mechanism without DCI change/update between FDD band and Dual DL. Based on the RAN1 conclusion, RAN will decide whether to support dynamic switching mechanism or not.

Proposal 2: To reduce workload and minimize other WGs impacts, RAN4 do not consider the cross-carrier scheduling between each carrier/band (only self-carrier scheduling is assumed) in Rel-20.

- Work Item Objectives for Low-band DL CA via switching

- Specify UE RF requirements for semi-static switching mechanism or Study phase for dynamic switching mechanism without DCI updates/changes, including at least switching gap, and corresponding physical layer procedures to switch between {Case 1, Case 3} [RAN4, RAN1]

  - Case 1: 1 Tx/2 Rx on FDD carrier 1 and 0 Tx/0 Rx on SDL carrier 2

  - Case 3: 0 Tx/2 Rx on FDD carrier 1 and 0 Tx/2 Rx on SDL carrier 2

    - Example Bands for LBDL CA: CA_n12A-29A, CA_n14A-n29A and CA_n28A-n67A.

* Note: RAN4 do not consider cross-carrier scheduling in Rel-20 to minimize other WGs impacts.

- Investigate on the UE preparation procedure time when RAN4 allow to support dynamic switching mechanism. [RAN1, RAN4]

# References

- RP-251879, “Way Forward on RAN4 R20 packages” RAN4 Chair (Apple), 3GPP TSG RAN Meeting #108, Prague, June 09-13, 2025.

- RP-251668, “Motivation for for low-band carrier aggregation via switching enhancement in Rel-20,” TELUS, Bell Mobility, Apple and others, 3GPP TSG RAN Meeting #108, Prague, June 09-13, 2025.

- RP-251337, “Enhancement of the low NR band aggregation via switching framwork”, Apple, 3GPP TSG RAN Meeting #108, Prague, June 09-13, 2025