3GPP TSG-RAN4 Meeting #116	R4-2509726

# Bengaluru, India, 25th – 29th August, 2025

Agenda item:			7.1.2.1.1

Source:	MediaTek Inc.

Title:	MSD simplification rules for 2UL for HPUE CA/DC UE

Document for:	Approval

# Introduction

In RAN4#115 meeting, the WF [1] has been approved and recommended to further discuss and encourage more companies’ input for each open issues i.e., MSD simplification and others for HP CA/DC UE.

Way forward [1]: - For new Rel-19 inter-band CA combinations, and for all valid 2UL band 2UL CC HPUE configurations, specify the 2 band 2 UL CC HPUE MSD requirements due to dual-UL CA intermodulation interference as: PCx MSD = PC31Tx MSD + MSD where, - PCx = PC2, or PC1.5, - PC31Tx MSD is the reference sensitivity exception specified for PC3 in Table 7.3A.4-1, or in Table 7.3A.4-4 or in Table 7.3A.6-1, and, - MSD is specified in seven LUT columns with a maximum MSD (“MSDmax”) of 6, 9, 12, 15, 18, 24, 30. One example of such LUT is captured below based on [5]. Coarser granularity and other tabulated values are not precluded. - and where the mapping of the dual-UL IMD order to the “MSDmax” LUT is specified with the following companion table for PC2 and PC1.5. This agreement applies to both 2UL/2DL inter-band CA and for 2UL/3DL inter-band CA for both HPUE achieved with 2Tx or 3Tx. Other table formats are not precluded for CR implementation. inter-band CA 2UL band 2CC power-class; MSDmax (dB). 6; 9; 12; 15; 18; 24; 30. PC2; IMD2; IMD3; IMD4; IMD5. PC1.5; IMD2; IMD3; IMD4; IMD5.

In this paper, we share our view on the MSD simplification methodology according to 2UL IMD interference sources in Rel-19.

# MSD simplification

## 2UL HPUE MSD simplification

For 2UL MSD analysis, the IMD products’ power is mainly influenced by factors such as changes in transmit power, variations in the power class of each PA, the number of TX paths, and the IMD index of the frequency bands involved in intermodulation distortion. Also, HP PA improvement in linearity can bring about by the increase will be leaded to a reduction in product power.

In Agreed WF on MSD simplification rules in RAN4 #113 meeting, RAN4 derived detail MSD calculation principle for 10 cases for total CA UE power class in Table 1 according to the number of Tx Chain, PA combinations and expected interference levels in own Rx band.

Table 1. HPUE MSD analysed table from Rel-15 to Rel-19

| case |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ULCA power class | Type | PC3 2Tx | PC2 2Tx | PC2 2Tx | PC2 3Tx | PC2 2Tx | PC2 3Tx | PC1.5 2Tx | PC1.5 3Tx | PC1.5 3Tx | PC1.5 3Tx |
|  | PCmax | 23 | 26 | 26 | 26 | 26 | 26 | 29 | 29 | 29 | 29 |
| per band power class | Bd1 | PC3 1Tx | PC3 1Tx | PC3 1Tx | PC3 1Tx | PC2 1TX | PC2 1TX | PC2 1TX | PC2 1TX | PC3 1Tx | PC2 1TX |
|  | Bd2 | PC3 1Tx | PC3 1Tx | PC2 1Tx | PC2 2Tx | PC2 1Tx | PC2 2Tx | PC2 1Tx | PC2 2Tx | PC1.x 2Tx | PC1.5 2Tx |
| Valid Release |  | R15 | R17 | R17 | R18 | R19 | R19 | R19 | R19 | R18 | R19 |
| 1UL power | Bd1 | 23 | 23 | 23 | 23 | 26 | 26 | 26 | 26 | 23 | 26 |
|  | Bd2 | 23 | 23 | 26 | 2x23 | 26 | 2x23 | 26 | 2x23 | 2x26 | 2x26 |
| 2UL power | Bd1 | 20 | 23 | 23 | 23 | 23 | 23 | 26 | 26 | 23 | 26 |
|  | Bd2 | 20 | 23 | 23 | 2x20 | 23 | 2x20 | 26 | 2x23 | 2x24.7 | 2x23 |
| 2UL interference level |  | Ref | +3xorder | <+3xorder | <+3xorder | +3dB | +3dB | +3dB +3xorder | +3dB +3xorder | <4.7xorder -1.7 | ≤+3dB +3xorder |
| Unified 2UL interference level |  | Ref | 3xorder |  |  | 3dB |  | +6xorder -3 |  | <4.7xorder -1.7 | +6xorder-3 |

RAN4 can consider the IMD product level will be increased as proportional by the IMD order and we can ignore the asymmetry IMD of UL band to derive the MSD analysis.

To consistency of 2UL MSD simplification rules, we prefer to define LUT per each power class according to the IMD order as follow

The original MSD proposed tables from [1] in agreed WF. It can be updated Table format as below to consistent with 1UL MSD table.

Table 2: 2UL Delta MSD Table for Power class 2 according to IMD order

| PC3 MSD (dB) | MSDmax / MSD(dB) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 6 | 9 | 12 | 15 | 18 | 24 | 30 |
| 0.1≤ PC3 MSD <0.5 | 1.2 | 2.4 | 3.9 | 5.8 | 7.9 | 13.2 | 18.8 |
| 0.5≤ PC3 MSD <1.0 | 1.7 | 3.5 | 5.4 | 7.5 | 10.0 | 15.6 | 21.4 |
| 1.0≤ PC3 MSD <2.0 | 2.8 | 4.8 | 7.1 | 9.6 | 12.4 | 18.0 | 24.1 |
| 2.0≤ PC3 MSD <3.0 | 3.5 | 5.7 | 8.2 | 10.9 | 13.7 | 19.5 | 25.5 |
| 3.0≤ PC3 MSD <4.0 | 3.9 | 6.3 | 8.9 | 11.7 | 14.5 | 20.5 | 26.5 |
| 4.0≤ PC3 MSD <5.0 | 4.3 | 6.8 | 9.4 | 12.4 | 15.2 | 21.2 | 27.2 |
| 5.0≤ PC3 MSD <6.0 | 4.5 | 7.1 | 9.9 | 12.8 | 15.8 | 21.7 | 27.7 |
| 6.0≤ PC3 MSD <7.0 | 4.8 | 7.6 | 10.4 | 13.3 | 16.2 | 22.3 | 28.2 |
| 7.0≤ PC3 MSD <8.0 | 5.0 | 7.8 | 10.7 | 13.6 | 16.6 | 22.5 | 28.5 |
| 8.0≤ PC3 MSD <9.0 | 5.2 | 8.1 | 11.0 | 14.0 | 16.9 | 22.8 | 28.8 |
| PC3 MSD ≥9.0 | 6.0 | 9.0 | 12.0 | 15.0 | 18.0 | 24.0 | 30.0 |

- PC2IMDx MSD = PC3IMDx MSD + PC2IMDx,

- PC1.5IMDx MSD = PC3IMDx MSD + PC1.5IMDx.

Proposal 1: For the 2UL MSD simplification rules of HPUE inter-band CA/DC UE, RAN4 can apply the above LUT based MSD simplification approach due to dual uplink IMD problems.

When the cases of PC3 MSD is not specified in TS38.101-1 or TS38.101-3, but MSD for PC2 CA/DC UE are specified in the specifications, then PC2 MSD shall be referred to derive MSD requirements for PC1.5 with 2Tx.

In the case, PC1.5 with 2Tx MSD will be calculated as following equation

- PC1.5 2Tx MSD = PC2_1TxMSD + MSDmax of PC2 (6,9,12,15)

Table 3. MSDmax for HPUE according to IMD order

| inter-band CA 2UL band 2CC power-class | MSDmax (dB) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 6 | 9 | 12 | 15 | 18 | 24 | 30 |
| PC2 | IMD2 | IMD3 | IMD4 | IMD5 |  |  |  |
| PC1.5 |  |  | IMD2 |  | IMD3 | IMD4 | IMD5 |

Proposal 2: For the PC3 MSD is not specified, but MSD for PC2 CA/DC UE are specified in the specifications, then PC2 MSD shall be referred to derive MSD requirements for PC1.5 with 2Tx.

- Conclusion

In this paper, we share our view on the MSD simplification methodology according to 2UL IMD interference sources in Rel-19. Based on these evaluate, we can propose as fo

Proposal 1: For the 2UL MSD simplification rules of HPUE inter-band CA/DC UE, RAN4 can apply the above LUT based MSD simplification approach due to dual uplink IMD problems.

Proposal 2: For the PC3 MSD is not specified, but MSD for PC2 CA/DC UE are specified in the specifications, then PC2 MSD shall be referred to derive MSD requirements for PC1.5 with 2Tx.

- PC1.5 2Tx MSD = PC2_1TxMSD + MSDmax of PC2 (6,9,12,15)

## References

- R4-2507935, WF on new MSD rules, Skyworks, Murata, Nokia, Samsung, Qualcomm, Apple, vivo, ZTE, MediaTek, RAN4 #115

- R4-2501955, 1UL MSD simplifications rules for Rel-19, Skyworks, Nokia RAN4#113

- R4-2506980, Discussion on MSD simplification of HPUE CA/DC UE, MediaTek, RAN4 #115

- R4-25xxxxx, RAN4 #115 Meeting report, RAN4 Chairman, RAN4 #116

- R4-2500218, Simplification of HPUE 2UL DeltaMSD, Skyworks, RAN4#114