3GPP TSG-RAN WG4 Meeting # 116	R4- 2509738

# Bengaluru, India, 25th – 29th August, 2025

Source:	LG Electronics

Title:	Discussion on simulation results for LP-WUS/WUR

Agenda item:	7.24.5.1

Document for:	Discussion

Introduction

In this contribution, we provide our updated simulation results for NR LP-WUS/WUR based on listed simulation assumptions in approved WF [1].

Discussion

- Topic #2: Simulation assumptions and results

Sub-topic 2-1 On simulation assumptions and parameters

Simulation assumptions

The simulation parameters that have been agreed upon so far are as below:

Note: Parameters highlighted by green color are agreed during the meeting. Other parameters are only for recommendation and information purpose. Table 1: General parameters Simulation parameters; Comments/values. Carrier frequency for Cell 1 and Cell 2; Agreement: 2.6 GHz initially. Prior knowledge of Cell 1 / Cell 2 by the UE; Interfering cell (Cell 2) is not known to UE. DRX; No applicable for LP-WUR. BS transmit antennas for LP-SS blocks; 1 Tx. UE receive antennas; 1 Rx. Data and control channel subcarrier spacing; [Data, SSB and LP-SS have the same SCS]. Subcarrier spacing; 30KHz. Measurement period (in number of measurement samples); [4, 5, other number could be studied upon a need]. LP-SS/SSB measurement interval; LP-SS: 320 ms SSB: 320 ms. LP-SS BW; 132 subcarriers for SCS=30kHz for LP-SS initially TBD for 15KHz SCS. SSS; 30KHz for SSS, TBD for 15KHz. Actual LP-SS transmissions; always transmitted. Guard band; 1 RB on each side of LP-SS/LP-WUS signal. Table 2: Cell-specific parameters Parameter; Unit; Cell 1; Cell 2. RF Channel number; -; Channel 1; Channel 1. NR-PSS, NR-SSS (OFDM based LP-WUR); -; To be indicated by companies; To be indicated by companies. LP-SS; Refer to conclusion of the Issue 2-1-4 in [R4-2502600] [3]; Refer to conclusion of the Issue 2-1-4 in [R4-2502600] [3]. PBCH and DMRS power offset with respect to NR-PSS, NR-SSS and LP-SS; dB; 0; 0. Data and control PSD relative to NR-PSS,NR-SSS and LP-SS; dB; 0; 0. RB Utilization; %; 100; 100. Data Modulation; -; QPSK; QPSK. Slot length; -; 14 symbols; 14 symbols. CP Length; -; Normal; Normal. Frequency offset relative to UE frequency reference; Hz; OFDM based receiver [5] ppm OOK based receiver [ 5 10] ppm; N/A. Timing error; Residual timing error + timing drift (frequency offset* 320ms (reference signal periodicity) Residual timing error: - T= 5us for OOK-1 - T= 2us for OOK-4 with M=2 - T= 1us for OOK-4 with M=4. 1)Relative Delay of 1st Path (synchronous); µs; 0; CP/2. 2) Relative Delay of 1st Path (asynchronous): Fixed delay; ms; 0; 3 ms. SNR; dB; SNR setting for serving and interference cell are derived based on agreement of Issue 2-1-1-1. When Ês/Iot = -3 dB - When SNR of cell 2 is 9 dB lower compared with cell 2; SNR = [-2.7] - When SNR of cell 2 is 6 dB lower compared with cell 2 SNR = [-2.4]; When Ês/Iot = -3 dB - When SNR of cell 2 is 9 dB lower compared with cell 2; SNR = [-11.7] - When SNR of cell 2 is 6 dB lower compared with cell 2; SNR = [-8.4]. When Ês/Iot = -0.5 dB Note: Determine the SNR based on based on agreement of Issue 2-1-1-1; When Ês/Iot = -0.5 dB Note: Determine the SNR based on based on agreement of Issue 2-1-1-1. When Ês/Iot = 2 dB Note: Determine the SNR based on based on agreement of Issue 2-1-1-1; When Ês/Iot = 2 dB Note: Determine the SNR based on based on agreement of Issue 2-1-1-1. When Ês/Iot = -6 dB; When Ês/Iot = -6 dB. Ês/Iot; dB; -3; -0.5dB; 2dB; N/A. Propagation conditions; -; FR1: AWGN TDL-C 300ns. UE speed; 3 km/h. Table 3: UE-specific parameters [Receiver Filter]; [3th/5th Order Butterworth with 3.96MHz bandwidth]. [Receiver ADC bit width]; [4 or 8-bitADC]. [Receiver Sampling Rate for LP-SS only]; [3.84 or 7.68MHz].

Based on the additional simulation assumption agreements, we tried to get additioanl simulation results. In this document, we share our simulation model and simulation results based on additional simulation assumptions.

Simulation model

Basic elements of the system model

- Waveform generation (OOK-1, OOK-2, OOK-4)

- Digital-to-analog (D/A) converter

- TX/RX LPF: Butterworth filter

- RF modulation/demodulation signal

- RSRP measurement: RSRPEstimated estimated based on the signal energy received during the ON time corrected for the energy received during the LP-SS OFF time, i.e. ∆E=EON-EOFF

  - is the mean value of the received power in the linear scale under the assumption of no noise and perfect channel knowledge

  - measured mean value of the received signal power in linear scale

RSRQ measurement accuracy=10log10RSRQEstimatedRSRQIdeal [dB]

Simulation assumption

Baseline assumption

| Parameters | Assumption |
| --- | --- |
| Carrier Frequency | 2.6GHz |
| LP-SS BW | 5 MHz (132 subcarriers) |
| Waveforms | OOK-1, OOK-2, OOK-4 |
| SCS | 30 kHz |
| Measurement period (in number of measurement samples) | 1, 2, 3, 4, 6, 8 |
| Code sequence length L | 4-32 bits |
| LP-SS measurement interval | 320 ms |
| Filter | Butterworth (5 Order) |
| ADC | 4 bits |
| Channel propagation | AWGN, TDLC300 |
| Mobility | 3 km/h |
| Antenna configuration | 1x1 |

Cell parameters

- We considered NR types of interference (same bandwidth with the useful signal)

Sub-topic 2-2 Simulation results alignment and summary

Issue 2-2-2: Summary on proposal/observations based on simulation results

Working assumption 1: The number of LP-SS samples for achieving ±3.5 dB accuracy for LP-SS RSRP at -3 dB SINR is [from 1 to 3]; When LP-SS samples number is larger than 3 at -3 dB SINR, the extra gain on accuracy for LP-SS RSRP is smaller; Working assumption 2: The number of LP-SS samples for achieving ±3.5 dB accuracy for LP-SS RSRQ for AWGN and TDL-C channel at -3 dB SINR is [2 or 3];

Simulation results summary

RSRP

| SINR | Channel | {M, L} | Required number of samples (Target accuracy 2.5dB) | Required number of samples (Target accuracy 3.5dB) |
| --- | --- | --- | --- | --- |
| -3dB | AWGN | {1, 4} | 1 | 1 |
|  |  | {1, 6} | 1 | 1 |
|  |  | {1, 8} | 1 | 1 |
|  |  | {2, 8} | 1 | 1 |
|  |  | {2, 12} | 1 | 1 |
|  |  | {2, 16} | 1 | 1 |
|  |  | {4, 16} | 1 | 1 |
|  |  | {4, 32} | 1 | 1 |
|  | TDL-C | {1, 4} | 8 (When remove bias 5) | 4 (When remove bias 3) |
|  |  | {1, 6} | 8 (When remove bias 5) | 4 (When remove bias 3) |
|  |  | {1, 8} | 8 (When remove bias 5) | 4 (When remove bias 3) |
|  |  | {2, 8} | 8 (When remove bias 6) | 4 (When remove bias 3) |
|  |  | {2, 12} | 8 (When remove bias 5) | 4 (When remove bias 3) |
|  |  | {2, 16} | 8 (When remove bias 5) | 4 (When remove bias 3) |
|  |  | {4, 16} | 8 (When remove bias 5) | 5 (When remove bias 3) |
|  |  | {4, 32} | 8 (When remove bias 5) | 5 (When remove bias 3) |

RSRQ

| SINR | Channel | {M, L} | Required number of samples (Target accuracy 2.5dB) | Required number of samples (Target accuracy 3.5dB) |
| --- | --- | --- | --- | --- |
| -3dB | AWGN | {1, 4} | 1 | 1 |
|  |  | {1, 6} | 1 | 1 |
|  |  | {1, 8} | 1 | 1 |
|  |  | {2, 8} | 1 | 1 |
|  |  | {2, 12} | 1 | 1 |
|  |  | {2, 16} | 1 | 1 |
|  |  | {4, 16} | 1 | 1 |
|  |  | {4, 32} | 1 | 1 |
|  | TDL-C | {1, 4} | 8 (When remove bias 5) | 4 (When remove bias 3) |
|  |  | {1, 6} | 8 (When remove bias 5) | 4 (When remove bias 3) |
|  |  | {1, 8} | 8 (When remove bias 5) | 4 (When remove bias 3) |
|  |  | {2, 8} | 8 (When remove bias 5) | 5 (When remove bias 3) |
|  |  | {2, 12} | 8 (When remove bias 5) | 4 (When remove bias 3) |
|  |  | {2, 16} | 8 (When remove bias 5) | 4 (When remove bias 3) |
|  |  | {4, 16} | 8 (When remove bias 5) | 5 (When remove bias 3) |
|  |  | {4, 32} | 8 (When remove bias 5) | 5 (When remove bias 3) |

Observation 1: In AWGN channel propagation, 1 sample is enough to meet the accuracy requirements at any scenarios. However in TDL-C channel propagation scenario, it is hard to meet the target accuracy 3.5 dB in 3 samples. If the bias is removed, then can meet the target accuracy 3.5 dB in 3 samples

Conclusion

In this contribution, we provide our views on the simulation results

Observation 1: In AWGN channel propagation, 1 sample is enough to meet the accuracy requirements at any scenarios. However in TDL-C channel propagation scenario, it is hard to meet the target accuracy 3.5 dB in 3 samples. If the bias is removed, then can meet the target accuracy 3.5 dB in 3 samples

Reference

[1] R4-2508280, “WF on RRM requirements for NR_LPWUS”, Moderator (vivo)