3GPP TSG-RAN WG4 Meeting # 116	R4- 2509750

# Bengaluru, India, 25th – 29th August, 2025

Source:	LG Electronics

Title:	Discussion of RAN4 impacts for RRM measurement prediction

Agenda item:	7.19.3.1

Document for:	Discussion

Introduction

In this contribution, we provide our views on issues for AI/ML mobility as below based on last RAN4 meeting’s moderator summary [2] and WF [1].

- Issue 2-1: Relative RSRP accuracy definition

- Issue 2-2: Accuracy requirements for the predicted RSRPs

- Issue 2-3: Ground truth definition for RSRP Accuracy for FR1

- Issue 2-4: Impacts of measurement error on prediction accuracy

Discussion

- Topic #2: RAN4 requirements for RRM Measuement prediction

### Issue 2-1: Relative RSRP accuracy definition

During last RAN4#115 meeting the relative RSRP prediction accuracy issue was discussed and made an agreement as below.

Agreement: - For intra-frequency L3-RSRP, - Relative accuracy of predicted L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), - cell 1 and cell 2 are on the same frequency - the reported RSRP of cell2 can be measured or predicted. - FFS on whether and how to consider time difference between the two reported RSRPs when applying the definition of the relative RSRP accuracy - Relative RSRP accuracy for Beam level measurements is FFS during WI phase depending upon RAN2 progress. - For inter-frequency L3-RSRP, - Relative accuracy of predicted L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), - cell 2 is on a different frequency than cell 1 but in the same FR as cell 1 - the reported RSRP of cell2 can be measured or predicted. - FFS on whether and how to consider time difference between the two reported RSRPs when applying the definition of the relative RSRP accuracy

For AI/ML mobility test, the time-varying channel model can be default test environment. The AI/ML model should work well in all real channel environments, including both static and time-varying channels. If we consider only static channel, the prediction rule can be very simple, and the prediction value can be obtained using a basic averaging method-like AI/ML model. However, when considering the time-varying channels, the channel changes over time. As a result, the ground truth of RSRP also can vary with time. Therefore, to define the RRM requirement of relative accuracy of predicted L3-RSRP, RAN4 need to restrict the time difference between the two reported RSRPs. Additionally, an extra accuracy margin caused by the measurement time difference also necessary. If the reporting times between the two reported RSRPs are guaranteed to be aligned,then such an extra accuracy margin is not necessary.

For intra-frequency scenario, the reporting times of the two RSRP measurements can be aligned, so no additional margin is needed. However, for inter-frequency scenario, RAN4 need to restrict the time difference between the two reported RSRPs and an extra accuracy margin caused by the measurement time difference also necessary.

Proposal 1: For intra-frequency scenario, the reporting times of the two RSRP measurements can be aligned, so no additional margin is needed. However, for inter-frequency scenario, RAN4 need to restrict the time difference between the two reported RSRPs and an extra accuracy margin caused by the measurement time difference also necessary.

### Issue 2-2: Accuracy requirements for the predicted RSRPs

- Proposals and Observations: - Proposal 1 (Ericsson): Predicted measurements should at least meet the accuracy requirements that are defined for the non-AI/ML based RRM measurement reporting for mobility purposes. - Proposal 2 (Nokia): For temporal domain Case-B prediction, RAN4 may consider accuracy performance requirements similar to those of the legacy relative and absolute accuracy performance defined for measurement. - Proposal 3 (ZTE): RAN4 should discuss whether to define separate prediction accuracy for each sub-use case or define one prediction accuracy requirement to cover 3 cell level sub-use cases.

The accuracy requirements for the predicted RSRPs should be at least meet the accuracy requirements of non-AI/ML based RRM measurement reporting for mobility purpose. Because the relaxed accuracy requirements can impact to other handover related threshold. And we don’t see any reason to define the accuracy requirements separately. So, our preference is to define one prediction accuracy requirements to cover 3 cell level sub-use cases.

Proposal 2: Support proposal 1. Predicted measurements should at least meet the accuracy requirements that are defined for the non-AI/ML based RRM measurement reporting for mobility purposes.

Proposal 3: RAN4 to define one prediction accuracy requirements to cover 3 cell level sub-use cases.

### Issue 2-3: Ground truth definition for RSRP Accuracy for FR1

Following agreement was reached in RAN4#114bis meeting. And in the last RAN4#115 meeting, RAN4 discussed about this issue but still remained open issue. The agreement in RAN4#114bis and two alternatives are as below:

- Alt 1: Ground truth for FR1 will be based on the transmitted or reception power - Alt 2: Ground truth for FR1 will be based on the reported measurement value under certain conditions RAN4#114bis meeting Agreement: During the SI, RAN4 to provide analysis on potential issue and possible solution to the two Alternatives. If RAN4 cannot make decision during the SI phase, RAN4 will discuss in the WI phase. - At least capture the definition of the two Alternatives in the TR.

During last RAN4#114bis meeting, many companies showed their views regarding two alternaitves. Among them, we have similar view with Nokia’s proposal as below:

|  | Issues | Impact to Alternative 1 | Impact to Alternative 2 |
| --- | --- | --- | --- |
| 1. | The information at TE side, on the timing UE performs and/ or finishes the measurement and/or prediction | TE may not be aware of the exact time period of the prediction performed at UE. This may lead to inaccuracies of ground truth derivation at TE due to dynamic channel conditions | NO |
| 2. | Impact of L3 filtering | L3 filtering methods, filtering co-efficient are configurable parameters, hence known to TE that can be utilized for deriving the ground truth. However, this may increase the complexity at TE when compared to legacy test setup. | NO |
| 3. | Number of samples of L1 filtering | The L1 filtering method, filtering co-efficient and the number of samples for filtering are not known by the TE since it is UE proprietary implementation, hence it will impact the derivation of ground truth at TE. | NO |
| 4. | Timing window, if it exists, where UE can’t measure the ground truth in Alt2 | NO | Depending upon the UE capabilities, some UEs may not be able to measure and predict at the same time. However, this can be handled during testing as described above under Alternative 2 description. |
| 5. | Channel condition | With fading channel, the dynamic channel variations in addition to L1/L3 filtering will have to be considered at TE for deriving the ground truth which may not be feasible due to proprietary algorithms used at UE. | NO |
| 6. | Avoid UE cheat in the test | NO | UE cheating can be an issue in this alternative |
| 7. | S(I)NR variations in the test at different time instances (for Alt2) | NO | One option is to measure at higher SINR level for more accurate ground truth and the SINR may be lower during the prediction with reference to the average SINR. |

Based on the analysis, our preference is alternative 2. Because the issues which impact to alternative 2 are can be overcome. But the issues which impact to alternative 1 are not easy to find the solution to overcome the impact.

Proposal 4: Alternative 2 is more reasonable and feasible method for ground truth definition for RSRP accuracy for FR1.

### Issue 2-4: Impacts of measurement error on prediction accuracy

During last RAN4#114bis meeting, RAN4 could not come to any conclusion for this issue. We think that the error model incorporates RF error and BB error, so that simply considering it as a Gaussian does not lose its generality. Legacy link-level simulator also do not consider the complicated error model. They just have used Gaussian error model, and it has already been proven that it reflects the actual situation well. We don’t think AI/ML case are much different.

Proposal 5: RAN4 to consider only Gaussian error model for RF and BB error model.

- Conclusion

In this contribution, we provide our views on issues for AI/ML mobility based on last RAN4 meeting’s moderator summary [2] and WF [1], and we propose

Proposal 1: RAN4 need to restrict the time difference between the two reported RSRPs and the additional accuracy margin caused by the measurement time difference also necessary.

Proposal 2: Support proposal 1. Predicted measurements should at least meet the accuracy requirements that are defined for the non-AI/ML based RRM measurement reporting for mobility purposes.

Proposal 3: RAN4 to define one prediction accuracy requirements to cover 3 cell level sub-use cases.

Proposal 4: Alternative 2 is more reasonable and feasible method for ground truth definition for RSRP accuracy for FR1.

Proposal 5: RAN4 to consider only Gaussian error model for RF and BB error model.

Reference

[1] R4-2508268, “WF on AI/ML Mobility Part1,” Moderater (Nokia)

[2] R4-2505564, “Topic summary for [115][215] FS_NR_AIML_Mob_Part1,” Moderator (Nokia)