3GPP TSG-RAN WG4 Meeting #116							         	       	R4-2509759

# Bengaluru, India, 25th – 29th August, 2025

Title: 	Proposal on 1UL band 2CC and 2UL band 3CC HPUE MSD

Source: 	Skyworks Solutions, Inc.

Agenda Item:	7.1.2.1.1	General aspects (incl. duty cycle solutions, MSD rules)	[NR_ENDC_RF_Ph4-Core]

Document for:	Approval

# 1	Background

In RAN#115, a major step was achieved in the simplification for 1UL and 2 UL HPUE MSD in a draft CR for 1UL [1] and way forward [2] for 2UL. However, this MSD simplification is only valid for 1CC per band and thus there is no agreed rule for HPUE one UL band 2CC or two UL band 3CC cases as was discussed in [3]. In this contribution, we propose to discuss the potential for rules and guidance for these two less common cases.

# Discussion

## 2.1 Proposal for one UL band 2CC case

The MSD for the one UL band 2CC cases (UL CA_nXX class B/C or 2A) is related to:

- The odd order IMDs of the 1RB+1RB allocation in each CC for victim bands in the proximity of the UL band with:

  - For TDD bands: IMD3 to IMD9 for class B or C and IMD3 to IMD7 for class 2A considering MPR

  - For FDD bands, higher order odd IMDs may be considered, especially when the two band are in the same band group.

- The IMD4 and IMD6 for victim bands in proximity of 2nd harmonic ranges of the UL band

- The IMD5 for victim bands in proximity of 3rd harmonic ranges of the UL band

- IMD4 of class 2A falling into low bands DL for band with >300MHz BW (NR bands n77/n78/n79)

Since the IMD3 and IMD5 are depend on applying MPR to meet SEM mask for TDD bands, their level is independent of the power class. This is also the case for any higher order than 3 IMDs since they decay in a similar or faster way compared to PC3.

This means that for TDD bands it is safe to consider that HPUE MSDs due to IMDs of one UL band with 2CC have the same value as PC3.

Proposal on HPUE one UL band 2CC MSD:

- For TDD UL bands, the HPUE MSD is equal to PC3: PC2 and PC1.5 ΔMSD(dB)=0

- For FDD UL bands, the HPUE MSD is studied case-by-case.

- A CR[4] is agreed to correct the legacy related HPUE MSDs accordingly

- Companies are encouraged to use this guideline for their HPUE TPs/CRs.

## 2.2 Proposal for two UL band 2CC case

The MSD for the two UL band 3CC cases (UL CA_nXXA-nYYB/C) is related to the triple beat (IMD3) between the 1CC band and the 2CC band allocations.

For PC2 inter-band HPUE the configuration goes from 20dBm+20dBm for PC3 to 23dBm+23dBm. Thus one could assume that the interference follows the IMD3 9dB ΔMSDmax from the 2UL MSD simplification LUT. However, to meet emissions for IMD3 and IMD5, MPR is allowed, thus reducing the power for the band with 2 UL CCs. The same happens for the PC1.5 26+26dBm case.

It is unlikely that HPUE intra-band contiguous UL CA, which is the only type considered in triple beat cases, will meet SEM without some backoff applied (but not necessarily the full amount allowed by the specification), the amount of backoff increase between PC3 and PC2 or PC3 and PC1.5 is likely to follow the what is defined for MPR.

For the 1RB+1RB outer2 allocations used for the triple beat test points, compared to PC3:

- PC2 1Tx sees a 0.5dB higher MPR and PC2 2Tx sees a 1.5dB higher MPR

- PC1.5 2Tx sees a 3.5dB higher MPR.

To target the same simplification for the 2Tx and 3Tx cases as for the two UL band inter-band 2CC case, we propose to use 1dB MPR for PC2 cases and 3dB for PC1.5 case, meaning that:

- For PC2:

  - 2CC band power increases by 2dB versus PC3

  - 1CC band power increases by 3dB versus PC3

  - Thus, triple beat interference power increases by 2*3+1*2 =8dB => PC2 triple beat ΔMSDmax is 8dB

- For PC1.5:

  - 2CC band power increases by 3dB versus PC3

  - 1CC band power increases by 6dB versus PC3

  - Thus, triple beat interference power increases by 2*6+1*3 =15dB => PC2 triple beat ΔMSDmax is 15dB

Proposal on HPUE two UL band 3CC MSD:

- PC2 2UL band 3CC triple beat (IMD3) ΔMSDmax is 8dB

  - Alternatively, for further simplification, the PC2 2UL 2CC IMD3 9dB ΔMSDmax LUT is used

- PC1.5 2UL band 3CC triple beat (IMD3) ΔMSDmax is 15dB and can reuse the PC2 2UL 2CC IMD5 15dB ΔMSDmax LUT

- A CR[4] is agreed to correct the legacy related HPUE MSDs accordingly

- Companies are encouraged to use these guidelines for their HPUE TPs/CRs.

# Conclusion

In this contribution, we analysed the HPUE delta MSDs for the one UL band 2CC IMDs and two UL band 3CC triple beat PC2 and PC1.5 cases and make the following proposals.

Proposal on HPUE one UL band 2CC MSD:

- For TDD UL bands, the HPUE MSD is equal to PC3: PC2 and PC1.5 ΔMSD(dB)=0

- For FDD UL bands, the HPUE MSD is studied case-by-case.

- A CR[4] is agreed to correct the legacy related HPUE MSDs accordingly

- Companies are encouraged to use this for their HPUE TPs/CRs.

Proposal on HPUE two UL band 3CC MSD:

- PC2 2UL band 3CC triple beat (IMD3) ΔMSDmax is 8dB

  - Alternatively, for further simplification, the PC2 2UL 2CC IMD3 9dB ΔMSDmax LUT is used

- PC1.5 2UL band 3CC triple beat (IMD3) ΔMSDmax is 15dB and can reuse the PC2 2UL 2CC IMD5 15dB ΔMSDmax LUT

- A CR[4] is agreed to correct the legacy related HPUE MSDs accordingly

- Companies are encouraged to use these guidelines for their HPUE TPs/CRs.

# References

[1] R4-2507916 DraftCR to TS38101-1-j10: Introduction of the 1UL band 1CC HPUE MSD requirements simplification, Skyworks Solutions Inc., Nokia, Samsung, RAN4#115

[2] R4-2507935 WF on new MSD rules, Skyworks Solutions Inc., RAN4#115

[3] R4-2507118 On HPUE MSD simplification scope and needed corrections for 1UL 2CC and 2UL 3CC, Skyworks Solutions Inc., RAN4#115

[4] R4-2511523 Draft CR to R19 38101-1 to correct HPUE MSDs due to one UL band 2CC IMDs, Skyworks Solutions Inc., RAN4#116