3GPP TSG-RAN WG4 Meeting # 116	R4-2509760

Bengaluru, India, August 25th – 29th, 2025

Source: 	Xiaomi

Title: 	Discussion on general aspects of AI/ML testability and interoperability

Agenda Item:	7.17.2

Document for:	Discussion

# 1 Introduction

In this contribution, we will discuss about general aspects of AI testability and interoperability.

# 2 Discussion

In last meeting, there are WF about deactivation and activation:

- LCM requirements - Agreement: - Activation delay: Activation delay requirements created for the periodic, semi-persistent and aperiodic CSI reporting for beam prediction; It is FFS for CSI prediction; For Option A, RAN4 will send LS to RAN2 to clarify the related mechanism; For semi-persistent CSI reporting: Activation delay starts at the reception of the MAC-CE/DCI; For aperiodic CSI reporting: Activation delay starts at the reception of the DCI - Switching delay and Fallback delay: Deprioritize the discussion on switching and fallback delay requirements and focus on activation/deactivation delay requirement, including both AI/nonAI.

AI beam reporting functionality activation procedure is agreed in RAN1#119:

Agreement - In Step 3, following configurations are provided from NW to UE: - UE is allowed to do UAI reporting via OtherConfig, - The applicability report is based on A) and/or B) - It is up to RAN 2 to design the container - A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied) - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3 - B) One set or multiple sets of inference related parameters for applicability report only (not for inference) - It is up to RAN2 to design the container. - The set of inference related parameters selected from the IEs in/or the IEs referred by CSI-ReportConfig as a starting point, e.g., - the associated ID - Note: this doesn’t imply the associated ID is mandatory - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction - In Step 4, UE reports applicability for all the above A) one or more CSI-ReportConfig and/or B) set(s) of inference related parameters - FFS on whether/what other information along with the applicability is needed - If A) is configured in Step 3, - Applicable aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after the applicability reported. - Applicable periodic CSI Report is considered as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete. - In Step 5, NW can optionally configure CSI-ReportConfig for inference configuration in RRCReconfiguration, where the associated ID may be configured in CSI framework as working assumption applied. - Note: Step 5 may be optional if UE has already been configured with CSI-ReportConfig in Step 3 Agreement For beam management, multiple CSI reports for inference for UE-side model can be configured/activated/triggered, which is up to UE capability. Conclusion For the CSI-ReportConfig for inference configuration provided in Step 5, - aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after RRCReconfigurationComplete. - periodic CSI Report is considered as activated after RRCReconfigurationComplete. - Note: UE is not expected to be configured with a CSI-ReportConfig for inference configuration for a non-applicable set of inference parameters or a non-applicable CSI-ReportConfig - Any specification impact is a separate discussion

RAN2 # 129 agree related signalling:

Agreements - Inference configuration/parameters can be signalled in step 3 and/or Inference configuration can be signalled in step 5 (i.e. option a and option b from RAN1). - The full inference configuration is sent in CSI-ReportConfig. - Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability. - FFS signaling details for option B (e.g. whether it is signaling in CSI-Report Config or otherconfig)

- Agreements applicability reporting and management Support the explicit reporting of applicability/inapplicability in initial report and subsequent reporting it reports only applicability it changed. FFS if we report explicit cause If option A is configured in Step 3, for periodic CSI reporting, the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 (i.e. without need to wait RRCReconfiguration in Step 5). The provided periodic CSI configuration should be consistent with reported UE capabilities FFS option B Semi-persistent and aperiodic CSI reporting of applicable functionality is activated following legacy CSI framework: - Semi-persistent reporting, activated by MAC CE/DCI - Aperiodic CSI reporting, activated by DCI

When Option A (i.e., CSI-ReportConfig for inference configuration) is provided in Step 3, and the UE reports applicability for the configured CSI-ReportConfig(s) via RRCReconfigurationComplete in Step 4, this applicability report does not imply that the full activation procedure on the UE side is already completed. Rather, it indicates that the UE considers the configuration applicable and is ready to activate the corresponding reporting functionalities without requiring any additional explicit activation signaling from the network (e.g., via a subsequent RRC message). In other words, for periodic CSI reporting, the UE autonomously proceeds with activation upon applicability reporting, while for semi-persistent and aperiodic CSI reporting, activation still follows the legacy mechanism via MAC-CE or DCI, respectively.

Therefore, we think that UE doesn’t need to finish the activation before sending RRCReconfigurationComplete.

Proposal 1: UE doesn’t need to finish the activation before sending RRCReconfigurationComplete.

Deactivation Delay Requirements

Switching delay and Fallback delay: Deprioritize the discussion on switching and fallback delay requirements and focus on activation/deactivation delay requirement, including both AI/nonAI, discussion

The current legacy CSI framework provides well-established deactivation mechanisms:

- For periodic CSI reporting, deactivation occurs through RRC reconfiguration

- For semi-persistent CSI reporting, deactivation is achieved via MAC CE or DCI signaling

- Aperiodic CSI reporting automatically terminates after single-shot transmission

Observation 1: In legacy, no de-activation delay is defined for CSI reporting in RAN4.

If switching delay and fallback delay are deprioritized or not considered, then the standalone definition of deactivation delay becomes less meaningful. This is because the primary significance of deactivation lies in its coordination with other procedures: for example, deactivation followed by activation of another CSI reporting configuration essentially constitutes a switching operation; deactivation followed by enabling non-AI measurements constitutes a fallback. In both cases, the deactivation delay directly contributes to the overall transition time. Therefore, if switching and fallback behaviors are out of scope, defining deactivation delay in isolation lacks practical utility. In legacy, no de-activation delay is defined for CSI reporting in RAN4.

Observation 2: Without considering switching or fallback procedures, defining deactivation delay in isolation may lack practical relevance.

Proposal 2: For AI BM, legacy CSI-ReportConfig framework will be used, don’t need to define deactivation delay.

Monitoring reporting delay and accuracy: Discuss how to define delay and accuracy requirements for UE sided monitoring where the UE reports the metric: BM-Case1 and possibly BM-Case 2; Performance monitoring type 1 option 2; If it proves to be not possible to define, then reverse the agreement; Note: For BM-Case2, it may not be completed in Rel-19. If it is not covered in Rel-19, there would not be a monitoring delay requirment. If it would be in Rel-19, a monitoring delay requirement is needed

Monitoring reporting delay

RAN1 agreement about monitoring report is as below:

Agreement(RAN 1 #120b) For UE-sided model monitoring Type 1 option 2, support the following combination for inference report type and monitoring report type: Monitoring report type Inference report type; P report; SP report; AP report. AP report; Not support; Not support; Support. SP report; Not support; Support; Support. P report; Support; Support; Support.

It shows the supported combination of reporting type for reference reporting and performance monitoring. And performance monitoring will make use of the result of reference. Then the whole structure of performance monitoring reporting is similar as reference reporting. And reference reporting is based on CSI reporting. In legacy CSI reporting, we defined measured period for periodic report and SP report and didn’t define reporting delay. reporting periodicity is configured by NW. For inference reporting, we don’t define reporting delay and we only consider measurement period. performance monitoring reporting is similar as inference reporting. Therefore, we suggest not to define reporting delay for performance monitoring delay.

Proposal 3: RAN4 not to define performance monitoring delay for BM-Case1 and BM-Case2.

Monitoring reporting accuracy

Agreement(RAN 1 #120bis) For UE-sided AI/ML model for beam management, for Option 2 (UE-assisted performance monitoring), the performance metric of Top 1 or Top K beam prediction accuracy is defined as: - At least one of the Top M beam(s) of the resource set(s) for monitoring is among Top-K predicted beam(s) of Set A (e.g., linked to at least one of the Top-K predicted beam(s) of Set A based on certain rule or signalling) - Where K is the number of predicted beam(s) in the corresponding inference report per time instance - Where Top M beam(s) is the best M beam(s) based on L1-RSRP measurements of the resource set(s) for monitoring - M is configured by NW in CSI report configuration for monitoring - M= 1, 2 - FFS: detailed rule or signaling Agreement(RAN 1 121) - At least for the monitoring Type 1 Option 2 of UE-side model monitoring, for calculation of metric for monitoring, - for BM-Case 1, the measurement result of nth (n = 1,..,N) latest transmission occasion of the CSI-RS/SSB resources for monitoring is linked with an inference report, where the CSI reference resource of the corresponding inference report has the minimal slot offset to the nth transmission occasion of the CSI-RS/SSB resources for monitoring. - Wherein, the corresponding inference report, and the transmission occasion of the CSI-RS/SSB resources for monitoring are no later than the CSI reference resource corresponding to the CSI report for monitoring - Predefines a threshold X = 64 for the minimal slot offset, which is configured by RRC, where the minimal slot offset k is no larger than X; otherwise, the transmission occasion for monitoring has no linked inference report. - Note: CSI reference resource corresponding to the CSI report for monitoring and inference is determined based on legacy for all types (P/SP/AP) of CSI report carrying L1-RSRP, considering discussing associated timeline separately - The associated working assumption made in RAN1#120b will not be confirmed. Agreement (RAN 1 #121) For beam prediction accuracy report for monitoring, the report quantity RS-PAI is Np (0 ≤ Np≤ N) - Where Np is the total count of accurate reference signal prediction instance(s) that meets the condition, among N latest transmission occasion(s) of monitoring resources that no later than CSI reference resource corresponding to the CSI report for monitoring - condition: - for the transmission occasion of monitoring resources, it has a linked inference report - at least one of the nrofBestBeamforMonitoring-r19 identified CSI-RS resources, or SS/PBCH Block resources mapped to one of the nrofreportedpredictedrs-r19 reported P-CRI(s) or P-SSBRI(s), of the linked report of the CSI Reporting Setting for inference - if this condition is met, the transmission occasion is counted as an accurate reference signal prediction instance; otherwise, it is not counted as an accurate reference signal prediction instance. - Where N = 1, 3, 7, 15 is configured in CSI-ReportConfig - the size of CSI field associated with the RS-PAI is log2(N+1) Agreement For BM-Case 2, at least support to report one beam prediction accuracy for one configured time instance, configured by one CSI-ReportConfig for monitoring, - only one resource set is configured in the CSI-ReportConfig - the one configured time instance (i.e. f-th time instance of the time instance in one inference report) for metric calculation is configured in the CSI-ReportConfig for monitoring - the performance metric of the f-th time instance is calculated based on N latest transmission occasion(s) of monitoring resource, no later than CSI reference resource corresponding to the CSI report for monitoring - The measurement result of nth (n = 1,..,N) latest transmission occasion of the CSI-RS/SSB resources for monitoring is linked with the f-th time instance for prediction, where the f-th time instance has the minimal slot offset to the nth transmission occasion of the CSI-RS/SSB resources for monitoring. - Wherein, the corresponding inference reports, and the transmission occasions of the CSI-RS/SSB resources for monitoring, are no later than the CSI reference resource corresponding to the CSI report for monitoring - Predefines a threshold X =64, where the minimal slot offset k is no larger than X; otherwise, the transmission occasion for monitoring has no linked time instance. - Note: CSI reference resource corresponding to the CSI report for monitoring is determined based on legacy for all types (P/SP/AP) of CSI report carrying L1-RSRP The associated working assumption made in RAN1#120b will not be confirmed.

Beam Prediction Accuracy Definition for performance monitoring

1.Top-K Accuracy (Option 2 – UE-side Monitoring)

- Top-M beams: Best M beams based on L1-RSRP from monitored resources (M=1 or 2, configured by NW)

- Top-K predictions: K beams predicted by UE model in each inference report

- Accuracy condition: If at least one of the Top-M beams matches any of the Top-K predicted beams, it's an accurate prediction

2. RS-PAI Metric (BM-Case 1)

- Reports N_p, the count of accurate predictions among N latest monitoring occasions (N=1,3,7,15)

- Accurate if:

  - A linked inference report exists (slot offset ≤ 64)

  - One of the Top-M measured beams matches a predicted beam in that report

3.Linking Rule

- Each monitoring resource links to the nearest inference report in time (slot offset ≤ 64)

- Both must be no later than the CSI reference resource of the report

4.BM-Case 2 (Single-Time Reporting)

- Reports accuracy for one configured time instance (f-th)

- Same linking and matching logic as BM-Case 1

- Only one resource set is allowed

Proposal 4: Beam prediction accuracy is defined as the ratio of the number of correctly predicted beam instances to the total number of evaluated monitoring occasions over a configured time window, i.e. Np/N.

Where,

- N is the number of latest CSI-RS or SSB transmission occasions used for performance evaluation, as configured by the network.

- Np ​is the number of those occasions for which the beam prediction is considered accurate according to the matching rule (e.g., at least one of the Top-K predicted beams matches one of the Top-M strongest beams measured based on L1-RSRP).The values of K, M, and N are network-configurable parameters.

Issue 1-4-4: Relation to legacy RAN4#112bis WF: Agreement: - For CSI prediction, AI/ML based performance requirement should not be worse than the legacy (non-AI/ML based) if a relevant equivalent legacy requirement/feature exists. - FFS which legacy requirement/feature are relevant - FFS whether AI/ML based performance should be better than the legacy - FFS on target for AI/ML based beam management - Proposals - Option 1: Keep same side conditions for legacy and AI/ML (Apple proposal 18, CATT proposal 1 option 1, CMCC proposal 1) - Option 2: Measurement error may have an impact on whether AI can exceed “legacy” requirements (Xiaomi proposal 1) - Option 3: Allow larger margin (presumably more relaxed requirement) for higher overhead reduction (CATT proposal 1 option 2) - For CSI compression, better than legacy with the same overhead, and no worse than legacy with reduced overhead (Huawei proposal 1) - For positioning, re-use legacy requirement for case 2a/3a if AI and non-AI cannot be distinguished (Huawei proposal 3) - for CSI prediction, it is proposed that AI/ML based performance requirement should not be worse than the legacy PMI requirements for 'typeII-doppler-r18 (CMCC proposal 2) - Option 4: Legacy performance is the baseline; allow margin for scenarios in which legacy is not defined compared to the baseline (ZTE proposals 3, 4). - Option 5: When a test is defined for AI for which there is no legacy test, need to decide whether to (i) Investigate non-AI performance in that scenario and set AI requirement greater or (ii) set an absolute requirement on AI (without considering non-AI) (Huawei proposal 5) - Recommended WF Discuss the following: - Same side conditions as legacy - What does this mean in practice ? When no existing requirement exists, or some other types of condition should be tested - Does measurement error impact comparison to non-AI - If no non-AI requirement exists for some functionality/condition, is it necessary to consider non-AI performance for comparison to AI, or sufficient just to set an absolute requirement for AI ? - When there is a non-AI requirement, if the AI functionality operates with lower overhead, should the same requirement be met or a more relaxed requirement ?

For AI based beam management, currently, there are several candidate test metrics:

Issue 2-1: Metrics/KPIs for beam ID prediction Agreement: - Metrics/KPIs for beam ID prediction, at least for the case if only beam ID is reported: The successful rate for the correct prediction, - The correct prediction is considered as maximum ground-truth RSRP among top-K predicted beams larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – x dB, - FFS on top-N (N</=K) predicted beams have to be considered in the success rate evaluation - FFS on N values - FFS on x values Note: if x=0 and N=1, it means Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams"

From our understanding, whether the AI requirements exceed “legacy” requirements depends on whether the same metric and side condition is defined for AI and non-AI legacy requirement.

For example, if test metric is beam ID prediction accuracy, in legacy requirement, there is no such metric. We prefer just to set a specific requirement for AI.

Proposal 5: For beam ID prediction accuracy, there is no performance requirement in legacy. Set a specific requirement for AI.

If test metric is absolute RSRP prediction accuracy for the same beam. In legacy non-AI, there is also similar metric. For this metric, predicted RSRP or legacy measured RSRP will be compared with ground truth. RSRP delta is the accuracy requirement.

However, currently, the impact of measurement error is studied. The AI input is measured L1-RSRP with error, it will degrade the RSRP prediction performance. It depends on how the AI model is trained. there are different implementations:

- AI input is measured RSRP without error, label is ground RSRP without error

- AI input is measured RSRP with error, label is measured RSRP with error

- AI input is measured RSRP with error, label is measured RSRP without error

Whether the accuracy of prediction RSRP can still outperform the accuracy of legacy measured RSRP is an issue which needs to be studied.

Proposal 6: For RSRP prediction accuracy, measurement error may have impact on whether AI requirements can exceed “legacy” requirements.

# 3 Conclusion

In this contribution, we provide the following proposals:

Proposal 1: UE doesn’t need to finish the activation before sending RRCReconfigurationComplete.

Observation 1: In legacy, no de-activation delay is defined for CSI reporting in RAN4.

Observation 2: Without considering switching or fallback procedures, defining deactivation delay in isolation may lack practical relevance.

Proposal 2: For AI BM, legacy CSI-ReportConfig framework will be used, don’t need to define deactivation delay.

Proposal 3: RAN4 not to define performance monitoring delay for BM-Case1 and BM-Case2.

Proposal 4: Beam prediction accuracy is defined as the ratio of the number of correctly predicted beam instances to the total number of evaluated monitoring occasions over a configured time window, i.e. Np/N.

Where,

- N is the number of latest CSI-RS or SSB transmission occasions used for performance evaluation, as configured by the network.

- Np ​is the number of those occasions for which the beam prediction is considered accurate according to the matching rule (e.g., at least one of the Top-K predicted beams matches one of the Top-M strongest beams measured based on L1-RSRP).The values of K, M, and N are network-configurable parameters.

Proposal 5: For beam ID prediction accuracy, there is no performance requirement in legacy. Set a specific requirement for AI.

Proposal 6: For RSRP prediction accuracy, measurement error may have impact on whether AI requirements can exceed “legacy” requirements.

# 4 Reference

- TR38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interfa