3GPP TSG-RAN WG4 Meeting # 116	R4-2509764

Bengaluru, India, August 25th – 29th, 2025

Source: 	Xiaomi

Title: 	Discussion on impacts on RAN4 requirement for measurement event prediction in AI mobility

Agenda Item:	7.19.3.2

Document for:	Discussion

# 1 Introduction

In this contribution, we will discuss RRM impact on AI mobility for measurement event prediction.

# 2 Discussion

2.1 Core requirement

As agreed in TR, there are two methods to predict measurement event, namely indirect and direct measurement event prediction.

In legacy, RAN4 defined event triggered L3 reporting delay in core part and define corresponding test case to verify whether UE will correctly detect the event by L3 reporting.

Observation 1: In legacy, RAN4 defined event triggered L3 reporting delay in core part.

Similarly, for measurement event prediction, it may have impact on predicated event triggered L3 reporting delay. RAN4 also needs to verify UE behaviour whether event can be correctly predicted. It’s FFS how to define requirement for predicated event triggered L3 reporting, RAN4 needs to wait for more RAN2 progress.

Proposal 1: Measurement event prediction may have impact on predicated event triggered L3 reporting delay.

2.2 Performance requirement

- 2.2.1 RSRP predication accuracy requirement

- Issue 3-1: Performance metric for event prediction use case - Agreement: In case of indirect measurement event prediction use case, if the report includes the predicted RSRPs [corresponding to the predicted event occurrence time], RAN4 will define requirement for the absolute and/or relative accuracy of predicted RSRP

1. Unclear reporting format: The specific formats of RRM prediction reports and event prediction reports have not yet been finalized in RAN2. They may include values at a single time point, or multiple future time points, making it impossible to define targeted evaluation requirements.

2. Strong association between evaluation requirements and reporting format: Existing accuracy evaluation rules (such as single-value threshold judgment, independent verification of multiple values, and relative accuracy between multiple reports) need to be designed based on a clear reporting format. An undefined format will render the rules inoperable.

3. Lack of definition for multi-value scenarios: For reports containing multiple values, how to set overall validity requirements (e.g., whether all values need to meet the threshold or only some) and the calculation standards for relative accuracy between multiple reports both need to be further clarified based on the design results of RAN2.

Proposal 2: For event-triggered reporting, we propose waiting for RAN2 progress to define requirements based on finalized formats, clarifying rules for multi-value scenarios and inter-report relative accuracy.

TTT length

From RAN2 simulation, the time interval from measurement to prediction increase, the prediction accuracy decreases. For example, the predicted RSRP accuracy of the first point in PW is quite different from that of the last point in PW due to the fact that T2 is larger than T1, as shown below:

Fig.1 different L3-RSRP predication accuracy in TTT

Observation 2: The time interval from measurement to prediction increase, the prediction accuracy decreases. The prediction accuracy of individual measurement results in a time window could be quite different.

Proposal 3: When define event prediction accuracy requirement, the TTT length will be larger than 1.

2.2.2 Event accuracy

For AI based indirect event prediction, performance metric F1 score is defined in RAN2

F1 score = 2*Precision*Recall/(Precision + Recall) Where: Precision = n3/(n1+n3) Recall =n3/(n2+n3) For indirect prediction, the counter n1, n2 and n3 in the formula are defined as following: - Counter n3(true event prediction): it increases by 1 when a ground-truth event occurs around a predicted event with ETD, whose range is [0, maximum ETD] or vice versa - Counter n1(false event detection): it increases by 1 when no ground-truth event occurs around a predicted event with ETD, whose range is [0, maximum ETD] - Counter n2(missed event detection): it increases by 1 when no event is predicted around a ground-truth event with ETD, whose range is [0, maximum ETD] The ETD i.e. timing difference between ground-truth event and predicted event is illustrated in Figure 5.3.1-1: Figure 5.3.1-1: illustration of ETD As illustrated in Figure 5.3.1-1, only if the ETD between a predicted event and a ground-truth event e.g. ground-truth event 2 is less than or equal to maximum ETD, the ETD can still be tolerated. Otherwise, both false event and missed event are detected.

From RAN2, performance metric F1 score is defined, which considered both precision and recall.

Besides, ETD is also defined in RAN2, which is a timing difference between predicated event and ground truth event.

- max ETD also be interpreted as the event prediction time margin in the test.

- max ETD can be interpreted as timing accuracy requirement for event prediction.

Event prediction time accuracy = predicated event time – ground truth event time

If timing difference is smaller than max ETD, UE has correctly predicted the event.

RAN4 needs to discuss how to consider the impact of ETD.

Observation 3: In RAN2, performance metric F1 score is defined for event prediction accuracy, which considered both precision and recall.

Observation 4: In RAN2, a timing difference between predicated event and ground truth event is defined.

While in legacy RAN4, in test for verify whether event is correctly detected, only precision = 90% is used. Precision considers the correct reporting ratio for all triggered events, which is shown below:

##### A.4.6.1.1.3 Test Requirements The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 800 ms from the beginning of time period T2. The UE is not required to read the neighbour cell SSB index in this test. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

Observation 5: In legacy RAN4, in test to verify event prediction accuracy, only precision = 90% is used and no timing difference between predicated event and ground truth event is considered.

Proposal 4: RAN4 to only consider precision and not consider recall in event prediction accuracy KPI.

- Conclusion

In this contribution, we provide the following proposals:

Observation 1: In legacy, RAN4 defined event triggered L3 reporting delay in core part.

Proposal 1: Measurement event prediction may have impact on predicated event triggered L3 reporting.

Observation 2: The time interval from measurement to prediction increase, the prediction accuracy decreases. The prediction accuracy of individual measurement results in a time window could be quite different.

Proposal 2: For event-triggered reporting, we propose waiting for RAN2 progress to define requirements based on finalized formats, clarifying rules for multi-value scenarios and inter-report relative accuracy.

Proposal 3: For event triggered reporting, RAN4 to define relative accuracy requirement. If there are multiple predicted RSRPs in one report, evaluate relative RSRP accuracies continuously over multiple time instances.

Observation 3: In RAN2, performance metric F1 score is defined for event prediction accuracy, which considered both precision and recall.

Observation 4: In RAN2, a timing difference between predicated event and ground truth event is defined.

Observation 5: In legacy RAN4, in test for verify event prediction accuracy, only precision = 90% is used and no timing difference between predicated event and ground truth event is considered.

Proposal 4: RAN4 to only consider precision and not consider recall in event prediction accuracy KPI.