3GPP TSG-RAN WG4 Meeting # 116	 R4-2509766

Bengaluru, India, August 25th – 29th, 2025

Source: 	Xiaomi

Title: 	Discussion on CSI-RS based L1 measurement for mobility

Agenda Item:	7.25.3.2

Document for:	Discussion

# 1 Introduction

In this contribution, we will mainly discuss the following issues for Rel-19 Mobility.

- Definition of intra-frequency and inter-frequency - Pre-requisite of CSI-RS based L1 measurement

- Discussion

Issue 3-2: whether step 2 is skipped for serving cell? Candidate options: - Option A: RAN4 requirements regarding ‘skipping step 2’ do not apply for the case CSI-RS based L1 measurement for LTM candidate cell is configured in legacy CSI-RS based L1 measurement in serving cell in FR2. (QC, HW) - Option A1: Requirement regarding ‘skipping step 2’ is not applicable if an FR2 candidate cell, from which CSI-RS is configured with repetition set to ‘off’, is one of the FR2 serving cells. (QC, OPPO) - Option A2: The CSI-RS based L1-RSRP measurement requirement is not applicable if an FR2 candidate cell, from which CSI-RS is configured with repetition set to 'off', is one of the FR2 serving cells (QC). - Option B: keep the current measurement procedure unchanged for serving cell when it is in the candidate cell for LTM. (MTK, Apple, E///)

In FR2-1, SSB-based L1 measurements play a foundational role in supporting both mobility management and physical-layer beam management procedures, including initial beam acquisition, beam tracking, beam recovery, and beam failure detection. While skipping Step 2 (SSB-based measurement) for neighbor cells may be considered under certain UE capabilities and CSI-RS configurations (e.g., when CSI-RS suffices for LTM purposes), the same logic does not hold for the serving cell.

Unlike neighbor cells, the serving cell acts as the primary anchor for beam management operations. Relying solely on CSI-RS—particularly when configured with repetition set to ‘off’—may lead to reduced measurement robustness, as CSI-RS does not always provide full spatial coverage or sufficient redundancy for reliable beam operation. Skipping SSB measurements under such configurations risks degrading beam management reliability and recovery mechanisms.

Therefore, to ensure consistent behavior, robust performance, and alignment with PHY-layer requirements, it is critical to retain Step 2 SSB-based L1 measurements for the serving cell in FR2-1, irrespective of CSI-RS repetition or other CSI-related parameters.

Proposal 1: For the serving cell in FR2-1, Step 2 (SSB-based L1 measurement) shall not be skipped, regardless of the CSI-RS configuration, including the repetition setting.

Issue 3-3: Sharing factor for resource collision between serving and neighbour cell Candidate options: - For FR2 CSI-RS L1 measurement requirement, - if the CSI-RS resources configured for L1 RSRP measurement in serving cell and neighbour cell are colliding in time domain,. - Option 1: sharing factor shall be introduced. (HW, QC, E///) - Option 2: introduce measurement restriction as legacy. (MTK, vivo)

In FR2, CSI-RS resources are highly configurable in the time and frequency domains, offering significant flexibility to avoid collisions across cells. This is in contrast to SSB resources, which have more rigid time-domain positions and therefore require defined sharing mechanisms when overlap occurs.

Given this configurability, collisions between CSI-RS resources configured for L1-RSRP measurement in the serving cell and neighbor cells can be effectively avoided through proper network planning and resource allocation. Therefore, it is unnecessary to introduce additional complexity, such as a sharing factor mechanism, to resolve such cases.

Instead, leveraging measurement restriction mechanisms—as done in legacy systems—offers a simpler and already understood approach. This avoids over-specification and ensures consistency with established practices.

Proposal 2: In case of time-domain collision between serving and neighbor cell CSI-RS resources for L1-RSRP measurement in FR2, apply measurement restriction as in legacy, without introducing a sharing factor.

Issue 3-6: Measurement restriction Candidate options: - Option 1: Apple, ZTE, Ericsson - In FR2, or in FR1 when CSI-RS based L1 measurement would cause scheduling restriction, all CSI-RS resources within a 40ms window on one intra-frequency layer should be configured within up to two separate windows, each lasting up to 5ms. - Option 2: vivo - When CSI-RS based L1 measurements cause scheduling restriction, all CSI-RS resources on one intra-frequency layer are configured within up to two separate windows where each window is up to 5 ms and the two windows, if configured, are separated by 20ms.

CSI-RS resources are highly configurable in time domain, and a simple bounding rule helps avoid measurement overload and scheduling conflict. Option 1 provides a clear and practical constraint by limiting all CSI-RS resources within a 40 ms window and up to two measurement windows (each ≤5 ms). This aligns with typical RAN4 measurement restriction principles and supports consistent behavior across FR2 and FR1.

Proposal 3: Limit CSI-RS resources on one intra-frequency layer within up to two ≤5 ms windows inside a 40 ms span when measurement restriction applies.

Issue 3-9: early CSI acquisition Candidate options: - Option 1: Define timeline for baseline early CSI acquisition. (vivo, Nokia, E///) - Option 2: Define scheduling/measurement restriction requirements for optional early CSI acquisition. (QC) - Option 3: not define RAN4 requirements for early CSI acquisition. (MTK, HW, Apple)

Early CSI acquisition mainly serves data transmission preparation, which is closely tied to RAN2-controlled scheduling and activation behavior. RAN4's primary scope is measurement-related requirements rather than pre-scheduling optimizations. Defining early acquisition in RAN4 risks crossing into RAN2 procedures and constraining implementation flexibility. Since early CSI acquisition is not essential for interoperability and often implementation-specific, it is preferable not to define RAN4 requirements at this stage.

Proposal 4: Do not define RAN4 requirements for early CSI acquisition.

- Conclusion

In this contribution, we provide the following proposals：

Proposal 1: For the serving cell in FR2-1, Step 2 (SSB-based L1 measurement) shall not be skipped, regardless of the CSI-RS configuration, including the repetition setting.

Proposal 2: In case of time-domain collision between serving and neighbor cell CSI-RS resources for L1-RSRP measurement in FR2, apply measurement restriction as in legacy, without introducing a sharing factor.

Proposal 3: Limit CSI-RS resources on one intra-frequency layer within up to two ≤5 ms windows inside a 40 ms span when measurement restriction applies.

Proposal 4: Do not define RAN4 requirements for early CSI acquisition.

- Reference